3GPP TSG-RAN WG4 Meeting # 112	R4-2412204

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.18.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impact for Rel-19 MIMO

Document for:	Discussion

# Introduction

According to the latest WID [1] for Rel-19 MIMO, RAN4 shall start work for the objectives listed in the WID from this meeting. In this contribution, we provide our views regarding the RAN4 impacts for the WI.

# Discussion

Based on the latest WID [1], the objectives are shown as follows:

The detailed objectives are as follows: RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. Acronyms: - sTRP: single TRP (transmit-receive point) - mTRP: multiple TRP (transmit-receive point) - CJT: coherent joint transmission - DU: distributed unit - CRI: CSI-RS resource indicator RAN2: Specify higher layer support of the enhancements listed above. RAN4: Specify necessary core requirements for the enhancements listed above.

It could be observed there 5 objectives in the WID. We present our analysis on potential RRM impacts in case by case manners.

## Objective#1

For Objective#1, it is to introduce UE-initiated/event-driven Beam management to reduce overhead and latency. The relevant RAN1 agreements so far are shown as follows:

Event and RS for measurement

Agreement On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, RAN1 further study at least the following aspects: quality metrics, event-definition and threshold. - Further study trigger events, including the following example as a starting point - Event-1: Quality of the current beam is worse than a certain threshold. - Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam. - Event-3: Quality of a new beam is better than a certain threshold. - Event-4: Quality of the current beam is worse than a threshold 1, and quality of at least one new beam is better than a threshold 2. - Others are not precluded. - Note: Companies are encouraged to provide details on procedure (e.g. how it is used) related to their preferred event Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported: - Mode A (dynamically scheduling UCI by gNB): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report - FFS: Request format, e.g., SR or a new UCI type. - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report. - Step 3: Beam report is transmitted in second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - This mode is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature). - No new DCI format is introduced. - Mode B (UCI in pre-configured resource(s) for second UL channel): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report - FFS: Notification format, e.g., SR or a new UCI type. - Step 2: UE transmits the beam report in the second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - The notification in Step1 is in a separate reporting instance from the beam report in Step 2. FFS: Whether UE receives acknowledge information with response to each step for all modes For above procedures, cross-CC beam reporting is supported for both modes. - FFS: Details. Agreement On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, at least support Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam. - At least L1-RSRP is supported as quality metrics used for Event-2 - FFS: How the L1-RSRP is used to determine the triggering event (e.g. timer, counter, filter coefficient) - FFS: Whether the network controls how the L1-RSRP is used to determine the triggering event - Regarding RS measurement for the new beam for Event-2, down-select one or more of the following: - Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured by RRC (e.g., reusing legacy configuration of RS measurement or in TCI-State) or MAC-CE - Option-3b (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s). - Option-3c (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of configured TCI state(s). - Note-1: ‘New/current beam’ is for discussion purpose. - Note-2: Other trigger events/quality metrics (e.g., L1-SINR) are not precluded. - Note-3: For above implicit manner(s), if there are two QCL RSs in a TCI state, the measurement RS is derived from RS w.r.t. QCL-TypeD, if applicable. Agreement On UE-initiated/event-driven beam reporting, regarding Event-2, the threshold value is RRC configured Agreement On UE-initiated/event-driven beam reporting, regarding Event-2, ‘current beam’ is a beam corresponding to the indicated TCI state. - Regarding RS measurement for the current beam for Event-2, Option-2a is supported: - Option-2a (implicit manner): The RS for current beam is implicitly derived from a QCL RS of indicated TCI state. - FFS: The RS for current beam can be either the QCL RS in the indicated TCI state or the SSB which is QCLed with the QCL RS in the indicated TCI state. - FFS: Option-2c (explicit manner): The RS for current beam is explicitly configured by RRC or MAC-CE. - Note: SSB or CSI-RS can be configured Agreement Regarding RS measurement for the current beam for Event 2, for Option-2a, support the both schemes as follows. - Scheme-1: RS for current beam is the QCL RS in the indicated TCI state - FFS: Whether/How to handle the case if only one TRS is configured in the indicated TCI state. - Scheme-2: the RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state. - Enabling one of either Scheme-1 or Scheme-2 is selected by NW. - FFS: The above selection is via an explicit RRC parameter or an implicit manner, e.g., if the RS(s) for new beam are CSI-RS, Scheme-1 is enabled; otherwise, Scheme-2 is enabled. - (Working Assumption) Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam. - The above QCL RS is the RS w.r.t. QCL-TypeD, if there are two QCL RSs in the indicated TCI state. Agreement Regarding RS measurement for the new beam for Event 2, at least Option-3a is supported - Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured - FFS: Option-3b/3c - Option-3b: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s). - Option-3c: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of TCI state(s) in a configured subset of the legacy RRC-configured TCI state list

Based on the agreement achieved so far in RAN1, it is agreed to introduce at least Event-2, which is the event when quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam, and at least L1-RSRP is supported as metrics.

Observation 1: Following Event-2 is agreement in RAN1 so far with at least L1-RSRP as the quality metrics.

- Event-2: quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam

RAN1 has also discussed and concluded on the definition of how to determine a current beam and a new beam. According to the agreements so far, current beam is determined is implicitly from a QCL RS of indicated TCI state (QCL RS or SSB QCL-ed with QCL RS). For new beam, at least the explicitly configuration is supported.

Observation 2: For the current beam and new beam determination, at least followings are agreed in RAN1 so far:

- Current beam: Implicitly derived from a QCL RS of indicated TCI state (QCL RS or SSB QCL-ed with QCL RS)

- New beam: Explicitly configured

Measurement

Agreement On UE-initiated/event-driven beam reporting, at least support L1-RSRP as a measurement quantity on SSB for intra-cell and inter-cell, and periodic CSI-RS for beam management - Notes: measurement results may be contained in the beam report and/or used as quality metric(s) to initiate/trigger the reporting. - FFS: Semi-persistent CSI-RS and aperiodic CSI-RS. - FFS: Whether/how to support L1-SINR measurement, assuming legacy RS or RS combination (e.g., CMR only, CMR+ZP/NZP-IMR) for Rel-16 SINR is reused. - FFS: Whether/how to specify filtering operation for L1-RSRP. Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification.

Regarding the RAN1 agreement relevant to measurement, it is agreed that at least SSB-based L1-RSRP for intra-cell and inter-cell and periodic CSI-RS for beam management are supported.

Observation 3: At least SSB-based L1-RSRP for intra-cell and inter-cell and periodic CSI-RS for beam management are supported.

Regarding the determination for Event-2, the basic feature is that once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs. An enhanced feature is that beam report occurs only number of Event-2 instance is greater than configured number within a window.

Observation 4: For Event-2, following two triggering determinations are supported:

- Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs

- If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

Report

Agreement On UE-initiated/event-driven beam report, at least of following aspects should be included: - Trigger-event detection for beam reporting by UE - UE monitors RS to assess if a beam-reporting trigger condition has been met - FFS: Trigger condition for declaring beam-reporting event - Beam-report transmission by UE - Signaling contents in the beam report - Down-selection one or more options (strive for one) between the following options as signaling medium/container for beam report transmission - MAC-CE - UCI - Others are not precluded. On UE-initiated/event-driven beam report, the following aspects may be included: - UE requesting UL resource(s) for the beam report - UE notifying transmission of beam report - gNB preconfigured resources Other procedure(s) as required Agreement On UE-initiated/event-driven beam reporting, regarding signaling content(s), at least support DL RS resource indicator and L1-RSRP - FFS: Study and decide whether additional contents can be supported. - FFS: L1-RSRP format, e.g., absolute and/or differential value. - Note: Above does not imply to preclude discussion on L1-RSRP filtering. - The actual reported content depends on the triggering event - Support of one or multiple events will be discussed separately Agreement On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, at least Option-3 is supported - Option-3: N ≥ 1 beam(s) are reported in the report instance, - At least one of N reported beam(s) should satisfy the condition of Event-2 - N is configured by gNB - FFS: candidate value of ‘N’. - FFS: RRC can enable or disable whether current beam is always reported in addition to the N beams - FFS: Option-1/1a/1b/2. - Above applies at least for the single CC case Agreement On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, for a report instance where N ≥ 1 beam(s) are reported, the following is supported. - RRC can enable or disable whether current beam is always reported - When enabled by RRC, the current beam + N beams from the measurement RSs for new beam(s) are reported - Note: The reported current beam is NOT counted in the N reported beams. - When disabled by RRC, N beams are reported.

RAN1 has also discussed on the reporting procedure and content for UE-initiated/event-driven beam reporting. But from our understanding, it is quite limited impacts on RAN4 RRM impacts.

Based on above analysis, the RRM impacts for Objective#1 is obvious. RAN4 shall define corresponding requirements for UE-initiated/event-driven beam reporting.

Proposal 1: RAN4 to define RRM requirements for Objective#1 (UE-initiated/event-driven beam reporting).

In details, based on observation 1 – 4, RAN4 start with Event-2 with L1-RSRP as the quality metrics.

Proposal 2: RAN4 start with Event-2:

- L1-RSRP as the quality metrics

- SSB and periodic CSI-RS as RS for beam management

- Following two triggering determinations

  - Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs

  - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

Note: Can be updated based on further RAN1 progress.

## Objective#2

For Objective#2, which is to support CSI for up to 128 CSI-RS reports. From our understanding, there is no RRM impacts for this objective.

Proposal 3: No RRM impacts for Objective#2.

## Objective#3

For Obective#3, which is to support UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul. Based on RAN1 agreements so far, following uses cases are considered:

- For per-TRP delay offset reporting: - Use case 1.1: TRP selection - Use case 1.2: delay offset compensation for at least one TRP to ensure the CJT-composite delay spread doesn’t exceed a pre-defined dynamic range/threshold - For per-TRP frequency offset (FO) reporting: - Use case 2.1: TRP selection - Use case 2.2: per-TRP FO compensation at NW side - For per-TRP DL/UL Rx-Tx phase misalignment reporting: - Use case 3.1: TRP selection - Use case 3.2: per-TRP DL/UL Rx-Tx phase compensation at NW side for reciprocity (e.g. using both CSI-RS and SRS for measurement) - Use case 3.3: For TDD reciprocity, timing offset report for at least one pair of TRPs to assist TRP synchronization (i.e. to align TRP inherent timing without propagation delay)

From our understanding, it is quite similar as Rel-18 TDCP reporting. In Rel-18 MIMO discussion, RAN4 perform feasibility study on whether to define accuracy requirements for TDCP reporting. RAN4 shall discuss whether to define Core requirements for CJT calibration reporting for delay offset/frequency offset/phase offset.

Proposal 4: RAN4 to discuss whether to define core requirements for CJT calibration reporting for delay offset/frequency offset/phase offset.

## Objective#4

For objective#4, which is to specify non-coherent UL codebook for 3-antenna-port codebook-based transmission. It is also clearly stated in the WID that there is no enhancement on SRS resource. Thus, from our understanding, there is no RRM impacts for objective#4.

Proposal 5: No RRM impacts for Objective#4.

## Objective#5

For objective#4, which to support sTRP/UL mTRP. The enhancement is to support two closed loop PC adjustment with pathloss offset configuration. It is stated in the WID in the pothloss RS is from DL sTRP. Thus, for PL-RS measurement perspective, there is no significant difference from legacy cases. The only difference is to apply the UL power with the additional PL offset. Thus, we also don’t foresee significant RRM impacts for objective#5.

Proposal 6: No RRM impacts for Objective#5.

# Conclusions

Observation 1: Following Event-2 is agreement in RAN1 so far with at least L1-RSRP as the quality metrics.

- Event-2: quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam

Observation 2: For the current beam and new beam determination, at least followings are agreed in RAN1 so far:

- Current beam: Implicitly derived from a QCL RS of indicated TCI state (QCL RS or SSB QCL-ed with QCL RS)

- New beam: Explicitly configured

Observation 3: At least SSB-based L1-RSRP for intra-cell and inter-cell and periodic CSI-RS for beam management are supported.

Observation 4: For Event-2, following two triggering determinations are supported:

- Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs

- If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

Proposal 1: RAN4 to define RRM requirements for Objective#1 (UE-initiated/event-driven beam reporting).

Proposal 2: RAN4 start with Event-2:

- L1-RSRP as the quality metrics

- SSB and periodic CSI-RS as RS for beam management

- Following two triggering determinations

  - Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs

  - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

Note: Can be updated based on further RAN1 progress.

Proposal 3: No RRM impacts for Objective#2.

Proposal 4: RAN4 to discuss whether to define core requirements for CJT calibration reporting for delay offset/frequency offset/phase offset.

Proposal 5: No RRM impacts for Objective#4.

Proposal 6: No RRM impacts for Objective#5.

# References

[1] RP-234007 New WID: NR MIMO Phase 5