3GPP TSG-RAN WG4 Meeting # 112	R4-2412205

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.21.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impact for Rel-19 NES

Document for:	Discussion

# Introduction

According to the latest WID [1] for Rel-19 NES, RAN4 shall start work for the objectives listed in the WID from this meeting. In this contribution, we provide our views regarding the RAN4 impacts for the WI.

# Discussion

Based on the latest WID [1], the relevant objectives are shown as follows:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105 - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

It could be observed that there are three main objectives: on-demand SSB, on-demand SIB1 and adaptation of common signal/channel transmission. In the following part of the contribution, we analyze the impacts of each objectives.

## On-demand SSB

Regarding the On-demand SSB, the relevant RAN1 agreements so far are summarized as follows:

Scenario:

Agreement Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study: - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell - FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted. FFS: Which scenario the above applies for Agreement For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios. - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321) - Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321) - This does not preclude SCell for which activation is completed - FFS: The case where SCell activation is completed FFS: Application timing between NW triggering message and on demand SSB transmission Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

Configuration and triggering:

Agreement - For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable Agreement - For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported. - Frequency of the on-demand SSB - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Periodicity of the on-demand SSB - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE - FFS: Whether the RRC is newly introduced or existing RRC is reused Agreement For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - FFS: Whether the value of T is predefined or indicated/configured by gNB - FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB” Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

L1/L3 measurement:

Agreement - For a cell supporting on-demand SSB SCell operation, - L1 and/or L3 measurement based on on-demand SSB is supported for the cell. - FFS further details on L1 and/or L3 measurement Agreement - At least support L1 measurement based on on-demand SSB - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported. - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB

Regarding the scenario, it is important to clarify the scope of RAN4 work at the beginning. According to RAN1 agreements, following four cases are already agreed:

- Case A: Scenario #2 and Case #1

- Case B: Scenario #2 and Case #2

- Case C: Scenario #2A and Case #1

- Case D: Scenario #2A and Case #2

For Scenario 3A and Scenario 3B, they are still FFS in RAN1 discussion. For initial RAN4 discussion, we could postpone the discussion until RAN1 has further agreement.

Proposal 1: RAN4 starts work on following RAN1 agreed scenario and cases, and postpone the discussion on Scenario #3A and Scenario #3B until RAN1 has further progress.

- Case A: Scenario #2 and Case #1

- Case B: Scenario #2 and Case #2

- Case C: Scenario #2A and Case #1

- Case D: Scenario #2A and Case #2

For above four cases, the potential impacts on RAN4 RRM requirements are as follows:

Case A: Scenario #2 and Case #1

For case A, which is shown in following Fig.1. OD SSB is transmitted on the SCell before it is activated. The benefits of the Scenario are not that straightforward as OD-SSB based SCell activation as Case C and Case D. For a deactivated SCell, the only procedure is to perform L3 measurement on the deactivated SCell. When there is OD SSB, if UE can perform L3 measurement based on OD SSB for the deactivated SCell, UE can maintain the measurement and T/F synchronization. UE can achieve known SCell activation upon receiving SCell activation command.

Fig.1 Scenario #2 and Case #1

Case B: Scenario #2 and Case #2

Fig.2 Scenario #2 and Case #s

For case B, which is shown in following Fig.2. Similar as case A, UE can maintain the measurement on the deactivated SCell based on OD SSB.

Observation 1: For Case A and Case B, if UE can perform L3 measurement based on OD SSB, UE can maintain the measurement for the deactivated SCell without always on SSB in Case A or with long period always on SSB. Thus, it is beneficial for fast SCell activation and achieving NW energy saving at the same time.

Based on above analysis, in details, the promising scenario in RAN4 is known SCell activation and SCell activation based on L3 reporting introduced in Rel-18.

Proposal 2: For Case A (Scenario #2 and Case #1) and Case B (Scenario #2 and Case #2), RAN4 to work on L3 measurement on deactivated SCell based on OD-SSB to enable known SCell activation and SCell activation based on L3 reporting triggered by SCell activation command introduced in Rel-18.

Case C: Scenario #2A and Case #1

As shown in following Fig.3, the OD-SSB will be triggered when receiving SCell activation command. The targeting scenario and gain is straightforward, which is to speed up the SCell activation procedure by utilizing OD-SSB and achieve energy saving by avoiding always on SSB. In Rel-18, RAN4 introduced the SCell activation enhancement by utilizing the L3 measurement result without reporting to achieve known SCell activation delay. For OD-SSB based SCell activation, it could be used to reduce the SCell activation delay for pure unknown case (with L3 measurement and reporting). In other words, UE has to perform necessary procedures, e.g. AGC/Cell search/L1-RSRP, but benefiting from shorter SSB periodicity, UE can complete SCell activation with much lower latency.

Fig.3 Scenario #2A and Case #1

Case D: Scenario #2A and Case #2

For Case D, which is shown in following Fig.4, the benefits are similar as case C. The only difference maybe that there could be mix of OD-SSB and always on periodic SSB. The details are subject to further RAN1 design.

Fig.4 Scenario #2A and Case #2

Observation 2: The target Scenario and gain for Case C and Case D is straightforward, which is to enable SCell activation based on OD-SSB with potential delay reduction to achieve NW energy saving and guarantee the SCell activation performance at the same time.

In previous release, the RAN4 SCell activation has been enhancement in different aspects, e.g. multiple SCell activation, SCell activation based on A-TRS, PUCCH SCell activation, SCell activation based on L3 reporting. As analysis above, the beneficial Scenario is for pure unknown. To have converged discussion at the initial stage, we suggest to prioritize the discussion for Case C and Case D from single SCell activation.

Proposal 3: For Case C (Scenario #2A and Case #1) and Case D (Scenario #2A and Case #2), RAN4 starts work from single SCell activation unknown case (i.e. without L3 measurement/report before SCell activation) for:

- SCell activation (8.3.2)

- PUCCH SCell activation (8.3.12)

And multiple SCell activation requirements can be discussed after the discussion on single SCell activation is concluded.

For detailed OD-SSB configurations and triggering, it is still under discussion in RAN1/2. RAN4 shall wait for further RAN1 progress, and RAN4’s work does not have much dependency on such details.

Observation 3: RAN4 shall wait for further RAN1/2 progress on OD-SSB configurations and triggering.

Another aspect of potential RAN4 impact is L1/L3 measurement. RAN1 has agreed to support L1 measurement based on OD-SSB. From our understanding, RAN1 will further discuss the details of OD-SSB based L1 measurement, e.g. type of L1 measurement, triggering/reporting framework. Thus, it is suggested to wait for more progress on this issue. For L3 measurement, there is no further RAN1/2 progress, for which we believe RAN4 is more responsible. The scenario for L3 measurement shall be further clarified. For L3 measurement on a serving cell with OD-SSB, as mentioned in Case A and Case B, there is benefits as analysed above. But if there is a neighbour cell with OD-SSB, the benefits and feasibility is not clear. For a neighbour cell, if there is always on periodic SSB, we believe current L3 measurement requirements and UE behaviour still apply since UE can perform L3 measurement in legacy manner base on these always on SSB, no mater whether there is OD SSB or not. However, if there is a Cell with only OD-SSB, it is not clear whether the Cell is expected to be detected by other UEs base on neighbour measurement. Since the OD-SSB could be quite dynamic, it is not possible to dynamically cooperate among many Cell about the on-off state of OD-SSB, and transmit the information to all UE under these Cells. And from UE perspective, it is also questionable whether UE can tell if the OD-SSB is transmitted of not.

Observation 4: For L3 measurement based on OD SSB for serving cell (deactivated SCell), there could be potential benefits as analysed for Case A and Case B.

Observation 5: For L3 measurement based on OD SSB for neighbour cell, it is feasibility and reliability are questionable and gain is not clear.

Proposal 4: For L1 measurement based on OD SSB, which is already agreed in RAN1, RAN4 to wait for further RAN1 progress.

Proposal 5: RAN4 to deprioritize the discussion for L3 measurement based on OD SSB for neighbour cell, as feasibility and reliability are questionable and gain is not clear.

## On-demand SIB1

For On-demand SIB1, which is still under study and it will be checked in RAN#115. Thus, RAN4 shall postpone the discussion.

Proposal 6: RAN4 to postpone the discussion for On-demand SIB1 since it is still under study in other WGs.

## Adaptation of common signal/channel transmission

For adaptation of common signal/channel transmission, there are mainly two aspects under discussion: PRACH and SSB. The relevant RAN1 agreements are summarized as follows:

PRACH

Agreement For adaptation of PRACH in time-domain, support at least the following: - Adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any) - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs - Configuration of additional PRACH resources is provided by semi-static signalling - FFS: details including whether there is overlap of additional PRACH resources and PRACH resources for legacy UEs - FFS: adaptation mechanism for additional PRACH resources - Note: No change to the existing PRACH configuration tables in 38.211 Agreement For adaptation of PRACH in time-domain, support the following: - SSB-RO mapping for the additional PRACH resources is separate from the SSB-RO mapping of the PRACH resources for legacy UEs (if any) - FFS: whether/how to handle SSB-RO mapping if the additional PRACH resources overlap in both time and frequency with the PRACH resources for legacy UEs - Note: SSB-RO mapping of the PRACH resources for legacy UEs is not impacted if Rel-19 UE uses these PRACH resources - FFS: SSB-RO mapping for the additional PRACH resources Agreement Support adaptation mechanisms of PRACH in time-domain for following: - UE in idle/inactive mode - UE in connected mode Agreement For adaptation of PRACH in time-domain, support at least the following case(s) - Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 2: time-domain overlap but no overlap in frequency domain between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 3: additional PRACH resources for NES-capable UEs and legacy PRACH resources overlap neither in time nor frequency domains - FFS: whether additional conditions are needed to support the above cases - FFS: Additional case whether full/partial overlap in both time and frequency is allowed - Above does not preclude discussion for the case where the configuration for additional PRACH resources contains legacy PRACH resources Agreement At least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule. - Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes. - Note: This mapping is not impacted by time domain PRACH adaptation - Validation rules for the additional PRACH resources follow the legacy validation rules for PRACH resources configured for legacy UEs.

SSB

Agreement For indication of adaptation of SSB in time-domain, - Support at least SSB adaptation provided by gNB without UE trigger Agreement Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s): - For cell with both legacy UEs and Rel-19 NES-capable UEs - Rel-19 NES-capable UE’s PCell (Connected mode) - Study from the following options: - Option A1: adaptation for CD-SSB - Option A2: adaptation for SSB that is not CD-SSB - Option A3: adaptation for SSB not on sync raster - Rel-19 NES-capable UE’s SCell - Study from the following options: - Option B1: adaptation for CD-SSB - Option B2: adaptation for SSB that is not CD-SSB - Option B3: adaptation for SSB not on sync raster - FFS: Rel-19 NES-capable UE in idle/inactive mode - Note: Impact to idle/inactive UEs shall be minimized

For adaptation of PRACH, it was concluded that time-domain PRACH adaptation is considered. From RAN4 RRM requirements perspective, there is no significant impacts as PRACH is usually only referred to and the end point of certain RRM activities, and the detailed PRACH occasion is directly referred to other spec.

Observation 6: For PRACH adaptation, from RAN4 RRM requirements perspective, there is no significant impacts as PRACH is usually only referred to and the end point of certain RRM activities, and the detailed PRACH occasion is directly referred to other spec.

For SSB adaptation, clearly there is RAN4 RRM impacts, and the impacted requirements/procedure includes both PCell/SCell and also IDLE/INACTIVE mode. However, the agreement/conclusion in RAN1 is quite generic now. RAN4 shall start the work with further RAN1 conclusion when the scenario is clearer.

Proposal 7: For SSB adaptation, there are RAN4 RRM impacts in different aspects. RAN4 shall start the work with further RAN1 conclusion when the scenario becomes clearer.

# Conclusions

Proposal 1: RAN4 starts work on following RAN1 agreed scenario and cases, and postpone the discussion on Scenario #3A and Scenario #3B until RAN1 has further progress.

- Case A: Scenario #2 and Case #1

- Case B: Scenario #2 and Case #2

- Case C: Scenario #2A and Case #1

- Case D: Scenario #2A and Case #2

Observation 1: For Case A and Case B, if UE can perform L3 measurement based on OD SSB, UE can maintain the measurement for the deactivated SCell without always on SSB in Case A or with long period always on SSB. Thus, it is beneficial for fast SCell activation and achieving NW energy saving at the same time.

Proposal 2: For Case A (Scenario #2 and Case #1) and Case B (Scenario #2 and Case #2), RAN4 to work on L3 measurement on deactivated SCell based on OD-SSB to enable known SCell activation and SCell activation based on L3 reporting triggered by SCell activation command introduced in Rel-18.

Observation 2: The target Scenario and gain for Case C and Case D is straightforward, which is to enable SCell activation based on OD-SSB with potential delay reduction to achieve NW energy saving and guarantee the SCell activation performance at the same time.

Proposal 3: For Case C (Scenario #2A and Case #1) and Case D (Scenario #2A and Case #2), RAN4 starts work from single SCell activation unknown case (i.e. without L3 measurement/report before SCell activation) for:

- SCell activation (8.3.2)

- PUCCH SCell activation (8.3.12)

And multiple SCell activation requirements can be discussed after the discussion on single SCell activation is concluded.

Observation 3: RAN4 shall wait for further RAN1/2 progress on OD-SSB configurations and triggering.

Observation 4: For L3 measurement based on OD SSB for serving cell (deactivated SCell), there could be potential benefits as analysed for Case A and Case B.

Observation 5: For L3 measurement based on OD SSB for neighbour cell, it is feasibility and reliability are questionable and gain is not clear.

Proposal 4: For L1 measurement based on OD SSB, which is already agreed in RAN1, RAN4 to wait for further RAN1 progress.

Proposal 5: RAN4 to deprioritize the discussion for L3 measurement based on OD SSB for neighbour cell, as feasibility and reliability are questionable and gain is not clear.

Proposal 6: RAN4 to postpone the discussion for On-demand SIB1 since it is still under study in other WGs.

Observation 6: For PRACH adaptation, from RAN4 RRM requirements perspective, there is no significant impacts as PRACH is usually only referred to and the end point of certain RRM activities, and the detailed PRACH occasion is directly referred to other spec.

Proposal 7: For SSB adaptation, there are RAN4 RRM impacts in different aspects. RAN4 shall start the work with further RAN1 conclusion when the scenario becomes clearer.

# References

[1] RP-241650 Revised WID: Enhancements of network energy savings for NR