3GPP TSG-RAN WG4 Meeting # 112	R4-2412207

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.26.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impacts for R19 IoT NTN

Document for:	Discussion

# Introduction

The new WID of NTN IoT was approved in [1] and revised [2]. In this contribution, we provide our initial analysis of RRM impacts of the WI.

# Discussion

The RRM related objectives are shown as follows:

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

According to the discussion during RP about the new proposed revision, the intention of the item is about the timing related aspect for EDT, which is related to the LS from RAN2 [3]. The content of the LS is shown as follows:

In RAN2#126 meeting, the following agreements related to Msg3 transmission for uplink capacity enhancement for R19 IoT NTN have been achieved: Agreements: => RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources). => If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3.. Based on the above agreements, RAN2 kindly asks RAN4 and RAN1: Q1: Whether an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2? Q2: If the answer for Q1 is no, from RAN4 and RAN1 perspective, how the required timing accuracy for Msg3 transmission can be satisfied in this case?

Proposal 1: The work scope of RRM for Rel-19 IoT NTN is to discuss whether and how to define timing requirements for Msg3 transmission without msg1/ Random Access Response (RAR).

From our understanding, the situation mentioned above is similar to PUR transmission introduced in previous release, where UE maintains the timing in IDLE mode and meets the timing requirements. The corresponding timing requirements for PUR is summarized as follows:

### 7.20A.1 Introduction The Category NB1 UE shall have capability to follow the frame timing change of the connected eNode B. The uplink frame transmission takes place NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×Ts before the reception of the first detected path (in time) of the corresponding downlink frame from the reference NB-IoT cell. UE shall use the serving NB-IoT cell as the reference cell for deriving the UE transmit timing. UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are defined in the following requirements. ### 7.20A.2 Requirements The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.20A.2-1. This requirement applies when it is the first transmission in a DRX cycle or the first transmission in a repetition period (R>1) for NPUSCH and NPRACH, the first transmission after an uplink transmission gap in a repetition period (R>1) for NPUSCH and NPRACH transmission, or it is the transmission on PUR. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the serving NB-IoT cell minus NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×Ts. The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the serving NB-IoT cell. NTA_Ref for NPRACH is defined as 0. NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific (in Ts units) for other channels is the difference between UE transmission timing and the Downlink timing immediately after when the last timing advance in clause 7.22A was applied. NTA_Ref for other channels is not changed until next timing advance is received. Table 7.20A.2-1: Te Timing Error Limit Downlink Bandwidth (MHz); Te_. 0.18; 97*TS. Note 1: TS is the basic timing unit defined in TS 36.211.

Observation 1: For PUR transmission, UE shall meet the timing transmitting timing requirements defined 7.20A using latest acquired NTA.

Regarding how to validate the TA, there was RAN4 discussion in Rel-18 that whether to define NRSRP-based TA validation in NTN. The controversial part is whether there is necessity to verify the TA by NRSRP measurement, which is introduced for TN. For NTN, UE can explicitly estimate the change of by GNSS and ephemeris and compensate it accordingly, thus there is no need to implicit verify the TA by NRSRP change. Thus, RAN4 achieved following agreements:

#### Issue 2-3-1: PUR, RSRP-based TA validation - Agreement - Do not define requirements for the legacy RSRP-based TA validation for PUR for both GEO and LEO

Observation 2: RAN4 did not define RSRP-based TA validation for PUR for IoT NTN.

Compared with PUR transmission, the scenario mentioned in RAN2 LS is even simpler. There is no maintained NTA and TA validation. For other part (i.e. NTAoffset, NTA,common,NTA, UE-specific), UE’s behaviour and performance are exactly the same as PUR transmission.

Observation 3: Compared with PUR transmission, UE may not have pre-configured NTA for Msg3 without Msg1/RAR.

Observation 4: For Msg3 without Msg1/RAR , UE determines reference point of the UL transmit timing for Msg3 without Msg1/RAR in the same manner as PUR, which is the downlink timing of the serving NB-IoT cell minus NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×Ts. The only difference part is the value of NTA_Ref.

From our understanding, UE can only assume NTA_Ref equal to 0 as NPRACH transmission if there is no further RAN1/2 procedure, and UE shall be able to meet the same timing error requirements.

Observation 5: UE can only assume NTA_Ref equal to 0 NPRACH transmission if there is no further RAN1/2 procedure, and UE shall be able to meet the same timing error requirements.

Based on analysis above, for Msg3 transmission without Msg1/Msg2, from RAN4 perspective in terms of UE UL transmit timing error requirements, UE shall met same timing error requirements as defined for IoT NTN, where the reference point for the is the downlink timing of the serving NB-IoT cell minus NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×Ts, and NTA_Ref is assumed as 0 if there is no further RAN1/2 agreements.

Proposal 2: For Msg3 transmission without Msg1/Msg2, from RAN4 perspective in terms of UE UL transmit timing error requirements, UE shall meet same timing error requirements as defined for IoT NTN, where the reference point is the downlink timing of the serving NB-IoT cell minus NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×Ts, and NTA_Ref is assumed as 0 if there is no further RAN1/2 agreements.

We provide the Reply LS in the Annex.

# Conclusions

Proposal 1: The work scope of RRM for Rel-19 IoT NTN is to discuss whether and how to define timing requirements for Msg3 transmission without msg1/ Random Access Response (RAR).

Observation 1: For PUR transmission, UE shall meet the timing transmitting timing requirements defined 7.20A using latest acquired NTA.

Observation 2: RAN4 did not define RSRP-based TA validation for PUR for IoT NTN.

Observation 3: Compared with PUR transmission, UE may not have pre-configured NTA for Msg3 without Msg1/RAR.

Observation 4: For Msg3 without Msg1/RAR , UE determines reference point of the UL transmit timing for Msg3 without Msg1/RAR in the same manner as PUR, which is the downlink timing of the serving NB-IoT cell minus NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×Ts. The only difference part is the value of NTA_Ref.

Observation 5: UE can only assume NTA_Ref equal to 0 NPRACH transmission if there is no further RAN1/2 procedure, and UE shall be able to meet the same timing error requirements.

Proposal 2: For Msg3 transmission without Msg1/Msg2, from RAN4 perspective in terms of UE UL transmit timing error requirements, UE shall meet same timing error requirements as defined for IoT NTN, where the reference point is the downlink timing of the serving NB-IoT cell minus NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×Ts, and NTA_Ref is assumed as 0 if there is no further RAN1/2 agreements.

# References

[1] RP-234077 New WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

[2] RP-241624 Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

[3] R2-2405769 LS on UL synchronization for contention based Msg3 transmission without Msg1/Msg2

# Annex

3GPP TSG-RAN WG4 Meeting #112	R4-24xxxxx

Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	Reply LS on UL synchronization for contention based Msg3 transmission without Msg1/Msg2

Response to:	R2-2405769

Release:	Rel-19

Work Item:	IoT_NTN_Ph3-Core

Source:	RAN4

To:	RAN2

Cc:	RAN1

Contact Person:

#### Name:

E-mail Address:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN4 thanks RAN2 for the LS (R2-2405769) on UL synchronization for contention based Msg3 transmission without Msg1/Msg2. RAN4 discussed the questions in RAN4#112 and achieved following agreements.

Q1: Whether an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2?

Answer: From RAN4 perspective, in terms of UE UL transmit timing error requirements, UE shall meet same timing error requirements as defined for IoT NTN, where the reference point is the downlink timing of the serving NB-IoT cell minus NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×Ts, and NTA_Ref is assumed as 0 if there is no further RAN1/2 agreements.

Q2: If the answer for Q1 is no, from RAN4 and RAN1 perspective, how the required timing accuracy for Msg3 transmission can be satisfied in this case?

Answer: As replied to Q1.

2. Actions:

To RAN4 group:

ACTION: RAN4 respectfully ask RAN2 to take the above information into account

3. Date of Next TSG-RAN WG4 Meeting:

TSG-RAN4 Meeting #112bis	October 14 – October 18, 2024 	Hefei, China

TSG-RAN4 Meeting #113	October 14 – October 18, 2024 	Orlando, US