3GPP TSG-RAN WG4 Meeting # 112	R4-2412209

Maastricht, Netherlands, 19th – 23rd August, 2024

Title: 	Discussion on maintenance for L1/L2-based inter-cell mobility

Source: 	Huawei, HiSilicon

Agenda item:	5.24.1

Document for:	Discussion

Introduction

L1/L2 based inter-cell mobility was discussed in last meeting [1]. This contribution provides further analysis on maintenance requirements on L1/L2 based inter-cell mobility from RRM requirement perspective.

Discussion

Early TCI state activation

The issue on early TCI state activation delay is still open [1]. In the endorsed big CR, a new clause for SSB based TCI state activation delay is introduced [R4-2410399]. There are two open leftovers, one is whether to consider early TCI state activation for multiple cells at the same time (Tfirst-SSB_List or Tfirst-SSB is to be used), the other is whether and how to support unknown TCI state in FR2.

8.x.3 SSB based TCI state activation delay In FR1, when the UE receives LTM candidate cell TCI state activation command at slot n, the UE shall have completed the LTM TCI state list update in slot n + THARQ +3Nslotsubframe,µ + [ TOk*(Tfirst-SSB_List or Tfirst-SSB + TSSB-proc) / NR slot length].In FR2, when all the target TCI states in the active TCI state list are known, when the UE receives TCI state activation command at slot n, UE shall have completed the LTM TCI state list update in slot n + THARQ +3Nslotsubframe,µ + [ TOk*(Tfirst-SSB_List or Tfirst-SSB + TSSB-proc) / NR slot length]. Editor’s Note: FFS the definition of Tfirst-SSB_List or Tfirst-SSB Editor’s Note: FFS whether and how to consider unknown TCI state in FR2 Where: - THARQ is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]. - TOk = 1 if the target TCI state is not in the serving cell active TCI state list or LTM candidate cell active TCI state list, 0 otherwise. - Tfirst-SSB is the time to first SSB occasion after slot n + THARQ +3Nslotsubframe,µ when the target SSB is within active BWP, upon receiving PDSCH carrying MAC-CE activation command in slot n; - Tfirst-SSB is the time to first SSB occasion overlapping with MGL after slot n + THARQ +3Nslotsubframe,µ + [2ms] if the SSB needs to be measured with MG, upon receiving PDSCH carrying MAC-CE activation command in slot n; - FFS: this is only applicable to UE supporting inter-frequency L1 measurement with MG. - TSSB-proc = 2 ms.

Issue 1-3-1: Whether to consider early TCI state activation for multiple cells at the same time - Proposals - Option 1 (Huawei, vivo, Nokia, MTK): - Early TCI state activation delay requirements to be defined for one or more TCI states for a single candidate cell, because one MAC-CE activates TCI states only for a single candidate cell. - Option 2 (Ericsson, QC): - If all the target LTM TCI states in the active TCI state list are known, if the UE receives TCI state activation command at slot n, UE shall have completed the LTM TCI state list update in slot n + THARQ +3Nslotsubframe,µ + TOk*(Tfirst-SSB_List + TSSB-proc) / NR slot length. - If any of the target TCI states in the active TCI state list are unknown, if the UE receives TCI state activation command at slot n, UE shall have completed the LTM TCI state list update in slot n+ THARQ +3Nslotsubframe,µ + (TL1-RSRP_list +TOuk*(Tfirst-SSB_List+ TSSB-proc)) / NR slot length. - In the TCI state activation requirements for LTM candidate cells, T L1-RSRP = 0 for FR1 - In the TCI state activation requirements for LTM candidate cells, Tfirst-SSB_List is given by - For FR1, Tfirst-SSB_List = max (Tfirst-SSB_LTM1, Tfirst-SSB_LTM2, .. , Tfirst-SSB_LTMn). - For FR2, - Tfirst-SSB_List = Tfirst-SSB_LTM1 + Tfirst-SSB_LTM2 + ... +Tfirst-SSB_LTMn, if the time to first SSB associated to LTM candidate TCI states are overlapped in FR2. - Tfirst-SSB_List = max (Tfirst-SSB_LTM1, Tfirst-SSB_LTM2, .. , Tfirst-SSB_LTMn.) if the time to first SSB associated to LTM candidate TCI states are not overlapped. - Where, the Tfirst-SSB_LTMn is the SSB periodicity of LTM candidate cell n. < Way Forward > - Early TCI state activation delay requirements are defined for one or more TCI states for a single candidate cell.

- When only one LTM TCI activation MAC CE is received

In R17 Tfirst-SSB_List is used for Active Downlink TCI state list update delay. Herein serial SSB periodicity is considered when the target TCI states of both serving cell and a cell with different PCI are in the active TCI state list. However in R18 LTM, as per RAN2 spec, a LTM candidate cell TCI state activation command indicates TCI state (s) of one candidate cell. So UE shall separately process TCI state activation procedure from different candidate cell.

- 8.15.5 Active Downlink TCI state list update delay <Omit no related requirements> If all the target TCI states in the active TCI state list are known, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall have completed the TCI state list update in slot n + THARQ +3Nslotsubframe,µ + TOk*(Tfirst-SSB_List + TSSB-proc) / NR slot length. <Omit no related requirements> - If the number of cells associated with the target TCI states in the active TCI list is 2, and time to first SSBs associated to the TCIs are overlapped in FR2, - Tfirst-SSB_List = Tfirst-SSB_SC + min(TSSB_SC , TSSB_CDP). - If the number of cells associated with the target TCI states in the active TCI list is 2, and time to first SSBs associated to the TCIs are not overlapped in FR1 or FR2, - Tfirst-SSB_List = max(Tfirst-SSB_SC , Tfirst-SSB_CDP) -Otherwise, - Tfirst-SSB_List = Tfirst-SSB_SC.

- When multiple LTM TCI activation MAC CEs are received at the same time

If we consider the scenario that multiple LTM TCI state activation commands are received at the same time, some requirements shall be further updated. Herein some examples are provided,

- In FR1, if the candidate neighbor cells TCI states associated are on different frequency layers and the SSB from these candidate cells are overlapped within MGL, UE is only able to acquire the SSB from one neighbor cell at one MGL duration. In this case, the number of frequency layers would be scaled when defining TCI state activation delay.

- Still in FR1, if RTD between TCI states associated candidate cells is larger than CP, UE with [capability of measurement with RTD>CP] needs to measure SSB on multiple intra-frequency layers in sequence. In this case, the number of intra-frequency layers is to be scaled as well when defining TCI state activation delay.

- In FR2, if the TCI state activation commands from multiple candidate cells are indicated, UE is not able to acquire SSB from different cells at the same time, as the cells are from different directions. In this case, the Rx beam number is supposed to be scaled when defining TCI state activation delay.

In our understanding, the number of LTM candidate cell is limited, it is not a typical configuration that network simultaneously indicates multiple LTM TCI activation MAC CE for multiple frequency layers or multiple cells. Moreover based on the above analysis, such configuration would severely prolong TCI state activation delay. Therefore we suggest not to specify requirements for multiple LTM TCI activation MAC CEs reception scenarios.

Proposal 1: No requirements of early TCI state activation delay are specified for the case that multiple LTM TCI activation commands are received at the same time.

In FR1, it is agreed to support both known and unknown TCI state in FR1in RAN4#110bis meeting. From TCI state activation delay perspective, the requirements for known case and unknown case in FR1 are the same. However it shall be noted that for unknown TCI state in FR1, UE shall have sent a valid L3 measurement report with SSB index of the TCI associated target cell within 1280ms before the LTM TCI state activation command. In our understanding, 1280ms is usually used for beam related scenario, like SCell activation procedure.

Proposal 2: Unknown TCI state in FR1, provided the following conditions are met:

-	UE has reported L3 measurement result with the associated SSB index of the TCI state within 1280ms before the LTM TCI state activation command.

-	SNR of the associated SSB is above -3dB.

In FR2, as we know, UE would keep tracking the timing of a candidate TCI state after the TCI state is indicated to be activated. It is not a typical case that network blindly activates one candidate TCI state from neighbor cell as it has no any information of the to be activated TCI. Moreover in FR2 if the candidate TCI state is unknown, UE needs to sweep RX beam to perform L1-RSRP measurement, then the time delay for activating one unknown TCI state in FR2 would be extremely long.

Proposal 3: Early TCI state activation delay is not applicable to FR2 unknown case.

LTM L1-RSRP measurement requirements

Report the unmeasured candidate cells

One open issue is regarding the L1 measurement report for unmeasured candidate cells. The potential scenario is that some of configured RS resources in LTM-CSI-ResourceConfig are not measured, as condition of the resource is not satisfied from UE side, e.g. due to bad SINR or not falling in UE Rx beam.

Issue 2-4-1: L1 report for unmeasured candidate cells <Way Forward >: Further discuss the following options: - Option 1 (vivo): - In L1-RSRP measurement report, for unmeasured candidate cells, UE sends invalid L1-RSRP in PUCCH if needed, i.e. the reported value corresponds to one of the invalid codepoints for L1-RSRP in Table 10.1.6.1-1 or DIFFRSRP_15 in Table 10.1.6.1-2. - In this case, the SSB or the cell it reflects remains unknown to the UE. - Option 2 (CTC): - In L1-RSRP measurement report, for unmeasured candidate cells, UE reports measured quantity value corresponding to any of the invalid codepoints in Table 10.1.6.1-1. - Option 3 (Nokia): RAN4 to send LS to RAN1 - Question: From RAN1 point of view, is it acceptable for the UE to report invalid values (from table 10.1.6.1-1 of TS 38.133) or value DIFFRSRP_15 (from table 10.1.6.1-2 of TS 38.133) for LTM candidate cells that are not measured by the UE? At least the following should be considered: - When none of the candidate cells are measured within the given periodicity, UE reports a 7 bit “Not valid” value. - DIFFRSRP_15 is reported when at least one LTM candidate cell was measured and at least one configured candidate cell was unmeasured. - The reported values for unmeasured cells do not meet any measurement requirements as they are unmeasured. - Option 4 (Ericsson, QC): - In L1-RSRP measurement report, for unmeasured candidate cells, UE reports DIFFRSRP_15 in Table 10.1.6.1-2.

As per RAN1 agreement, the largest measured value of L1-RSRP is reported by 7-bit value in the range [-140, -44] dBm with 1dB step size according to Table 10.1.6.1-1 in TS38.133, and the other measurement results adapt differential scheme. The differential L1-RSRP value is computed with 2 dB step size with a reference to the largest measured L1-RSRP value where the differential L1-RSRP value mapping is defined in Table 10.1.6.1-2 in TS38.133.

Table 10.1.6.1-2: Differential SS-RSRP and CSI-RSRP measurement (for L1 reporting and L3 reporting) report mapping

| Reported value | Measured quantity value (difference in measured RSRP from strongest RSRP) | Unit |
| --- | --- | --- |
| DIFFRSRP_0 | 0≥ΔRSRP>-2 | dB |
| DIFFRSRP_1 | -2≥ΔRSRP>-4 | dB |
| DIFFRSRP_2 | -4≥ΔRSRP>-6 | dB |
| DIFFRSRP_3 | -6≥ΔRSRP>-8 | dB |
| DIFFRSRP_4 | -8≥ΔRSRP>-10 | dB |
| DIFFRSRP_5 | -10≥ΔRSRP>-12 | dB |
| DIFFRSRP_6 | -12≥ΔRSRP>-14 | dB |
| DIFFRSRP_7 | -14≥ΔRSRP>-16 | dB |
| DIFFRSRP_8 | -16≥ΔRSRP>-18 | dB |
| DIFFRSRP_9 | -18≥ΔRSRP>-20 | dB |
| DIFFRSRP_10 | -20≥ΔRSRP>-22 | dB |
| DIFFRSRP_11 | -22≥ΔRSRP>-24 | dB |
| DIFFRSRP_12 | -24≥ΔRSRP>-26 | dB |
| DIFFRSRP_13 | -26≥ΔRSRP>-28 | dB |
| DIFFRSRP_14 | -28≥ΔRSRP>-30 | dB |
| DIFFRSRP_15 | -30≥ΔRSRP | dB |

Obviously, the largest measured value is not from the unmeasured cell. So the L1-RSRP reporting of unmeasured cells shall use differential reporting. It shall be noted that in Table 10.1.6.1-2, there is no reserved codepoint for invalid value. Therefore we propose to select DIFFRSRP_15 to be used in this scenario, as DIFFRSRP_15 is rare to be used in practice. To ensure the some understanding between network and UE, some clarification on DIFFRSRP_15 is required.

Proposal 4: DIFFRSRP_15 in Table 10.1.6.1-2 to be used for measurement reporting for unmeasured LTM L1-RSRP resources. To ensure the some understanding between network and UE, some clarification on DIFFRSRP_15 is required.

LTM Cell switch delay requirements

TRS as QCL source in cell switch delay

Issue 3-1-1-1: T/F fine tracking when TRS as QCL source in cell switch delay <Way Forward >: Further discuss the following options: - Option 1 (MTK): UE is supposed to perform T/F tracking (if needed) based on SSB during cell switch delay in R18 LTM. - Option 2 (Nokia): Add TRS as a possible QCL source for T/F tracking in RAN4 cell switch delay requirements.

In current LTM cell switch delay requirement, only SSB is considered for T/F fine tracking. If SSB is configured as a source RS in TCI state which is QCL type-C, according to RAN1 specification UE does not expect to be indicated type-A properties, and assumes PDCCH/PDSCH receptions based on SSB is QCL-type A. Therefore the current RAN4 requirements where SSB is used for T/F tracking are well aligned with RAN1.

TS38.213 clause 21

Another question is if TRS is configured as a resource RS in TCI state which is QCL type-A, whether new requirements are to be introduced. For RACH based LTM cell switch, as during legacy L3 handover delay, SSB is used for T/F tracking, it is sufficient to use SSB to perform T/F tracking for LTM cell switch as well. For RACH-less based LTM cell switch, either SSB or TRS can work to obtain T/F synchronization. As UE is switched to a new cell, the conservative implementation is using low MCS in the initial stage. Using SSB does not violate RAN1 spec, and using TRS may shorten cell switch delay if TRS has shorter periodicity.

Proposal 5: If TRS is configured as a resource RS in TCI state,

-the current RACH based LTM cell switch delay can keep unchanged with using SSB for T/F tracking.

-for RACH-less based LTM cell switch delay, either using SSB or using TRS for T/F tracking can work.

Extra time for PL-RS measurement

The issue of extra time for PL-RS measurement is still under discussion. RACH based cell switch is alike legacy handover procedure where the PRACH transmission are based on SSB. There is no additional time for PL-RS measurement when specifying legacy handover delay requirements.

Regarding RACH-less cell switch, from implementation perspective, initial UL signal can be transmitted without pathloss-RS measurement where the initial transmit power can be set a large value. It means that uplink power control during cell switch procedure is not critical. Moreover it is supported in RAN1 that PL-RS is maintained before cell switch command. In this case, no additional time for PL-RS measurement. As we know one motivation of LTM is to reduce the cell switch delay, therefore it is not expected to add extra time for PL-RS measurement and RAN4 only consider PL-RS maintained case.

Proposal 6: RAN4 only consider PL-RS maintained case and no extra time is expected for PL-RS measurement in LTM cell switch delay.

Early ASN.1 decoding and validity/compliance check

In LTM cell switch delay, TLTM_RRC-processing is the time for ASN.1 decoding and validity/compliance check for the RRC configuration of the LTM target cell indicated in the LTM cell switch command. RAN4 is discussing the conditions / cases for TLTM_RRC-processing =0.

Issue 3-1-4-1: Which cell(s) TLTM-RRC-processing = 0 apply to when candidate cells configured are more than UE capability? <Way Forward >: Further discuss the following options: - Option 1 (Apple): - NW is not expected to trigger TCI activation or RACH toward candidate cell unless cell switch is likely to happen soon. - TLTM_RRC-processing in TS38.133 is zero only applicable to the cells with early TCI activation or early TA acquisition, provided the number of these cells doesn’t exceed UE capability regarding number of cells for early ASN.1 decoding. - Option 2 (Huawei): - TLTM-RRC-processing = 0, if the UE supports [earlyDecodingAndValidityCheck] capability, and the maximum number of serving cell(s) and candidate cell(s) and the maximum number of LTMCandidateConfigs does not exceed [number of candidate cells for early ASN.1 decoding and validity check]. Early TCI state activation or PDCCH order RACH does not necessarily trigger early decoding and validity check. - Option 3 (Nokia): - When TCI state activation MAC-CE or PDCCH order is sent for more cells than UE capability for fast processing, the cells for which the UE received TCI state activation MAC-CE or PDCCH order the most recently before cell switch command are the ones that are pre-processed - Option 4 (MTK): - TLTM_RRC-processing in TS38.133 is zero only applicable to the cells whose TCI state(s) are activated, if early TCI state activation on any candidate cell is triggered, and the number of cells to perform early RRC decoding and validity check does not exceed UE capability. - TLTM_RRC-processing in TS38.133 is zero only applicable to the latest cell on which PDCCH-order RACH is triggered if NW doesn’t active TCI state of any candidate cell - Option 5 (Ericsson, QC) - The ltm-CandidateConfig IEs associated with at least one active TCI state - The ltm-CandidateConfig IEs associated with previously performed PDCCH-order PRACH. - If the number of the ltm-CandidateConfig IEs associated with active TCI state and PDCCH-order PRACH transmission is larger than maxLTMCandidateConfig, the ltm-CandidateConfig IEs for fast RRC processing are chosen in reverse chronological order of Candidate Cell TCI States Activation MAC CE and PDCCH-order PRACH, i.e. maxLTMCandidateConfig ltm-CandidateConfig IEs with the most recently activated TCI states and PDCCH-order PRACH transmission. - The current serving cells and the cells inside the ltm-CandidateConfig, chosen by the above condition, across cell groups (i.e. MCG and SCG) is not larger than maxServingAndCandidteCells

It is previously agreed in RAN4 that early TCI state activation and PDCCH order RACH can be used to trigger early ASN.1 decoding and validity check. Due to UE limited storage memory size, there are some discussion on the coordination of the cells which performed early TCI state activation and/or PDCCH order PRACH. Certain principles are proposed. However in our understanding, to perform LTM cell switch, network has already got some information of the candidate cells. When network indicates UE to perform early TCI state activation (pre DL synchronization) and/or PDCCH order RACH (pre UL synchronization) on candidate cells, the cell switch command would come soon. It implies that the cells with early TCI activation or early PDCCH order RACH are all promising candidates.

As we know, the specified capability of [Fast processing of LTM candidate cell RRC configuration] indicates the number of cells that UE can store the configurations and number of LTMCandidateConfigs that UE can support fast processing. Therefore we think TLTM_RRC-processing =0 applies to the cells with early TCI activation or early PDCCH order RACH provided that the total number of serving cells and the candidate cells in the LTM candidate cell configuration, the cells with early TCI activation, and the cells with early PDCCH order RACH does not exceed the capability [Fast processing of LTM candidate cell RRC configuration].

| 39. NR_Mob_enh2 | 39-6 | Fast processing of LTM candidate cell RRC configuration | 1. Indicates the maximum number of serving cell(s) and candidate cell(s), including serving SpCell(s), serving SCell(s) in MCG and SCG, SpCell in LTMCandidateConfig(s) and Scell(s) in LTMCandidateConfig(s) for MCG and SCG, that UE can store the configurations. 2. Indicates the maximum number of LTMCandidateConfigs that UE can support fast processing. |
| --- | --- | --- | --- |

In case the candidate cells configured are more than UE capability, UE would keep the early ASN.1 decoding and validation check of the cells corresponding to the latest TCI activation/PDCCH order RACH configurations before LTM cell switch command.

Proposal 7: TLTM_RRC-processing =0 applies to the cells with early TCI activation or early PDCCH order RACH, provided that the total number of serving cells and the candidate cells in the LTM candidate cell configuration, the cells with early TCI activation, and the cells with early PDCCH order RACH does not exceed the capability [Fast processing of LTM candidate cell RRC configuration].

Proposal 8: TLTM_RRC-processing =0 applies to the latest cell(s) with early TCI activation or early PDCCH order RACH before LTM cell switch command, if the total number of serving cells and the candidate cells in the LTM candidate cell configuration, the cells with early TCI activation, and the cells with early PDCCH order RACH exceed the capability [Fast processing of LTM candidate cell RRC configuration].

In RAN4#111 meeting, RAN4 also agreed that for FR1 LTM requirements are applicable to the case without L1 measurement and reporting. The current LTM cell switch requirements are defined when target cell and target TCI are known. While the current known condition for TCI state is only related with L1-RSRP measurement. In the following we discuss how to accommodate LTM cell switch without L1 measurement in FR1. In LTM procedure, L3 measurement shall be performed in advance. As long as the measurement result with SSB index is reported within 1280ms before LTM TCI state activation command for FR1, network would know the suitable transmit beam and indicate TCI in cell switch to UE. So the current LTM cell switch switching delay can be applied for FR1 without early L1 measurement.

Proposal 9: The current LTM cell switch switching delay can be applied for FR1 without early L1 measurement, when UE has reported L3 measurement result with the associated SSB index of the TCI state within 1280ms before the LTM TCI state activation command.

Conclusions

This contribution provides further analysis on maintenance requirements on L1/L2 based inter-cell mobility. The following proposals are provided:

Proposal 1: No requirements of early TCI state activation delay are specified for the case that multiple LTM TCI activation commands are received at the same time.

Proposal 2: Unknown TCI state in FR1, provided the following conditions are met:

-	UE has reported L3 measurement result with the associated SSB index of the TCI state within 1280ms before the LTM TCI state activation command.

-	SNR of the associated SSB is above -3dB.

Proposal 3: Early TCI state activation delay is not applicable to FR2 unknown case.

Proposal 4: DIFFRSRP_15 in Table 10.1.6.1-2 to be used for measurement reporting for unmeasured LTM L1-RSRP resources. To ensure the some understanding between network and UE, some clarification on DIFFRSRP_15 is required.

Proposal 5: If TRS is configured as a resource RS in TCI state,

-the current RACH based LTM cell switch delay can keep unchanged with using SSB for T/F tracking.

-for RACH-less based LTM cell switch delay, either using SSB or using TRS for T/F tracking can work.

Proposal 6: RAN4 only consider PL-RS maintained case and no extra time is expected for PL-RS measurement in LTM cell switch delay.

Proposal 7: TLTM_RRC-processing =0 applies to the cells with early TCI activation or early PDCCH order RACH, provided that the total number of serving cells and the candidate cells in the LTM candidate cell configuration, the cells with early TCI activation, and the cells with early PDCCH order RACH does not exceed the capability [Fast processing of LTM candidate cell RRC configuration].

Proposal 8: TLTM_RRC-processing =0 applies to the latest cell(s) with early TCI activation or early PDCCH order RACH before LTM cell switch command, if the total number of serving cells and the candidate cells in the LTM candidate cell configuration, the cells with early TCI activation, and the cells with early PDCCH order RACH exceed the capability [Fast processing of LTM candidate cell RRC configuration].

Proposal 9: The current LTM cell switch switching delay can be applied for FR1 without early L1 measurement, when UE has reported L3 measurement result with the associated SSB index of the TCI state within 1280ms before the LTM TCI state activation command.

Reference

- R4-2410302, WF on NR mobility enhancements (part 1), MediaTek

- R4-2410399, Draft Big CR to TS 38.133 on core requirement maintenance for Further NR mobility enhancements, MediaTek Inc, Apple