3GPP TSG-RAN WG4 Meeting # 112							                                     R4-2412218

Maastricht, Netherlands, 19th – 23rd August, 2024

Title: 					Discussion on SSB-less SCell operation

Source: 	Huawei, HiSilicon

Agenda item:	5.29.1

Document for:	Discussion

Introduction

This contribution provides analysis on some maintenance in core requirements for network energy saving.

Discussion

Inter-band SSB-Less operation

Power difference conditions

In RAN4#111 meeting, the side condition of power difference was further discussed. The agreement and way forward are duplicated as below [1],

Issue 1-1-1: Power difference conditions Agreement: Clarify that EPRE difference is based on the EPRE normalized by SCSs of SSB of reference Cell and A-TRS/P-TRS of SSB-less Cell. Only change [9] dB to [12] dB in this meeting. Further discuss whether and how to support larger EPRE difference in next meeting: - Option 1: Clarify that EPRE difference is smaller or equal to [12] dB + ¦20*log (f1/f2)¦ - Margin, where f1 and f2 is the center frequency of reference Cell and SSB-less Cell and the value of Margin is FFS. - Option 2: Further increase [12] dB and the value is FFS - Option 3: Update the requirements with one more P-TRS sample for larger EPRE difference - Other options are no precluded.

Considering different BW and carrier frequency separation from SSB-less SCell and the reference cell, the received power may be large at UE side, especially if the frequency domain separation is large. Some kind of AGC compensation would be executed at UE side. In requirements perspective, the definition on “the compensation for AGC” is suggested to be explicitly specified to guide UE implementation and network deployment.

As per the current requirements, EPRE at UE side focus on the received power per resource element, then the BW difference has almost no impact when using ERPE as unit. The EPRE difference is mainly coming from pathloss. According to pathloss models specified in TS 38.901, typical pathloss is modelled by 20log10(X), X is the ratio of center frequency of the SSB-less operation band pair. Based on the above equation the pathloss difference from two inter-band bands can be simulated and compensated at UE. Taking some UE implementation margin into account, 4dB margin can be added on top of pathloss difference.

Proposal 1: EPRE difference is smaller or equal to [12] dB + |20*log (f1/f2)| - Margin, where f1 and f2 is the center frequency of reference Cell and SSB-less Cell and SSB-less Cell and the margin can be 4dB.

Multiple SSB-less SCells activation

One remaining issue is the SCell activation requirements when UE is configured with more than one SSB-less SCells in one SCell activation command. It is agreed in RAN4#110bis meeting that the single SSB-less activation requirements specified in R18 can be applied to each to-be-activated SSB-less SCell respectively when to-be-activated SSB-less SCells are in different bands [2].

RAN4#110bis Issue 1-1-7: Multiple SSB-less SCells activation Agreement: - When to-be-activated SSB-less SCells are in different bands - Existing single CC requirement can apply to each to-be-activated SSB-less SCells respectively. - The reference cell to the multiple SSB-less Scells in difference bands may or may not be different.

In RAN4#111 meeting, the requirements were discussed when all to-be-activated SCells are SSB-Less cell and these SSB-less SCells are contiguous on the same band [1].

RAN4#111 Issue 1-1-3: Multiple SSB-less SCells activation Agreements: - Also define requirements for following scenario: - When all to-be-activated SCells are SSBless on the same band and SCells are contiguous. - All to-be-activated SCells have same QCL source cell. - P-TRS is used. - Option 1: The multiple SCell activation delay requirements are based on TRS with the shortest periodicity. The detailed requirements can be discussed in the CR. - Option 2: UE activate each to-be-activated SCell based on the TRS on the SCell, and the requirements to be defined accordingly (i.e. single CC delay) - FFS A-TRS based requirements.

When the to-be activated SSB-less SCells are on the same band, option 1 is a kind of further enhancements on the activation delay. As it is a very late stage of the WI, we prefer to define general and minimum requirements to cover both scenarios (i.e., to-be-activated SSB-less SCells are in different bands and are on the same band.)

Proposal 2: When all to-be-activated SSB-less SCells are contiguous in same band, the single SSB-less SCell activation delay can be applied to each to-be-activated SSB-less SCell respectively.

Relation to R15 Intra-band SSB-less operation

In current spec, both R15 intra-band contiguous SSB-less SCell activation delay and R18 inter-band SSB-less SCell activation delay requirements are defined. In RAN4#111 meeting, some discussion are proposed for the UE supporting both R18 inter-band SSB-less and R15 intra-band contiguous SSB-less operation.

The scenario is that there is one intra-band contiguous activated SCell for the to-be-activated SSB-less SCell, and in parallel there is also another inter-band collocated activated SCell for the to-be-activated SSB-less SCell. The controversial point is what’s the QCL configuration under such scenario and what kind of requirement is to be applied.

In RAN4#111 meeting, the agreement is achieved for the case when NO R18 reference cell indication is configured (shown as below [1]). It is still open for the case that when R18 reference cell indication is configured. When R18 reference cell indication is configured, it means that network intend UE to perform inter-band SSB-less operation, otherwise no indication would be signalled. Then the network would configure QCL resources to associate with the inter-band reference SCell. Naturally Rel-18 SSB-less requirement is applied. We don’t think the network would configure with QCL sources to both intra-band contiguous and inter-band Cells. In other words, it is an invalid scenario.

Issue 1-1-5: Relation to R15 intra-band SSB-less Agreement: From RAN4 understanding, for UE supports both R18 inter-band SSB-less and R15 intra-band contiguous SSB-less, - It is not expected that the to-be-activated SCell is configured with QCL source to both intra-band contiguous and inter-band Cells [when the Rel-18 reference cell indication is not configured]. - When R18 reference cell indication is configured, network configure with QCL source to both intra-band contiguous and inter-band Cells, there is no RAN4 agreement on whether Rel-15 or Rel-18 requirement is applied. - The above RAN4 understanding does not intend to trigger any further RAN2 discussion.

Proposal 3: For UE supports both R18 inter-band SSB-less and R15 intra-band contiguous SSB-less, when R18 reference cell indication is configured, network shall configure QCL source to associate with the inter-band Cell.

Proposal 4: For UE supports both R18 inter-band SSB-less and R15 intra-band contiguous SSB-less, when R18 reference cell indication is configured, Rel-18 SSB-less requirement is applied.

NES-based CHO

Regarding NES-based CHO requirements, how to precisely formulate the wording to cover all cases are still under discussion. Some candidate options are proposed in last meeting as captured in summary [3]. RAN2 has relevant discussion in previous meeting as following agreements.

Agreements on CHO Understanding is that “The fulfilment of the event includes the case that the NES mode indication has been received and the entry condition is now fulfilled”. No need to update the specifications No further clarification to continue measurement and CHO evaluation is necessary for the case UE now receives NES mode-enabled indication while NES CHO event is not yet fulfilled, and for the case NES CHO event is now fulfilled and UE has received a NES mode-enabled indication.

There was no sufficient discussion in last meeting, thus we provide the analysis in this meeting, and hope the issue could be concluded. Based on previous RAN4 discussion in previous meeting, all companies are aligned on the UE behavior. The controversial part is how to describe different cases clearly in the requirements. With RAN2 further clarification, the NES-based CHO are triggered when following conditions are met:

Condition 1: NES mode indication has been received.

Condition 2: The entry condition is now fulfilled.

Observation 1: The NES-based CHO is triggered when following conditions are met:

Condition 1: NES mode indication has been received.

Condition 2: The entry condition is now fulfilled.

Based on the current requirements, we didn’t consider the case when condition has been met but not met anymore. It could be divided into following two cases w.r.t whether the DCI is received before or after.

Fig.1 Conditions vs CHO triggering when condition is kept fulfilled.

However, if we want to model the case that condition could be dropped after it is fulfilled. As shown in following Fig.2, the CHO shall be triggered as point A.

Fig.2 Conditions vs CHO triggering when condition is not met anymore.

Based on the discussion during RAN4#110bis meeting, companies proposed to modify the TEvent_DU as follows:

TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until - a condition exists at the measurement reference point which will trigger the NES-based conditional handover, and - the condition keeps existing within Tidentify before UE successfully decodes DCI 2-9 with NES-mode indication

However, it can not handle the case when DCI 2_9 is received earlier as shown in Fig.3

Fig.3 Conditions vs CHO triggering when condition is not met anymore.

Then, one may propose to further modify the TEvent_DU as the delay uncertainty until the condition keeps existing for Tidentify. However, there is still missing case. As shown in following Fig. 4, the condtion has been met for Tidentify from point B to point C. However, the condtion is dropped again. Then based on above modification and current requirements, UE shall be able to trigger NES based CHO at point D. However, the minimum requirements shall be defined based on that UE triggers NES CHO at Point E.

Fig.4 Conditions vs CHO triggering when condition is not met anymore.

Thus, based on the analysis above, if we want to model the procedure clearly, it could be divided into following two cases:

- NES based CHO is triggered when DCI 2-9 with NES-mode indication is decoded when condition keeps existing within Tidentify before UE successfully decodes DCI 2-9 with NES-mode indication

- Otherwise, NES based CHO is triggered until condition keeps existing for Tidentify after DCI 2-9 with NES-mode indication is decoded.

Based on the analysis above, we found it is very difficult to define Tevent_DU based on current framework ignoring the receiving of DCI 2-9.

Observation 2: If we want to model the procedure clearly, it could be divided into following two cases:

- NES based CHO is triggered when DCI 2-9 with NES-mode indication is decoded when condition keeps existing within Tidentify before UE successfully decodes DCI 2-9 with NES-mode indication

- Otherwise, NES based CHO is triggered until condition keeps existing for Tidentify after DCI 2-9 with NES-mode indication is decoded.

Observation 3: It is very difficult to define Tevent_DU based on current framework which ignores the receiving of DCI 2-9.

Thus, if RAN4 would like to consider the complicated case when the condition is variable, we propose to define the NES based CHO as follows, which is quite different from legacy CHO framework:

##### 6.1.4.2.1 Handover delay Procedure delays for all procedures that can command a conditional handover are specified in TS 38.331 [2]. When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within DCHO seconds from the end of the last TTI containing the RRC command. DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution Where: TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [2]. TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until - a condition exists at the measurement reference point which will trigger the conditional handover, or - UE successfully decodes DCI 2-9 with NES-mode indication <<unchanged part>> ##### 6.1.4.2.2 Measurement time The measurement time delay is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts. For conditional intra-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra with index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2. For conditional inter-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify_inter_with_index or Tidentify_inter_without_index defined in clause 9.3.4 or clause 9.3.9. For NES-based conditional intra-frequency handover: - If a condition exists at the measurement reference point which fulfills the conditions for NES-based conditional handover, and it keeps existing for Tidentify_intra_with_index or Tidentify_intra_without_index before UE successfully decodes the DCI 2-9 with NES-mode indication, Tmeasure = 0 - Otherwise, Tmeasure equal to the time span from the end of TEvent_DU until a condition keeps existing for Tidentify_intra_with_index or Tidentify_intra_without_index which can fulfill the NES-based conditional handover.

With above test proposals, the examples given in this paper can be precisely described as follows

Fig.5 Description using new method for difference cases

Proposal 5: Modify the requirements for NES-based CHO as follows:

##### 6.1.4.2.1 Handover delay Procedure delays for all procedures that can command a conditional handover are specified in TS 38.331 [2]. When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within DCHO seconds from the end of the last TTI containing the RRC command. DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution Where: TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [2]. TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until - a condition exists at the measurement reference point which will trigger the conditional handover, or - UE successfully decodes DCI 2-9 with NES-mode indication <<unchanged part>> ##### 6.1.4.2.2 Measurement time The measurement time delay is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts. For conditional intra-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra with index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2. For conditional inter-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify_inter_with_index or Tidentify_inter_without_index defined in clause 9.3.4 or clause 9.3.9. For NES-based conditional intra-frequency handover: - If a condition exists at the measurement reference point which fulfills the conditions for NES-based conditional handover, and it keeps existing for Tidentify_intra_with_index or Tidentify_intra_without_index before UE successfully decodes the DCI 2-9 with NES-mode indication, Tmeasure = 0 - Otherwise, Tmeasure equal to the time span from the end of TEvent_DU until a condition keeps existing for Tidentify_intra_with_index or Tidentify_intra_without_index which can fulfill the NES-based conditional handover.

Conclusions

This contribution provides analysis on maintenance for network energy saving. According to the analysis, we propose the following proposals:

Proposal 1: EPRE difference is smaller or equal to [12] dB + |20*log (f1/f2)| - Margin, where f1 and f2 is the center frequency of reference Cell and SSB-less Cell and SSB-less Cell and the margin can be 4dB.

Proposal 2: When all to-be-activated SSB-less SCells are contiguous in same band, the single SSB-less SCell activation delay can be applied to each to-be-activated SSB-less SCell respectively.

Proposal 3: For UE supports both R18 inter-band SSB-less and R15 intra-band contiguous SSB-less, when R18 reference cell indication is configured, network shall configure QCL source to associate with the inter-band Cell.

Proposal 4: For UE supports both R18 inter-band SSB-less and R15 intra-band contiguous SSB-less, when R18 reference cell indication is configured, Rel-18 SSB-less requirement is applied.

Observation 1: The NES-based CHO is triggered when following conditions are met:

Condition 1: NES mode indication has been received.

Condition 2: The entry condition is now fulfilled.

Observation 2: If we want to model the procedure clearly, it could be divided into following two cases:

- NES based CHO is triggered when DCI 2-9 with NES-mode indication is decoded when condition keeps existing within Tidentify before UE successfully decodes DCI 2-9 with NES-mode indication

- Otherwise, NES based CHO is triggered until condition keeps existing for Tidentify after DCI 2-9 with NES-mode indication is decoded.

Observation 3: It is very difficult to define Tevent_DU based on current framework which ignores the receiving of DCI 2-9.

Proposal 5: Modify the requirements for NES-based CHO as follows:

##### 6.1.4.2.1 Handover delay Procedure delays for all procedures that can command a conditional handover are specified in TS 38.331 [2]. When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within DCHO seconds from the end of the last TTI containing the RRC command. DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution Where: TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [2]. TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until - a condition exists at the measurement reference point which will trigger the conditional handover, or - UE successfully decodes DCI 2-9 with NES-mode indication <<unchanged part>> ##### 6.1.4.2.2 Measurement time The measurement time delay is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts. For conditional intra-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra with index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2. For conditional inter-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify_inter_with_index or Tidentify_inter_without_index defined in clause 9.3.4 or clause 9.3.9. For NES-based conditional intra-frequency handover: - If a condition exists at the measurement reference point which fulfills the conditions for NES-based conditional handover, and it keeps existing for Tidentify_intra_with_index or Tidentify_intra_without_index before UE successfully decodes the DCI 2-9 with NES-mode indication, Tmeasure = 0 - Otherwise, Tmeasure equal to the time span from the end of TEvent_DU until a condition keeps existing for Tidentify_intra_with_index or Tidentify_intra_without_index which can fulfill the NES-based conditional handover.

References

- R4-2410269, WF on RRM requirements for NR network energy saving, Huawei

- R4-2406301, WF on RRM requirements for NR network energy saving, Huawei

- R4-2408023 Topic summary for [111][226] Netw_Energy_NR