3GPP TSG-RAN WG4 Meeting # 112							                                         R4-2412220

Maastricht, Netherlands, 19th – 23rd August, 2024

Title: 					Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Source: 	Huawei, HiSilicon

Agenda item:	8.15.2.2

Document for:	Discussion

Introduction

The discussion on WI NR Radio Resource Management (RRM) Phase 5 [1] was carried out since RAN4#110bis. The objective of the WI is SSB based L3 measurement delay reduction for connected mode is to be discussed since this meeting. The content is duplicated as below [1].

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2

This contribution provides further analysis on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC.

Discussion

Application requirements

In current specification, CSSFoutside gap is applied for CSSFintra, CSSFinter and CSSFinterRAT, where

- CSSFintra is for intra-frequency measurement without gap which is defined since Rel-15;

- CSSFinter is for inter-frequency measurement without gap (either legacy gap or NCSG). Multiple cases are included herein:

  - R16 Inter-frequency measurement without gap where SSB is completely contained in active BWP;

  - R17 NCSG measurement with ‘nogap-noncsg’;

  - R18 NeedForGaps measurement with ‘no-gap-no-interruption’ or with “no-gap-with-interruption”,

- CSSFinterRAT is for inter-RAT measurement without gap if the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoEUTRA for the inter-RAT measurement.

CSSFoutside_gap of the above five cases refer to the same table which is specified in 9.1.5.1. So the enhanced CSSFoutside_gap is applied to these requirements as well.

Proposal 1: L3 measurement delay reduction by optimizing CSSFoutsidegap can be applied in:

- CSSFintra for intra-frequency measurement without gap which is defined since Rel-15;

- CSSFinter for inter-frequency measurement without gap (either legacy gap or NCSG). Multiple cases are included herein:

  - R16 Inter-frequency measurement without gap where SSB is completely contained in active BWP;

  - R17 NCSG measurement with ‘nogap-noncsg’;

  - R18 NeedForGaps measurement with ‘no-gap-no-interruption’ or with “no-gap-with-interruption”,

- CSSFinterRAT for inter-RAT measurement without gap if the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoEUTRA for the inter-RAT measurement.

Application Scenarios

In realistic commercial deployment, FR1+FR2 is an extremely critical and promising one. FR1 can guarantee coverage and FR2 can further increase capacity. So the optimized CSSF shall be considered at least in both SA FR1+FR2 CA and FR1+FR2 DC scenario. As in current TS38.101, there is no FR2+FR2 NRDC configuration, it is no need to consider FR2+FR2 DC scenario.

Proposal 2: L3 measurement delay reduction by optimizing CSSFoutsidegap can be applied in

- SA: FR2+FR2 CA, FR1+FR2 CA

- NRDC:  FR1+FR2 DC

Candidate Solutions

After several meetings of discussion, some directions were proposed by companies for optimizing CSSFoutside gap.

- Direction 1: Reducing the number of frequency layers involved in competing searchers

- Direction 2: Leveraging the searcher used for within gap for outside gap measurement (then total 3 searchers available) under certain scenario

- Direction 3: Optimizing the searcher occupancy ratio of candidate frequency layers

The target of the WI is to figure out a feasible and gainful solution. All directions are deserved to be further studied and discussed and all kinds of UE implementation is not supposed to be precluded. In the following, we provide analysis on each potential direction.

One proposed solution of direction 1 is UE only needs to measure one serving CC per FR2 band and UE is not required to measure SSBs on other serving cells in the same band. As per the existing RAN4 specification (in red font), in FR2, UE would measurement on serving cell for each of the other intra-frequency layer(s) in the same band. The proposed solution is an enhancement of the below requirement in red font.

#### 9.2.3.2 Requirements for FR2 For one single intra-frequency layer in a band, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least: - 6 identified cells, and - 24 SSBs with different SSB index and/or PCI, where this single intra-frequency layer shall be: - PCC when UE is configured with SA NR operation mode with PCC in the band; or - PSCC when UE is configured with EN-DC with PSCC in the band; or - PSCC when UE is configured with NR-DC with PSCC in the band; or - One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation. The UE shall also be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least 2 SSBs on serving cell for each of the other intra-frequency layer(s) in the same band.

As we know servingCellMO is assigned value as a MO index. If servingCellMO is indicated for a SCell, the SCC should be first configured as a MO. If we are planning not to perform on other serving cells in the same band, network can directly not configure MO associated with the SCC. On the other hand, if network configures a SCC on other serving cell as a MO and servingCellMO, it means that network is willing to know the measurement results of the indicated frequency layer. It is not reasonable that UE refuse to perform measurement. In summary, if network want to know the information of other serving cell, it would configure servingCellMO on the SCell, otherwise no MO is configured on the other intra-frequency layers in the same band. Therefore we think the proposed solution can be implemented by network configuration.

Another proposed solution of direction 1 is UE only measures one FR2 carriers among the layers which are competing searcher, for instance, NSCC_SSB, Y, Z in Table 9.1.5.1.2-1.

- NSCC_SSB is for intra-frequency measurement without gap;

- Y is for inter-frequency measurement without gap (either legacy gap or NCSG) and includes the below scenarios:

  - R16 Inter-frequency measurement without gap where SSB is completely contained in active BWP;

  - R17 NCSG measurement with ‘nogap-noncsg’;

  - R18 NeedForGaps measurement with ‘no-gap-no-interruption’ or with “no-gap-with-interruption”,

- Z is for inter-RAT measurement without gap if the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoEUTRA for the inter-RAT measurement.

Table 9.1.5.1.2-1: CSSFoutside_gap,i scaling factor for SA mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSF outside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap | CSSFoutside_gap,i for E-UTRA inter-RAT MO with no measurement gap [TBD] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 only CA | 1+NPCC_CSIRS + NPCC_CCA_RSSI/CO | NSCC_SSB +Y+Z+2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO | N/A | N/A | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO | NSCC_SSB +Y+Z+2x NSCC_CSIRS |
| FR2 only intra band CA | N/A | N/A | 1+NPCC_CSIRS | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS |
| FR2 only inter band CA | N/A | N/A | 1+NPCC_CSIRS | 2*(1+ NSCC_CSIRS_FR2_NCM) Note 3,5 | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) |
| FR1 +FR2 CA (FR1 PCell) Note 1 | 1+NPCC_CSIRS | 2×( NSCC_SSB +Y+Z+2* NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | N/A | 2x(1+ NSCC_CSIRS_FR2_NCM) Note 3,5 | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) | 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM) |
| FR1 +FR2 CA (FR2 PCell) Note 1 | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS | 1+NPCC_CSIRS | N/A | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS | NSCC_SSB +Y+Z+2x NSCC_CSIRS |
| Note 1: Only one FR1 operating band and one FR2 operating band are included for FR1+FR2 inter-band CA. Note 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. Note 3: CSSFoutside_gap,i =1 if only one SCell is configured and no inter-frequency MO without gap and only SSB based L3 measurement is configured on SCC; CSSFoutside_gap,i =2 if only one SCell is configured and no inter-frequency MO without gap and either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement is configured on SCC. Note 4: Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG; otherwise, it is 0. Note 5: Only two NR FR2 operating bands are included for FR2 inter-band CA. Note 6: NPCC_CSIRS=1 if PCC is with either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement configured; otherwise, NPCC_CSIRS =0. Note 7: NSCC_CSIRS=Number of configured SCell(s) with either both SSB and CSI-RS based L3 measurement configured or only CSI-RS based L3 measurement configured Note 8: NSCC_CSIRS_FR2_NCM=1 if FR2 SCC, where neighbour cell measurement is required, is with either both SSB and CSI-RS configured or only CSI-RS measurement configured; otherwise, NSCC_CSIRS_FR2_NCM=0. Note 9: NSCC_SSB=Number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG. Note 10: NPCC_CCA_RSSI/CO= 1 if PSCC is configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping; NSCC_CCA_RSSI/CO = Number of MOs for SCell(s) configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping. Note 11: Z is the number of configured E-UTRA inter-RAT MOs without MG that are being measured outside of MG; otherwise, it is 0. |  |  |  |  |  |  |  |

In essence, NSCC_SSB, Y, Z are the number of configured (Measurement Object) MOs (intra-f, inter-f or inter-RAT). It is not clear to us what criteria UE shall follow to select one among multiple MOs. However it is easier for network to control. If network think the measurement result on one MO is sufficient, network can configure only one MO on the FR2 band. Therefore we think the proposed solution can be implemented by network configuration as well.

Proposal 3: The candidate solutions of optimizing CSSF (Reducing the number of frequency layers involved in competing searchers) are network implementation.

The proposed solution of direction 2 is regarding three searchers. Herein an example in figure 1 is provided where in FR1+FR2 scenario, two CCs (e.g., PCC and SCC) in FR1 are to be measured outside gap, and at the same time one inter-frequency layer in FR2 is to be measured within gap. For a UE which supports per-FR gap, as per the existing requirements, intra-frequency outside measurement with CSSFoutsidegap_PCC =1 and CSSFoutsidegap_SCC=1 are performed and at the same time the inter-frequency with gap measure is performed. It means that UE shall be able to measure these three frequency layers simultaneously.

Figure 1. CSSF for within gap and for outside gap for UE supporting per FR gap

In essence, in above scenario, a UE is supposed to be equipped with 3 searchers in order to meet the legacy requirements for measurement within gap in FR2 and outside gap in FR1 respectively, otherwise the legacy measurement requirements can not be satisfied.

Observation 1: In order to meet the legacy requirements for measurement within gap in FR2 and outside gap in FR1, a UE is supposed to be equipped with 3 searchers.

For the same UE, if three to-be-measured CCs are outside gap and no gap is needed, as per the existing requirements where 2 searchers are assumed:

- one dedicated searcher (searcher #1) is allocated to PCC, then CSSF for PCC=1,

- the other two SCCs share the other searcher (searcher #2), then CSSF for SCC1=2, CSSF for SCC2=2

However for the UE, as no gap is needed, the searcher used for within gap in Figure 1 can be leveraged for outside gap. Therefore total 3 searchers are used for the measurement outside gap. Then CSSF is well optimized and shorter measurement delay for serving cell measurement can be achieved:

- one dedicated searcher (searcher #1) is allocated to PCC, then CSSF for PCC=1,

- the other two searchers (searcher #2 and #3) are allocated for SCC1 and SCC2, then CSSF for SCC1=1, CSSF for SCC2=1

Figure 2. CSSF for outside gap for UE supporting per FR gap

In our understanding as per current requirements, three searchers capability is naturally supported by UE who supports per FR gap. No any enhanced/additional capability is required. However during the offline discussion, there are different UE product specification due to storage, processing restriction. So we are open to further discuss the side conditions of the solution and whether a new capability (rather than reusing per FR capability) is needed.

Proposal 4: For UE supporting per FR gap, when all MOs are to be measured outside gap, the searcher used for within gap can be leveraged for outside gap. CSSFoutsidegap can be optimized, as these MOs can share totally three searchers.

Proposal 5: We are open to further discuss the applicable condition of the three searcher solution and whether a new capability is needed.

The proposed solution of direction 3 is to optimize the searcher occupancy ratio of candidate frequency layers. Until now no detailed solution is observed. As we know PCC is more important for mobility.  So one separate searcher in the existing requirements is allocated to PCell is reasonable. For the other MOs without gap (e.g., SCC, inter-f MO without gap or NCSG, and inter-RAT MO without gap), we are open to further discuss the conditions in which measurement on other MOs can reduce the searcher occupancy ratio.

Proposal 6: More details need to be revealed of the solution of optimizing the searcher occupancy ratio of candidate frequency layer.

Conclusions

This contribution provides some considerations on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC. According to the analysis, we propose the following proposals:

Proposal 1: L3 measurement delay reduction by optimizing CSSFoutsidegap can be applied in:

- CSSFintra for intra-frequency measurement without gap which is defined since Rel-15;

- CSSFinter for inter-frequency measurement without gap (either legacy gap or NCSG). Multiple cases are included herein:

  - R16 Inter-frequency measurement without gap where SSB is completely contained in active BWP;

  - R17 NCSG measurement with ‘nogap-noncsg’;

  - R18 NeedForGaps measurement with ‘no-gap-no-interruption’ or with “no-gap-with-interruption”,

- CSSFinterRAT for inter-RAT measurement without gap if the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoEUTRA for the inter-RAT measurement.

Proposal 2: L3 measurement delay reduction by optimizing CSSFoutsidegap can be applied in

- SA: FR2+FR2 CA, FR1+FR2 CA

- NRDC:  FR1+FR2 DC

Proposal 3: The candidate solutions of optimizing CSSF (Reducing the number of frequency layers involved in competing searchers) are network implementation.

Observation 1: In order to meet the legacy requirements for measurement within gap in FR2 and outside gap in FR1, a UE is supposed to be equipped with 3 searchers.

Proposal 4: For UE supporting per FR gap, when all MOs are to be measured outside gap, the searcher used for within gap can be leveraged for outside gap. CSSFoutsidegap can be optimized, as these MOs can share totally three searchers.

Proposal 5: We are open to further discuss the applicable condition of the three searcher solution and whether a new capability is needed.

Proposal 6: More details need to be revealed of the solution of optimizing the searcher occupancy ratio of candidate frequency layer.

References

- RP-240830, New WID: WI resulting from discussion on RRM enhancements

- R4-2410260, Way Forward for [111][228] NR_RRM_Ph5, Apple