3GPP TSG-RAN WG4 Meeting #112	R4-2412221

Maastricht, Netherlands, 19th – 23rd August, 2024

Title: 	Discussion on RRM requirements for supporting intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s)

Source: 	Huawei, HiSilicon

Agenda item:	8.5.3

Document for:	Discussion

Introduction

Supporting of intra-band non-collocated EN-DC/NR-CA deployment for Type 2 UE has been completed in Rel-18. The requirements for FR1 inter-band non-contiguous EN-DC with overlapping or partially overlapping bands and FR1 intra-band non-contiguous NR-CA for Type 2 UE were specified.

The new R19 WI is to further support intra-band non-collocated EN-DC/NR-CA deployment scenarios for Type 4 FWA UE. The objectives of the WID are duplicated as below [1].

- 4.1 Objective of SI or Core part WI or Testing part WI The core part of the work item includes: - Specify the core requirements for type 4a/4b capable FWA UE that supports non-co-located scenarios for FR1 inter-band non-contiguous EN-DC with overlapping or partially overlapping bands and/or FR1 intra-band non-contiguous NR-CA for up to 4-layer DL MIMO per CC, where MRTD>CP is assumed. - RF power imbalance requirements of up to 25dB - RRM requirements including MTTD and MRTD requirements and other identified requirement such as scheduling restriction/availability, SCell operation related delay and interruption. - Work is limited to EN-DC/NR-CA for bands 42, n77/n78 - Discuss and if needed, specify the UE behavior when the UE supports type 4a/b but operates in co-located scenarios depending on whether network signaling is provided. R18 handling of UE behavior can be used as a reference.

The contribution provides some analysis for supporting intra-band non-collocated EN-DC/NR-CA deployment scenarios from RRM requirements perspective.

Discussion

Impact on RRM requirements

UE types for non-collocated non-contiguous intra-band NR-CA and inter-band EN-DC are defined in R4-2217734 [2]. The assumed UE architectures for Type 2 and Type 4a/4b are listed in Table 1.

Table 1: UE Types for 4-layer MIMO case [2]

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | Frequency Separation between 2cc | NRCA/ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 shared |  | 4 shared | 4 shared | 4Rx | ≤ X MHz | NRCA,ENDC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 |  |  |  |  | 4Rx |  |  |  |  |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 3a | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 2 | 2 | 2Rx |  |  |  |  |
| 3b | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 4 | 4 | 4Rx |  |  |  |  |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 4 |  | 4 | 4 | 4Rx |  |  |  |  |

Type 4a/4b UE has separated RF chain for each DL CC. The difference between the type 4a and type 4b lies in fact that the former one supports up to 2 DL MIMO layers in one of the CCs and the latter one supports to 4 DL MIMO layers on both CCs. It is observed that the RF chains of Type 4 UE are not shared on different CCs. Similar as Type 2 UE, the existing MRTD/MTTD requirements defined in 38.133 for inter-band case can be reused for Type 4 UE.

Proposal 1: R18 MRTD/MTTD requirements defined for Type 2 UE in non-co-located scenarios can be reused for Type 4 capable FWA UE, that is

- MRTD/MTTD requirements for non-collocated FR1 inter-band synchronous EN-DC with overlapping DL bands for Type 4 UE are

- MRTD=33us

- MTTD=35.21us

- MRTD/MTTD requirements for non-collocated FR1 intra-band non-contiguous NR-CA for Type 4 UE are

- MRTD=33us

- MTTD=34.6us

In R18 intra-band non-collocated WI, besides MRTD and MTTD, the requirements including scheduling restriction, interruption, SCell activation delay and BFD/CBD are impacted by Type 2 UE as well. Taking scheduling restriction during BFD as an example, two cases for Type 2 UE are considered. One case (Case#1) is when nonCollocatedTypeNR-CA-r18 is not provided, and the other case (Case#2) is when the IE is provided. In Case #1, UE operates as a Type 2 UE in non-collocated scenario. Case #2 is a collocated scenario, and UE falls back to Type 1 UE capability. The scheduling restriction are different in two cases (see below).

TS 38.133 When intra-band non-contiguous carrier aggregation is configured for a UE indicating intraBandNR-CA-non-collocated-r18 and if nonCollocatedTypeNR-CA-r18 is not provided, there are no scheduling restrictions on FR1 serving cell(s) configured on the non-contiguous CC(s) in the same band. Otherwise, the scheduling restrictions on one serving cell apply to all other serving cells in the same band on the symbols that fully or partially overlap with the restricted symbols if nonCollocatedTypeNR-CA-r18 is provided.

In R19 intra-band non-collocated WI for Type 4 UE, RF session has discussed for two meeting cycles earlier than RRM session. Some progress/agreements in RF session are achieved in [3]. In RF it is agreed that when UE with Type 4 capability operates in collocated scenario, it can fall back to Type 1 with 8 MIMO layer..

< Issue 2-3-2: UE having Type 4 capabilities in non-collocated/collocated scenarios > Agreement: - UE having Type 4 capability support the following capabilities (Set 1) when operating in non-collocated scenarios. - Maximum 4 MIMO layers on each CC - Maximum power imbalance of 25dB - UE having Type 4 capability support the following capabilities (Set 2=Type 1 capability) when operating in collocated scenarios. - Maximum 8 MIMO layers on each CC - It may be optionally supported - Maximum PSD imbalance of 6dB

In RF session, some additional fallback scenarios are still under discussion, e.g., “Type 4 UE falls back to type 2” or “Type 4 UE falls back to Type 1 with 4Rx”. However from RRM perspective, we think the following principle is supposed to be followed:

- when UE operates with separate RF chain (in “ Type 4” or “ fall back to Type 2”(not decided)) in non-collocated scenario, the corresponding requirements defined for type 2 UE in non-collocated scenario in R18  can be reused,

- when UE operates with shared RF chains (“ fall back to Type 1 with 4 Rx (not decided)” or “ fall back to Type 1 with 8 Rx”) in collocated scenario, the corresponding requirements defined for type 2 UE in collocated scenario in R18 can be reused.

As the discussion on Type 4 UE capability and new BS signalling are ongoing, the related requirements description for Type 2 UE defined in R18 may need corresponding update as per discussion progress.

Proposal 2: The following principle is supposed to be followed:

- when UE operates with separate RF chain (in “ Type 4” or “ Type4 fall back to Type 2 (not decided yet)”) in non-collocated scenario, the requirements (including scheduling restriction, interruption, SCell activation delay) defined for type 2 UE in non-collocated scenario in R18 can be applied;

- when UE operates with shared RF chains (“ fall back to Type 1 with 4 Rx(not decided yet)” or “fall back to Type 1 with 8 Rx”) in collocated scenario, the requirements (including scheduling restriction, interruption, SCell activation delay) defined for type 2 UE in collocated scenario in R18 can be applied.

Regarding the requirements of SCell BFD/CBD, in non-collocated scenario, the channel quality on one serving cell may be different with that of the other non-collocated serving cell in the same band. So the number of serving cells per band for Type 4 UE operating in non-collocated scenario is up to 2 which is the same as Type 2 UE.

For UE supporting intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided for the configured FR1 intra-band non-contiguous CA, the requirements in this clause apply when UE is required to perform beam failure detection on no more than 2 serving cells per band if these 2 serving cells are in non-contiguous carriers, and no more than 1 serving cell per band otherwise.

Proposal 3: Type 4 UE when operating in non-collocated scenario can be configured with BFD/CBD measurements on up to two serving cells in the same band.

UE Capability/UE behavior and network signaling for Type 4 EN-DC/NR-CA

In R18 non-collocated WI, two new BS signalling are introduced, i.e., nonCollocatedTypeMRDC or nonCollocatedTypeNR-CA. When nonCollocatedTypeMRDC or nonCollocatedTypeNR-CA is indicated, it implies that UE is operating in collocated scenario and it can fall back to Type 1 UE with shared LNA. When nonCollocatedTypeMRDC and nonCollocatedTypeNR-CA are NOT indicated, it implies that UE is operating in NON-collocated scenario and UE is operating in Type 2 UE with separate RF chain for each CC.

nonCollocatedTypeMRDC This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands. If this field is present, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [14] and inter-band RF requirements. If this field is absent, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [14] and inter-band RF requirements when indicating support of interBandMRDC-WithOverlapDL-Bands-r16. nonCollocatedTypeNR-CA This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD intra-band NRCA. If this field is present, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15]. If this field is absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band noncollocated NR-CA including 7.10A in TS 38.101-1 [15] when indicating support of intraBandNR-CA-non-collocated-r18.

As the R18 BS signaling is used to indicate UE for switching between “separate RF chain” implementation and “shared RF chain” implementation”, the similar principle can be inherited for R19. In R19 when Type 4 UE fallback to Type 1 with 8RX, either a new BS signalling (e.g., nonCollocatedTypeNR-CA-r19) or extend existing nonCollocatedTypeNR-CA-r18 to make it applicable for UEs configured with maxMIMO-Layers ≤ 4 can work. RAN2 can decide which scheme shall be adopted.

Proposal 4: For type 4 UE fallback between non-collocated scenario and collocated scenario, either a new BS signalling (e.g., nonCollocatedTypeNR-CA-r19) or extend existing nonCollocatedTypeNR-CA-r18 to make it applicable for UEs configured with maxMIMO-Layers ≤ 4 can work. RAN2 can decide which scheme shall be adopted.

For Type 4 UE, if UE is in non-collocated scenario (e.g., nonCollocatedTypeNR-CA-r18 is not provided, assuming R18 signalling is reused), UE shall work with separate RF chains. Then there are two possible cases:

- State 1-1: UE works with Type 4 capability (max 4 MIMO layers/CC),

- State 1-2: UE fall back to Type 2 capability (max 2MIMO layers /CC).

In RF session, the below agreement was achieved at RAN4#111. Type 2 capability is the by default capability for Type 4 capable UE.

< Issue 2-2-2: Whether to support Type 2 capability by UE having Type 4 capabilities > Agreement: - If UE reports the Type 4 capability, Type 2 capability shall be deemed as support by default regardless of whether UE indicates Type-2 capability or not. - If UE reports the Type 4 capability, it is not necessary for UE to report Type 2 capabilities. - If the issue is identified for these bullets, RAN4 will revisit them.

As we know the intention of “state1-2” is for power saving. As type 4 UE is FWA UE, power consumption is not a bottleneck. Moreover there are alternatives methods to avoid more power assumption. For example, network can deactivate one CC and UE enters to a lower power mode with single carrier. To some extent, UE in state 1-2 is not very necessary.

Observation 1: In non-collocated scenario, Type 4 UE falling back to Type 2 capability is not very necessary.

If much more benefit of state 1-2 than single carrier state is observed, we are also fine to keep it. However we don’t think it is necessary to introduce an additional new BS signaling indication for the fallback. As we know, the max MIMO layer adaption can be performed by BWP switching procedure which is supported in R16. In DL, there are at most four BWPs, and max MIMO layer number is configured on each BWP. Figure 1 depicts MIMO layer adaption in different BWP. When network indicates UE to switch from BWP1 to BWP2 by MAC CE, UE can turn off 2 Rx antenna and fall back to Type 2 UE to save power. Therefore there is no additional new BS indication is needed.

Observation 2: Type 4 UE can fall back to Type 2 UE through BWP switch procedure which is supported in R16, if falling back to Type 2 is needed.

Figure 1. Max MIMO layer adaption

One may raise concern that whether UE enters to Type 1 with 2RX mode upon receiving the BWP switch command. We think there is no any ambiguous, because the prerequisite of the scenario is that UE is in non-collocated scenario, UE shall work with separate RF chains and would not enter to Type 1 capability.

Proposal 5: In non-collocated scenario, UE is operating with separate chains (Type 4 or Type 2 UE). There is no need to introduce additional new BS signalling to enable Type 4 UE fallback to Type 2 UE.

For Type 4 UE, if UE is in collocated scenario (e.g., if nonCollocatedTypeNR-CA-r18 is provided), UE would change its hardware to use shared RF chains. Then there are also two possible cases with shared RF chain:

- State 2-1: Type 4 fall back to Type 1 with 8RX (max 8 MIMO layers/CC),

- State 2-2: Type 4 fall back to Type 1 with 4Rx(max 4 MIMO layers/CC)

For Type 4 UE, the benefit of fallback to state 2-1 is for throughput improving and the benefit of fallback to state 2-2 is for power saving.

The analysis is similar as Proposal 4. If nonCollocatedTypeNR-CA-r18/r19 is provided, it means that UE now in collocated scenario. If network indicates Type 4 UE to switch from BWP1 to BWP3 (in Figure 1), it means that UE would fallback to state 2-1. If no BWP switch command is received (i.e., UE stays at BPW1), it means that UE would fallback to state 2-2. Therefore no additional new BS signalling is needed for this kind of fallback.

Proposal 6: In collocated scenario, UE is operating with shared chains (Type 1 with 4 Rx or Type 1 with 8RX). There is no need to introduce additional new BS signalling to distinguish Type 4 UE fallback to “Type 1 with 4RX” or “Type 1 with 8RX”.

The above UE states operating in non-collocated and collocated scenarios are summarized as below:

In non-collocated scenario (e.g., if nonCollocatedTypeNR-CA-r18 is NOT provided)

- State 1-1: Type 4 capability (max 4 MIMO layers/CC)

- State 1-2: Type 2 capability (max 2MIMO layers /CC)

In collocated scenario (e.g., if nonCollocatedTypeNR-CA-r18 is provided)

- State 2-1: Type 4 fall back to Type 1 with 8RX (max 8 MIMO layers/CC)

- State 2-2: Type 4 fall back to Type 1 (max 4 MIMO layers/CC)

The state switching can be implemented by R16 BWP switch procedure ( in Table 1 and Table 2). With the existing R18 BS signalling, both BS and UE have clear understanding on which state UE shall work in.

Table 1. Capability UE operating and BWP# in non-collocated scenario

| Non-collocated | State 1-1(Type 4) | State 1-2 (Type 2) |
| --- | --- | --- |
| Indicate UE work on BPW# | BWP1 | BWP 2 |

Table 2. Capability UE operating and BWP# in collocated scenario

| Collocated | State 2-1(Type 1 with 8Rx) | State 2-2 (Type 1 with 4Rx) |
| --- | --- | --- |
| Indicate UE work on BPW# | BWP3 | BWP 1 |

Conclusions

In this paper we provided our analysis on RRM requirements for supporting intra-band non-collocated EN-DC/NR-CA deployment for Type 4 UE. The following proposals are provided.

Proposal 1: R18 MRTD/MTTD requirements defined for Type 2 UE in non-co-located scenarios can be reused for Type 4 capable FWA UE, that is

- MRTD/MTTD requirements for non-collocated FR1 inter-band synchronous EN-DC with overlapping DL bands for Type 4 UE are

- MRTD=33us

- MTTD=35.21us

- MRTD/MTTD requirements for non-collocated FR1 intra-band non-contiguous NR-CA for Type 4 UE are

- MRTD=33us

- MTTD=34.6us

Proposal 2: The following principle is supposed to be followed:

- when UE operates with separate RF chain (in “ Type 4” or “ Type4 fall back to Type 2 (not decided yet)”) in non-collocated scenario, the requirements (including scheduling restriction, interruption, SCell activation delay) defined for type 2 UE in non-collocated scenario in R18 can be applied;

- when UE operates with shared RF chains (“ fall back to Type 1 with 4 Rx(not decided yet)” or “fall back to Type 1 with 8 Rx”) in collocated scenario, the requirements (including scheduling restriction, interruption, SCell activation delay) defined for type 2 UE in collocated scenario in R18 can be applied.

Proposal 3: Type 4 UE when operating in non-collocated scenario can be configured with BFD/CBD measurements on up to two serving cells in the same band.

Proposal 4: For type 4 UE fallback between non-collocated scenario and collocated scenario, either a new BS signalling (e.g., nonCollocatedTypeNR-CA-r19) or extend existing nonCollocatedTypeNR-CA-r18 to make it applicable for UEs configured with maxMIMO-Layers ≤ 4 can work. RAN2 can decide which scheme shall be adopted.

Observation 1: In non-collocated scenario, Type 4 UE falling back to Type 2 capability is not very necessary.

Observation 2: Type 4 UE can fall back to Type 2 UE through BWP switch procedure which is supported in R16, if falling back to Type 2 is needed.

Proposal 5: In non-collocated scenario, UE is operating with separate chains (Type 4 or Type 2 UE). There is no need to introduce additional new BS signalling to enable Type 4 UE fallback to Type 2 UE.

Proposal 6: In collocated scenario, UE is operating with shared chains (Type 1 with 4 Rx or Type 1 with 8RX). There is no need to introduce additional new BS signalling to distinguish Type 4 UE fallback to “Type 1 with 4RX” or “Type 1 with 8RX”.

Reference

- RP-240838, New WID on Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s)

- R4-2217734, WF on NonCol_intraB_ENDC_NR_CA for New Type UE, KDDI