3GPP TSG-RAN WG4 Meeting # 112	R4-2412222

Maastricht, Netherlands, 19th – 23rd August, 2024

Title: 	Discussion on NR mobility enhancements Phase 4

Source: 	Huawei, HiSilicon

Agenda item:	8.23.2

Document for:	Discussion

Introduction

R19 mobility WI was agreed in [1]. The objectives are duplicated as below.

- 4.1 Objective of SI or Core part WI or Testing part WI - Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - As secondary priority, support the case when NR-DC is configured and CU is acting as SN and MCG is unchanged - As secondary priority, support the case when NR-DC is configured, CU is acting as MN and SCG is unchanged or SCG is released - Note: The case that LTM is configured in both MCG and SCG is excluded - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1] - Specify support of conditional LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Prioritise intra-CU LTM - Checkpoint to review objective at RAN#105. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

This contribution provides preliminary analysis on NR_Mob_Ph4 from RRM requirement perspective.

Discussion

The first objective is to enable LTM operation between cells of different gNBs (i.e. inter-CU).  This objective is regarding high layer interaction and has no RRM impact.

Event trigger reporting

So far only periodic reporting, semi-persistent reporting and aperiodic reporting are specified in measurement reporting requirements, as event triggering of measurement results is not supported by L1 measurement in R18 LTM. The R19 mobility enhancement is to support event triggered L1 measurement reporting. There are some progress in RAN2 during the last 2 meetings.

Agreements on measurements:

- L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signaling.

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

Therefore the “Event Triggered Reporting” would be introduced in section 9.14.3 and section 9.15.3 for LTM L1-RSRP measurement requirements in R19.

Proposal 1: “Event Triggered Reporting” for LTM L1 measurement is to be introduced in section 9.14.3 and section 9.15.3.

CSI-RS measurements for LTM procedures

Regarding the objective of supporting for CSI-RS measurements for LTM procedures, CSI-RS based L1 measurement of neighbour cell would impact on RRM requirements. According to WID, the CSI-RS measurement is at least for beam management. Other CSI-RS measurement operations depend on RAN1 progress. RAN4 can first carry out work on early CSI-RS based L1 measurement on candidate cells before LTM cell switch. Before go to detailed requirements discussion, RAN4 shall first have unified understanding on the procedure of CSI-RS based L1 measurement for candidate cells.

For FR1, the procedure is straight forward, as UE in FR1 has omnidirectional RX reception. UE first performs SSB based L3 measurement on the neighbour cells. Through SSB based L3 measurement, the coarse timing is obtained, the UE can correctly receive configured CSI-RS resource of candidate neighbour cells, and perform CSI-RS based L1-RSRP measurements.

Proposal 2: In FR1, UE shall first perform SSB based L3 measurement on candidate neighbour cells. Then UE performs configured CSI-RS L1 measurement on these candidate neighbour cells.

In FR2, it is a bit complicated. It may have two possible options for UE performing CSI-RS based L1 measurement on neighbour cells.

Option 1: Firstly UE still needs to perform SSB based L3 measurements and acquire SSB index information on the candidate cells. In our understanding, it is most likely that CSI-RS resources for LTM measurement are to be associated with SSB (RAN1 is responsible for the discussion). UE can acquire SSB index through SSB L3 measurement. And then, one option is that UE receives the configured CSI-RS resources and perform CSI-RS measurement with Rx beam sweeping. As UE has identified the associated SSB, UE can sweep Rx beam with CSI-RS resources within a limited angle, and further achieve the CSI-RS level finer Rx beam.

Option 2: Another option is inherited from R15 CSI-RS based L1 measurement on serving cell. In R15, Rx beam sweeping is not applied to CSI-RS based L1 measurement for beam management when repetition is OFF.

TS38.133 section 9.5.4.2 For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply if qcl-InfoPeriodicCSI-RS is configured for all the resources in the resource set and for each resource one RS has QCL-TypeD with - SSB for L1-RSRP measurement, or - another CSI-RS in resource set configured with repetition ON. - For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set) for Table 9.5.4.2-2, where Nres_per_set is number of resources in the resource set. The requirements apply provided qcl-InfoPeriodicCSI-RS is configured with QCL-TypeD for all resources in the resource set.

If the above principle is followed, UE would not sweep the RX beam for CSI-RS resources. For CSI-RS based L1 measurement on candidate neighbour cell, UE needs to perform SSB based L3 measurement and acquires the SSB index information, and then UE performs SSB based L1 measurement to refine the RX beam. It shall be noted that the SSB based L1 measurement on candidate cells is the same as R18 LTM where UE shall sweep RX beam based on SSB. Afterwards UE perform measurements on the configured CSI-RS resources of candidate cells where each CSI-RS resource is QCL-typeD with SSB for L1-RSRP measurement. Herein UE performs L1 CSI-RS based measurement without RX beam sweeping.

The above two options lead to different UE implementation and requirements definition. RAN4 shall decide which direction would go with.

Proposal 3: For L1 CSI-RS measurement on candidate neighbour cells in FR2, RAN4 needs to further discuss two possible options:

Option 1: UE performs SSB based L3 measurements and acquires SSB index information of the candidate cells. And then UE receives the configured CSI-RS resources and perform L1 CSI-RS measurement with Rx beam sweeping on candidate cells.

Option 2: UE performs SSB based L3 measurements and acquires SSB index information of the candidate cells. And then UE performs SSB based L1 measurement to refine the RX beam (with RX beam sweeping) which is the same as R18 LTM. Afterwards UE perform measurements without RX beam sweeping on the configured CSI-RS resources of candidate cells where each CSI-RS resource is QCL-typeD with SSB for L1-RSRP measurement.

Conditional LTM

For the third objective, according to RANP timeline, RAN WG does not start the work of conditional LTM before RAN#105.

Proposal 4: The discussion on the requirements of conditional LTM in RAN4 does not start before RANP#105.

Conclusion

The contribution provides preliminary analysis on NR_Mob_Ph4 from RRM requirement perspective. The following proposals are proposed:

Proposal 1: “Event Triggered Reporting” for LTM L1 measurement is to be introduced in section 9.14.3 and section 9.15.3.

Proposal 2: In FR1, UE shall first perform SSB based L3 measurement on candidate neighbour cells. Then UE performs configured CSI-RS L1 measurement on these candidate neighbour cells.

Proposal 3: For L1 CSI-RS measurement on candidate neighbour cells in FR2, RAN4 needs to further discuss two possible options:

Option 1: UE performs SSB based L3 measurements and acquires SSB index information of the candidate cells. And then UE receives the configured CSI-RS resources and perform L1 CSI-RS measurement with Rx beam sweeping on candidate cells.

Option 2: UE performs SSB based L3 measurements and acquires SSB index information of the candidate cells. And then UE performs SSB based L1 measurement to refine the RX beam (with RX beam sweeping) which is the same as R18 LTM. Afterwards UE perform measurements without RX beam sweeping on the configured CSI-RS resources of candidate cells where each CSI-RS resource is QCL-typeD with SSB for L1-RSRP measurement.

Proposal 4: The discussion on the requirements of conditional LTM in RAN4 does not start before RANP#105.

Reference

- R4-2406502, WF on NR mobility enhancements (part 1), MediaTek

- R4-2406513, Draft Big CR to TS 38.133 on core requirement maintenance for Further NR mobility enhancements, MediaTek Inc, Apple