3GPP TSG-RAN WG4 Meeting #112	R4-2412223

Maastricht, Netherlands, 19th – 23rd August, 2024

Title: 	Discussion on UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4

Source: 	Huawei, HiSilicon

Agenda item:	8.1.2

Document for:	Discussion

Introduction

For R19 WID on UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, one of the objectives is to enable 6Rx in FR1 [1]. In this contribution, we will provide the discussion on the possible RRM impact from SRS antenna switching and RLM/BFD test cases perspective for 6Rx capable UEs.

Discussion

R19 NR_ENDC_RF_Ph4 work item includes three objectives: High power UE (HPUE) for CA in terrestrial network (TN), Power boosting and/or MPR reduction, 6Rx for handheld and FWA UE. The first two objectives have no impact on RRM requirements. The contribution would provide analysis on the third objective regarding 6Rx which is duplicated as below from RRM perspective.

6Rx for handheld and FWA UE - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

SRS antenna switching

In Rel-17, the interruption requirements at NR SRS antenna port switching are defined for the following two scenarios:

- Scenario 1: when X=1 SRS symbol is configured in a slot for SRS antenna port switching

- Scenario 2: otherwise, using X=6 SRS symbols in a slot as assumption of SRS transmission time

Symbol level interruption requirement is considered for scenario 1 for synchronized case. For all other cases, interruption requirement is defined in number of slots. In our understanding the current interruption requirements for SRS antenna port switching are generally applicable to xTyR (y>=x) UE which supports supportedSRS-TxPortSwitch capability. Therefore the current interruption requirements at SRS antenna switching are also applicable to 6RX capable UE. Moreover as the requirements are general, no specification impact due to 6Rx is observed.

There are some enhancements in SRS antenna switching in earlier release, e.g., SRS starting in any symbol within a slot is supported since Rel-16, and 8, 10, 12 and 14 consecutive repetition symbols in one slot in one SRS resource since Rel-17, etc. However we don’t think defining new requirements for these further enhancements shall take place in the 6Rx work item. If the requirements are defined for enhanced SRS antenna switching in this WI, it means that the requirements are only applied for specific 6Rx capable UE. If companies have interest on specifying interruption for enhanced SRS antenna switching, a new work item is required and the requirements shall be generally applied for xTyR UE.

Proposal 1: The existing interruption requirements at SRS antenna switching are applicable to 6RX capable UE, and no specification impact is observed.

RLM/BFD test cases for 6Rx capable UEs

As usual, we think there is no impact on core requirements of RLM/BFD due to 6Rx capable UE. The following discussion will focus on performance requirements. It may be a bit earlier to carry out the discussion on performance, so the discussion can be regarded as a warm up.

All the RRM test cases defined in section A.4 and A.6 for FR1 are configured with two Rx antenna ports for UEs supporting 2RX. For RLM and BFD test cases, most of the test parameters are the same for different Rx capable UEs. But the SNR levels configured for RLM and BFD tests are derived from the assumed values of Qin/Qout/Qout_LR which are impacted by the number of Rx antenna ports.

The antenna connection method for applying 2RX tests to UEs supporting 4RX antenna ports is specified in section A.3.6.1. In section A.3.6.1, the testing principles and the modified SNR levels for RLM and BFD tests are specified for 4Rx capable UEs. 4RX capable UE supporting 4RX on some but not all bands is only required to be tested on 2RX band with 2 RX antenna connection. 4RX capable UEs supporting 4RX on all bands is required to be tested on any one of 4RX band with 4 RX antenna connection, and the SNR levels of RLM and BFD test cases need to be modified according to table A.3.6.1.1.2.1-1, table A.3.6.1.1.2.1-2 and table A.3.6.1.1.2.1-3.

Table A.3.6.1.1.2.1-1: Modified parameters for RLM out of sync testing with 4 RX antenna connection

| Test case | SNR during T3 (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Test 1 | Test 2 | Test 3 | Test 4 |
| A.4.5.1.1 | -18 | N/A | N/A | N/A |
| A.4.5.1.3 | -18 | N/A | N/A | N/A |
| … | … | … | … | … |

Table A.3.6.1.1.2.1-2: Modified parameters for RLM in sync single carrier testing with 4 RX antenna connection

| Test case | SNR during T3 (dB) |  | SNR during T4 (dB) |  |
| --- | --- | --- | --- | --- |
|  | Test 1 | Test 2 | Test 1 | Test 2 |
| A.4.5.1.2 | -18 | N/A | -8 | N/A |
| A.4.5.1.4 | -18 | N/A | -8 | N/A |
| … | … | … | … | … |

Table A.3.6.1.1.2.1-3: Modified parameters for Beam Failure Detection and Link Recovery testing with 4 RX antenna connection

| Test case | SNR for RS in set q0 during T3, T4 and T5 (dB) |
| --- | --- |
|  | Test 1 |
| A.4.5.5.1 | -15 |
| A.4.5.5.2 | -15 |
| … | … |

In R18 NR_ENDC_RF_FR1_enh2, the test principle and SNR levels of RLM/BFD tests for 8 RX capable UEs follow that of 4 Rx UEs because only 4Rx is used for PDCCH reception even if UE has more RX antennas. Likewise, the RLM and BFD testing for 6Rx capable UE shall reuse SNR levels specified for 4Rx capable UE as well.

Proposal 2: RAN4 introduce the antenna connection for 6Rx capable UEs in TS 38.133 A.3.6, where the following aspects are specified:

  - Testing principles for 6Rx capable UEs

  - For RLM and BFD testing, reuse SNR levels specified for 4Rx capable UE

Conclusions

In this paper we provided our analysis on the possible RRM impact from SRS antenna switching and RLM/BFD test cases perspective for 6Rx capable UEs. The following is provided.

Proposal 1: The existing interruption requirements at SRS antenna switching are applicable to 6RX capable UE, and no specification impact is observed.

Proposal 2: RAN4 introduce the antenna connection for 6Rx capable UEs in TS 38.133 A.3.6, where the following aspects are specified:

  - Testing principles for 6Rx capable UEs

  - For RLM and BFD testing, reuse SNR levels specified for 4Rx capable UE

Reference

- RP-240828, “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, HiSilicon