3GPP TSG-RAN WG4 #112	R4-2412229

Maastricht, 19th – 23th August 2024

Source: 	Huawei, HiSilicon

Title:  	Discussions on RRM requirements for Rel-19 ATG

Agenda Item:	8.10.4

Document for:	Discussion

Introduction

In Rel-18, RAN4 completed the specification work to support ATG for single-cell scenario. In Rel-19, the New WI is approved in [1] to address the increasing demand for the deployment of ATG. The carrier aggregation technology can provide a larger downlink bandwidth. In this paper, we present our observations and views on RRM impacts to support CA in ATG.

General

The objectives of the WI [2] are shown as follows:

The core part of the work item includes: - Specify the RF and RRM core requirements for intra-band co-located and inter-band co-located DL CA [RAN4]: - FR1 intra-band contiguous CA - Example band combination: CA_n79C - FR1+FR1 inter-band CA - Example band combination: CA_n3A-n39A - Specify the RF requirements for support of UL MIMO with 2TX for single CC for UE. [RAN4] - Specify signalling and UE capabilities if necessary [RAN2].

RAN4 RF has started the corresponding discussion in RAN4#110bis and RAN4#111 [3][4]. The discussion was mainly about operation band combinations/supporting of UL MIMO/UE antenna type for CA.

The RAN4 RF has already concluded the antenna type of the ATG UE which is shown as follows. In the RRM discussion, we need to analyse each possible antenna type of the ATG UE comprehensively.

Issue 3-1: clarify the antenna type for each band for inter-band CA Agreement: - No limitation on antenna types for ATG CA - FFS on whether to assume omni-antenna type can be assumed for both band n3 and n39 in DL CA_n3-n39 - FFS on whether new capability is needed for ATG CA

The omnidirectional antenna provides wide coverage. Despite the fast-moving nature of the aircraft, the omnidirectional antenna maintains a stable connection between the ATG BS and the ATG UE. The wireless link between the ATG BS and the aircraft UE generally experiences less multi-path interference. Therefore, the omnidirectional antenna is well-suited to cope with minimal multi-path interference that might exist. On the other hand, the antenna array shows significant advantages in signal strength due to its high antenna gain in a specific direction. Meanwhile, if this type of antenna is equipped the ATG UE is capable of interference suppression.

Observation 1: There could be different antenna types for inter-band CA as agreed in RF session.

Proposal 1: RAN4 to discuss the antenna types for inter-band CA and corresponding impacts on RRM requirements.

RRM core requirement analysis

Compared with the requirements defined in Rel-18 ATG for single-Cell, we provide our analysis on the potential RRM impacts to support CA in case by case manner in following part.

MRTD

It is straightforward that MRTD requirements shall be defined for ATG-supporting cases. The existing MRTD requirements as shown as follows for intra-band NCCA and inter-band CA.

As stated in the WID and further clarified in RF discussion, the applicable scenario in Rel-19 is co-located intra-band contiguous CA and inter-band CA, meaning that the UE will receive both the PCC and SCC signals from the same base station. In this configuration, the UE receiving signals from both the PCell and SCell will experience negligible latency and time difference may not be the same as the general CA.

Proposal 2: RAN4 to define MRTD requirements for inter-band and intra-band CA scenarios for Rel-19 ATG.

Proposal 3: RAN4 to discuss whether to reuse the same MRTD requirements.

SCell activation/deactivation

In previous releases, different types of SCell activation enhancement were specified. For instance, direct SCell activation/PUCCH SCell activation/fast SCell activation/multiple SCell activation. Based on WID and RAN4 RF discussion so far, there are no multiple SCells scenarios. Considering the characteristic of ATG scenario, we think at least following SCell activation requirements shall be defined:

- SCell activation

- Direct SCell activation at SCell addition

- Direct SCell activation at Handover

Proposal 4: RAN4 to define SCell activation for single-SCell for at least:

- SCell activation

- Direct SCell activation at SCell addition

- Direct SCell activation at Handover

Interruption requirements

For UE supporting CA, it is straightforward that there could be an interruption in one Cell caused by certain procedures from the other Cells. Based on the current interruption requirements in chapter 8.2 of 38.133, it is evident that there is no distinction made between different scenarios or UE types. Following the same logic, we think current interruption requirements can also apply to ATG-supporting CA without the need for any additional specifications. If there are ATG new procedures which are not covered in current interruption requirements, RAN4 could discuss whether to introduce new interruption requirements.

Proposal 5: Existing interruption requirements can also apply to ATG supporting CA. RAN4 could discuss whether to introduce new interruption requirements if there are ATG new procedures which are not covered in current interruption requirements.

Other requirements

In Rel-18 ATG, RAN4 defined L3 intra-frequency and inter-frequency measurement. For Rel-19 ATG supporting CA, RAN4 shall also consider the case for intra-frequency measurement on deactivated SCell.

Proposal 6: RAN4 to define intra-frequency measurement requirements for deactivated SCell.

Conclusion

Observation 1: There could be different antenna types for inter-band CA as agreed in RF session.

Proposal 1: RAN4 to discuss the antenna types for inter-band CA and corresponding impacts on RRM requirements.

Proposal 2: RAN4 to define MRTD requirements for inter-band and intra-band CA scenarios for Rel-19 ATG.

Proposal 3: RAN4 to discuss whether to reuse the same MRTD requirements.

Proposal 4: RAN4 to define SCell activation for single-SCell for at least:

- SCell activation

- Direct SCell activation at SCell addition

- Direct SCell activation at Handover

Proposal 5: Existing interruption requirements can also apply to ATG supporting CA. RAN4 could discuss whether to introduce new interruption requirements if there are ATG new procedures which are not covered in current interruption requirements.

Proposal 6: RAN4 to define intra-frequency measurement requirements for deactivated SCell.

References

[1] RP-241213 Revised WID on Enhancements for Air-to-ground network for NR

[2] RP-240839 Enhancements for Air-to-ground network for NR

[3] R4-2406594 WF on Rel-19 ATG UE requirements

[4] R4-2410687 WF on Rel-19 ATG UE requirements