3GPP TSG RAN WG4 Meeting # 112                         R4-2412230

Maastricht, NL, Aug 19 - Aug 23, 2024

Title:	RRM Core requirements on Further NR mobility enhancements

Source:	China Telecom

Agenda item:			5.24.1

Document for:	Discussion

- Introduction

In the RAN4 #111 meeting, the L1/L2 based inter-cell mobility were discussed and WF [1] has been approved. In this paper, some issues are further discussed.

- Discussion

Issue 1-3-1: Whether to consider early TCI state activation for multiple cells at the same time

- Option 1 (Huawei, vivo, Nokia, MTK): - Early TCI state activation delay requirements to be defined for one or more TCI states for a single candidate cell, because one MAC-CE activates TCI states only for a single candidate cell. - Option 2 (Ericsson, QC): - If all the target LTM TCI states in the active TCI state list are known, if the UE receives TCI state activation command at slot n, UE shall have completed the LTM TCI state list update in slot n + THARQ +3Nslotsubframe,µ + TOk*(Tfirst-SSB_List + TSSB-proc) / NR slot length. - If any of the target TCI states in the active TCI state list are unknown, if the UE receives TCI state activation command at slot n, UE shall have completed the LTM TCI state list update in slot n+ THARQ +3Nslotsubframe,µ + (TL1-RSRP_list +TOuk*(Tfirst-SSB_List+ TSSB-proc)) / NR slot length. - In the TCI state activation requirements for LTM candidate cells, T L1-RSRP = 0 for FR1 - In the TCI state activation requirements for LTM candidate cells, Tfirst-SSB_List is given by - For FR1, Tfirst-SSB_List = max (Tfirst-SSB_LTM1, Tfirst-SSB_LTM2, .. , Tfirst-SSB_LTMn). - For FR2, - Tfirst-SSB_List = Tfirst-SSB_LTM1 + Tfirst-SSB_LTM2 + ... +Tfirst-SSB_LTMn, if the time to first SSB associated to LTM candidate TCI states are overlapped in FR2. - Tfirst-SSB_List = max (Tfirst-SSB_LTM1, Tfirst-SSB_LTM2, .. , Tfirst-SSB_LTMn.) if the time to first SSB associated to LTM candidate TCI states are not overlapped. - Where, the Tfirst-SSB_LTMn is the SSB periodicity of LTM candidate cell n.

In our understanding, when MAC-CE activates TCI states, the Cell ID is necessary, and there is only one Cell ID. In this case, one MAC-CE activates TCI states only for a single candidate cell, so we think early TCI state activation delay requirements are defined for one or more TCI states for a single candidate cell.

Proposal 1: Early TCI state activation delay requirements are defined for one or more TCI states for a single candidate cell.

Which cell(s) UE performs early RRC decoding and check before cell switch command when candidate cells configured are more than UE capability

In our understanding, for reducing the delay of cell switch, early RRC decoding and check is important. When the number of cells to perform early RRC decoding and validity check does not exceed UE capability, UE is supposed to decoding RRC of all the cells whose TCI state(s)are activated or early RACH is triggered.

If the number of cells to perform early RRC decoding and validity check exceeds UE capability, UE is supposed to decoding RRC of the lasted ones within UE capability which the UE received TCI state activation MAC-CE or PDCCH order.

Proposal 2: UE is supposed to decoding RRC of all the cells whose TCI state(s)are activated or early RACH is triggered, provided the number of these cells doesn’t exceed UE capability.

If the number of these cells exceeds UE capability, UE is supposed to decoding RRC of the lasted ones within UE capability whose TCI state(s)are activated or early RACH is triggered.

- Summary

In this paper, we provide our views on L1/L2 based inter-cell mobility. From this discussion we have derived the following proposals:

Proposal 1: Early TCI state activation delay requirements are defined for one or more TCI states for a single candidate cell.

Proposal 2: UE is supposed to decoding RRC of all the cells whose TCI state(s)are activated or early RACH is triggered, provided the number of these cells doesn’t exceed UE capability.

If the number of these cells exceeds UE capability, UE is supposed to decoding RRC of the lasted ones within UE capability whose TCI state(s)are activated or early RACH is triggered.

- References

[1] R4-2410302, WF on NR mobility enhancements (part 1) _v2, MediaTek Inc.