3GPP TSG-RAN WG4 Meeting #112	R4-2412231

Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	Ericsson

Title:  	Testability and interoperability issues for beam management

Agenda Item:	8.17.2

Document for:	Discussion

# Introduction

In last meeting 3GPP TSG RAN WG4 Meeting #111 RAN4 discussed RRM requirements for AI/ML based beam management and the discussion summary is listed in [1]. Following the discussions, we herein in this contribution present our views and proposals on the remaining open issues.

# Discussion

## Sub-topic 2-1 Metrics/KPIs for Beam management

Issue 2-1: Metrics/KPIs for Beam Management requirements/tests

Different metrics/KPIs have been discussed and were captured in the TR 38.843: For metrics for beam management requirements/tests, the following test metrics are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy - Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" - Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" - Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: combinations of above options In RAN4 meeting #111bis: Issue 2-1: Metrics/KPIs for BM Proposals - Option 1: Use Option 1 - Option 2: Use Option 2 - Option 3: Use Option 1,2,3 depending on use case - Option 4: Neither Option 1, 2, 3 is appropriate, a new metric is needed - Option 5: Other - Recommended WF - To be discussed Companies suggesting to use different metrics(new) should come up with a concrete proposal

Recapping the discussions in last meeting, at least some companies observed that the Metrics/KPIs for beam management requirements/tests shall be based on the input/output of the AI/ML model which is (to be) defined in RAN1, which is captured by the following agreements.

Agreement For UE-sided model, at least for BM-Case1, for content in the report of inference results, support - Opt 1: Beam information on predicted Top K beam(s) among a set of beams - Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams - At least K=1 and more, FFS on max value - FFS on beam information - FFS on the definition of predicted Top K beam(s) - FFS on definition of reported RSRP when applicable - FFS on other information in the report with potential down selection among the following options - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams - FFS on the quantization method of probability information - Probability information is the probability of the beam to be the Top 1 or Top K beam - Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP - FFS on definition of reported RSRP - FFS on the definition and quantization method of confidence information - Other options are not precluded. where the set of beams is Set A, i.e., the beams for UE prediction.

Agreement For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options: - Option A: Predicted RSRP - Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement - Where the predicted RSRP is based on AI/ML output - Note: Support both Option A and Option B is not precluded.

We consider the Metrics/KPIs for beam management requirements/tests as follows with below options separately:

- In the case of beam information only in the content in the report of AI/ML model inference results, the Metrics/KPIs for beam management requirements/tests must be derived by the beam prediction accuracy information.

- In the case of beam information + RSRP in the content in the report of AI/ML model inference results, the Metrics/KPIs for beam management requirements/tests must take both of the beam prediction accuracy and beam information into account, i.e., the UE shall fulfil beam prediction accuracy and beam information simultaneously.

In addition, in the second option, the beam prediction accuracy definition is still ambiguous to some extent since the content of RSRP has not been exposed in TR or in RAN1 discussions. In last meeting, at least one company proposed that predicted RSRP accuracy is a set of predicted RSRP accuracy of each beam in the Top-K predicted beams, on the contrary, at least one company proposed that the individual predicted RSRP accuracy of each beam in the Top-K predicted beams isn’t critical to the purpose of beam management framework. We understand the rationale of the latter one from the applicability perspective. However, we intend towards a precise approach to verify each beam from conformance test perspective, and it's worth noting that it exactly is the way we treat measured RSRP and define RSRP accuracy for plenary beams in legacy.

Proposal 1: Metrics/KPIs for beam management requirements/tests separately in below options:

- In the case of beam information only in the content in the report of AI/ML model inference results

  - The Metrics/KPIs for beam management requirements/tests must be based on the beam information.

- In the case of beam information + RSRP in the content in the report of AI/ML model inference results

  - The Metrics/KPIs for beam management requirements/tests must take both of the beam prediction accuracy and beam information into account, i.e., the UE shall fulfil beam prediction accuracy and beam information accuracy simultaneously.

Proposal 2: In the case of beam information + RSRP in the content in the report of AI/ML model inference results, the predicted RSRP accuracy of each beam in the Top-K predicted beams shall be individually verified with respect to the requirement to be defined.

Furthermore, in the above, the RSRP accuracy is referred to as the absolute RSRP accuracy, which is consistent with the definition of legacy requirements on the absolute RSRP accuracy. In same manner, the relative RSRP accuracy shall be defined by RAN4 as well. The legacy relative RSRP accuracy definition in Clause 10.1.20.1.2 in TS38.133 is copied as follows:

﻿The relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB compared to the largest measured value of L1-RSRP among all SSBs of the serving cell.

Proposal 3: Relative RSRP accuracy, in same way of definition in legacy, shall be defined for Metrics/KPIs for beam management requirements/tests.

Besides, another possible situation shall be considered as well. According to RAN1 agreements to date, for BM-Case 1, if a beam in Set B is configured for measurement, then the AI/ML model may produce the measured RSRP not the predicted RSRP. In this case, if beam information and RSRP both are in the content in the report of AI/ML model inference results, the Metrics/KPIs for beam management requirements/tests must take both of the measured RSRP accuracy and beam information into account, where measured RSRP accuracy shall follow the legacy requirements.

Proposal 4: For BM-Case 1, if a beam is configured for measurement in Set B, then the AI/ML model may produce the measured RSRP instead of the predicted RSRP. In this case, if beam information and RSRP both are in the content in the report of AI/ML model inference results, the Metrics/KPIs for beam management requirements/tests must take both of the measured RSRP accuracy and beam information into account, where measured RSRP accuracy shall follow the legacy requirements.

## Sub-topic 2-2 RSRP Prediction accuracy

Issue 2-2: RSRP Prediction accuracy

- Proposals - Option 1: Difference between the predicted RSRP(of the predicted beam) and the measured RSRP of the same Tx beam - Option 2: Difference between the predicted RSRP(of the predicted beam) and the ideal RSRP of the same Tx beam - Option 3: Difference between the predicted RSRP(of the predicted beam) and the ideal RSRP of the beam that should be predicted (genie-aided predicted beam) - Option 4: Difference between the predicted RSRP(of the predicted beam) and the measured RSRP of the beam that should be predicted (genie-aided predicted beam) - Option 5: Difference between the ideal RSRP(of the predicted beam) and the ideal RSRP of the beam that should be predicted (genie-aided predicted beam) - Option 6: Difference between the ideal RSRP(of the predicted beam) and the measured RSRP of the beam that should be predicted (genie-aided predicted beam) - Option 7: Difference between the measured RSRP(of the predicted beam) and the ideal RSRP of the beam that should be predicted (genie-aided predicted beam) - Option 8: Difference between the measured RSRP(of the predicted beam) and the measured RSRP of the beam that should be predicted (genie-aided predicted beam) - Option 9: RSRP difference between the RSRP of the genie-aided Top-1 beam and the measured RSRP of the predicted Top-K - Option 10: - Recommended WF - To be discussed

With respect to the agreement in Issue 2-3, by defining the ground truth for the predicted RSRP, predicted RSRP accuracy in Issue 2-2 can be refined as the difference between the predicted RSRP (of the predicted beam) and the ideal measurement of RSRP of the same Tx beam.

Proposal 5 RSRP prediction accuracy is defined as the difference between the predicted RSRP (of the predicted beam) and the ideal measurement of RSRP of the same Tx beam.

## Sub-topic 2-3 Measured/reported value and ground truth

Issue 2-3: Reported measurements and ground truth

Agreement: - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded

Below are some preliminary analyses on the FFS factors in the issue:

- SNR:

In last meeting, the SNR side condition for sufficient accuracy of reported measured RSRP was proposed to be checked. According to Table 10.1.20.1.2-1 in TS 38.133, the UE’s RSRP measurement accuracy requirements only apply down to the SNR of -3 dB. To get sufficient accuracy of reported RSRP, the SNR condition shall be increased. However, the exact SNR level which may rely on RS periodicity, density, bandwidth, channel model and subcarrier spacing, a totally safe SNR to deal with any configuration/condition isn’t easy to be determined and may be difficult to be generated in test environment. Recapping some discussions in L1-RSRP measurement accuracy for beam management, e.g., some content in R4-1809883, the simulation results show that the measurement accuracy is mostly harden, i.e., the slope of measurement accuracy with respect to SNR is almost flat, when SNR is close to 12dB.

Moreover, how to add ideal measurement of RSRP with high SNR condition into tests is an issue to be handled when designing the test.

  - To realize high SNR, the probes shall be able to generate signals at high output power level.  A problem is that the wanted signal level can sometimes become too high to create inn the OTA environment. In that case, the AWGN level must be reduced. If it is reduced too far then the noise floor becomes dominant, which is not predictable.

  - After that, to our understanding, it requests the UE to be perform prediction/measurement with same channel condition (except for SNR condition) for comparison of their RSRP, it may need sync. between test environment and UE to keep an adequate time period for prediction/measurement report and comparison.

- Multiple-AoA test setup:

No specific treatment, it shall be same as channel models for AI/ML beam management test.

- Channel condition:

No specific treatment, it shall be same as channel models for AI/ML beam management test.

- The ground truth will be changing or not:

No specific treatment, the ground truth shall keep same unless test procedure updates the predicted Tx beam.

Proposal 6: The achievable maximal SNR level for ideal measurement of RSRP may need to be checked by test environment vendors.

Proposal 7: Adding ideal measurement of RSRP with high SNR condition into tests is an issue to be handled.

- It requests the UE to perform prediction/measurement with same channel condition (except for SNR condition) respectively for comparison of their RSRP.

- Eventually, it may need a design between test environment and UE for a set of interactions and operations involving prediction, prediction report, increasing SNR, measurement, measurement report and comparison between measurement and prediction.

Proposal 8: Regarding the question on multiple-AoA test setup, channel condition, the ground truth will be changing or not, in our opinion, there is no specific treatment. They shall be the same as channel models for AI/ML beam management test.

## Sub-topic 2-4 Channel models in tests

Issue 2-4: Channel models

- Proposals - Option 1: CDL channels are needed (with fading) - Option 2: Only use AWGN channels - Option 4: multipath fading is needed, FFS how this can be emulated in the chamber

No agreement was achieved in the last meeting, various points of view were put forth, CDL is preferred because of multiple AoAs while TDL is preferred due to its simplicity.

Referring TR38.901, as duplicated in below, 3D Multi-Probe Anechoic Chamber (MPAC) for FR2 MIMO OTA testing is a good start point as reference methodology. And CDL channel realized by MPAC which is mainly for MIMO evolution may be a proper reference channel for AI/ML because of similarity between each other. Although TDL channel can emulate multipath effect, it is not suitable for multi-antenna transmission scheme.

Apart from that, some discussions took place regarding the difference between time-varying channel and time-static channel. We think the time-varying channel may reflect some characteristics of AI/ML model which can’t be observed in time-static channel test.

Given above, we support option 1, that is take CDL channel as reference. Generating a CDL channel is derived from at least the following parameters.

- Departure angles (AOD, ZOD)

- Arrival angles (AOA, ZOA)

- Coupling of rays within a cluster for both azimuth and elevation

- Cross polarization power ratios

- Channel coefficient

For BM-case1, which predicts the best beams in Set B with another set of beam A at the same time domain, channel variation isn’t important. Therefore, generating 2 channels, channel #1 for training and generating  channel #2 with same parameters for inference, is enough, as regenerating channel introduces randomness but keeps certain correlation between two channels, which is demonstrated in Figure 1.

Figure 1 Channel update for BM-case 1

For BM-case2, which harnesses historical measurement results with Set B to anticipate the best beam in Set A for one or more future time instances. From test perspective, it needs some trackable change, e.g., temporal correlation of channels to mimic UE motion. Apparently, only random change between the two channels isn’t enough. In practice, UE travelling direction and UE velocity may determine the temporal correlation, but temporal correlation in test must rely on the consequent change of the channel parameters. At the least, generating a set of channel #1 to channel #m for training and generating a set of channel #m+1 to channel #n with a set of parameters which update sequentially and gladfully for inference is necessary, as depicted in Figure 2. In which, the number of m and n depend on the measurement window and prediction window of BM-case2.

Figure 2 Channel update for BM-case 2

Wherein, the parameters needing updates between channels may at least comprise:

- Departure angles (AOD, ZOD)

- Arrival angles (AOA, ZOA)

- Channel coefficient

In an illuminative example, AOD in channel #1 is 10 degrees, AOD in channel #2 is 20 degrees, and AOD in channel #k is (10*k) degree.

In the last meeting, at least one company proposed adjusting departure angles in BM-case2. We agree on the essential intention, but a more general approach is covering more parameters in CDL channel as above.

Proposal 9: Take 3D Multi-Probe Anechoic Chamber (MPAC) for FR2 MIMO OTA testing as the start point of reference methodology for AI/ML tests.

Proposal 10: Take CDL channel realized by MPAC as the start point of reference channel for AI/ML tests.

Proposal 11: Regarding Channel generation in tests (for each test configuration, a test case may contain more than one test configuration):

- For BM-case1, channel change in time isn’t important. Therefore, generating channel #1 for training and generating channel #2 with same parameters for inference is enough.

- For BM-case2, from test perspective, it needs some trackable change to mimic UE motion in test, which must rely on a set of consequent change of the channel parameters.

  - At least, it includes generating a set of channels, from channel #1 to channel #m, for training and generating a set of channels, from channel #m+1 to channel #n, along with sequent and grade updates of a set of parameters is necessary.

    - The number of m and n depend on the measurement window and prediction window of BM-case2.

    - The parameters may comprise one or more than one of

      - Departure angles (AOD, ZOD)

      - Arrival angles (AOA, ZOA)

      - Channel coefficient

      - Others aren’t precluded

### 6.2.3 3D Multi-Probe Anechoic Chamber (MPAC) for FR2 The 3D MPAC test method is the reference methodology for FR2 NR MIMO OTA testing. By arranging an array of antennas around the Equipment Under Test (EUT), a spatial distribution of angles of arrival in the 3D MPAC system may be simulated to expose the EUT to a near field environment that appears to have originated from a complex multipath far field environment. As illustrated schematically in Figure 6.2.3-1, signals propagate from the base station/communication tester to the EUT through a simulated multipath environment known as a spatial channel model, where appropriate channel impairments such as Doppler and fading are applied to each path prior to injecting all of the directional signals into the chamber simultaneously through the probe array. The resulting field distribution in the test zone is then integrated by the EUT antenna(s) and processed by the receiver(s) just as it would do so in any non-simulated multipath environment. The 3D MPAC system with 6 dual-polarized probes (illustrated with black dots in Figure 6.2.3-1) placed on a sector with minimum radius of 0.75m from the centre of the test zone is permitted for NR FR2 MIMO OTA testing. Figure 6.2.3-1: 3D MPAC system layout for NR FR2 MIMO OTA testing The exact probe locations with respect to the OTA test system coordinate system are tabulated in Table 6.2.3-1. Table 6.2.3-1. FR2 3D MPAC Probe Locations in OTA test system coordinate system Probe Number; Theta [deg]; Phi [deg]. 1; 0.0; 0.0. 2; 11.2; 116.7. 3; 20.6; -104.3. 4; 20.6; 104.3. 5; 20.6; 75.7. 6; 30.0; 90.0. The 3D MPAC probes in Table 6.2.3-1 can be implemented using conventional millimetre-wave probes as well as IFF-based probes as long as the same probe configuration and same number of probes is used. The channel model parameters and probe locations for channel model implementation are defined in a channel model coordinate system, which is illustrated in figure 6.2.3-2. The channel model coordinate axes xCM, yCM, and zCM correspond to the OTA test system coordinate axes z, y, and -x, respectively. Figure 6.2.3-2: Channel Model Coordinate Axes The probe locations with respect to channel model coordinate axes are tabulated in table 6.2.3-2. Table 6.2.3-2. FR2 3D MPAC Probe Locations in Channel Model Coordinate System Probe Number; Theta [deg]; Phi [deg]. 1; 90; 0. 2; 85; 10. 3; 85; -20. 4; 85; 20. 5; 95; 20. 6; 90; 30. The channel model rotations assumed for this probe configuration are tabulated in Table 6.2.3--3. Table 6.2.3-3. Channel Model Rotations InO CDL-A; UMi CDL-C. Phi [deg]; Theta [deg]; Phi [deg]; Theta [deg]. -5; -2.0; 32; 15.0. These channel model rotations assume the relative orientations of BS and UE antennas displayed in Figure 6.2.3-3, i.e., the DUT antenna is pointed towards the BS in channel model coordinate system. In order to avoid positioning ambiguities, the turntable implementing the rotation in  shall match the intended DUT  for P0 Orientation 1 without the re-positioning approach, as defined in Annex A.3, applied. With the re-positioning approach applied, the relative orientation between the DUT and the probes for P0 Orientation 2 shall be the same the relative orientation between DUT and probes as for P0 Orientation 1. Figure 6.2.3-3: Relative orientations of BS and UE antennas. Since the test points are uniformly spaced in 3D already, Table 6.2.3.2-1, there is no need to adjust/rotate the DUT rotations by the channel model rotations

## Sub-topic 2-5 Test environment limitation/requirements

Issue 2-5: Test environment

Agreement: maximum number of set B Tx beams that test system should be able to emulate: [8-16] maximum number of set A Tx beams that test system should be able to emulate: [64-128] FFS on AoAs UE rotation during the test: FFS UE rotation/repositioning between different tests: Yes

Based on R18 evaluations,  assuming that the maximum number of set A is 256, the accuracy of set B typically comprised a ratio that is 1/4 or 1/8 of Set A for spatial beam prediction, and set B equal to set A for BM-case2 in some evaluations.  It can be imagined that a same AI/ML model can achieve prediction accuracy m% with 8 beams in Set B and accuracy n% with 32 beams in Set B, herein, m% < n%.

- To the case of set A: 256 and set B: 8, it’s of course the toughest prediction case which may need advanced neural network and result in lower accuracy since less training input. But in a certain sense, the model is compatible to more UEs.

- To the case of set A: 256 and set B: 32, it requires UE of more computational power/complexity for the AI/ML model, because more beams in Set B may require larger neural network potentially. The requirement to the UE capability is higher but its accuracy could be higher.

Given above, the choice of set A and set B is essentially a tradeoff between prediction accuracy and AI/ML model compatibility. Further, we support the maximum number of set A to be as large as possible. Thus, we suggest the maximum number of set A = 256 and the maximum number of set B = 32.

Proposal 12: RAN4 to define the maximum number of set B Tx beams = 32, maximum number of set A Tx beams = 256, as the starting point.

To evaluate AI/ML model performance in different areas, more than one set of number of set A and set B can be defined as test configurations, therefore, different prediction accuracy requirements may be applied.

Proposal 13:  At least two sets of number of set A and set B are provided. Below are two examples:

- For verifying UE computational power of the AI/ML model, one example is number of set A = 256, number of set B = 32.

- For verifying UE performance (e.g., accuracy) of the AI/ML model, one example is number of set A = 256, number of set B = [8].

- Different accuracy requirements may be applied for different sets of of number of set A and set B.

Proposal 14: Regarding AoAs, CDL channels may be the reference.

Proposal 15: Regarding UE rotation during the test, we can refer to definitions in MPAC, no UE rotation is needed.

Proposal 16: Regarding UE rotation/repositioning between different tests, we can refer to definitions in MPAC, no UE rotation/repositioning is needed.

Proposal 17: RAN4 to define the number of future time instances to predict, i.e., prediction window, for temporal beam prediction (BM-case 2).

## Sub-topic 2-6 Data sets

Issue 2-6:	Datasets for training/testing

Agreement: Vendors may take into account the test environment/conditions defined by RAN4 when training the UE - sufficient test environment/conditions should be defined to enable vendors to create the data needed for training FFS on proposals to augment training data to avoid overfitting of UE models to the test environment RAN4 to strive to make the test conditions similar to field deployment conditions

The intention of the agreement is that the coverage of datasets for training/testing shall be broad adequately to meet practical field usage as much as possible, in other words, the AI/ML model shall be robust enough to against more than test environments. In this manner, we suggest considering defining test environment/conditions which can demonstrate the robustness of the AI/ML model.

Proposal 18: In order to avoid overfitting of UE models to the test environment only, the test (including data sets) shall be able to demonstrate the robustness of the AI/ML model.

There still is an open question on training data set. The training is executed by the AI/ML model directly, but the training data set, which is collected with specific hardware, doesn’t match all possible hardware impairment, noise and measurement error prior to AI/ML model in an individual UE, and it may further include some information not in the datasets. In a test to a individual UE, the training data may not match perfectly to the hardware of this UE. One way to deal with the issue is taking the hardware relevant error into AI/ML model metrics/KPIs, besides of measurement error.

Proposal 19: RAN4 to clarify whether training data set covers or doesn’t cover the hardware relevant error prior to the input of AI/ML model in UE.

Proposal 20: RAN4 shall study how to deal with hardware relevant error in training data. One option is taking the hardware relevant error into account the overall error.

## Sub-topic 2-7 Beamforming consistency

Issue 2-7:	Consistency

multiple companies brought up the need to discuss the consistency between set A and set B, otherwise it is not expected that beam prediction would work - Proposals - Option 1: consistency between set A and set B is reflected by below aspects: - Same NW antenna/beam configurations for set A and set B, and that its configurations don’t change during training and inference. - Same channel model for set A and set B, and that its configurations don’t change during training and inference. (Question: if consistency is valid for a non-static(time-varying) channel for set A and set B?) - Option 2: Consistency should be defined in a different way - Option 3: no need for any consistency definition - Recommended WF - To be discussed

We suppose it’s a common understanding that, from NW and UE perspective, some potential requirements to keep the consistency includes the following:

- Same size of Set A of beams

- Same size of Set B of beams

- Same DL spatial TX-filter for each beam among of all Set A of beams

- Same DL spatial TX-filter of for each beam among all Set B of beams

- Same indexing/ordering of all Set A of beams

- Same indexing/ordering of all Set B of beams

- Same Set B pattern(s) (i.e., association between Set A and Set B)

- Same QCL assumptions for resources configured, including Set B and Set A

- Same deployment scenarios e.g., ISD, antenna height, down tilt and NLOS probability

Given the above, their configurations shall be same in set A and set B at least by UE side, besides, we shall take the necessary changes for test purpose in network side, which have been exposed in the second bullets in Option 1 in the proposals.

Proposal 21: Consistency between training and inference shall cover below aspects:

- Same size of Set A of beams

- Same size of Set B of beams

- Same DL spatial TX-filter for each beam among of all Set A of beams

- Same DL spatial TX-filter of for each beam among all Set B of beams

- Same indexing/ordering of all Set A of beams

- Same indexing/ordering of all Set B of beams

- Same Set B pattern(s) (i.e., association between Set A and Set B)

- Same QCL assumptions for resources configured, including Set B and Set A

- Same deployment scenarios e.g., ISD, antenna height, down tilt and NLOS probability

Proposal 22: Consistency between training and inference shall consider the channel for test as follows:

- At least in BM-case2, the channel may vary between during training and inference, and UE’s position and rotation may vary as well.

## Sub-topic 2-8 Measurement error impact

Issue 2-2: Measurement error impact

Multiple companies brought up the need to evaluate the impact that the UE measurement error and the error in the training data have on inference accuracy Issue 2-8: Measurement error impact - Proposals - Option 1: RAN4 should study the impact of measurement error, companies should bring proposals for the next meeting on how to proceed with such a study - Option 2: RAN4 should postpone the discussion on the impact of the measurement error for now - Option 3: No need to evaluate the measurement error impact - Recommended WF - To be discussed

At least one company in the last meeting proposed that the prediction accuracy maybe not be worse than the current accuracy in case that L1-RSRP is one of AI/ML model outputs.

Depending on the type of the output of AI/ML model, the output type, such as label (classifier network) or RSRP (CNN or MLP), the request to the measurement error is different.

- To the AI/ML model with label (classifier network) output, measurement error may have no impact to the prediction.

- To the AI/ML model with RSRP output, it appears that the predicted RSRP could not be more accurate than or equal to the measured RSRP, which is also proved by the simulation which we have presented in last meeting as copied as follows.

This clause takes spatial beam prediction as reference. The measurement error impact to prediction is mainly manifested by introducing variants on training data to the AI/ML model, where training data are based on real measurements in field. The measurement accuracy is defined in Clause 10.1.20 of TS 38.133 for L1-RSRP accuracy requirements for FR2. Figure 2 shows the performance comparison of non-AI and AI method with and without measurement error. Where, baseline 1 and baseline 2 are defined as follows: • Baseline 1: Select the best beam within Set A of beams based on the measurement of all RS resources or all possible beams of beam Set A (exhaustive beam sweeping) • Baseline 2: Select the best beam within Set B of beams based on the measurement of all RS resources or all possible beams of beam Set B For the results with measurement error, the beam is selected based on the measured RSRP value including the error, however the real RSRP of the selected beam is used for calculating the RSRP difference metric. In both AI and non-AI scenario, Set B consists of 8 CSI-RS beams out of all 32 Tx beams (1/4 of all Tx beams), and set A contains of all 32 Tx beams. Also, for this evaluation we only perform beam selection algorithm among the Tx beam and it is assumed that the Rx beam with highest RSRP is used in all cases. Details of the parameter settings can be found in R1-2306930. Figure 3 Performance comparison of non-AI baselines and AI method with and without measurement error The results show that the measurement error impact on RSRP prediction accuracy of AI/ML model (the difference between the black solid curve and the green solid curve is about 7dB at 90%) is considerably larger than the legacy method 1, i.e. the exhaustive search of all beams in set A (the difference between the red dashed curve and the y-axis is about 3dB at 90%), or the legacy method 2, i.e. selecting the best beam among all measured beams in set B (the difference between the red solid curve and the blue solid curve is about 1 dB at 90%).

Proposal 23: In case of AI/ML model with RSRP output, measurement error impact to prediction shall be verified, since the predicted RSRP is always worse than the measured RSRP since measurement error is introduced in training/inference phase of AI/ML model.

Proposal 24: Measurement error impact to prediction shall be verified (e.g. by simulation) to guarantee an acceptable prediction performance.

Proposal 25: If tightening measurement error under current side condition is difficult from UE implementation perspective, RAN4 can add another side condition for tightened measurement error for enabling AI-ML model.

Proposal 26: In case of AI/ML model with label (classifier network) output, measurement error may not have direct impact to the prediction, if the reporting of the UE only relies on the output of AI/ML model without explicit or implicit assistance of measurement result.

## Sub-topic 2-9 UE reporting for network side models

Issue 2-9:	UE reporting for network side models

One company brought up a possible need for RAN4 to introduce different reporting schemes to help train the network side models. - Proposals - Option 1: RAN4 shall introduce the necessary core requirement on supporting data collection for NW-side AI/ML model inference/training (for BM-Case1 & 2), by considering: - Potential enhancement on L1 measurement/reporting for inference: e.g., beam reporting for more than 4 beam in L1 signaling, and overhead reduction; - Potential enhancement on MDT-based measurement/reporting for training. - Option 2: RAN4 cannot introduce new reporting schemes, proposal should be made in another WG - Option 3: others - Recommended WF - To be discussed

Proposal 27: Regarding UE reporting for network side models, RAN4 can wait for a more concrete proposal to be made in other WGs, e.g., RAN1 or RAN2.

## Sub-topic 2-10 Performance monitoring

Regarding the performance monitoring for UE-sided model, RAN1 has the agreements as follows.

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring

Particularly, let us focus on the available statements in the agreement, type 1 performance monitoring is performed in the NW-side. In which, the UE in Option 1 shall perform measurement of RS to be defined by network, from RRM requirements perspective, it hasn’t fundamental difference from the legacy measurement scheme; the UE in Option 2 calculate performance metric based on measurement and prediction, which may introduce new RRM requirements on the performance metric.

Proposal 28: Regarding different types of performance monitoring for UE-sided model, RAN4 requirement may be provided as follows:

- For Type 1, Option 1 (NW-side performance monitoring): From RRM requirements perspective, it hasn’t fundamental difference from the legacy measurement scheme.

- For Type 1, Option 2 (UE-assisted performance monitoring): New RRM requirements on the performance metric.

- For Type 2 (UE-side performance monitoring): Wait for further clarification in RAN1.

# Conclusion

Proposal 1: Metrics/KPIs for beam management requirements/tests separately in below options:

- In the case of beam information only in the content in the report of AI/ML model inference results

  - The Metrics/KPIs for beam management requirements/tests must be based on the beam information.

- In the case of beam information + RSRP in the content in the report of AI/ML model inference results

  - The Metrics/KPIs for beam management requirements/tests must take both of the beam prediction accuracy and beam information into account, i.e., the UE shall fulfil beam prediction accuracy and beam information accuracy simultaneously.

Proposal 2: In the case of beam information + RSRP in the content in the report of AI/ML model inference results, the predicted RSRP accuracy of each beam in the Top-K predicted beams shall be individually verified with respect to the requirement to be defined.

Proposal 3: Relative RSRP accuracy, in same way of definition in legacy, shall be defined for Metrics/KPIs for beam management requirements/tests.

Proposal 4: For BM-Case 1, if a beam is configured for measurement in Set B, then the AI/ML model may produce the measured RSRP instead of the predicted RSRP. In this case, if beam information and RSRP both are in the content in the report of AI/ML model inference results, the Metrics/KPIs for beam management requirements/tests must take both of the measured RSRP accuracy and beam information into account, where measured RSRP accuracy shall follow the legacy requirements.

Proposal 5 RSRP prediction accuracy is defined as the difference between the predicted RSRP (of the predicted beam) and the ideal measurement of RSRP of the same Tx beam.

Proposal 6: The achievable maximal SNR level for ideal measurement of RSRP may need to be checked by test environment vendors.

Proposal 7: Adding ideal measurement of RSRP with high SNR condition into tests is an issue to be handled.

- It requests the UE to perform prediction/measurement with same channel condition (except for SNR condition) respectively for comparison of their RSRP.

- Eventually, it may need a design between test environment and UE for a set of interactions and operations involving prediction, prediction report, increasing SNR, measurement, measurement report and comparison between measurement and prediction.

Proposal 8: Regarding the question on multiple-AoA test setup, channel condition, the ground truth will be changing or not, in our opinion, there is no specific treatment. They shall be the same as channel models for AI/ML beam management test.

Proposal 9: Take 3D Multi-Probe Anechoic Chamber (MPAC) for FR2 MIMO OTA testing as the start point of reference methodology for AI/ML tests.

Proposal 10: Take CDL channel realized by MPAC as the start point of reference channel for AI/ML tests.

Proposal 11: Regarding Channel generation in tests (for each test configuration, a test case may contain more than one test configuration):

- For BM-case1, channel change in time isn’t important. Therefore, generating channel #1 for training and generating channel #2 with same parameters for inference is enough.

- For BM-case2, from test perspective, it needs some trackable change to mimic UE motion in test, which must rely on a set of consequent change of the channel parameters.

  - At least, it includes generating a set of channels, from channel #1 to channel #m, for training and generating a set of channels, from channel #m+1 to channel #n, along with sequent and grade updates of a set of parameters is necessary.

    - The number of m and n depend on the measurement window and prediction window of BM-case2.

    - The parameters may comprise one or more than one of

      - Departure angles (AOD, ZOD)

      - Arrival angles (AOA, ZOA)

      - Channel coefficient

      - Others aren’t precluded

Proposal 12: RAN4 to define the maximum number of set B Tx beams = 32, maximum number of set A Tx beams = 256, as the starting point.

Proposal 13:  At least two sets of number of set A and set B are provided. Below are two examples:

- For verifying UE computational power of the AI/ML model, one example is number of set A = 256, number of set B = 32.

- For verifying UE performance (e.g., accuracy) of the AI/ML model, one example is number of set A = 256, number of set B = [8].

- Different accuracy requirements may be applied for different sets of of number of set A and set B.

Proposal 14: Regarding AoAs, CDL channels may be the reference.

Proposal 15: Regarding UE rotation during the test, we can refer to definitions in MPAC, no UE rotation is needed.

Proposal 16: Regarding UE rotation/repositioning between different tests, we can refer to definitions in MPAC, no UE rotation/repositioning is needed.

Proposal 17: RAN4 to define the number of future time instances to predict, i.e., prediction window, for temporal beam prediction (BM-case 2).

Proposal 18: In order to avoid overfitting of UE models to the test environment only, the test (including data sets) shall be able to demonstrate the robustness of the AI/ML model.

Proposal 19: RAN4 to clarify whether training data set covers or doesn’t cover the hardware relevant error prior to the input of AI/ML model in UE.

Proposal 20: RAN4 shall study how to deal with hardware relevant error in training data. One option is taking the hardware relevant error into account the overall error.

Proposal 21: Consistency between training and inference shall cover below aspects:

- Same size of Set A of beams

- Same size of Set B of beams

- Same DL spatial TX-filter for each beam among of all Set A of beams

- Same DL spatial TX-filter of for each beam among all Set B of beams

- Same indexing/ordering of all Set A of beams

- Same indexing/ordering of all Set B of beams

- Same Set B pattern(s) (i.e., association between Set A and Set B)

- Same QCL assumptions for resources configured, including Set B and Set A

- Same deployment scenarios e.g., ISD, antenna height, down tilt and NLOS probability

Proposal 22: Consistency between training and inference shall consider the channel for test as follows:

- At least in BM-case2, the channel may vary between during training and inference, and UE’s position and rotation may vary as well.

Proposal 23: In case of AI/ML model with RSRP output, measurement error impact to prediction shall be verified, since the predicted RSRP is always worse than the measured RSRP since measurement error is introduced in training/inference phase of AI/ML model.

Proposal 24: Measurement error impact to prediction shall be verified (e.g. by simulation) to guarantee an acceptable prediction performance.

Proposal 25: If tightening measurement error under current side condition is difficult from UE implementation perspective, RAN4 can add another side condition for tightened measurement error for enabling AI-ML model.

Proposal 26: In case of AI/ML model with label (classifier network) output, measurement error may not have direct impact to the prediction, if the reporting of the UE only relies on the output of AI/ML model without explicit or implicit assistance of measurement result.

Proposal 27: Regarding UE reporting for network side models, RAN4 can wait for a more concrete proposal to be made in other WGs, e.g., RAN1 or RAN2.

Proposal 28: Regarding different types of performance monitoring for UE-sided model, RAN4 requirement may be provided as follows:

- For Type 1, Option 1 (NW-side performance monitoring): From RRM requirements perspective, it hasn’t fundamental difference from the legacy measurement scheme.

- For Type 1, Option 2 (UE-assisted performance monitoring): New RRM requirements on the performance metric.

- For Type 2 (UE-side performance monitoring): Wait for further clarification in RAN1.

# References

- WF on AI/ML, R4-2401570

- 3GPP TR 38.827, v16.8.0, “Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)”.