3GPP TSG-RAN WG4 Meeting #112	R4-2412232

Maastricht, NL, August 19-23, 2024

Source: 	Ericsson

Title:  	Discussion on RRM requirements on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

Agenda Item:	8.26.3

Document for:	Discussion

# Introduction

The WI Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3 targeting release Rel-19 was approved, which can be found in [1].

In this contribution, we present our initial views on the underlying impacts to RRM requirements mandated by the objectives in the WI, starting from the study content and taking agreements in RAN1 and RAN2 into account.

# Discussion

## Topic #1: Store&Forward

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2, RAN4] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

In RAN2#125bis and 126 meetings, the agreements below were reached.

- S&F implies that at least the full eNB will be onboard - An IoT NTN network shall be able to inform UE(s) whether S&F Satellite operation is applied, either via NAS or AS (wait for SA2 progress on this) - The S&F satellite operation is common for NB-IoT and eMTC. - The S&F satellite operation is applied to both CP solution and UP solution (for the UP solution pending on SA2 conclusions on the architecture)

In our opinion, in S&F operation, relying on regenerative payload, the satellite could temporarily store the UL data when the feeder link is unavailable and forward the UL data to the NTN gateway when it becomes available.  The same concept applies to DL transmission, when the feeder link is available and UE is not in the coverage, the satellite can store the DL data and later forward the DL data to UE when the UE enters coverage (Noting that, the feeder link may not be available at this time).

From RRM perspective, the S&F operation may define plenty of time instants for indicating starting or ending of the S&F operation, the UE may potentially update measurement behavior/limitation accordingly.

Proposal 1: Regarding S&F, RAN4 to monitor and wait for more concrete agreements in other WGs before determining impact to RRM requirements.

## Topic #2: Capacity enhancements for uplink

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

In our opinion, EDT is a higher-layer feature, and we don’t define RRM requirements for this. In TN, EDT feature is also supported for IoT devices in LTE (NB-IoT, eMTC) in RAN2, but no RRM requirements were defined in RAN4.

During discussions in RAN2, the reduction of the necessary uplink and downlink signaling, e.g., contention based Msg3 transmission without Msg1/Msg2, may potentially incur checking of timing requirements. But the solution still isn’t mature in RAN1 and RAN2.

Proposal 2: Regarding capacity enhancements for uplink, RAN4 to monitor and wait for more concrete agreements in other WGs.

# Conclusion

Proposal 1: Regarding S&F, RAN4 to monitor and wait for more concrete agreements in other WGs before determining impact to RRM requirements.

Proposal 2: Regarding capacity enhancements for uplink, RAN4 to monitor and wait for more concrete agreements in other WGs.

# References

- RP-234077, Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, Deutsche Telekom