3GPP TSG-RAN WG4 Meeting #112	R4-2412233

Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	Ericsson

Title:	Discussion on reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2

Agenda item:	8.26.3

Document for:	Discussion

# 1	Introduction

In RAN2#126 meeting, a LS was sent to RAN4 and RAN1 [1]:

… 1. Overall Description: In RAN2#126 meeting, the following agreements related to Msg3 transmission for uplink capacity enhancement for R19 IoT NTN have been achieved: Agreements: => RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources). => If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3.. Based on the above agreements, RAN2 kindly asks RAN4 and RAN1: Q1: Whether an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2? Q2: If the answer for Q1 is no, from RAN4 and RAN1 perspective, how the required timing accuracy for Msg3 transmission can be satisfied in this case? 2. Actions: To RAN4 and RAN1: ACTION: RAN2 respectfully requests RAN4 and RAN1 to take above RAN2 progress into account and provide feedback. …

In this paper we analyze the issue brought up by the LS and propose an LS reply.

# 2	Discussion

In TS 36.133 V18.6.0, the requirements relevant to the pre-compensated TA for NB-IoT NTN, which are stated in clause 7.20A.1, is duplicated as below.

… The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.20A.2-1. This requirement applies when it is the first transmission in a DRX cycle or the first transmission in a repetition period (R>1) for NPUSCH and NPRACH, the first transmission after an uplink transmission gap in a repetition period (R>1) for NPUSCH and NPRACH transmission, or it is the transmission on PUR. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the serving NB-IoT cell minus NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×Ts. The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the serving NB-IoT cell. NTA_Ref for NPRACH is defined as 0. NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific (in Ts units) for other channels is the difference between UE transmission timing and the Downlink timing immediately after when the last timing advance in clause 7.22A was applied. NTA_Ref for other channels is not changed until next timing advance is received. Table 7.20A.2-1: Te Timing Error Limit Downlink Bandwidth (MHz); Te_. 0.18; 97*TS. Note 1: TS is the basic timing unit defined in TS 36.211. …

Furthermore, the UE shall be able to adjust the timing of its transmissions with the adjustment accuracy defined in In TS 36.133 V18.6.0, which is duplicated in the below.

#### 7.22A.2.2 Timing Advance adjustment accuracy The UE shall adjust the timing of its transmissions with a relative accuracy better than or equal to ±13.33* TS seconds to the signalled timing advance value compared to the timing of preceding uplink transmission. The timing advance command is expressed in multiples of 16* TS and is relative to the current uplink timing.

Similarly, the requirements relevant to the pre-compensated TA for MTC, which are stated in clause 7.24A.2, is duplicated as below.

… The UE initial transmission timing error shall be less than or equal to Te_NTN_M1 where the timing error limit value Te_NTN_M1 is specified in Table 7.24A.2-1. This requirement applies when it is the first transmission in a DRX cycle, eDRX_CONN cycle, or the first transmission in a repetition period (R>1) for PUCCH, PUSCH, and SRS, or the first transmission after an uplink transmission gap in a repetition period (R>1) for PUCCH or PUSCH, or it is the PRACH transmission, or it is the transmission on PUR, or the first transmission in each segment of PUSCH/PUCCH for eMTC in a repetition period. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the serving cell minusNTA_Ref+NTA-offset+NTA,adjcommon+NTA,adjUE×Ts. The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the serving cell. NTA_Ref for PRACH is defined as 0. NTA_Ref+NTA-offset+NTA,adjcommon+NTA,adjUE(in Ts units) for other channels is the difference between UE transmission timing and the Downlink timing immediately after when the last timing advance in clause 7.3 was applied. NTA_Ref for other channels is not changed until the next timing advance is received. Table 7.24A.2-1: Te_NTN_M1 Timing Error Limit CE Mode; Te_NTN_M1_. A; 41*TS. B; 65*Ts. Note 1: TS is the basic timing unit defined in TS 36.211. Note 2: This requirement applies regardless of the downlink carrier bandwidth.. …

Furthermore, the UE shall be able to adjust the timing of its transmissions with the adjustment accuracy defined in In TS 36.133 V18.6.0, which is duplicated in the below.

#### 7.28A.2.2 Timing Advance adjustment accuracy The requirements defined in clause 7.3.2.2 also apply for this section.

#### 7.3.2.2 Timing Advance adjustment accuracy The UE shall adjust the timing of its transmissions with a relative accuracy better than or equal to ±4* TS seconds to the signalled timing advance value compared to the timing of preceding uplink transmission. The timing advance command is expressed in multiples of 16* TS and is relative to the current uplink timing.

Recall the timing relevant steps in a contention based RACH procedure as follows,

- A UE is able to receive SI from the serving cell to gather necessary satellite information and perform SSB measurement before Msg1 transmission to fulfill initial transmission timing error Te for Msg1 transmission.

- Subsequently, the UE monitors the PDCCH for the Random Access Response (RAR) message (Msg2) which may contains UE specific TA if applicable.

- After applying the TA to reduce the UL timing error, the UE then can guarantee msg3/PUCCH transmission of the UE not to interfere with other UE’s UL signals/channels.

In RAN4 specification, it appears that,

- For Msg1 transmission, the UE shall guarantee transmission timing error Te referring to the downlink timing of the reference cell minus NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×Ts. NTA_Ref for PRACH is defined as 0.

- For Msg3/PUCCH and any later PUCCH/PUSCH transmission, the UE shall follow the requirements in clause 7.1.2.1, and guarantee transmission timing error Te referring to the downlink timing of the reference cell minus NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×Ts.

We understand the question in LS from RAN2 in fact inquires whether UE can guarantee transmission timing error Te for Msg3/PUCCH and any later PUCCH/PUSCH transmission with any TA provided by network. Actually, the inquiry is the same as examining whether the transmission timing error range error range plus channel dispersion needs to be covered by the CP. In other words, we need to determine whether the following expression is fulfilled:

CP-2*(Te + TAadjerr) - channel_dispersion ≥ 0

Where,

- CP is the cyclic prefix length of Msg3 PUSCH, channel_dispersion is the max channel dispersion.

- TAadjerr is the accuracy of adjustment referring to TA  = ±13.33*Ts.

- Te is UE initial transmission timing error, including all UE specific estimation accuracy.

- Channel_dispersion

The max channel dispersion can be assumed to be 245 ns ≈ 7.53*Ts. The reason is expressed as follows. In TR 38.811, section 6.7.2, the worst case LOS scenario for 20 degrees or higher elevation angle has a logarithmic delay spread lgDS lgDS = log10(DS/1s) with mean µlgDS = -7.28 and standard deviation σlgDS = 0.67. This means that µlgDS + σlgDS = -7.28 + 0.67 = -6.61 and DS < 0.245 µs with 68 % probability, if Gaussian statistics is assumed (TR 38.811, Table 6.7.2-1a). At the end, 10^(-6.61) ≈ 0.245 µs.

For NB-IoT, Msg3 is transmitted using NPUSCH Format 1. NPUSCH Format 1 can be used with two different subcarrier spacings, 15   and 3.75 kHz. With respect to TS 36.211, the below can be acquired:

- For 15 kHz SCS, the CP length is 160*Ts ≈ 5.2083 µs for the first symbol in a slot and 144*Ts ≈ 4.6875 µs for the remaining symbols.

- For 3.75 kHz SCS, the CP length is 256*Ts ≈ 8.3333 µs.

Inserting the values, we get (in Ts units):

NB-IoT with 3.75 kHz SCS:	256-2*(97 + 13.33) – 7.53 = 27.81 > 0 i.e., PASS

NB-IoT with 15 kHz SCS:		144-2*(97 + 13.33) – 7.53 = -84.19 < 0 i.e., FAIL

For MTC, The CP duration is 160*Ts ≈ 5.2083 µs for the first symbol in a slot and 144*Ts ≈ 4.6875 µs for the remaining symbols (see TS 36.211).

Inserting the values, we get (in Ts units):

LTE-MTC, CE mode A:		144-2*(41 + 4) – 7.53 = 46.47 > 0 i.e., PASS

LTE-MTC, CE mode B:		144-2*(65 + 4) – 7.53 = -1.53 < 0 i.e., FAIL

Observation 1: Taking timing adjustment error and channel dispersion into account, the existing timing requirements, for NB-IoT with 15 kHz SCS and LTE-MTC CE mode B, cannot satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2.

On the contrary, taking NB-IoT as example, Te for NB-IoT(from Rel-13) equaling to 80*TS was determined by not larger than CP/2, it implies estimated error-free and no channel path delay spread in order for the NPUSCH to reach at eNB within cyclic prefix window. The decision was made with consideration for the NB-IoT UE's practical power consumption and complexity. After that, NB-IoT NTN took NTN scenario specific margin into Te based on that.

In such a way (expect for the CP lengths which still is same as the above ones), the inquiries are rewritten in the below:

For NB-IoT,

NB-IoT with 3.75 kHz SCS:	256-2*97 = 62 > 0 i.e., PASS

NB-IoT with 15 kHz SCS:		144-2*97 = -50< 0 i.e., FAIL

For MTC,

LTE-MTC, CE mode A:		144-2*41= 62 > 0 i.e., PASS

LTE-MTC, CE mode B:		144-2*65 = 14 > 0 i.e., PASS

Observation 2: Not taking timing adjustment error and channel dispersion into account, the existing timing requirements, for NB-IoT with 15 kHz SCS, cannot satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2.

Anyhow, applying the same formula and with the same parameters, Msg3 transmission without Msg1/Msg2 has comparable timing behavior with initial transmission timing error. Since initial transmission timing error may cause ISI in some extreme cases, in this manner, Msg3 transmission without Msg1/Msg2 may be acceptable.

Proposal 1: Reply to Q1 as follows:

- For NB-IoT with 3.75 kHz SCS and for LTE-MTC, CE mode A, the existing timing requirements (i.e., initial transmission timing error) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2.

- For NB-IoT with 15 kHz SCS and for LTE-MTC CE mode B with max (245 ns) channel dispersion, the existing timing requirements (i.e., initial transmission timing error) cannot satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2.

# 3 	Summary

Observation 1: Taking timing adjustment error and channel dispersion into account, the existing timing requirements, for NB-IoT with 15 kHz SCS and LTE-MTC CE mode B, cannot satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2.

Observation 2: Not taking timing adjustment error and channel dispersion into account, the existing timing requirements, for NB-IoT with 15 kHz SCS, cannot satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2.

Proposal 1: Reply to Q1 as follows:

- For NB-IoT with 3.75 kHz SCS and for LTE-MTC, CE mode A, the existing timing requirements (i.e., initial transmission timing error) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2.

- For NB-IoT with 15 kHz SCS and for LTE-MTC CE mode B with max (245 ns) channel dispersion, the existing timing requirements (i.e., initial transmission timing error) cannot satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2.

# References

[1] R2-2405769, LS on UL synchronization for contention based Msg3 transmission without Msg1/Msg2, ZTE

# Appendix draft LS reply

3GPP TSG-RAN WG4 Meeting #112		R4-xxxx

Maastricht, Netherlands, 19th – 23th August, 2024

Title:	Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2

Response to:	R2-2405769

Release:	Release 19

Work Item:	IoT_NTN_Ph3-Core

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:

Contact person:

Name:	Ming Li

E-mail Address:	ming.l.li@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1 Overall description

RAN4 would like to thank RAN2 for their LS R2-2405769.

RAN4 has the relevant transmission timing requirements,TS 36.133 V18.6.0, the requirements relevant to the pre-compensated TA for NB-IoT, which are stated in clause 7.20A.1, is duplicated as below.

… The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.20A.2-1. This requirement applies when it is the first transmission in a DRX cycle or the first transmission in a repetition period (R>1) for NPUSCH and NPRACH, the first transmission after an uplink transmission gap in a repetition period (R>1) for NPUSCH and NPRACH transmission, or it is the transmission on PUR. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the serving NB-IoT cell minus NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×Ts. The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the serving NB-IoT cell. NTA_Ref for NPRACH is defined as 0. NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific (in Ts units) for other channels is the difference between UE transmission timing and the Downlink timing immediately after when the last timing advance in clause 7.22A was applied. NTA_Ref for other channels is not changed until next timing advance is received. Table 7.20A.2-1: Te Timing Error Limit Downlink Bandwidth (MHz); Te_. 0.18; 97*TS. Note 1: TS is the basic timing unit defined in TS 36.211. …

Taking the above requirements into the RACH procedure, the UE shall fulfill the transmission timing in RACH procedure in a consistent manner as follows:

- For Msg1 transmission, the UE shall guarantee transmission timing error Te referring to the downlink timing of the reference cell minus NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×Ts. NTA_Ref for PRACH is defined as 0.

- For Msg3/PUCCH and any later PUCCH/PUSCH transmission, the UE shall follow the requirements in clause 7.1.2.1, and guarantee transmission timing error Te referring to the downlink timing of the reference cell minus NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×Ts.

We understand the question in LS from RAN2 in fact inquires whether UE can guarantee transmission timing error Te for Msg3/PUCCH and any later PUCCH/PUSCH transmission with any TA provided by network.

Given that, RAN4 provides the answer to Q1 as follows:

- For NB-IoT with 3.75 kHz SCS and for LTE-MTC, CE mode A, the existing timing requirements (i.e., initial transmission timing error) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2.

- For NB-IoT with 15 kHz SCS and for LTE-MTC CE mode B with max (245 ns) channel dispersion, the existing timing requirements (i.e., initial transmission timing error) cannot satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2.

# 2 Actions

To RAN2:

ACTION: 	RAN4 respectfully requests RAN2 to take the above feedback into consideration.

# 3 Dates of next TSG-RAN WG2 meetings

RAN4#112bis	from 2024-10-14 to 2024-10-18		Hefei, China

RAN4#113	from 2024-11-18 to 2024-11-22		Orlando, US