3GPP TSG-RAN WG4 Meeting #112	R4-2412234

Maastricht, NL, August 19-23, 2024

Source: 	Ericsson

Title:  	Discussion RRM requirements on NTN for NR phase 3

Agenda Item:	8.25.4

Document for:	Discussion

# Introduction

The WI NR Non-Terrestrial Networks (NTN) for NR Phase 3 targeting release Rel-19 was approved, which can be found in [1] and [2].

In this contribution, we present our initial views on the underlying impacts to RRM requirements mandated by the objectives in the WI, starting from the study content and review the relevant agreements in RAN1 and RAN2.

# Discussion

## Topic #1: Downlink coverage enhancements

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

In RAN1#116 meeting, the below agreements were reached. The link budget calculation was discussed and evaluated, but no requirements have been agreed.

Agreement RAN1 to consider the following performance metrics for DL Coverage enhancement evaluation at system level: At least: - CDF of the received SINR - The dwell time and revisit time interval for each beam illumination across the coverage - Periodicity of common control channels (e.g. SSB, CORESET0/SIB1, SIB19) and corresponding coverage ratio Other metrics may be reported such as - CDF of the cell throughput - CDF of user perceived throughput (UPT) - CDF of Latency - Ratio of mean served cell throughput and offered cell throughput, denoted by 𝜌 (refer to TR36.889) For system level study based on analytical evaluation: - N1 beam footprints are in state “off” - These beam footprints are not served by any signal (no satellite service in this area) - N2 beam footprints are in state “common messages only” - These beam footprints do not have any active user traffic, and are served the necessary information for cell discovery and initial access. - Optionally, companies may consider user arrival (e.g. RACH access) in this type of cell, and should describe how this is taken into account in the analytical evaluation - N3 beam footprints are in state “active traffic” - These beam footprints have X active (e.g. VoNR) users each. - These beam footprints are also served the necessary information for cell discovery and initial access - N1 + N2 + N3 = “Total number of beam footprints “ - N1, N2, N3, X are to be reported by companies. - Resource utilization obtained under the assumptions above is to be reported by companies. - Other assumptions made in the evaluation are to be reported by companies, e.g. power sharing scheme, beam hopping scheme, etc.

In RAN2#125bis and 126 meetings, the agreements below were reached.

- With regard to link level enhancement, RAN2 waits for RAN1 agreement on the DL channels to enhance before starting any RAN2 work. - We will continue the discussion on RAN2 aspects of DL coverage enhancements (e.g. cell level / beam level DTX/DRX mechanism, etc.) in the next meetings, trying to identify questions to RAN1 for aspects where we need their input

- Based on the solution being investigated in RAN1, RAN2 will further discuss whether/how legacy UEs might operate in a cell supporting DL coverage enhancements. - RAN2 assumes that both EFC and EMC are supported

According to the targets of the objective and agreements in RAN1 and RAN2, the solution serving the purpose is increasing the coverage across the satellite footprint by certain power sharing schemes. In this manner, limited by the power budget and number of RF chains available at the satellite payload, and the number of active beams can’t be increased beyond certain limit, this eventually leads to the adaptation of beam hopping.

For DL coverage enhancement objective, network energy saving should be considered as baseline for study. However, the Rel-18 network energy saving features in Rel-18 may not be reused in NTN scenario directly, since in NTN a beam covers a spot only at same time, as depicted in Figure 1.

Figure 1 Beam coverage by beam hopping

It is up to the gNB to schedule either the downlink or the uplink channels/signals in any cell that is being active via a specific illuminated satellite beam. It is essential to consider the network operation both in terms of DL and UL while devising the beam hopping. At the least, there are two kinds of SSB periodicity extension from usage perspective:

- SSB periodicity extension for cell search, which could affect its initial access and connection establishment. Furthermore, if the SSB periodicity is extended to a larger value than the default value of 20ms, then it is essential to investigate its impact on the periodicity of the remaining physical channels and signals (e.g., SIB, paging, etc.) by considering the network operation both in terms of downlink and uplink.

- SSB periodicity extension relevant in rate matching and measurement timing configurations. Unlike in cell search, where the UE lacks any information before the decoding of SSB, the UE is explicitly informed about the SSB periodicity in these cases as follows:

  - The SSB periodicity in ms indicated in the information element ssb-periodicityServingCell is used for the rate matching purpose, which can be either 5ms, 10ms, 20ms, 40ms, 80ms, or 160ms. If the field is absent, UE assumes the periodicity of 5ms.

  - In the context of neighbour cell, SMTC is used configure measurement timing configurations, i.e., timing occasions at which the UE measures SSBs. The periodicity can be either 5ms, 10ms, 20ms, 40ms, 8ms, or 160ms.

The scheme of beam hopping, e.g. granularity of beam switch in time domain, including periodicity, on-duration and off-duration, relationship to SSB/SMTC/DRX, availability of common signals in off-duration, still is under evaluation in other WGs. Given the status, we don’t expect any actual progress in RAN4 since it’s the first meeting for the WI.

Even though we note several RRM requirements that the potential beam hopping approach is likely to affect straightforwardly.

Firstly, to SSB periodicity extension for cell search, even cell search may not impact RRM requirements explicitly, extending the periodicities of the physical channels and signals may cause changes of RRM requirements.

Proposal 1: RAN4 to check if RRM requirements could be impacted by SSB periodicity extension to a value larger than 20ms during cell search, including the remaining physical channels and signals (e.g., SIB, paging, etc.).

Secondly, to SSB periodicity extension relevant in rate matching and measurement timing configurations, an underlying impact to the below RRM requirements is, due to the extension of SSB/SMTC/DRX periodicity, the measurement time delay is scaled, which may subsequently degrade measurement accuracy, which may include L1/L3 measurements, especially for the earth_moving cell case.

Proposal 2: RAN4 to check if RRM requirements could be impacted by scaled measurement time delay due to extension of SSB/SMTC/DRX periodicity.

At the end, in latest RAN1 agreement, there is a beam delay assumption as below.

- Agreement - For NR NTN Rel-19 DL coverage evaluation, a value of beam steering latency equal to 0 at least if phase array antenna is assumed. - Values different from 0 can be optionally reported

In the case of analog beamforming, there may be additional delay around 20ms caused by setting the phase coefficient for different antenna elements. So, the beam switching delay not zero for analog beamforming may incur the system overhead as there is no data will be transmitted during the beam switching period.

Furthermore, the beam switching time may not be perfectly aligned with the time frame. For example, the synchronization error between the gNB/gateway and satellite in transparent payload make beam switching either delay or advance referencing to an ideal switching time at gNB. The impact of the synchronization error can be the following. The advance of the beam switching may cut off the DL/UL data at the last subframe before switching to new beam. The delay of the beam switching may cut off the DL/UL data at the first subframe after switching to the new beam. There is no such concern on regenerative payload as the gNB is on board the satellite. In legacy TDD network, the 3ms is specified for a maximum time error between different network nodes. Though the interface between gNB and satellite is not standardized, a similar performance could be expected. The synchronization error may cause a similar system overhead in the beam hopping system and can be considered together.

Proposal 3: RAN4 to check if beam steering latency other than 0 may impact RRM requirements.

Proposal 4: RAN4 to check if timing offset/error (between beam switching timing and DL/UL framework) may impact RRM requirements.

Another observation is RRM requirements for quasi-earth_fixed cell and earth_ moving cell both may be affected by the beam hopping scheme, but with different influence.

Proposal 5: If applicable, the requirements for quasi-earth_fixed cell and earth_ moving cell both shall be studied with respect to the beam hopping scheme.

## Topic #2: Uplink Capacity/Throughput Enhancement

- Uplink Capacity/Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Study then specify, if beneficial, DFT-s-OFDM PUSCH enhancements via Orthogonal Cover Codes (OCC) - Determine the achievable capacity improvement to be targeted taking into account realistic impairments (e.g. Doppler, time variation, phase distortion, etc) - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed - Note: The study can consider orthogonal cover codes across OFDM symbols, across slots, and/or within an OFDM symbol. - Note: the study phase is targeted to be completed by RAN#104 - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design

Since there are no RRM requirements for the uplink channels which are impacted by these enhancements, it appears that no study is necessary.

Proposal 6: No RRM requirements are affected by uplink enhancement.

## Topic #3: Regenerative payload

- Support of regenerative payload [RAN3, RAN2, RAN4] - Specify the support of gNB on board in TS 38.300 - Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3] - Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

For support of regenerative payload, the satellite includes on-board processing to demodulate and decode the received signal and regenerate the signal before sending it back to the earth. When applied to general 3GPP architecture and terminology, the regenerative payload architecture means that the gNB is located in the satellite. Furthermore, the regenerative payload provides the feasibility of ISL between the satellites, and the ISL could be a radio interface, such as the Xn interface. The UE served by a gNB on the satellite could access the 5G core network (CN) via ISL.

Going through the study items for the topic and the framework of regenerative payload, as described in above, in our opinion, no RRM requirements are affected by regenerative payload.

Proposal 7: Regenerative payload has no effect on RRM requirements as of yet, we may continue to monitor progress in other WGs.

## Topic #4: Support of Rel-17 RedCap and Rel-18

In RAN plenary #104 meeting the WID was updated to include HD-FDD RedCap. Moreover, for HD-FDD RedCap UE and eRedCap UE it was agreed to focus on HD collisions cases (cases 3 and 4):

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

In RAN1#116bis and RAN1#117 meeting, the observations and conclusions listed below, were reached.

### RAN1#116 Agreement Study at least the following scenarios for (e)RedCap HD-FDD UEs for NTN: - Whether existing handling rules for the following cases should be reused or updated when taking into account TA mismatch between actual TA used by UE and assumed TA at the gNB based on available TA report: - Case 1: Dynamically scheduled DL reception collides with semi-statically configured UL transmission - Case 2: Semi-statically configured DL reception collides with dynamically scheduled UL transmission - Case 3: Semi-statically configured DL reception collides with semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Case 5: Configured SSB collides with dynamically scheduled or configured UL transmission - Case 6: Dynamic or semi-static DL collides with valid RO - Case 7: Collision due to direction switching - At least the following potential issues can be further considered for (e)RedCap HD-FDD UEs - Error cases in case 3 and case 4 - SIB19 reception collides with UL transmission - Slot counting for UL repetition transmission colliding with SSB reception - Invalid symbol determination for PUSCH repetition type B - Actual TDW determination due to the collision between DL reception and UL transmission with DMRS bundling - CPU occupation due to omitted DL reception or UL transmission Note: Both GSO and Non-GSO should be considered. ### RAN1#116bis Observation To avoid the occurrence of error cases 3 and 4 through network scheduling, there are less resources available for a scheduled HD-FDD RedCap/eRedCap UE in NTN compared to TN when there is TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB. Observation For collision cases 1, 2, 5 and 6, when there is TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB, there might be less resources available for the scheduled HD-FDD RedCap/eRedCap UE in NTN compared to TN if gNB attempts to avoid the collision or there is a loss of DL/UL transmissions due to collision. Observation When there is TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB, there may be a BLER performance degradation for the reception of UL transmissions at the gNB for the scheduled HD-FDD RedCap/eRedCap UE in NTN compared to TN if gNB does not attempt to avoid the collision at least in the following cases: - UL transmission with repetitions due to different available slot counting at UE and gNB when colliding with SSB reception - PUSCH repetition type B due to different invalid symbol determination at gNB and UE when colliding with DL transmissions - UL transmission with DMRS bundling due to the different actual TDW determination at gNB and UE when colliding with DL transmissions Note: the above cases happen at least with one of collision cases 1, 2, 5, 6, and 7. ### RAN1#117 Conclusion For Rel-19 HD-FDD RedCap/eRedCap UE in NTN, the issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB should be mitigated for collision cases 3 and 4. - Note: further discussion on other cases is not precluded Conclusion For collision cases 1, 2, 5 and 6, the existing priority rules can be reused for a HD-FDD (e)RedCap UE in NTN. Observation TA reporting is beneficial to mitigate the TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB for HD-FDD RedCap/eRedCap UE in NTN from RAN1 perspective. - Note: complexity, power consumption and signaling overhead impact of TA reporting for (e)redcap UEs was not investigated in this work item

In general, RRM requirements for RedCap UE in NTN can be started by taking RedCap UE in TN as baseline.

On top of that, the requirements and enhancements for NR NTN, as what we have done for it in Rel-17 and Rel-18, shall be introduced for RedCap UE in NTN.

Proposal 8: RRM requirements for RedCap UE in NTN can be started by taking RedCap UE in TN as baseline.

Proposal 9: The requirements and enhancements for NR NTN in Rel-17 and Rel-18 shall be introduced for RedCap UE in NTN.

### HD-FDD (e)RedCap collision cases in NTN

There is ongoing work in RAN1 regarding the mechanism to handle HD-FDD (e)RedCap collision cases in NTN for collision cases 3 & 4. RAN1 does not yet have a solution ready. However, we prefer to restate our RAN1 view here, for RAN4 reference [3]. We also comment on some other options discussed in RAN1 and in RAN plenary.

Our position is that adding a fixed rule in the specification seems to be a suitable approach for cases 3 & 4 (just as it has been done for most of the existing collision cases).

The other options discussed in RAN1, i.e., leaving up to the UE implementation to decide the priority or introducing signaling for the network to decide the priority are less preferred.

The alternative to leave it up to the UE implementation to decide the priority, keeps the uncertainty and varies from time-to-time (i.e., as per UE’s discretion) what is to be prioritized.

Regarding signaling, we think that the information to be potentially prioritized (e.g., SIB19) takes place during both initial access and connected mode. Thus, using signaling to define priority rules for collision cases 3 and 4 likely will end-up impacting “common control information” for initial access and “UE specific control information” for when the UE has established RRC connection with the network.

- Observation 1: There have been preliminary proposals for adding prioritization rules for collision cases 3 and 4 using any of the following alternatives:

- Adding a fixed rule in the specification (i.e., no signalling).

- Leaving up to the UE implementation to decide the priority (i.e., Adding a statement in the specification like the one that exists for collision case 6 “the HD-UE can select based on its implementation whether to either transmit … or receive …”).

- Introducing signalling for the network to decide the priority.

Observation 2: In our understanding, the intention behind defining priorities for collision cases 3 and 4 is eliminating the current uncertainty of those cases being treated as error cases. Thus, adding a fixed rule in the specification seems to be a suitable approach (just as it has been done for most of the existing collision cases).

Observation 3: In relation to the previous observation. Our initial view is that the other alternatives (i.e., up to UE implementation, and signalling) for introducing the priorities of collision cases 3 and 4 are less preferred because of the following reasons:

- Leaving up to the UE implementation to decide the priority: In terms of eliminating the uncertainty, this alternative keeps the uncertainty and varies from time-to-time (i.e., as per UE’s discretion) what is to be prioritized.

- Introducing signalling for the network to decide the priority: The information to be potentially prioritized (e.g., SIB19) takes place during both initial access and connected mode. Thus, using signalling to define priority rules for collision cases 3 and 4 likely will end-up impacting “common control information” for initial access and “UE specific control information” for when the UE has established RRC connection with the network.

RedCap and eRedCap UE are less capable devices, by definition and we prefer not to add a more detailed total TA-report at this stage and instead propose to focus on adding a fixed rule in specification, in RAN1, in order to handle HD-FDD (e)RedCap collision cases in NTN for collision cases 3 & 4.

Proposal 10: 	RedCap and eRedCap UE are less capable devices, by definition and we prefer not to add a more detailed total TA-report at this stage and instead focus on adding a fixed rule in specification, in RAN1, in order to to handle HD-FDD (e)RedCap collision cases in NTN for collision cases 3 & 4.

# Conclusion

- Observation 1: There have been preliminary proposals for adding prioritization rules for collision cases 3 and 4 using any of the following alternatives:

- Adding a fixed rule in the specification (i.e., no signalling).

- Leaving up to the UE implementation to decide the priority (i.e., Adding a statement in the specification like the one that exists for collision case 6 “the HD-UE can select based on its implementation whether to either transmit … or receive …”).

- Introducing signalling for the network to decide the priority.

Observation 2: In our understanding, the intention behind defining priorities for collision cases 3 and 4 is eliminating the current uncertainty of those cases being treated as error cases. Thus, adding a fixed rule in the specification seems to be a suitable approach (just as it has been done for most of the existing collision cases).

Observation 3: In relation to the previous observation. Our initial view is that the other alternatives (i.e., up to UE implementation, and signalling) for introducing the priorities of collision cases 3 and 4 are less preferred because of the following reasons:

- Leaving up to the UE implementation to decide the priority: In terms of eliminating the uncertainty, this alternative keeps the uncertainty and varies from time-to-time (i.e., as per UE’s discretion) what is to be prioritized.

- Introducing signalling for the network to decide the priority: The information to be potentially prioritized (e.g., SIB19) takes place during both initial access and connected mode. Thus, using signalling to define priority rules for collision cases 3 and 4 likely will end-up impacting “common control information” for initial access and “UE specific control information” for when the UE has established RRC connection with the network.

Proposal 1: RAN4 to check if RRM requirements could be impacted by SSB periodicity extension to a value larger than 20ms during cell search, including the remaining physical channels and signals (e.g., SIB, paging, etc.).

Proposal 2: RAN4 to check if RRM requirements could be impacted by scaled measurement time delay due to extension of SSB/SMTC/DRX periodicity.

Proposal 3: RAN4 to check if beam steering latency other than 0 may impact RRM requirements.

Proposal 4: RAN4 to check if timing offset/error (between beam switching timing and DL/UL framework) may impact RRM requirements.

Proposal 5: If applicable, the requirements for quasi-earth_fixed cell and earth_ moving cell both shall be studied with respect to the beam hopping scheme.

Proposal 6: No RRM requirements are affected by uplink enhancement.

Proposal 7: Regenerative payload has no effect on RRM requirements as of yet, we may continue to monitor progress in other WGs.

Proposal 8: RRM requirements for RedCap UE in NTN can be started by taking RedCap UE in TN as baseline.

Proposal 9: The requirements and enhancements for NR NTN in Rel-17 and Rel-18 shall be introduced for RedCap UE in NTN.

Proposal 10: 	RedCap and eRedCap UE are less capable devices, by definition and we prefer not to add a more detailed total TA-report at this stage and instead focus on adding a fixed rule in specification, in RAN1, in order to to handle HD-FDD (e)RedCap collision cases in NTN for collision cases 3 & 4.

# References

- RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Huawei

- RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales

- R1-2406808, On HD-FDD RedCap for NTN