3GPP TSG-RAN WG4 Meeting #112	R4-2412235

Maastricht, NL, August 19-23, 2024

Source: 	Ericsson

Title:  	Discussion on RRM requirements for enhanced requirements and test methodology for NR and IoT NTN

Agenda Item:	8.8.3.4

Document for:	Discussion

# Introduction

The WI NR Enhanced requirements and test methodology for NR and IoT NTN, which targets release Rel-19 was approved, which can be found in [1].

In this contribution, we present our initial views on the underlying impacts to RRM requirements mandated by the objectives in the WI.

# Discussion

In the WI, between two objectives, less than 5MHz for NTN has definite request to update RRM requirements, instead, no RRM requirements shall be updated for introduction of high power UE (HPUE) for NTN.

### Observation 1: In the WI, only the objective: less than 5MHz for NTN has a definite demand to update RRM requirements.

## Topic #1: Less than 5MHz for NTN

Objective of Core part WI Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

As a reference, a UE operating on a cell with less than 5MHz BW in TN, in TS38.133 somewhere it is referred as ‘a target cell with 12 PRB SSB’, is required to fulfill the below core requirement updates other than 5MHz or wider BW, which are specified in Rel-18 (captured in R4-2321648, R4-2402624 … and compiled by TS38.133 V18.6.0).

- Handover, Tsearch for interruption time

- Radio Link Monitoring, PDCCH transmission parameters

- Intra/inter-frequency measurements, time period for time index detection

In general, RRM requirements for UE operating on a cell with less than 5MHz BW in NTN can be started from referring to the content in TN. On top of that, the requirements and enhancements for NR NTN, as what we have done for it in Rel-17 and Rel-18, shall be applied as well for UE operating on a cell with less than 5MHz BW in NTN.

Proposal 1: Studying RRM requirements for UE operating on a cell with less than 5MHz BW in NTN can be started from referring to the content in TN, and checking if further requirements may be impacted. As reference, below requirements contains the dedicated ones for UE operating on a cell with less than 5MHz BW in TN.

- Handover, Tsearch for interruption time

- Radio Link Monitoring, PDCCH transmission parameters

- Intra/inter-frequency measurements, time period for time index detection

Proposal 2:  The requirements and enhancements for NR NTN in Rel-17 and Rel-18 shall be introduced and applicable for UE operating on a cell with less than 5MHz BW in NTN.

# Conclusion

### Observation 1: In the WI, only the objective: less than 5MHz for NTN has a definite demand to update RRM requirements.

Proposal 1: Studying RRM requirements for UE operating on a cell with less than 5MHz BW in NTN can be started from referring to the content in TN, and checking if further requirements may be impacted. As reference, below requirements contains the dedicated ones for UE operating on a cell with less than 5MHz BW in TN.

- Handover, Tsearch for interruption time

- Radio Link Monitoring, PDCCH transmission parameters

- Intra/inter-frequency measurements, time period for time index detection

Proposal 2:  The requirements and enhancements for NR NTN in Rel-17 and Rel-18 shall be introduced and applicable for UE operating on a cell with less than 5MHz BW in NTN.

# References

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN