3GPP TSG-RAN WG4 Meeting #112	R4-2412236

Maastricht, NL, August 19-23, 2024

Source: 	Ericsson

Title:  	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Agenda Item:	8.15.2.1

Document for:	Discussion

# Introduction

The WI NR Radio Resource Management (RRM) enhancements (WID in [1]) for Rel-19 has been discussed RAN#4 111 meeting, after the on-line discussion, the conclusions on RRM requirements capturing open issues were list in WF [2].

In this contribution, we present our view on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor as follows.

# Discussion

## Topic #2-1: FR2-1 SSB based L3 measurement delay reduction for connected mode

Issue 2-1-1: Applicability requirement of L3 measurement delay reduction by optimizing Rx BSF

Applicability requirement:

Agreement: Baseline: L3 delay enhancements in Rel-19 by optimizing Rx BSF for UE supporting multi-rx simultaneous reception are applicable provided that: - the target carrier(s) to be measured: only one carrier in the single FR2-1 band is configured for L3 SSB measurement and - UE serving carrier(s): UE is configured with single carrier on FR2-1 band, i.e. FR2-1 PCell without CA/DC. Note: The ‘other UE CA/DC modes (e.g., 1 or 2 FR2-1 bands CA, or FR1+FR2 CA/DC, or EN-DC)’ and/or the ‘other number of target to-be-measured carrier(s) on FR2-1 band’ can be FFS after concluding the baseline above. These extra FFS parts will NOT delay the WI completion.

We aren’t concerned about the agreement. Moreover, we’d like to draw attention to a potential issue as follows if CA/DC configurations will be considered later. In legacy, limited by Rx beam sweeping scheme, a UE is assumed to use same Rx beam when multiple CCs are measured at the same time, and it restricts the applicable configurations of SMTC offset(s) among multiple CCs for measurement delay requirement. In the light of BSF enhancement, it is potentially possible to rethink the validity of the rule and consequent measurement delay requirement.

Observation 1: If CA/DC configurations in the WI is applicable, Rx beam sweeping scheme on multiple CCs may need to be checked, e.g., updating limitation on SMTC offset(s) among multiple CCs.

UE Power class:

FFS the following options and moderator option: - Option 1 (Apple, OPPO, LGE, vivo, HW, Samsung): - RAN4 to consider UE supporting FR2-1 power class 3 as first priority. - Option 1a (LGE): - but RAN4 should consider if other power classes could apply the outcome of the WI discussion - Option 2 (CATT): - It is proposed not to restrict the number of configured carriers and applied power class. - Option 3 (Ericsson): - We can have generic requirements for all power classes, maybe PC6 can be precluded, if no specific use cases for some power classes are pursued. - Option 4 (Moderator) - Baseline: RAN4 to consider UE supporting FR2-1 power class 3 as first priority. - Note: whether other power classes could apply the outcome of the WI discussion can be FFS after concluding on PC3. These extra FFS parts will NOT delay the WI completion.

After some offline discussions in the last meeting, we understand that the main concerns/motivations to prioritize certain power class(es) is gaining the most significant measurement delay reduction. Given that it’s fine with us to prioritize power class 3.

Proposal 1: To gain the most significant measurement delay reduction, we agree to prioritize power class 3.

Issue 2-1-2: Conditions to apply L3 measurement delay reduction by optimizing Rx BSF

- FFS: multi-Rx simultaneous reception of UE is in active mode, which is expected to follow the one specified in Rel-18 for multi-Rx simultaneous reception feature - Option 1 (Apple, CATT, ZTE, vivo, CTC, Intel): multi-Rx simultaneous reception of UE is in active mode, which is expected to follow the one specified in Rel-18 for multi-Rx simultaneous reception feature. - Option 1a (ZTE): The UE is in multi-Rx operation if following condition is met: UE is configured with group-based beam reporting (GBBR) report - Option 1b (Intel): The UE is considered activated in multi-Rx simultaneous reception mode when the UE is configured with group-based beam reporting. The UE is considered activated for L3 reporting when the GBBR is configured not long prior to the expected L3 reporting. - Option 2 (LGE): The L3 measurement delay enhancement requirements for UE supporting multi-Rx simultaneous reception can apply if multiple panels are activate and SSBs in a SMTC window can be measured with multiple beams. - Option 3 (CMCC): the conditions for UE to apply L3 measurement delay reduction by optimizing Rx BSF is that multi-Rx simultaneous reception of UE is in active mode, as for whether the condition is same as that for Rel-18 multi-Rx simultaneous reception can be further discussed. - Option 4 (HW, Nokia, Ericsson): Do not reuse the same applicable conditions specified in Rel-18 multi-Rx - Option 4a (QC) : R19 Rx BSF is independent to R18 multi-Rx simultaneous reception feature includes conditions and requirement. - Option 5 (MTK): The conditions in R18 multi-Rx WI do not limit the discussion on having further conditions in R19 for UE supporting multi-Rx to enhance FR2-1 SSB based L3 measurement delay

- FFS: Rel-19 L3 measurement with multi-Rx DL reception is irrelevant to multi-TRP operation deployment - Option 1 (vivo, Samsung): Rel-19 L3 measurement with multi-Rx DL reception is irrelevant to multi-TRP operation deployment

- FFS: cell-centre UE or cell-edge UE - Option 1 (Nokia, MTK): RAN4 to consider L3 FBS targeting cell edge scenarios

What we observe is that the statement ‘the one specified in Rel-18 for multi-Rx simultaneous reception feature’ only reflects the common understanding that the receiver architecture supporting multi-Rx simultaneous reception in Rel-18 could be reused as a baseline BSF optimization. But, from requirement perspective, multi-Rx for L3 and multi-Rx for L1 are irrelative.

Observation 2: Measurement enhancement by FBS may reuse full or part of the receiver architecture supporting multi-Rx simultaneous reception in Rel-18. However, multi-Rx for L3 and multi-Rx for L1 are irrelative features.

L1 measurement enhancement by multi-Rx in Rel-18 targeting multi-TRP scenario enables simultaneously data reception (e.g. PDSCH) from multiple TRPs. Apparently, it has no direct relationship with the L3 measurement enhancements by BSF optimization. They may happen at the same time or at a different time. In our earliest thought in the contribution for last meeting, the scenario of FBS for L3 measurement is to speed up L3 measurement for mobility (e.g., cell change) and it may mostly happen at cell edge. After some discussions in the last meeting, at least one company proposed that FBS for L3 measurement may happen in cell center also for other benefits, e.g., scheduling restriction reduction (which has not been proven yet). We generally agree on the possibility.

Proposal 2: As a feature, measurement enhancement by FBS doesn’t follow multi-Rx simultaneous reception for L1 measurement enhancement in Rel-18. Also, measurement enhancements by FBS can’t reuse the applicable conditions (e.g., multi-TRP, GBBR) specified in multi-Rx in Rel-18.

- FFS: Simultaneous operation between L3 and L1 measurements - Option 1 (ZTE): Simultaneous operation between L3 and L1 measurements by optimizing Rx BSF, simultaneous operation between L3 measurement and data reception by optimizing Rx BSF - Option 2 (Ericsson): Scenario where L3 measurement is reduced using reduced beam sweeping and scenario where L1 measurement is reduced using multiple-reception from multi-TRP for DL measurement/data are different scenarios and not expected to operate simultaneously. - L3 measurement delay reduction may be influenced by L1 measurement/data transmission scheme, wherein L1 measurement may be: - Legacy requirements or, - Enhanced requirements for Multi-Rx in Rel-18 - We prefer legacy requirements between them. Once L1 measurement is chosen, we shall further check L1/L3 sharing scheme.

Nevertheless, we recommend isolating multi-Rx for L1 measurement enhancement in Rel-18 and FBS for L3 measurement enhancement in Rel-19. One reason is to simplify the conditions for L3 measurement enhancement, another reason is to avoid in-depth study on harmonizing parallel L1 measurement enhancement and L3 measurement enhancement which may introduce more complexity. In a consistent manner, L3 measurement enhancement by FBS shall not take the conditions for multi-Rx for L1 measurement enhancement in Rel-18 e.g., multi-TRP, GBBR, into account.

Proposal 3: For sake of simplification while considering the target scenarios, when Rel-19 L3 measurement enhancement is enabled, it is assumed that L1 measurement enhancement in Rel-18 doesn’t work simultaneously. Subsequently, the sharing factor between L3 and L1 is defined with respect to the assumption of L3 measurement applying FBS and L1 measurement not applying multi-Rx in Rel-18, so legacy ﻿Psharing factor is applied.

Proposal 4: For sake of simplification, we don’t expect simultaneous operation between L3 measurement and data reception, if its interpretation is that L3 measurement on one panel and data reception on another panel on same resources at same time.

Given the aforementioned assumptions, a situation as one of results may occur is that a UE may switch between multi-Rx in Rel-18 and FBS in Rel-19 if both features are supported by NW and UE.

Observation 3: A UE may switch between multi-Rx in Rel-18 and FBS in Rel-19 if both features are supported by NW and UE.

As a result of the above analysis, it’s unclear if a UE needs transition time period between the operation of multi-Rx in Rel-18 and the operation of FBS in Rel-19, and between enabling and disabling FBS in Rel-19.

Proposal 5: RAN4 to study the requirements relevant to switching between different operations, including:

- Transition time period when enabling and disabling FBS in Rel-19.

- Transition time period when switching between FBS in Rel-19 and multi-Rx in Rel-18.

- FFS: UE’s mobility status, e.g., whether HST is precluded or not - Option 1 (Apple, ZTE(preclude HST), vivo, CTC, Samsung(preclude HST)): low mobility status, i.e., preclude HST - Option 2 (CATT, CMCC): enhanced BSF can also be used for HST - Option 3 (Ericsson): no scenario restriction (e.g., low mobility) is needed - Option 4 (MTK): For UE with high mobility

Regarding the issue, we have no preference on scenario. Since HST is indicated by at least one company, we express our view on it here. We understand HST may get benefits from BSF optimization, but it shall be noted that introducing HST support may cause the need of in-depth analysis on HST FR2 scenarios (set A/B, bi/uni-directional deployment).  In our understanding low mobility is the dedicated low mobility scenario in TS38.133 which is not identical with non-HST. That’s the reason we mentioned in the last meeting that low mobility isn’t mandatory.

Proposal 6: RAN4 to prioritize FBS for legacy normal (non-HST) requirements. FBS in HST is feasible from technical perspective but it might not be the WI's main concern.

- FFS: RRM measurement with two panels activated, two searchers are occupied by this single carrier - Option 1 (Apple): the legacy searcher assumption and legacy CSSF shall still be applied for L3 RRM measurement with two panels activated - Option 1a (Ericsson): - The WI shall prioritize the use case of the same receiver for search and measurement processing on one carrier simultaneously received from multiple panels, e.g. a single searcher receives and processes the same carrier on multiple panels. - Option 2 (vivo, HW): RRM measurement with two panels activated, two searchers are occupied by this single carrier

We understand two searchers occupied by a single carrier may enable such a measurement mechanism: one searcher for the search and measurement at one carrier on one panel and the other searcher for the search and measurement at same carrier on the other panel. If the interpretation is correct, we agree that it is technically valid for 2 carriers’ reception.

However, such configuration may depend on UE implementation. We assume the WI’s target is achieved by applying the same receiver to simultaneously measure the single carrier on multiple panels, e.g. a single searcher processes the same carrier received from multiple panels. We don’t see the advantage on measurement requirements when applying two searchers for single carrier in the target scenario ‘UE is configured with single carrier on FR2-1 band. only one carrier in the single FR2-1 band is configured for L3 SSB measurement’. Apparently, if two searchers are occupied by this single carrier, it may impact CSSF in CA/DC configurations but, as what we have agreed in other issues, CA/DC configurations shall not be the dependence of the study in the WI.

Proposal 7: Support Option1/1a. One searcher is able to handle the single carrier received from multiple panels, targeting the scenario: ‘UE is configured with single carrier on FR2-1 band. Only one carrier in the single FR2-1 band is configured for L3 SSB measurement.’

- FFS: SSB processing delay/time for processing multiple beams received in a SMTC - Option 1 (vivo, QC, Intel, Apple): needs SSB processing delay/time for processing multiple beams received in a SMTC

Other clarification on WID:

- FFS: - “For UE supporting multiple-Rx simultaneous reception for L3 delay enhancement” means UE supporting “simultaneous reception of multiple SSBs from different directions of the same target frequency layer inside a SMTC window. But it does not mean “UE can process multiple SSBs from different directions of the target frequency in parallel.”

We suppose the concern relies on the UE capability. In legacy, processing time may be needed instantly after the whole measurement time period, like ﻿Tmargin for SSB post-processing in handover interruption time, wherein Tmargin can be up to 2ms. On the basis of that, extra processing time in several ms may be acceptable depending on the gain by BSF enhancement.

Proposal 8: Extra SSB post-processing time in several ms, as side effect of BSF enhancement, may be acceptable, but it depends on how much we can gain with BSF enhancement.

- FFS: Power consumption issue - Option1 (Apple): considering power consumption, BSF reduction of L3 measurement will not trigger UE to activate multi-Rx. - Option 2 (LGE): For power consumption of multi-Rx operation, Rel-18 UAI ‘multiRx-PreferenceFR2’ for power saving can be considered as starting point. - Option 3 (Ericsson): To avoid unnecessary power consumption and computation load, enabling/disabling the parallel L3 measurement on multiple panels (if it is one of solutions addressing L3 measurement delay) may be determined by at the least one of the below options: - Option 3-1: NW indicates UE enabling parallel L3 measurement on multiple panels for serving L3 measurement delay reduction through L3 or lower layers signalings. - Option 3-2: UE determines to apply parallel L3 measurement on multiple panels for serving L3 measurement delay reduction, and enable it after acknowledged by NW. - Option 3-3: UE determines to apply parallel L3 measurement on multiple panels for serving L3 measurement delay reduction if a condition is fulfilled, e.g., at cell edge. NW may be aware of it by sending a ‘allowance’ signalling or not aware of it. - Option 4 (Nokia): Discuss FBS triggering conditions among the following options: - Option 4-1: Network configuration of FBS (similar as option 3-1) - Option 4-2: Mobility Event triggering FBS - Option 4-3: Conditional Handover configuration

We agree that the extra unneglectable power consumption for FBS is a valid concern, as one of the factors determining enabling/disabling FBS, to a UE. As reference, multi-Rx in Rel-18 assumes GBBR configuration from NW is the indication to enable multi-Rx simultaneous reception.

In this WI, we understand one of use cases of FBS is enabling fast measurement for mobility, it may happen when a UE receives low/poor signal level from the serving cell and is expected to change the serving cell. In this case, the NW serving the UE determines the UE to execute handover with respect to the received measurement reports.

Furthermore, since another use case of FBS is abbreviating scheduling restriction on a UE, it seems sense that enabling/disabling FBS shall be awared or indicated by the NW for determining scheduling to the UE.

Given above, it is rational to let the NW determine enabling/disabling the UE’s FBS, since the NW determines (e.g., by configurations and commands) the mobility and scheduling scheme of the UE.

In such sense, we think at least the below indication approaches can be applied for enabling/disabling FBS.

- Option 1: NW indicates the UE enabling/disabling FBS through L3 or lower layers signaling.

- Option 2: UE requests/reports enabling/disabling FBS to NW and NW acknowledge it.

- Option 3: UE determines FBS if a condition is fulfilled.

Observation 4: Enabling/disabling FBS may be determined by at the least one of the below options:

- Option 1: NW indicates UE enabling/disabling FBS through L3 or lower layers signaling.

- Option 2: UE requests/reports enabling/disabling FBS to NW and NW acknowledge it.

- Option 3: UE determines FBS if a condition is fulfilled.

Proposal 9: NW indicates UE enabling/disabling FBS through L3 or lower layers signaling.

- FFS: UE has prior knowledge on the cell to be measured - Option 1 (CATT): on top of the UE capability of supporting Multi-Rx, no additional conditions of prior knowledge for target cell is needed - Option 2 (vivo, CTC, MTK): UE needs prior knowledge on the cell to be measured - Option 2a (CTC): It’s proposed to consider conditions of prior knowledge on the cell to be measured and discuss whether the conditions of prior knowledge are applicable - Option 3 (Ericsson): UE may only measure part of spatial directions with one panel out of multiple panels. It reduces L3 measurement delay as well, upon acquiring prior knowledge on the cell to be measured, e.g., - The UE has measured the cell before in a time period. - The UE has knowledge on the absolute/relative location of the cell to be measured. - The UE has knowledge on its moving state (including rotation).

As various solutions serving the purpose, besides of parallel L3 measurement on multiple (e.g., 2) panels which have been described in the above context, we do recommend that other features shall be exploited.

For a UE, one of approaches to reduce L3 measurement delay by optimizing Rx BSF is relying on any prior knowledge on the cell to be measured. In particular, the UE has measured the cell before in a time period. Upon acquiring such information, the UE may only measure less spatial directions with one panel or multiple panels. It can reduce L3 measurement delay as well.

Proposal 10: A UE may only measure less spatial directions with one panel or multiple panels upon acquiring prior knowledge on the cell to be measured, e.g., The UE has done measurements before in a time period.

Deep into the particular requirements of intra/inter-frequency measurement, Rx beam sweeping factor appears more than one time in the Tidentify_intra_without_index formula, separately in the items: Tpss/sss_sync , TSSB_time_index_inter and Tssb_measurement_period. Under certain conditions, the Rx beam sweeping factor in the subsequent operation can be reduced compared to the full (legacy) Rx beam sweeping factor in the prior operation because the UE already acquires the Rx beam information in the prior operation.

Proposal 11: As a particular example of the last proposal, apply reduced Rx beam sweeping in the subsequent operation(s) compared to the full (legacy) Rx beam sweeping in the prior operation(s), e.g., in SSB based Intra/inter-frequency measurement, apply reduced Rx beam sweeping factor consequently in Tpss/sss_sync, TSSB_time_index_inter and Tssb_measurement_period.

- FFS: DRX is configured or not (newly added issue in this meeting based on Xiaomi’s proposal) - Option 1 (Xiaomi): SSB based L3 measurement delay reduction with DRX shall be deprioritized

We understand the motivation, but deprioritizing DRX case doesn’t gain anything.

Proposal 12: FBS may cover DRX cases, no need to deprioritize DRX case.

In some manner or other, the UE capabilities designed for the operation of Multi-Rx in Rel-18 may not be totally applicable for L3 measurement enhancement by FBS. At least one company proposed to build totally new UE capabilities different from multi-Rx in Rel-18, but a common understanding is that we shall maximally reuse the existing capabilities, surely if we can avoid introducing mess or misunderstanding.

Proposal 13: To support FBS, RAN4 to check if those capabilities for multi-Rx in Rel-18, e.g., faster RX beam sweeping, enhanced scheduling and measurement restrictions and multi-Rx preference indication, can be used directly.  If the NW indicates FBS and multi-RX in Rel-18 not parallelly, we can observe the possibility of reusing at least part of such capabilities.

One solution to reduce L3 measurement delay is executing parallel L3 measurement on multiple (e.g., 2) panels at UE. Then, the straightforward result is that RX beam sweep factor is 4, which is half of legacy number 8.

Proposal 14: As a result of FBS, RX beam sweep factor can be defined as [4].

FFS on following scenarios to decide which ones are considered to use L3 measurement delay reduction by optimizing Rx BSF:

FFS：Scenarios to use L3 measurement delay reduction by optimizing Rx BSF: - FFS: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - FFS: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - FFS: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - FFS: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - FFS: Handover - FFS: PSCell addition - FFS: RRC Re-establishment/RRC Connection Release with Redirection - FFS: SCell activation - FFS: SCG activation - FFS: CGI identification - FFS: CSI-RS based intra-/inter-frequency measurements, the CSI-RS is configured associatedSSB - The discussion on CSI-RS configured with associatedSSB could be revisited if SSB based L3 measurement delay reduction is concluded.

As stated in the above analysis, L3 measurement is the main target of the WI, e.g., for the UE mobility or scheduling restriction reduction. On top of that, at least one company brought up a concern in the last meeting indicating that the FBS solution shall take such scenarios into account. To our opinion, intra/inter-frequency measurements and handover shall be prioritized accordingly.

Proposal 15: Prioritize the following requirements since they are most important to improve mobility latency.

- SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter

- SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter,  TSSB_time_index_inter and TSSB_measurement_period_inter

- Handover

Apart from the aformentioned cases, we also are open to other cases if noticeable value is observed.

# Conclusion

Observation 1: If CA/DC configurations in the WI is applicable, Rx beam sweeping scheme on multiple CCs may need to be checked, e.g., updating limitation on SMTC offset(s) among multiple CCs.

Observation 2: Measurement enhancement by FBS may reuse full or part of the receiver architecture supporting multi-Rx simultaneous reception in Rel-18. However, multi-Rx for L3 and multi-Rx for L1 are irrelative features.

Observation 3: A UE may switch between multi-Rx in Rel-18 and FBS in Rel-19 if both features are supported by NW and UE.

Observation 4: Enabling/disabling FBS may be determined by at the least one of the below options:

- Option 1: NW indicates UE enabling/disabling FBS through L3 or lower layers signaling.

- Option 2: UE requests/reports enabling/disabling FBS to NW and NW acknowledge it.

- Option 3: UE determines FBS if a condition is fulfilled.

Proposal 1: To gain the most significant measurement delay reduction, we agree to prioritize power class 3.

Proposal 2: As a feature, measurement enhancement by FBS doesn’t follow multi-Rx simultaneous reception for L1 measurement enhancement in Rel-18. Also, measurement enhancements by FBS can’t reuse the applicable conditions (e.g., multi-TRP, GBBR) specified in multi-Rx in Rel-18.

Proposal 3: For sake of simplification while considering the target scenarios, when Rel-19 L3 measurement enhancement is enabled, it is assumed that L1 measurement enhancement in Rel-18 doesn’t work simultaneously. Subsequently, the sharing factor between L3 and L1 is defined with respect to the assumption of L3 measurement applying FBS and L1 measurement not applying multi-Rx in Rel-18, so legacy ﻿Psharing factor is applied.

Proposal 4: For sake of simplification, we don’t expect simultaneous operation between L3 measurement and data reception, if its interpretation is that L3 measurement on one panel and data reception on another panel on same resources at same time.

Proposal 5: RAN4 to study the requirements relevant to switching between different operations, including:

- Transition time period when enabling and disabling FBS in Rel-19.

- Transition time period when switching between FBS in Rel-19 and multi-Rx in Rel-18.

Proposal 6: RAN4 to prioritize FBS for legacy normal (non-HST) requirements. FBS in HST is feasible from technical perspective but it might not be the WI's main concern.

Proposal 7: Support Option1/1a. One searcher is able to handle the single carrier received from multiple panels, targeting the scenario: ‘UE is configured with single carrier on FR2-1 band. Only one carrier in the single FR2-1 band is configured for L3 SSB measurement.’

Proposal 8: Extra SSB post-processing time in several ms, as side effect of BSF enhancement, may be acceptable, but it depends on how much we can gain with BSF enhancement.

Proposal 9: NW indicates UE enabling/disabling FBS through L3 or lower layers signaling.

Proposal 10: A UE may only measure less spatial directions with one panel or multiple panels upon acquiring prior knowledge on the cell to be measured, e.g., The UE has done measurements before in a time period.

Proposal 11: As a particular example of the last proposal, apply reduced Rx beam sweeping in the subsequent operation(s) compared to the full (legacy) Rx beam sweeping in the prior operation(s), e.g., in SSB based Intra/inter-frequency measurement, apply reduced Rx beam sweeping factor consequently in Tpss/sss_sync, TSSB_time_index_inter and Tssb_measurement_period.

Proposal 12: FBS may cover DRX cases, no need to deprioritize DRX case.

Proposal 13: To support FBS, RAN4 to check if those capabilities for multi-Rx in Rel-18, e.g., faster RX beam sweeping, enhanced scheduling and measurement restrictions and multi-Rx preference indication, can be used directly.  If the NW indicates FBS and multi-RX in Rel-18 not parallelly, we can observe the possibility of reusing at least part of such capabilities.

Proposal 14: As a result of FBS, RX beam sweep factor can be defined as [4].

Proposal 15: Prioritize the following requirements since they are most important to improve mobility latency.

- SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter

- SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter,  TSSB_time_index_inter and TSSB_measurement_period_inter

- Handover

Apart from the aformentioned cases, we also are open to other cases if noticeable value is observed.

# References

- RP-240830, New WID: WI resulting from discussion on RRM enhancements, vivo (Moderator)

- R4-2406392, WF for [110bis][230] NR_RRM_Ph5, Apple