3GPP TSG-RAN WG4 Meeting #112	R4-2412237

Maastricht, NL, August 19-23, 2024

Source: 	Ericsson

Title:  	Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Agenda Item:	8.15.2.2

Document for:	Discussion

# Introduction

The WI NR Radio Resource Management (RRM) enhancements (WID in [1]) for Rel-19 has been discussed RAN#4 111 meeting, after the on-line discussion, the conclusions on RRM requirements capturing open issues were list in WF [2].

In this contribution, we present our view on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC as follows.

# Discussion

## Topic #2-2 L3 measurement delay reduction by optimizing CSSF for UE not in multiple-Rx simultaneous reception mode

Issue 2-2-1: Clarification on the bullets in WID for this CSSF optimization

Agreement: Rel-19 discussion on CSSF optimization starts for the case UE is not capable of Rel-18 multi-Rx simulaeous reception, further discuss whether/how it can be applied to the case UE is capable of Rel-18 multi-Rx simulaeous reception but work in single-Rx currently.

Recall the relative objective in WID in below.

- For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios Baseline assumption on number of searchers is 2

Some relevant proposal summaries in last meeting are presented as follows.

FFS: - Option 1 (Apple): Rel-19 CSSF optimization applies for the both cases: (1)UE is not capable of Rel-18 multi-Rx simultaneous reception, (2)UE is capable of Rel-18 multi-Rx simultaneous reception but work in single-Rx currently. - Option 2 (ZTE): Besides the case UE is not capable of R18 multi-Rx simultaneous reception, R19 CSSF optimization is also applied to the case that UE is capable of multi-Rx but not configured with GBBR report. - Option 3 (Ericsson): Clarify whether ‘the case UE is capable of Rel-18 multi-Rx simultaneous reception but work in single-Rx currently’. - Does it indicate that the UE only can use a single panel (and subsequent receiver including baseband) out of multiple panels for reception and measurement? If so, is there any degradation of reception and measurement from measuring on a single panel? - Option 4 (Nokia): Rel-19 discussion on the scenarios for CSSF optimization will be considered in CA/DC scenarios, independently of the UE support of multi-Rx capabilities.

It appears that the agreement is to deal with the concern that ‘the case UE is capable of Rel-18 multi-Rx simultaneous reception but work in single-Rx currently’ may have different capability from ‘the case UE is not capable of Rel-18 multi-Rx simultaneous reception’.

Based on the agreement, we suggest deprioritizing ‘the case UE is capable of Rel-18 multi-Rx simultaneous reception but work in single-Rx currently’ since the UE capability in this case and the impact to requirements are unclear. If such a case really results in different CSSF requirement from ‘the case UE is not capable of Rel-18 multi-Rx simultaneous reception’, we may initiate to study it. But we haven’t seen the proof at least until now.

Proposal 1: RAN4 to deprioritize ‘the case UE is capable of Rel-18 multi-Rx simultaneous reception but work in single-Rx currently’.

Issue 2-2-2: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF

Agreement: - Only consider CSSF outside MG case. Proposal for further discussion: - Scenarios to use L3 measurement delay reduction by optimizing CSSF: - SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter and TSSB_measurement_period_inter

FFS: - Option 1: the following scenarios in CA/DC to use L3 measurement delay reduction by optimizing CSSF shall be prioritized: - SSB based Intra-frequency measurement without MG (Apple, CATT, CMCC, vivo, CTC, HW, Nokia, MTK) - Alt1: including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Alt2: including TPSS/SSS_sync_intra, TSSB_time_index_intra and TSSB_measurement_period_intra - SSB based Inter-frequency measurement without MG, (Apple, CATT, CMCC, vivo, CTC, HW, Nokia, MTK) - Alt1: including TPSS/SSS_sync_inter and TSSB_measurement_period_inter - Alt2: including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Inter-RAT SSB measurement without MG (CMCC, Ericsson) - NeedForGaps measurement without MG, including both with and without interruption (Ericsson) - NCSG measurement without MG without interruption (Ericsson)

As we know, from Rel-16 to Rel-18, 3GPP spent a long time to develop different types of measurement without gap features as follow.

- Rel-16 NeedForGaps

- Rel-17 NCSG

- Rel-18 NeedForInterruption

- Rel-18 Inter-RAT measurement wo MG

In our understanding, all these features are meaningful and can be applied by CSSF outside MG. Some of the features have already been implemented by products and some of the features are highly possible to be developed. Thus, we propose to further consider the measurement without gap features into the Rel-19 CSSF outside MG optimization WI.

Proposal 2: RAN4 to study the following scenarios applying L3 measurement delay reduction by optimizing CSSF:

- SSB based Intra-frequency measurement without MG

- SSB based Inter-frequency measurement without MG

Proposal 3: RAN4 further to take below scenarios into account:

  - FFS on NeedForGaps measurement without MG, including both with and without interruption

  - FFS on NCSG measurement without MG without interruption

  - FFS on Inter-RAT measurement without MG

Issue 2-2-3: Applicability requirement of L3 measurement delay reduction by optimizing CSSF

FFS: - Option 1 (Apple, CATT, OPPO, CMCC, ZTE): RAN4 to consider following CA/DC mode for L3 measurement delay reduction by optimizing CSSFoutside_gap,i - EN-DC (Apple, CATT, OPPO, CMCC, ZTE, Ericsson): - Intel proposed to deprioritize EN-DC - NE-DC (Apple, CATT, OPPO, CMCC, ZTE, Intel, Ericsson): - SA (Apple, CATT, OPPO, CMCC, ZTE, CTC, Intel, HW): - FR2+FR2 CA (HW), FR1+FR2 CA (HW, Ericsson), FR2 only intra band CA (Ericsson), FR2 only inter band CA (Ericsson) - NR-DC (Apple, CATT, OPPO, CMCC, ZTE, CTC, Intel) - FR1+FR2 NR-DC (HW, Ericsson)

As per the available formulas of CSSF, one question shall be clarified is what is the scope of CSSF for ‘FR2-1 L3 measurement’. With reference to the CSSF definition in TS38.133, we may conclude that CSSF optimization only considers the below configurations: FR2 only intra band CA, FR2 only inter band CA, FR1+FR2 CA, FR1+FR2 NR-DC, and also EN-DC or NE-DC cases.

Proposal 4: CSSF optimization considers the configuration below: FR2 only intra band CA, FR2 only inter band CA, FR1+FR2 CA, FR1+FR2 NR-DC, and also EN-DC or NE-DC cases.

Proposal 5: RAN4 to check contiguous and/or non-contiguous FR2 CC configuration in CSSF optimization.

﻿Traditionally, the assumption of searchers allocation is that one searcher for PCC and the other one is for other (P)SCC(s). In terms of the assumption, what we can optimize CSSF may comprise the below options:

- Reuse measurements on one CC for another or other multiple CC.

- Prioritize one or multiple CC among all CCs.

First of all, we shall check the issue as follows. In the last meeting, a viewpoint was raised to prove that UE can determine the measurements on one CC or more than one CC, referring to the table below which is duplicated from TS 38.133.

- 9.2.3.2 Requirements for FR2 For one single intra-frequency layer in a band, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least: - 6 identified cells, and - 24 SSBs with different SSB index and/or PCI, where this single intra-frequency layer shall be: - PCC when UE is configured with SA NR operation mode with PCC in the band; or - PSCC when UE is configured with EN-DC with PSCC in the band; or - PSCC when UE is configured with NR-DC with PSCC in the band; or - One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation. The UE shall also be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least 2 SSBs on serving cell for each of the other intra-frequency layer(s) in the same band.

We understand the statements indicate rules for determining which SCC shall be reported for one single intra-frequency layer in a band, which is a limitation for a FR2 UE in legacy Rel-15 requirements. But eventually it can't be simply referred to as the natural assumption of CSSF enhancement in this WI. Apparently, in this WI, we suggest checking the validity and seeking for any enhancement for the study target.

Apparently, the NW hasn’t any information on the channel quality of the whole band if the UE doesn’t report SSBs on each  intra-frequency layer(s) in serving cell(s), but the same or similarity of channel quality on the whole band isn’t always valid. In this sense, the NW expects to acquire channel quality of the other intra-frequency layer(s) other than the one reported by the UE, for further scheduling and management improvements. Above all, from NW perspective, it is rational that the NW may determine the serving cell(s) to be measured, if necessary, the NW may signal the UE of updating measurement configuration to measure another serving cell(s).

Observation 1: For CSSF enhancement, it is possible for UE only to measure one serving CC in one band in case there are multiple CCs in the band.  But on the contrary, we do believe that NW as well can configure/indicate measurement only for one serving CC in the band to UE.

Proposal 6: RAN4 to study the mechanism of CSSF optimization, indicated by NW configuration/indication, enabling/disabling measurement on particular CC(s).

To the end, some detailed thoughts are presented as follows for designing the scheme of measurement (including reporting) configurations on all CCs and the indicated CC(s).

Proposal 7: If UE only measures one CC (e.g., determined by either UE or NW) out of multiple CCs, regarding measurement (including reporting) configurations to the CC and other CC(s), some promising approaches to indicate the CC(s) to be measured (or not to be measured) are listed as follows:

- Option 1: NW measurement configuration only covers the CC to be measured, i.e., doesn’t cover the CC(s) not to be measured.

- Option 2: NW measurement configuration covers all CC, by further (e.g. dynamical) indication,

  - Option 2.1. No measurement report or measurement configuration is configured for the CC(s) which are not to be measured.

  - Option 2.2: If measurement configuration is configured for the CC(s) which not to be measured, the report on the CC(s) reuses the measured result of the CC to be measured.

Yet another further question is, we shall clarify the scope of CSSF enhancement.

From the understanding to the on-line discussions in the last meeting, until now, the scope focused on reducing CSSF for SCC(s) where neighbor cell measurement isn’t required. Since NSCC_SSB in CSSF definition table in TS 38.133 mainly appears in the formula for the case in which neighbor cell measurement isn’t required, we may focus on such case.

Proposal 8: RAN4 to clarify the scope of CSSF enhancement, e.g., the scope covers the below:

- Includes CSSF for SCCs where neighbor cell measurement isn’t required.

- Doesn’t include CSSF on SCCs where neighbor cell measurement is required.

- Doesn’t include CSSF on SCCs for inter-frequency without gap.

Another approach optimization CSSF is prioritizing one or more than one CC in CSSF provided there are multiple CCs to be measured. ‘Prioritizing’ herein is increasing the ratio of measurement.

Proposal 9: CSSF enhancement also includes: prioritizing the CSSF for one or more than one CC out of multiple CCs to be measured, i.e., different CSSF inequivalently applies on different CCs.

Since the purpose of the objective is to reduce the number of CSSF, the importance is evident only when the original CSSF number or CC number is large. Otherwise, for example, CSSF=2 or CC number=2 can’t show the advantage to the measurement delay requirements compared to the conventional CSSF formula.

Proposal 10: RAN4 to study the minimal CC number to apply CSSF enhancement.

## Topic #2-3 Common aspects for L3 measurement delay reduction

Issue 2-3-1: whether and/or which previous release feature shall also be considered in “FR2-1 SSB based L3 measurement delay reduction for connected mode”

FFS in next meeting: - Moderator note: - encourage companies to provide view on which features in previous release shall be considered for “FR2-1 SSB based L3 measurement delay reduction for connected mode” by optimizing Rx beam sweeping factor - encourage companies to provide view on which features in previous release shall be considered for “FR2-1 SSB based L3 measurement delay reduction for connected mode” by optimizing CSSF outside gap

FFS: - Option 1 (Apple): - for “FR2-1 SSB based L3 measurement delay reduction for connected mode” by optimizing Rx beam sweeping factor, R18 feature of FR2 multi-Rx reception shall be considered - for “FR2-1 SSB based L3 measurement delay reduction for connected mode” by optimizing CSSF outside gap, both R16 inter-frequency measurement without MG and R18 inter-RAT measurement without MG shall be considered. - Option 2 (LGE, QC): - RAN4 not to consider SSB based L3 measurement delay enhancement with previous release features. - R19 L3 measurement enhancement for both fast beam sweeping and CSSF optimization is independent to R18 multi-Rx feature. (QC) - Option 3 (Ericsson): - Only Rel-18 multi-Rx scenario should be assumed, and we should not assume that the UE is also supporting other features.

Proposal 11: Regarding FBS, only Rel-18 multi-Rx scenario should be assumed, and we should not assume that the UE is also supporting other features. But it is noted that it doesn’t mean L1 measurement enhancement by multi-Rx in Rel-18 is mandatory for FBS.

Proposal 12: Regarding CSSF enhancement, below features shall be checked:

- Rel-16 NeedForGaps

- Rel-17 NCSG

- Rel-18 NeedForInterruption

- Rel-18 Inter-RAT measurement wo MG

# Conclusion

Observation 1: For CSSF enhancement, it is possible for UE only to measure one serving CC in one band in case there are multiple CCs in the band.  But on the contrary, we do believe that NW as well can configure/indicate measurement only for one serving CC in the band to UE.

Proposal 1: RAN4 to deprioritize ‘the case UE is capable of Rel-18 multi-Rx simultaneous reception but work in single-Rx currently’.

Proposal 2: RAN4 to study the following scenarios applying L3 measurement delay reduction by optimizing CSSF:

- SSB based Intra-frequency measurement without MG

- SSB based Inter-frequency measurement without MG

Proposal 3: RAN4 further to take below scenarios into account:

  - FFS on NeedForGaps measurement without MG, including both with and without interruption

  - FFS on NCSG measurement without MG without interruption

  - FFS on Inter-RAT measurement without MG

Proposal 4: CSSF optimization considers the configuration below: FR2 only intra band CA, FR2 only inter band CA, FR1+FR2 CA, FR1+FR2 NR-DC, and also EN-DC or NE-DC cases.

Proposal 5: RAN4 to check contiguous and/or non-contiguous FR2 CC configuration in CSSF optimization.

Proposal 6: RAN4 to study the mechanism of CSSF optimization, indicated by NW configuration/indication, enabling/disabling measurement on particular CC(s).

Proposal 7: If UE only measures one CC (e.g., determined by either UE or NW) out of multiple CCs, regarding measurement (including reporting) configurations to the CC and other CC(s), some promising approaches to indicate the CC(s) to be measured (or not to be measured) are listed as follows:

- Option 1: NW measurement configuration only covers the CC to be measured, i.e., doesn’t cover the CC(s) not to be measured.

- Option 2: NW measurement configuration covers all CC, by further (e.g. dynamical) indication,

  - Option 2.1. No measurement report or measurement configuration is configured for the CC(s) which are not to be measured.

  - Option 2.2: If measurement configuration is configured for the CC(s) which not to be measured, the report on the CC(s) reuses the measured result of the CC to be measured.

Proposal 8: RAN4 to clarify the scope of CSSF enhancement, e.g., the scope covers the below:

- Includes CSSF for SCCs where neighbor cell measurement isn’t required.

- Doesn’t include CSSF on SCCs where neighbor cell measurement is required.

- Doesn’t include CSSF on SCCs for inter-frequency without gap.

Proposal 9: CSSF enhancement also includes: prioritizing the CSSF for one or more than one CC out of multiple CCs to be measured, i.e., different CSSF inequivalently applies on different CCs.

Proposal 10: RAN4 to study the minimal CC number to apply CSSF enhancement.

Proposal 11: Regarding FBS, only Rel-18 multi-Rx scenario should be assumed, and we should not assume that the UE is also supporting other features. But it is noted that it doesn’t mean L1 measurement enhancement by multi-Rx in Rel-18 is mandatory for FBS.

Proposal 12: Regarding CSSF enhancement, below features shall be checked:

- Rel-16 NeedForGaps

- Rel-17 NCSG

- Rel-18 NeedForInterruption

- Rel-18 Inter-RAT measurement wo MG

# References

- RP-240830, New WID: WI resulting from discussion on RRM enhancements, vivo (Moderator)

- R4-2406392, WF for [110bis][230] NR_RRM_Ph5, Apple