3GPP TSG RAN WG4 Meeting #112	R4-2412243

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:			5.13.1

Source:	vivo

Title:	Remaining issues for FR2 multi-Rx core maintenance

Document for:	Discussion

# Introduction

In the RAN4#110bis meeting, there were discussions on core part for multi-Rx. The agreements and open issues were captured in the WF [1].

<Core maintenance> - For fast beam sweeping, the UE is in multi-Rx operation if following condition is met: - UE is configured with group-based beam reporting (GBBR) report. - For scheduling restriction, remove “UE is multi-Rx operation” from the spec. - For measurement restriction: - Way forward: Companies may bring further analysis, on whether to remove or not the following condition of measurement restriction relaxation for CSI-RS based L1 measurements - [The two CSI-RS resources and both PDSCHs are overlapped on the same OFDM symbol].

In this contribution, we further provide our views on remaining open issues for FR2 multi-Rx chain DL reception.

# Discussion

## 2.1 Condition of fast beam sweeping

A new UE capability of fast beam sweeping for layer-1 measurement when the UE is in multi-Rx operation was introduced for multi-Rx. It was agreed that for fast beam sweeping the UE is in multi-Rx operation if the UE is configured with group-based beam reporting (GBBR) report. In our understanding, the purpose is to enable fast beam sweeping in multi-TRP operation as GBBR is mainly for multi-TRP operation. The GBBR, however, is used for NW acquiring TCI states for multi-TRP and then scheduling the UE with multiple PDSCHs. It seems that fast beam sweeping could be enabled in two ways. One is when NW operates in multi-TRP mode to schedule the UE so that GBBR is always configured for tracking and updating TCI states. Another is NW does not intend to operate in multi-TRP mode and GBBR is configured only for enabling fast beam sweeping.

For the first one, there should be no problem for the UE. However, the second one would be problematic since GBBR may be configured for nothing but triggering fast beam sweeping. It is not expected as it costs UE power for GBBR measurements unnecessarily. It would be helpful for UE to have an exit condition. On the other hand, it also depends on how fast beam sweeping is implemented by UE, e.g., single-panel based or multi-panel based. If it is single-panel based, then it actually doesn’t need NW to configure GBBR at all to start fast beam sweeping. If it is multi-panel based, triggering by GBBR configuration is necessary. Since NW cannot differentiate the two types of UEs, it can just only triggering fast beam sweeping by configuring GBBR. In both cases, an exit condition is expected. A straightforward method is that if NW does not schedule the UE with dual TCI states after a time period, then the UE is allowed to exit fast beam sweeping.

Proposal 1: When dual TCI states are not indicated within [300] s since group-based beam reporting is configured, the UE is allowed to exit fast beam sweeping.

## 2.2 Measurement restrictions

- For measurement restriction: - Way forward: Companies may bring further analysis, on whether to remove or not the following condition of measurement restriction relaxation for CSI-RS based L1 measurements - [The two CSI-RS resources and both PDSCHs are overlapped on the same OFDM symbol].

For measurement restriction, following was summarized in [1].

Issue 1-3-2: measurement restriction for sDCI and mDCI Background: As per RAN1 spec highlighted in Huawei contribution R4-2408558 - timeDurationForQCL - SCS of 60 kHz: s7, s14, s28 - SCS of 120 kHz: s14, s28. - sTRP - Moderator Observation for sTRP: - Same slot scheduling: - PDSCH always have QCL assumption of lowest CORESET. QCL source corresponding to indicated TCI state in DCI is only used for cross slot scheduling. In other words, for same slot scheduling, PDCCH and PDSCH has same QCL assumptions - Cross slot scheduling: - If the scheduling offset is more than timeDurationForQCL, it is possible to use the beam indicated in TCI state as QCL source, else it is always the same beam as PDCCH or the lowest CORESETID is used for PDSCH reception. mTRP sDCI: - Moderator Observation for mTRP sDCI: - Same slot scheduling or Scheduling offset is lower than timeDurationForQCL: - PDSCH always use QCL assumption of lowest codepoint among the TCI code points containing two different TCI states. That means the TCI state indicated in DCI is not used for same slot scheduling. - Cross slot scheduling: - If the scheduling offset is more than timeDurationForQCL, beam indicated in TCI state are used. - Moderator Observation for mTRP mDCI: - For each TRP, sTRP observation applies.

At present, the overlapping condition for measurement restriction are as follows.

- Both CSI-RSs are not in any CSI-RS resource set with repetition ON, and - One CSI-RS has same QCL source as the active TCI state of one PDSCH, and the other CSI-RS has same QCL source as the active TCI state of the other PDSCH, and - [The two CSI-RS resources and both PDSCHs are overlapped on the same OFDM symbol], and - Resources of the active TCI states for the two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report.

Firstly, when the two CSI-RS resources and both PDSCHs are overlapped on the same OFDM symbol, it is feasible for the UE to perform simultaneous measurements and also simultaneous data reception.

When the two CSI-RS resources and one of the PDSCH, e.g., in partial overlapping case, are overlapped on the OFDM symbol, it is also feasible for UE to perform simultaneous measurements. It is similar to scheduling restriction.

When the two CSI-RSs are not overlapped with any PDSCHs, e.g., no PDSCHs are scheduled in the slot, the UE can perform simultaneous measurements on the two CSI-RSs as long as they are QCL-ed with typeD to reference signals in a resource group in the latest Rel-17 group based beam report. Typically, intra-slot scheduling could be used. In this case, UE shall buffer the data firstly with the default beam. Regardless of whether there is PDSCH actually being scheduled, we can consider the default beams for PDSCH reception as TCI state of the PDSCH.

In any case, what matters is that CSI-RS and PDSCH to be received in the same slot should have same QCL source. The PDSCH may not be actually scheduled. In such case, it can always assume that the two CSI-RS resources and both PDSCHs are overlapped on the same OFDM symbol. The condition of overlapping CSI-RS and PDSCH can be removed.

The updated conditions for measurement restrictions are proposed as below.

Proposal 2: Conditions for measurement restriction are

- Both CSI-RSs are not in any CSI-RS resource set with repetition ON, and

- Th two CSI-RSs are QCL-ed with typeD to reference signals in a resource group in the latest Rel-17 group based beam report, and

- One CSI-RS has same QCL source as the TCI state of one PDSCH, and the other CSI-RS has same QCL source as the TCI state of the other PDSCH.

## 2.3 TRP specific BFD requirements

For the TRP specific CSI-RS based BFD, the conditions for simultaneous measurements from two TRPs are defined as below.

For FR2-1, there is no measurement restriction allowed for UE supporting [TBD - multi-rx capability] according to the conditions described in clause 3.6.x, and the UE is required to measure both the CSI-RS for BFD and the other CSI-RS for RLM, BFD or L1-RSRP measurement, provided the following conditions are met: - Both CSI-RSs are not in any CSI-RS resource set with repetition ON - [FFS: The two CSI-RS resources are overlapped on the same OFDM symbol.] - [The two CSI-RS resources and both PDSCH are overlapped on the same OFDM symbol.] Editor’s note: FFS whether CSI-RS need to overlap with PDSCH for measurement restriction - The CSI-RS and both of the PDSCHs are on the same OFDM symbol(s). - One CSI-RS has same QCL source as the active TCI state of one PDSCH, and the other CSI-RS has same QCL source as the active TCI state of the other PDSCH - Resources of the active TCI states for the two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report. - [FFS how to capture UE is activated with multi-Rx operation]

In general, the similar conditions as for measurement restriction can be defined.

Proposal 3: Conditions for PTRP=1 for CSI-RS based TRP specific BFD requirements for multi-Rx operation is

- Both CSI-RSs are not in any CSI-RS resource set with repetition ON

- The two CSI-RSs are QCL-ed with typeD to reference signals in a resource group in the latest Rel-17 group based beam report, and

- The CSI-RS in set q0,0 has same QCL source as the TCI state of one PDSCH, and the CSI-RS in set q0,1 has same QCL source as the TCI state of the other PDSCH.

## 2.4 TCI state switching

The remaining open issues are as follows.

Issue 1-4-1: DCI based dual TCI state switch delay for mDCI: - Proposals: - Proposal 1: For DCI based dual TCI state switching in mDCI, the 2 TCI state switching procedures can be treated independently. In other words, UE should be able to receive PDSCH with a target TCI state of which the switching is finished earlier. - Proposal 2: If TCI 1 and TCI 2 are in a beam pair, UE to receive on TCI 1 and TCI 2 between C and D. After D, to receive on TCI 2 and TCI 4. Between C and D, UE capable of multi-Rx can receive overlapping PDSCH 0 and PDSCH 1 simultaneously. Otherwise, UE to receive on TCI 2 alone till D. After D, UE can receive on TCI 2 and TCI 4. - Proposal 3: No additional requirements are needed for DCI based dual TCI state switch delay for m-DCI. - Proposal 4: - For mDCI based dual DCI state switch, TCI state switch on each coreset is independent without any restriction on the DCI reception. - Between point C and D, UE to receive on TCI state 0 alone, if new TCI state 0 and old TCI state 1 are not in a beam pair. - Proposal 5: - Update the DCI based dual TCI state switch for m-DCI with “UE shall be able to receive PDSCHs with target TCI states simultaneously after slot max(n1, n2) + timeDurationForQCL - the UE can receive simultaneously TCI 1 and TCI 2 between points C and D if they have been reported as a beam pair using GBBR-17 - Recommended WF: - For DCI based TCI state switch, switching happens within CP and UE can receive entire timeDurationForQCL. Based on this assumption, please discuss following. - Please further discuss whether following is agreeable - If TCI 1 and TCI 2 are a beam pair. - UE to receive on TCI 1 and TCI 2 between C and D. After D, to receive on TCI 2 and TCI 4 - If TCI 1 and TCI 2 are not a beam pair. - No requirements till point D. i.e., UE is not expected to receive on TCI 2 till point D.

In the figure, TCI0 and TCI1 would actually the default beams for PDSCH0 and PDSCH1 reception, respectively. TCI2 and TCI2 are indicated beams for PDSCH0 and PDSCH1 reception after timeDurationForQCL, respectively. If TCI 1 and TCI 2 are a beam pair reported in a group in Rel-17 GBBR report, it is feasible for UE to receive PDSCH0 and PDSCH1 simultaneously between C and D. If TCI 1 and TCI 2 are not a beam pair, simultaneous reception with TCI 1 and TCI 2 between C and D is not possible. Current requirements do not preclude this. It seems not necessary to define specific requirements for this. After D, UE can receive PDSCH0 and PDSCH1 with TCI 2 and TCI 4 simultaneously if they are also a beam pair.

Proposal 4: No additional requirements are needed for DCI based dual TCI state switch delay for m-DCI.

# Summary

In this contribution, we further provided views on remaining issues for NR FR2 multi-Rx chain DL reception WI. Following proposals and observations are present.

Proposal 1: When dual TCI states are not indicated within [300] s since group-based beam reporting is configured, the UE is allowed to exit fast beam sweeping.

Proposal 2: Conditions for measurement restriction are

- Both CSI-RSs are not in any CSI-RS resource set with repetition ON, and

- Th two CSI-RSs are QCL-ed with typeD to reference signals in a resource group in the latest Rel-17 group based beam report, and

- One CSI-RS has same QCL source as the TCI state of one PDSCH, and the other CSI-RS has same QCL source as the TCI state of the other PDSCH.

Proposal 3: Conditions for measurement restriction are

- Both CSI-RSs are not in any CSI-RS resource set with repetition ON, and

- Th two CSI-RSs are QCL-ed with typeD to reference signals in a resource group in the latest Rel-17 group based beam report, and

- One CSI-RS has same QCL source as the active TCI state of one PDSCH, and the other CSI-RS has same QCL source as the active TCI state of the other PDSCH, when at least one of the PDSCHs is scheduled on the same OFDM symbol as both the CSI-RSs.

Proposal 3: Conditions for PTRP=1 for CSI-RS based TRP specific BFD requirements for multi-Rx operation is

- Both CSI-RSs are not in any CSI-RS resource set with repetition ON

- The two CSI-RSs are QCL-ed with typeD to reference signals in a resource group in the latest Rel-17 group based beam report, and

- The CSI-RS in set q0,0 has same QCL source as the TCI state of one PDSCH, and the CSI-RS in set q0,1 has same QCL source as the TCI state of the other PDSCH.

Proposal 4: No additional requirements are needed for DCI based dual TCI state switch delay for m-DCI.

- References

- R4-2408000	Topic summary for [111][203] FR2_multiRx_part1, Moderator (Ericsson)