3GPP TSG RAN WG4 Meeting #112	R4-2412247

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:			8.24.2

Source:	vivo

Title:	Reply LS on UE assistance information for XR

Document for:	Discussion

# Introduction

RAN1 had discussed UE assistance information related to measurement occasions for several meeting. In the RAN1#117 meeting, conclusions were made and an LS [1] were sent to RAN4 to trigger further discussions in RAN4.

Agreement RAN1 continues to discuss and decide whether or not to introduce new UE assistance information for solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. At least the following UE assistance information is considered for further study: - FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable; - FFS: UE assistance information related to channel conditions: - FFS: RSRP is below/above search threshold (s-MeasureConfig); - FFS: UE assistance information related to traffic: - FFS: PSI (PDU set importance); - FFS: UE assistance information related to UE mobility: - FFS: L3 parameters related to mobility, e.g., static or not Companies are encouraged to provide additional details (e.g. how often the UE assistance info is provided, timing, applicable scenarios, performance gains, etc) on their preferred scheme. Note: From specification point of view, there is no mandated gNB behavior in response to any of the UE assistance information. RAN1 to make decision, from RAN1 perspective, in RAN1#117 on the support of UE assistance information. Conclusion RAN1 does not further discuss new UE assistance information related to channel conditions, traffic, UE mobility. Conclusion There is no consensus in RAN1 to support UE assistance information related to measurements occasions. The reason for this situation is lack of consensus on the need/feasibility for UAI and lack of technical understanding on issues outside of RAN1 expertise (e.g. impact of RRM measurement performance). It is up to other working groups to trigger further work in RAN1 on UE assistance information.

In this contribution, we provide our views on UE assistance information regarding measurement occasions with restrictions on data scheduling.

# Discussion

## 2.1 gaps/restrictions

According to WID [2], one of the objectives of this WI aims to realize system capacity gains by enabling transmission/reception in gaps/restrictions that are caused by RRM measurements, while keeping impact to mobility performance limited. This was studied in TR 38.835 B.1.7.

RRM measurements can be categorized into L3 measurements and L1 measurements. L3 measurements can be further grouped as measurements within gaps and outside gaps. The scheduling restrictions are different for the three types of measurements.

The L3 measurements with gaps are performed when measurement gaps are configured. During the measurement gaps, the UE is not required to conduct reception/transmission from/to the corresponding serving cells. The entire measurement gap is considered to be used for measurement only by receiving reference signals such as SSB, PRS.

The L3 measurements outside gaps are performed on SMTC occasions outside of measurement gaps if they are not fully overlapped. The scheduling restrictions are summarized as below.

- For UE which do not support simultaneousRxDataSSB-DiffNumerology for SSB with different subcarrier spacing than PDSCH/PDCCH on FR1

  - The UE is not expected for transmission/reception on SSB symbols configured to be measured, and on 1 data symbol before and after each consecutive SSB symbols configured to be measured

    - intra-band carrier aggregation: apply to all other serving cells in the same band

    - intra-band non-contiguous carrier aggregation configured for a UE indicating intraBandNR-CA-non-collocated-r18: scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band if nonCollocatedTypeNR-CA-r18 is provided.

- TDD bands on FR1

  - The UE is not expected to transmit on SSB symbols configured to be measured, and on 1 data symbol before and after each consecutive SSB symbols configured to be measured

    - When TDD intra-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell also apply to all other serving cells

    - When intra-band non-contiguous carrier aggregation is configured for a UE indicating intraBandNR-CA-non-collocated-r18, apply to all other serving cells in the same band if nonCollocatedTypeNR-CA-r18 is provided

    - When TDD inter-band carrier aggregation is performed, apply to another serving cell in a different band if UE does not have the capability of supporting simultaneousRxTxInterBandCA for this band pair.

- FR2

  - The UE is not expected for transmission/reception on restricted symbols, on SSB symbols configured to be measured, and on 1 data symbol before and after each consecutive SSB symbols configured to be measured

    - intra-band carrier aggregation: apply to all other serving cells in the same band

    - inter-band carrier aggregation: for UE supporting IBM, similar restrictions as TDD bands on FR1

For L1 measurements, it has similar restrictions as L3 measurement outside gaps but the restrictions are only on reference signal symbols for L1 measurements. It is summarized as below.

- FR1

  - For UE which do not support simultaneousRxDataSSB-DiffNumerology SSB with different subcarrier spacing than PDSCH/PDCCH on FR1,

    - The UE is not expected for transmission/reception on symbols corresponding to the SSB indexes configured for L1-RSRP measurement

      - intra-band carrier aggregation: apply to all other serving cells in the same band

      - intra-band non-contiguous carrier aggregation configured for a UE indicating intraBandNR-CA-non-collocated-r18: scheduling restrictions due to a given serving cell also apply to all other serving cells in the same band if nonCollocatedTypeNR-CA-r18 is provided.

- FR2

  - The UE is not expected for transmission/reception symbols corresponding to the SSB indexes and/or periodic/semi-persistent/aperiodic CSI-RS resources which is NOT QCLed with active TCI state for PDCCH/PDSCH configured for L1 measurement

    - intra-band carrier aggregation: apply to all serving cells in the band on the symbols that fully or partially overlap with restricted symbols.

It is observed that the UE cannot be scheduled during the measurement gaps. For L3 measurements outside gaps when certain conditions are met, the UE cannot be scheduled on some or all of the symbols within SMTC. For L1 measurements when certain conditions are met, the UE cannot be scheduled on the reference signal symbols.

Observation 1: Scheduling restrictions are different for L3 measurements within gaps, L3 measurements outside gaps and L1 measurements.

## 2.2 RRM measurements within gaps

For the L3 measurements performed within gaps, the cell identification delay and measurement period for a frequency layer i is depending on MGRP, SMTC period, CCSFwithin_gap,i. It is further depending on DRX cycle if C-DRX is configured.

For example, the measurement period requirements for inter-frequency measures with gaps in FR1 is specified as in Table 9.3.5-1.

Table 9.3.5-1: Measurement period for inter-frequency measurements with gaps (Frequency FR1)

| Condition NOTE1,2 | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(200ms, Ceil(8 * Kgap) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(200ms, Ceil(8 1.5 * Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | Ceil(8 * Kgap) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: For a UE supporting concurrent measurement GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured. |  |

CCSFinter is determined according to CSSFwithin_gap,i, which is derived based on following procedures.

If measGapSharingScheme is equal sharing, CSSFwithin_gap,i= max(ceil(Ri×Mtot,i,j)), where j=0…(160/MGRP)-1 If measGapSharingScheme is not equal sharing and - measurement object i is an intra-frequency measurement object, CSSFwithin_gap,i is the maximum among - ceil(Ri×Kintra×Mintra,i,j) in gaps where Minter,i,j≠0, where j=0…(160/MGRP)-1 - ceil(Ri×Mintra,i,j) in gaps where Minter,i,j=0, where j=0…(160/MGRP)-1 - measurement object i is an inter-frequency or inter-RAT measurement object or NR PRS measurement on any one positioning frequency layer, CSSFwithin_gap,i is the maximum among - ceil(Ri×Kinter×Minter,i,j) in gaps where Mintra,i,j ≠0, where j=0…(160/MGRP)-1 - ceil(Ri×Minter,i,j) in gaps where Mintra,i,j=0, where j=0…(160/MGRP)-1 Where Ri is the maximal ratio of the number of measurement gap where measurement object i is a candidate to be measured over the number of measurement gap where measurement object i is a candidate and not used for a long-periodicity measurement defined above. Where： - Mtot,i,j = Mintra,i,j + Minter,i,j : Total number of intra-frequency, inter-frequency and inter-RAT frequncy layers and up to one NR PRS measurement on any one positioning frequency layer, inter-frequency L1-RSRP measurement layers, which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mtot,i,j equals 0.

Based on the above procedure and depending on measurement configuration, the definition of CCSFwithin_gap,i could lead to a long measurement delay requirements where some the gaps may not be used for measurements.

For the example of measurement configuration depicted in Fig 1, measurement gap is configured together with three inter-frequency layers where different SMTC periodicity is used. For this configuration, CCSFwithin_gap,i is 3 for all the frequency layers.

Fig 1. Configuration of measurements with gaps

In this case, some of the gaps may not always be used to meet minimum measurement period requirements. The requirements are assumed that there are always 3 candidates of MOs to be measured within a gap. For MO#1 and MO#2, it is not the case. So, some of the gaps could be left unused as long as measurement period requirements are satisfied.

Suppose 2 samples are needed for one measurement period. one of the possible measurement implementations is depicted in Fig 2. Another possible measurement implementation is depicted in Fig 3.

Fig 2. Measurement implementation example 1

Fig 3. Measurement implementation example 2

Based on above analysis, it is observed that some of the gaps may not be used for measurements and it is up to implementation which gaps are not used for measurements.

Observation 1: Depending on measurement configuration, e.g., gap, SMTC, MO etc., some of the gaps may not be used for measurements while measurement requirements being satisfied.

Observation 2: It is up to UE implementation which gaps are not used for measurements under current measurement configuration.

In the case as depicted in Fig 1, Fig 2 and Fig 3, the unused gaps are about 37.5% (9/24). It is significant gain if these gaps can be used for data transmission/reception. Since it is up to UE implementation which gaps are not used, the information can only be provided by UE. Therefore, there is obvious benefit if UE can provide assistance information about which gaps are not used for measurements.

Observation 3: Significant gain on throughput is observed if UE can provide information about unused gaps for measurement without impact on mobility performance.

Even for use case other than XR, these unused measurement gaps can be used for data transmission/reception, which means the gain comes at no cost.

Proposal 1: UE should provide assistance information about gaps not used for measurement for enabling transmission/reception in these gaps.

Then it comes the question that the reported gaps not used for measurement may not be enough for data transmission/reception considering QoS requirements. In some cases, there may be no gaps not used for measurement depending on measurement configuration. However, when enabling transmission/reception in gaps/restrictions that are caused by RRM measurements, it needs to keep impact to mobility performance limited. In order to achieve this goal, it needs UE to provide information of unused gaps for measurement. If it is up to NW indication, the mobility performance will definitely be degraded as gaps may have different priority for measurements, e.g., the most loaded gap has highest priority and it may be used to measure sparse reference signals and should not be used for data transmission/reception, or if gap sharing scheme is not equal sharing, some gaps may have high priority.

One possible solution is that NW indicates the percentage of gaps not used for measurement during a period of time. It is up to UE to decide which gaps are not used for measurement during the period of time. This could be the best balance between QoS requirements of data transmission/reception and UE mobility performance.

Proposal 2: NW may indicate the percentage of measurement gaps not used for measurement for UE to provide assistance information about gaps.

If the percentage is too large for UE to meet the minimum measurement requirements, RAN4 should further study whether and how to define new requirements.

## 2.3 RRM measurements outside gaps

For the L3 measurements performed outside gaps, the cell identification delay and measurement period for a frequency layer i is depending on SMTC period, Klayer1_measurement and CCSFoutside_gap,i. It is further depending on DRX cycle if C-DRX is configured.

For example, the measurement period requirements for intra-frequency measurements outside gaps in FR2 are specified as in Table 9.2.5.2-2.

Table 9.2.5.2-2: Measurement period for intra-frequency measurements without gaps (FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

CCSFintra is determined according to CSSFoutside_gap,i, which is based on configuration of number of CCs and inter-frequency/inter-RAT measurements outside gaps. The PCC has highest priority and would use a dedicated searcher. All the other CCs and inter-frequency/inter-RAT measurements will share another searcher, which means it will be measured sequentially.

Klayer1_measurement is sharing factor between L1 measurements and L3 measurements outside gaps when all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting are fully overlapped wither reference signal symbols, including 1 symbol before/after, for L3 measurements. The scaling factor is only needed in FR2 and it is 1 in FR1.

In general, when the SMTC for a MO, which can be measured outside gaps, are not fully overlapped with measurement gaps, the measurements are conducted on SMTCs not overlapped with measurement gaps. Otherwise, it becomes measurement within gaps. Furthermore, it the SMTCs not overlapped with measurement gaps are fully overlapped with reference signal symbols for L1 measurements, i.e., RLM, BFD, CBD or L1-RSRP for beam reporting, it will share the SMTC measurement occasions which only happens in FR2.

For the measurements on PCC, if sharing with L1 measurements are not considered, then all the SMTCs outside gaps would be used for measurements to meet minimum measurement requirements. It seems no strong motivation to provide UE assistance information about SMTC occasions not used for measurements. If there is sharing between L1 measurements and L3 measurements outside gaps, it could be useful to provide assistance information that some SMTC occasions during a period of time are used for L1 measurements since the scheduling restrictions are different between L3 measurements and L1 measurements.

For the measurements on SCCs, the CCSF is scaled by the number of CCs. It could leave to unused SMTC occasions depending on measurement configurations.

For the example of measurement configuration depicted in Fig 4, measurements on PCC and 3 SCCs are configured where different SMTC periodicity is used on SCCs. For this configuration, CCSFoutside_gap,i is 3 for all the 3 SCCs and is 1 for PCC.

Fig 4. Configuration of measurements outside gaps

In this case, one of the possible measurement implementations is depicted in Fig 5. the SMTC occasion filled with pattern is not used for measurements. All the SMTC occasions on PCC are used for L3 measurements. For SCC, UE just needs to measure 1 out of 3 SMTC occasions since CCSF is 3. It is obvious some of the SMTC occasions on SCCs are not used for measurement.

Fig 5. Configuration of measurements outside gaps

Observation 4: Depending on measurement configuration, e.g., SMTC, MO etc., some of the SMTC occasions may not be used for measurements while measurement requirements being satisfied.

Observation 5: It is up to UE implementation which SMTC occasions are not used for measurements under current measurement configuration.

Similar to UE assistance information about measurement gaps, it is beneficial to provide the SMTC occasions not used for measurements on each CC to NW so that NW can schedule the UE on these SMTC occasions by taking scheduling restriction requirements into consideration.

Proposal 3: UE should provide assistance information about SMTC occasions not used for measurement for enabling transmission/reception on the SMTC occasions.

Similar to UE assistance information about measurement gaps, the reported SMTC occasions not used for measurement may not be enough for data transmission/reception considering QoS requirements. In some cases, there may be no SMTC occasions not used for measurement depending on measurement configuration. One possible solution is that NW indicates the percentage of SMTC occasions not used for measurement during a period of time. It is up to UE to decide which SMTC occasions are not used for measurement during the period of time. This could be the best balance between QoS requirements of data transmission/reception and UE mobility performance.

Proposal 4: NW may indicate the percentage about SMTC occasions not used for measurement for UE to provide assistance information about SMTC occasions.

Proposal 5: The UE assistance information about SMTC occasions not used for measurement should be provided on each CC/MO.

If the percentage is too large for UE to meet the minimum measurement requirements, RAN4 should further study whether and how to define new requirements.

For L1 measurements, it includes RLM, BFD, CBD or L1-RSRP for beam reporting. The reference signals for L1 measurements includes SSB and CSI-RS and it may or may not be fully overlapped with SMTC occasions outside measurement gaps. If it is not fully overlapped with SMTC occasions, the measurement is performed on the reference signal symbols not overlapped with SMTC occasion. In any case, if such reference signal symbols are not used for L1 measurements, the minimum measurement requirements can not be met. There would be no difference between by UE or NW on deciding which L1 reference signal symbols not used for measurements. It is also very difficult to provide UE assistance information about L1 reference signals.

Proposal 6: No UE assistance information about L1 measurements is provided.

## 2.4 UE assistance information

There serval aspects about UE assistance information had been discussed in RAN1.

- FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable;

As discussed in section 2.2 and 2.3, the patterns of gaps/SMTC occasions not used for measurement within a time period should be provided to achieve best balance between QoS requirements and mobility performance. Other options will have unknown and unexpected impact on UE measurement.

Proposal 7: The patterns of gaps/SMTC occasions not used for measurement within a time period should be provided as UE assistance information.

- FFS: UE assistance information related to channel conditions: - FFS: RSRP is below/above search threshold (s-MeasureConfig);

s-MeasureConfig is the threshold for NR SpCell RSRP measurement controlling when the UE is required to perform measurements on non-serving cells. When serving cell RSRP is above the threshold, UE may not perform measurements on neighbour cells. However, UE still needs to perform measurements on SMTC occasions for serving cell, which are also the same for all neighbour cells on the serving cell frequency, if the measurements are performed outside gap. The channel condition of serving cell RSRP does not help UE to provide information about which gaps/SMTC occasions within a time period are not used for measurements.

Observation 6: Channel condition, e.g., RSRP, does not provide information about which gaps/SMTC occasions are not used for measurements.

- FFS: UE assistance information related to traffic: - FFS: PSI (PDU set importance);

The UE assistance information related to traffic is not RAN4 expertise. It would be better to be discussed in RAN2.

- FFS: UE assistance information related to UE mobility: - FFS: L3 parameters related to mobility, e.g., static or not

There is no L3 measurement requirements related to UE mobility. In connected mode, requirements for UE fulfilling relaxed measurement criteria are specified for RLM and BFD measurements. The relaxed measurement criteria include good serving cell quality and low mobility. There is no L3 measurement requirements related to UE mobility. Thus, UE behaviour on L3 measurement is not changed based on different UE mobility status. If UE mobility is to be used as additional UE assistance information, relaxed L3 measurement requirements should be specified accordingly.

Observation 7: UE mobility information does not help how gaps/SMTC occasions are used for measurement unless relaxed L3 measurement requirements are specified.

Based on aforementioned discussions on RRM measurements within gaps and outside gaps, the UE assistance information about measurement gaps/SMTC occasions not used for measurement is dependent on measurement configurations related to number of measurement objects, SMTC, MGRP, DRX, number of CCs etc. These parameters are configured and updated via RRC signaling RRCReconfiguration. Thus, UE should provide the assistance information about gaps/SMTC occasions not used for measurement in response to RRCReconfiguration.

Proposal 8: UE assistance information is reported in response to RRCReconfiguration.

There are different measurement gaps being introduced in multiple releases, e.g., The per-UE gap and per-FR gap in Rel-15, pre-configured measurement gap, concurrent measurement gaps, NCSG and MUSIM gaps in Rel-17. Combination of concurrent gap and pre-MG or NCSG was further enhanced in Rel-18. Regarding the UE assistance information about gaps not used for measurements, RAN4 needs to decide what type of measurement gaps are considered.

Proposal 9: RAN4 to further decide what measurement gaps are considered in this WI, e.g., per-UE gap and per-FR gap, pre-configured measurement gap, concurrent measurement gaps, NCSG and MUSIM gaps etc.

When DRX is configured, the UE may measure once within gaps or on SMTC occasions in one DRX cycle. Only the gaps/SMTC occasions overlapped with DRX on duration can be used data transmission/reception. There could be different reporting scheme for UE assistance information, e.g., only reporting gaps/SMTC occasions overlapped with DRX on duration within a time period.

Proposal 10: RAN4 to further study how UE assistance information is reported when DRX is configured.

# Summary

In this contribution, we provided views on UE assistance information regarding measurement occasions not used for measurements. Following proposals and observations are present.

Observation 1: Depending on measurement configuration, e.g., gap, SMTC, MO etc., some of the gaps may not be used for measurements while measurement requirements being satisfied.

Observation 2: It is up to UE implementation which gaps are not used for measurements under current measurement configuration.

Observation 3: Significant gain on throughput is observed if UE can provide information about unused gaps for measurement without impact on mobility performance.

Observation 4: Depending on measurement configuration, e.g., SMTC, MO etc., some of the SMTC occasions may not be used for measurements while measurement requirements being satisfied.

Observation 5: It is up to UE implementation which SMTC occasions are not used for measurements under current measurement configuration.

Observation 6: Channel condition, e.g., RSRP, does not provide information about which gaps/SMTC occasions are not used for measurements.

Observation 7: UE mobility information does not help how gaps/SMTC occasions are used for measurement unless relaxed L3 measurement requirements are specified.

Proposal 1: UE should provide assistance information about gaps not used for measurement for enabling transmission/reception in these gaps.

Proposal 2: NW may indicate the percentage of measurement gaps not used for measurement for UE to provide assistance information about gaps.

Proposal 3: UE should provide assistance information about SMTC occasions not used for measurement for enabling transmission/reception on the SMTC occasions.

Proposal 4: NW may indicate the percentage about SMTC occasions not used for measurement for UE to provide assistance information about SMTC occasions.

Proposal 5: The UE assistance information about SMTC occasions not used for measurement should be provided on each CC/MO.

Proposal 6: No UE assistance information about L1 measurements is provided.

Proposal 7: The patterns of gaps/SMTC occasions not used for measurement within a time period should be provided as UE assistance information.

Proposal 8: UE assistance information is reported in response to RRCReconfiguration.

Proposal 9: RAN4 to further decide what measurement gaps are considered in this WI, e.g., per-UE gap and per-FR gap, pre-configured measurement gap, concurrent measurement gaps, NCSG and MUSIM gaps etc.

Proposal 10: RAN4 to further study how UE assistance information is reported when DRX is configured.

- References

- R1-2405736	LS on UE assistance information, RAN1

- RP-240791	Revised WID: XR (eXtended Reality) for NR Phase 3, Nokia

- Appendix: Draft reply LS

3GPP TSG RAN WG4 Meeting #112	R4-24x

Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	Reply LS on UE assistance information

Response to:	R1-2405736 (R4-2411007)

Release:	Rel-19

Work Item:	NR_XR_Ph3-Core

Source:	RAN4

To:	RAN1

Cc:	RAN2

Contact Person:

Name:	Qian Yang

E-mail Address: qian9.yang@vivo.com

1. Overall Description:

RAN4 thanks RAN1 for the LS on UE assistance information.

RAN4 discussed the UE assistance information for reporting not used measurement gas/SMTC occasions to network. Following conclusions are made.

From UE measurements and mobility performance perspective, RAN4 agreed that it is beneficial if UE reports UE assistance information about not used measurement gaps/SMTC occasions to network and network indicates skipping gaps/SMTC occasions based on UE assistance information.

In order for NW and UE to have same understanding which gaps should be not used for measurement, the patterns of gaps/SMTC occasions not used for measurement within a time period should be provided as UE assistance information.

RAN4 will further study the details and additional conditions of UE assistance information. RAN1 will be informed once there are conclusions.

2. Actions:

To: RAN WG1

ACTION: 	RAN4 kindly asks RAN1 to take the above RAN4 agreements into account in the future work.

3. Reference:

- R1-2405736	LS on UE assistance information, RAN1

4. Date of Next TSG RAN WG4 Meetings:

TSG RAN WG4 Meeting #112bis		October 14 – October 18, 2024			Hefei, CN

TSG RAN WG4 Meeting #113	 	   November 18 – November 22, 2024			Orlando, US