3GPP TSG RAN WG4 Meeting #112	R4-2412248

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:			8.24.2

Source:	vivo

Title:	Discussion on RRM impacts for XR

Document for:	Discussion

# Introduction

A revised WID on XR (eXtended Reality) for NR [1] was approved in RAN#103 meeting. The objectives relevant to RAN4 are as follows.

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4] - Specify Enhancements for Scheduling, as follows: - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2]. NOTE: LCP implementation complexity need to be taken into account when evaluating solutions. NOTE: Check in RAN#105 - Specify the following user plane enhancements [RAN2] - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. - Specify Core requirements related to the above objectives as necessary [RAN4]

In this contribution, we provide our views on RRM requirements impact for XR in Rel-19.

# Discussion

This WI aims to realize system capacity gains by enabling transmission/reception in gaps/restrictions that are caused by RRM measurements, while keeping impact to mobility performance limited. This was studied in TR 38.835 B.1.7. The solution to achieve this aim is to specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc).

The discussions in RAN1 had been continued for several meetings on triggering scheme to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements and whether/what UE assistance information is needed. From RAN4 perspective, it needs to decide firstly what RRM measurements and corresponding scheduling restrictions are to be considered in the WI.

Summary of gaps/restrictions due to RRM measurements are summarized in [2]. It can be briefly summarized that the UE cannot be scheduled during the measurement gaps due to L3 measurements within gaps. For L3 measurements outside gaps when certain conditions are met, the UE cannot be scheduled on some or all of the symbols within SMTC. For L1 measurements when certain conditions are met, the UE cannot be scheduled on the reference signal symbols.

L3 measurements within gaps

L3 measurements within gaps could be intra-frequency measurements, inter-frequency measurements and inter-RAT measurements. As analyzed in [2], some of the gaps may not be used for measurements at all while measurement requirements being satisfied, depending on measurement configuration, e.g., gap, SMTC, MO etc. Then the UE can be scheduled in these gaps. It is also necessary, at least for XR use case, that some of the gaps which are supposed to be used for measurements depending on measurement configuration may also be used for data scheduling when NW indicates so. In such case, the UE cannot meet measurement requirement requirements.

In general, gaps can be enabled for data transmission/reception. Depending on various conditions, measurement requirements may or may not be satisfied. RAN4 should study whether and how to define new core requirements when some of gaps that need to be used for measurements are enabled for data transmission/reception.

Proposal 1: RAN4 to study whether and how to define new core requirements when some of gaps that need to be used for measurements are enabled for data transmission/reception.

There are different measurement gaps being introduced in multiple releases, e.g., The per-UE gap and per-FR gap in Rel-15, pre-configured measurement gap, concurrent measurement gaps, NCSG and MUSIM gaps in Rel-17. Combination of concurrent gap and pre-MG or NCSG was further enhanced in Rel-18. RAN4 needs to decide what type of measurement gaps are considered in this WI. It would have a lot of requirements to be defined if all the gap types are considered.

Proposal 2: RAN4 to decide what measurement gaps are considered in this WI, e.g., per-UE gap and per-FR gap, pre-configured measurement gap, concurrent measurement gaps, NCSG and MUSIM gaps etc.

L3 measurements outside gaps

L3 measurements outside gaps could also be intra-frequency measurements, inter-frequency measurements and inter-RAT measurements. As analyzed in [2], there could be some SMTC occasions for a MO within a time period are not used for measurements depending on measurement configuration. These SMTC occasions can be used for data transmission/reception. Similar to gaps, the SMTC occasions not used for measurements in the first place may not be enough for XR traffic. Additional SMTC occasions may be indicated by NW for data transmission/reception. Thus, it has similar situation as measurement within gap that RAN4 needs to study whether and how to define new core requirements when some of the SMTC occasions that need to be used for measurements are enabled for data transmission/reception when there are scheduling restrictions on the SMTC occasions.

Proposal 3: RAN4 to study whether and how to define new core requirements when some of SMTC occasions that need to be used for measurements outside gaps are enabled for data transmission/reception when scheduling restrictions exists.

L1 measurements

L1 measurements are performed outside gaps and SMTC occasions when L1-RS are not fully overlapped with gaps and SMTC occasions. It includes RLM, BFD, CBD or L1-RSRP for beam reporting. The reference signals for L1 measurements includes SSB and CSI-RS and it may or may not be fully overlapped with SMTC occasions outside measurement gaps. If it is not fully overlapped with SMTC occasions, the measurement is performed on the reference signal symbols not overlapped with SMTC occasion.

For L1 measurements, it includes RLM, BFD, CBD or L1-RSRP for beam reporting. The reference signals for L1 measurements includes SSB and CSI-RS and it may or may not be fully overlapped with SMTC occasions outside measurement gaps. If it is not fully overlapped with SMTC occasions, the measurement is performed on the reference signal symbols not overlapped with SMTC occasions. Otherwise, in FR2 it needs to share SMTC occasions for measurements with L3 measurements outside gaps. Since scheduling restriction for L1 measurements are only on L1-RS symbols which would not be many, the benefit of enabling data transmission/reception on the symbols is very small. Furthermore, it could have big impact on L1 measurement performance, especially for RLM/BFD measurements. Therefore, it is not preferred to enable L1-RS symbols for data transmission/reception when there are scheduling restrictions.

Proposal 4: RAN4 does not consider enabling data transmission/reception on L1-RS symbols that need to be used for L1 measurements if there is scheduling restriction.

Triggering/enabling scheme

In the RAN1#117 meeting, following agreements on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements were made as captured in [3].

- For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements consider the following alternatives or combinations for further down-selection: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap(s)/restriction(s) that are caused by RRM measurements. - FFS: Alt 1-1: Explicit indication by DCI to skip a particular gap(s)/restriction(s); - Indication is included as part of scheduling DCI: - FFS: Bit-field size is one bit; - FFS: Bit-field size is >1 bit; - Note: Minimum time offset(s) between the end of [the first] received dynamic indication and start of corresponding gap(s)/restriction(s) occasion that is going to be skipped shall be introduced. - FFS: Alt 1-2: Explicit indication by DCI to indicate a time window where to skip a particular gap(s)/restriction(s); - Note: Minimum time offset between the end of received dynamic indication and start of gap(s)/restriction(s) occasion in time window that is going to be skipped shall be introduced. - FFS: Alt 1-3: Implicit indication by DCI scheduling a transmission/reception overlapping with a gap(s)/restriction(s) to skip the gap(s)/restriction(s); - Note: Minimum time offset between the end of received dynamic indication and start of gap(s)/restriction(s) occasion that is going to be skipped shall be introduced. - FFS: DCI format, DCI content, DCI bit-field size; - FFS: Whether indication is for one or more occasions; - FFS: How to consider time offset between the end of received dynamic indication and start of gap(s)/restriction(s) occasion that is going to be skipped. - Alt. 3: Semi-static solution to enable TX/RX in gaps/restrictions that are caused by RRM measurements. - FFS: Alt 3-1: Configure a pattern(s) via RRC to indicate occasions where to skip gaps/restrictions; - FFS: Details of pattern: - FFS: Pattern is based on periodicity, offset and duration; - FFS: Pattern is based on a bitmap; - FFS: whether a pattern is applied to all or subset of configured MG configurations/scheduling restrictions. - FFS: Alt 3-3: Gaps/restrictions that are caused by RRM measurements are skipped if collided with particular semi-statically pre-configured Tx/Rx occasions. - FFS: Alt. 3-4: Gaps/restrictions that are caused by RRM measurements are skipped based on semi-statically configured priority information for particular semi-statically pre-configured Tx/Rx and/or particular gaps/restrictions.

Based on RAN1 discussions so far, the enabling data transmission/reception in gaps/SMTC occasions could be based either semi-static solution or dynamic solution. In general, UE schedules measurements after measurement and other configurations are updated via RRC reconfiguration. If enabling data transmission/reception in gaps/SMTC occasions are indicated via RRC, or other ways that the use of gaps/SMTC occasions is known to UE after RRC reconfiguration, then UE can still schedule the measurement well as before by taking the indication into account. If it is indicated in dynamic manner via DCI, then it is difficult for UE to schedule the measurements because the it is not known to UE when the gaps/SMTC occasions will be unavailable. Moreover, it is very difficult to define RRM requirements for such indications, if possible. Therefore, from RAN4 perspective, semi-static solution is a better solution.

Proposal 5: RAN4 concludes semi-static solution for indication of gaps/SMTC occasions not used for measurement is better from RAN4 perspective and LS to RAN1 if necessary.

There is not enough progress on solutions of how gaps/SMTC occasions not used for measurements are indicated. It is difficult to discuss RRM requirements for such solution for now. RAN4 can wait for more progress in RAN1.

# Summary

In this contribution, we provided views on RRM requirements impact for XR in Rel-19. Following proposals and observations are present.

Proposal 1: RAN4 to study whether and how to define new core requirements when some of gaps that need to be used for measurements are enabled for data transmission/reception.

Proposal 2: RAN4 to decide what measurement gaps are considered in this WI, e.g., per-UE gap and per-FR gap, pre-configured measurement gap, concurrent measurement gaps, NCSG and MUSIM gaps etc.

Proposal 3: RAN4 to study whether and how to define new core requirements when some of SMTC occasions that need to be used for measurements outside gaps are enabled for data transmission/reception when scheduling restrictions exists.

Proposal 4: RAN4 does not consider enabling data transmission/reception on L1-RS symbols that need to be used for L1 measurements if there is scheduling restriction.

Proposal 5: RAN4 concludes semi-static solution for indication of gaps/SMTC occasions not used for measurement is better from RAN4 perspective and LS to RAN1 if necessary.

- References

- RP-240791	Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia

- R4-2412247	Reply LS on UE assistance information for XR, vivo

- Draft_Minutes_report_RAN1#117_v030