3GPP TSG RAN WG4 Meeting #112	R4-2412249

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:			8.17.1

Source:	vivo

Title:	Discussion on general aspects for AI/ML

Document for:	Discussion

- Introduction

At the current stage in Rel-19 work item phase, as per the working plan in WID [1], RAN4 aims to specify requirements for LCM procedures for BM and positioning use cases based on the detailed designs in other WGs. In this contribution, we further provide our views on potential LCM procedures for AI/ML NR air interface based on the latest outcome reached in the last meeting [2].

- Discussion

- 2.1	LCM requirements

In the last meeting, RAN4 made the further progress on the issue about defining RRM requirements for LCM procedures and the corresponding requirement scope of procedures has been reached as follows.

Issue 1-5: LCM Requirements Agreement: - RAN4 considers defining RRM requirements for LCM procedures based on the specific use cases (beam management and positioning), and candidates are: - identification - selection - activation - deactivation - switching - fallback to non-AI operation - performance monitoring - Others are not precluded

For the above set of procedure types, discussions are also under way in other working groups. Next, we will provide our analysis for each procedure based on the latest progress from RAN1/2 specifically for UE-sided model.

For identification, the corresponding definition captured in TR38.843[3] is:

- For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models, model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side.

RAN1 made following agreement in RAN1#116bis meeting, the related discussions mainly focus on the association between Associated ID, Data collection related configurations and NW-sided additional conditions. For step D, it belongs to a part of model identification and further details need to be confirmed in RAN1 discussion. On the other hand, it is still under discussing in RAN2 on reporting of additional condition information as well. Thus, for model identification, RAN4 can wait for further progress in RAN1/2 about the details of procedure and potential signalling support.

From RAN1 perspective, for UE-sided model(s) developed (e.g., trained, updated) at UE side, following procedure is an example (noted as AI-Example1) of MI-Option1 for further study (including the feasibility/necessity) - A: For data collection, NW signals the data collection related configuration(s) and it/their associated ID(s) - Associated IDs for each sub use case in relation with NW-sided additional conditions - B: UE(s) collects the data corresponding to the associated ID(s) - C: AI/ML models are developed (e.g., trained, updated) at UE side based on the collected data corresponding to the associated ID(s). - D: UE reports information of its AI/ML models corresponding to associated IDs to the NW. Model ID is determined/assigned for each AI/ML model - relationship between model ID(s) and the associated ID(s) - How model ID(s) is determined/assigned, e.g., - Alt.1: NW assigns Model ID - Alt.2: UE assigns/reports Model ID - Alt.3: Associated ID(s) is assumed as model ID(s) - “Model ID is determined/assigned for each AI/ML model” in D is not needed - Alt.4: Model ID is determined by pre-defined rule(s) in the specification - FFS: how to report - Note: D is to facilitate AI/ML model inference - Note: Step A/B/C and additional interaction of associated IDs between UE and NW can be considered as a different solution for resolving the consistency without model identification.

For selection, this process is to select an AI/ML model for activation among multiple models for the same AI/ML enabled feature, which may or may not be carried out simultaneously with model activation. It can be observed that model selection is possible to couple with activation procedure. Given that there are many details to be determined for activation and it is possible no separate requirement is needed for this procedure, it is suggested to wait for more progress for this part.

For activation/deactivation, RAN2 made the relevant agreements in RAN2#125bis and 126 meeting as follows.

RAN2#125bis Agreements 1 Which AI/ML-enabled Features/FGs and functionalities are supported should be standardized. The details wait for RAN1’s progress. “supported” means that the UE is capable of supporting the functionality and doesn’t mean necessarily that the UE has the model available. FFS what functionality refers to. 2 Supported AI/ML-enabled Features/FGs and supported functionalities are included in UE capability. RAN2#126 Agreements 1 RAN2 will support functionality activation/deactivation after inference configuration. FFS initial state of configuration and how activation/deactivation is achieved. FFS what Deactivation refers to: examples discussed: 1) fall back to legacy 2) switching, etc. 2 We will work offline on the definitions for functionality types and define what is availability. 3 The UE will indicate the gNB/LMF whether the AI/ML functionality is available/applicable. For a functionality to be applicable at least there should at least one model available within it. FFS other details on what is applicability/non-applicability. 4 For NW-side additional conditions, RAN2 assumes that RRC signalling from gNB to UE can be designed for consistency between inference and training. RAN2 will wait for RAN1 input for further details. FFS if the same applies to positioning 5 For BM use case, as a baseline the UE determines whether a functionality is applicable. Existing UAI framework is used at least for proactive reporting of applicable functionality. FFS reactive

Among them, there are several terminologies being discussed:

‘supported’: RAN2 clarified which means UE is capable of supporting the functionality and doesn’t mean necessarily that the UE has the model available. Based on this assumption, it was also agreed that the supported AI/ML-enabled Features/FGs and supported functionalities are included in UE capability.

‘functionality’: it refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. Based on RAN1 understanding, the association between functionality and model can be one to many. Each model type is differentiated via associated ID. The exact definition and granularity are still FFS.

‘available/applicable’: For the supported functionality, NW cannot know which functionality or model is available/applicable under the same functionality only based on UE capability. Thus, it needs UE to indicate the gNB/LMF whether the AI/ML functionality is available/applicable.

Based on the agreements from RAN2, the interactions including reporting the supported functionalities indication on applicability of functionality and inference configuration are performed before the functionality/model activation. So far, it is still unclear which kind of information and parameters will be conveyed during these interactions, which affects how the activation process is conducted and which signalling will be used for activation (e.g., RRC or MAC CE or DCI).

For deactivation/ fall-back to legacy/ switching, as per the agreements as above, it is still unclear about the relationship among these three procedures. Deactivation may not be the separate action from the last two, but directly refers to one of the 1) fall back to legacy 2) switching. From this perspective, it should not be necessary to define requirements for all three processes.

For performance monitoring, regarding the use cases on BM and positioning, the details on monitoring method are still under discussion in other WGs. Here we would like to provide some analysis for CSI use case based on the latest progress in RAN1. For CSI, RAN1 figured out the concrete monitoring method as follows. There are mainly two candidate monitoring options, one is NW-side monitoring and the other is UE-side monitoring. Both NW side and UE side monitoring can be supported for CSI compression with two-sided models. For NW-side monitoring, consider target CSI reporting by UE via legacy eT2 codebook or eT2-like high-resolution codebook. For UE-side monitoring, consider monitoring based on either direct estimation of SGCS (Case 2-1) or the output of reconstruction model at the UE (Case 2-2). For provision of the reference model for UE-side monitoring purpose, it could directly follow the corresponding inter-vendor collaboration methods for Option 3, e.g., proxy model transfer or directly include monitoring related output as the model output.

Agreement Further study following monitoring options in Rel-19, including the necessity and feasibility, - NW-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the target CSI reported by the UE via legacy eT2 codebook or eT2-like high-resolution codebook (Case 1) - SRS-based monitoring - UE-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the output of the CSI reconstruction model at the UE (Case 2-1) - Note: CSI reconstruction model at the UE-side can be the same as the actual CSI reconstruction model used at the NW-side, a reference model provided by NW, or a proxy model developed by the UE side. - Via direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI (Case 2-2) - Via estimation of monitoring output other than intermediate KPI without reconstructing a target CSI - Based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model - Based on the output of the CSI reconstruction model indicated by the NW via legacy eT2 codebook or eT2-like high-resolution codebook Regarding monitoring metrics: - Monitoring accuracy also includes generalization considerations, if applicable. - Complexity also includes LCM complexity, if applicable. - Monitoring overhead, latency, complexity, and accuracy analysis may have to consider using at N>1 CSI feedback occasions. - Testability of UE reported metrics Discussion may include the following aspects: - Consideration of Options 1-5 and their sub-options for alleviating / resolving the issues related to inter-vendor training collaboration - Temporal domain aspects of CSI compression - How the above monitoring approaches or combination of them may help identifying the cause (e.g., NW side, UE side, data drift) of the performance degradation Note: for UE-side monitoring, the final reported monitoring output, if specified, may be different, e.g., be further derived based on the output of the above approaches. Note: implementation-based monitoring solutions can be considered in assessing the necessity of the above monitoring approaches.

Based on the analysis on each candidate LCM procedure, it can be observed that some process may occur sequentially or be included in other process. It may be not necessary to define requirements for all potential processes. Given that there still many details pending on the input from other WGs, from RAN4 perspective, we can revisit the requirements scope based on the version agreed in the last meeting, when RAN1/2 draw the whole picture on how LCM procedures work.

Proposal 1: RAN4 to revisit the RRM requirements scope for LCM procedures based on the following agreement when RAN1/RAN2 have enough inputs for LCM procedures

- RAN4 considers defining RRM requirements for LCM procedures based on the specific use cases (beam management and positioning), and candidates are:

- identification

- selection

- activation

- deactivation

- switching

- fallback to non-AI operation

- performance monitoring

- Others are not precluded

- Summary

In this contribution, we provided our views on general aspects for AI/ML. Based on above analysis, following proposals are present.

Proposal 1: RAN4 to revisit the RRM requirements scope for LCM procedures based on the following agreement when RAN1/RAN2 have enough inputs for LCM procedures

- RAN4 considers defining RRM requirements for LCM procedures based on the specific use cases (beam management and positioning), and candidates are:

- identification

- selection

- activation

- deactivation

- switching

- fallback to non-AI operation

- performance monitoring

- Others are not precluded

- References

- Qualcomm Incorporated, RP-234039, “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN#102, Dec.11-15, 2023.

- R4-2401570	WF on AI/ML, Qualcomm

- 3GPP TR38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”, V18.0.0, 2023-12.