3GPP TSG RAN WG4 Meeting #112	R4-2412250

Maastricht, NL, 19 Aug - 23 Aug, 2024

Agenda item:			8.17.2

Source:	vivo

Title:	Discussion on testability and interoperability issues for beam management

Document for:	Discussion

- Introduction

In RAN4#111 meeting, there are some further discussion on key issues including KPIs related, the impact of measurement error on prediction accuracy and testability. The outcome was recorded in latest WF [1].

In this contribution, we further provide our views on the candidate KPIs/ Test Metrics and testability aspects for beam management.

- KPIs/ Test Metrics for Beam Management

In the previous meeting, the issue about KPIs for evaluating the AI/ML based BM are discussed many times and no agreements were reached. Next, we will provide some analysis on beam prediction accuracy and RSRP accuracy based on the simulation results.

For metrics for beam management requirements/tests, the following test metrics are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy - Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" - Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" - Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: combinations of above options

Firstly, for the beam prediction accuracy, all the metrics in Option 2 including Top-1 (%), Top-K/1 (%) and Top-1/K (%) are considered. From the simulation results listed in the first column, it can be observed that except for the accuracy of Top-1 (%), there are over 90% beam prediction accuracy under other metrics. One the top of it, we also try to verify the effect of introducing measurement error in training/validation data on prediction performance. The measurement errors are modelled as ±4.5 dB and ±6 dB truncated Gaussian distribution, respectively.

Table 1 Evaluation results of prediction accuracy for BM-Case1 DL Tx beam prediction by considering measurement error

| Simulation results |  | Prediction accuracy (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | No error | ±4.5dB error in validation data | ±4.5dB error in training/validation data | ±6dB error in validation data | ±6dB error in training/validation data |
| Top 1 | +0dB | 77.96 | 34.66 | 39.14 | 31.72 | 34.24 |
|  | +1dB | 93.72 | 51.81 | 57.00 | 48.59 | 45.58 |
| Top 2/1 | +0dB | 90.87 | 60.80 | 64.00 | 56.06 | 57.35 |
|  | +1dB | 95.23 | 72.04 | 74.64 | 66.32 | 64.52 |
| Top 4/1 | +0dB | 94.62 | 86.68 | 87.60 | 83.42 | 84.06 |
|  | +1dB | 97.09 | 90.45 | 90.54 | 87.45 | 86.39 |
| Top 5/1 | +0dB | 96.24 | 89.48 | 90.22 | 86.43 | 87.23 |
|  | +1dB | 97.89 | 92.50 | 92.50 | 89.75 | 89.09 |
| Top 1/2 | +0dB | 91.61 | 59.01 | 64.02 | 54.07 | 58.09 |
|  | +1dB | 96.50 | 79.12 | 80.36 | 73.06 | 71.01 |
| Top 1/4 | +0dB | 97.35 | 84.50 | 87.34 | 80.06 | 84.07 |
|  | +1dB | 99.23 | 94.90 | 95.16 | 92.00 | 91.35 |
| Top 1/5 | +0dB | 98.40 | 89.41 | 91.93 | 85.91 | 89.82 |
|  | +1dB | 99.48 | 97.07 | 97.39 | 95.17 | 94.98 |

It can be observed that with the number of K increases, the prediction accuracy also increases. Also, add 1dB margin will bring a noticeable gain on the prediction accuracy, especially for the Top 1 case. When the measurement error is considered in training/validation data, the performance of the prediction accuracy will degrade a lot at most 30%-40%. Besides, the training and validation dataset inconsistency due to the measurement error will slightly impact the prediction accuracy, where 2-4% performance gain is obtained for the case of Top 1 +dB margin.

Observation 1: There is a non-negligible performance degradation on beam prediction accuracy when considering measurement error modelling in training/validation data.

Observation 2: 1dB margin could bring a noticeable gain on the prediction accuracy.

Observation 3: Training/validation dataset inconsistency due to the measurement error slightly impact the prediction accuracy.

And for RSRP accuracy, before determining the concrete test metrics, RAN4 shall firstly align with the definition of RSRP accuracy. The following is the candidate two definition of RSRP difference.

- Alt1: RSRP difference only pertains to the case if AI/ML predicted beam ID is consistent with corresponding genie beam, or

- Alt2: RSRP difference between predicted Top-K beam with corresponding Top-K genie beam

Here we will firstly assume that the RSRP accuracy only focuses on the RSRP of predicted Top-1 beam. We will take the following case depicted in the Fig.1 as the example:

Fig. 1 The possible comparations between predicted beam and genie-aid beam

The first understanding is shown as the definition 1 depicted with green block: RSRP accuracy is not limited the scenario with the consistent beam ID. The predicted best beam is beam 1 while the actual best beam is beam 4, the RSRP accuracy refers to the comparison between RSRP 1 and RSRP 4’. The similar logic was ever adopted in the metric average L1-RSRP difference used for RAN1 SI evaluation, which is listed as the underlined sentence in part 1. The comparison is between the ideal L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam, where the beam IDs of the two are not necessarily the same.

Excerpts from TR38.843 Part 1: - Average L1-RSRP difference of Top-1 predicted beam: - The difference between the ideal L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam Part 2: For AI/ML models, which provide L1-RSRP as the model output, the accuracy of predicted L1-RSRP is to be evaluated. Companies optionally report average (absolute value)/CDF of the predicted L1-RSRP difference, where the predicted L1-RSRP difference is defined as the difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the same beam.

And another understanding is the depicted definition 2 depicted with red font: the comparison between predicted RSRP and actual RSRP shall be firstly based on the precondition that the beam ID is consistent. The RSRP accuracy refers to the comparison between predicted RSRP of Top-1 beam and the actual RSRP of same genie beam (e.g., between [beam 1+ predicted RSRP 1] and [beam 1+ actual RSRP 1’]) regardless of whether beam 1 is the actual best beam or not. This metric was also referred in TR38.843 as part 2 described.

Next, in order to evaluate the performance difference between the above two metrics, we will provide the corresponding RSRP accuracy simulation results using Alt1 and Alt2 as the KPI, respectively.

Table 2 and Table 3 shows the simulations results of the RSRP accuracy. Table 2 shows 90%-tile RSRP difference between the predicted L1-RSRP of the Top-1 and Top-5 predicted beams and the ideal L1-RSRP of the corresponding genie-aided beam. As for the Table 3, it shows 90%-tile RSRP difference between the predicted L1-RSRP of the Top-1 and Top-5 predicted beams and the ideal L1-RSRP of the same beams. Note that the simulation is performed based on the case of ground truth without measurement error, while the measurement errors are only added in model inference input.

Table 2 90%-tile RSRP difference between the predicted L1-RSRP of the Top-1 and Top-5 predicted beams and the ideal L1-RSRP of the corresponding genie-aided beam

| Simulation results | RSRP difference (dB) without considering errors |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1st Beam | 2nd Beam | 3rd Beam | 4th Beam | 5th Beam |
| Top 1 | 0.63 |  |  |  |  |
| Top 5/1 | 0.63 | 0.64 | 0.72 | 0.91 | 0.87 |
| Top 1/5 | 0.63 | 1.43 | 3.62 | 7.23 | 13.58 |
| RSRP difference (dB) considering errors ±4.5dB errors (only add in input) |  |  |  |  |  |
| Top 1 | 3.38 |  |  |  |  |
| Top 5/1 | 3.38 | 3.03 | 3.06 | 3.22 | 3.92 |
| Top 1/5 | 3.38 | 3.76 | 5.15 | 8.20 | 14.86 |
| RSRP difference (dB) considering errors ±6dB errors (only add in input) |  |  |  |  |  |
| Top 1 | 4.43 |  |  |  |  |
| Top 5/1 | 4.43 | 4.21 | 4.23 | 4.27 | 4.69 |
| Top 1/5 | 4.43 | 4.59 | 5.60 | 8.21 | 14.90 |

Table 3 90%-tile RSRP difference between the predicted L1-RSRP of the Top-1 and Top-5 predicted beams and the ideal L1-RSRP of the same beams

| Simulation results | RSRP difference (dB) without considering errors |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1st Beam | 2nd Beam | 3rd Beam | 4th Beam | 5th Beam |
| Top 1 | 0.63 |  |  |  |  |
| Top 5/1 | 0.63 | 0.68 | 0.78 | 1.00 | 0.87 |
| RSRP difference (dB) considering errors ±4.5dB errors (only add in input) |  |  |  |  |  |
| Top 1 | 4.70 |  |  |  |  |
| Top 5/1 | 4.70 | 3.91 | 4.12 | 4.61 | 5.12 |
| RSRP difference (dB) considering errors ±6dB errors (only add in input) |  |  |  |  |  |
| Top 1 | 6.65 |  |  |  |  |
| Top 5/1 | 6.65 | 5.44 | 5.56 | 5.91 | 6.31 |

As can be seen from the results in above two tables, the performance difference on RSRP accuracy under these two definitions is relatively small. Taking an example for Top 5/1 beam with no measurement error, the relative difference of 5 RSRP difference values are smaller than 0.1dB. Even when 6dB errors are considered, the maximum difference is smaller than 2.3dB. From Table 2, it can also be observed that the 5th beam RSRP difference is larger than 12dB for Top 1/5 case even no measurement errors are considered.

Observation 4: There is no large performance gap on the RSRP accuracy between the following two definitions of RSRP difference

- Alt1: RSRP difference only pertains to the case if AI/ML predicted beam ID is consistent with corresponding genie beam, or

- Alt2: RSRP difference between predicted Top-K beam with corresponding Top-K genie beam

Based on this observation, from the better alignment of legacy definition of L1-RSRP accuracy perspective, we prefer the RSRP accuracy is based on the premise that the beam IDs between predicted RSRP and actual RSRP are the same. If we go with Alt 2, Our concern is that there may be with misalignment of beam ID when compared. Consequently, it is quite tricky to compare the corresponding RSRP values if the predicted RSRP of beam 1 is very different from the ideal RSRP of beam 2.

Besides, from the simulation results in Table 1,2 and 3, even the beam prediction accuracy is high, the corresponding RSRP accuracy is still within a reasonable range. However, similar conclusions cannot be reached in reverse. Therefore, prediction accuracy could at least be considered as the metric to evaluate the performance of AI/ML based BM. Moreover, RAN4 could also consider to use RSRP accuracy requirement for evaluating the AI prediction performance, and the predicted RSRP values corresponding to all of Top-K predicted beams to be reported shall meet the RSRP accuracy requirements. Considering it is still pending on whether RSRP is mandatory to be reported in the report content, RAN4 can further discuss if RSRP accuracy needs to be considered as the KPI after RAN1 have the clear conclusion.

Observation 5: It can be observed that when beam prediction accuracy is high, the corresponding RSRP accuracy is also within reasonable range

Proposal 1: RAN4 to use beam prediction accuracy as the baseline of KPI and whether RSRP accuracy can be the KPI depends on if the RSRP is mandatory to be reported in the report content

- Testability issues for beam management

- 3.1 Test setup and channel model

In the last meeting, the following agreements are captured in the WF

Issue 2-3: Reported measurements and ground truth Agreement: - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded

For the performance requirement, the ideal measurement of RSRP is used as the ground truth for the predicted RSRP. While from the testing perspective, based on the discussion in the last meeting, RAN4 assume that the reported RSRP can be regarded as the approximate of the ground truth. For the reported RSRP, Rx beamforming gain is the essential factor to be considered, which may bring some uncertainty in the testing. Therefore, before RAN4 determine the certain SNR level, it may be necessary to perform simulation with consideration of SNR level and Rx beamforming gain.

Proposal 2: Before determining the certain SNR level, the simulations of whether high SNR level is necessary and the impact due to Rx beamforming gain may be needed.

For the multi-AoA test setup, no matter what kind of test framework will be chosen as the baseline, e.g., multi-Rx test framework or FR2 MIMO OTA test framework, the testing principle should be to emulate the spatial and power characteristics of receiving environment. However, different test framework will bring different impact on AoA setup, for example, the MIMO OTA will use multiple probe to emulate 1 beam with the channel model, but multi-Rx test only use 1 probe to emulate 1 AoA. Therefore, more progress of the test system is needed for discussing the impact of multiple-AoA test setup.

Observation 7: The multiple-AoA test setup depends on the test framework progress.

For the channel condition, the channel coefficients can be realized and fixed in channel emulator and other parameters can also be fixed in any time instance. Thus, a same channel condition can be realized and the UE could report the measurement values before or after the prediction. However, what king of the “same” for the channel condition is still a question if the test system is not decided yet. For example, if we keep the small-scale parameters as the same for CDL channel mode, there are still some random parameters and noise to impact the generation of the channel matrix, so as to impact the L1-RSRP measurement. But if we keep all the channel coefficients the same, there is no need to consider the mentioned questions.

Observation 8: “Same” channel condition meaning is diverse, e.g., same channel coefficient, same channel large/small scale parameter, etc.

Proposal 3: Same channel conditions are needed when testing to ensure the RSRP values are unchanging and comparable.

Issue 2-5: Test setup needs Agreement: maximum number of set B Tx beams that test system should be able to emulate: [8-16] maximum number of set A Tx beams that test system should be able to emulate: [64-128] FFS on AoAs UE rotation during the test: FFS UE rotation/repositioning between different tests: Yes

We provide a simulation results for the configuration of set B and set A in Table 4.

Table 4 Evaluation results of predict accuracy for BM-Case1 DL Tx beam prediction for different set A and set B configurations

| Simulation results |  | Predict accuracy (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Set A 64 |  | Set A 128 |  |  |
|  |  | Set B 8 | Set B 16 | Set B 16 | Set B 32 | Set B 64 |
| Top 1 | +0dB | 59.3 | 75.7 | 61.5 | 74.7 | 79.4 |
|  | +1dB | 75.8 | 93.9 | 80.6 | 90.1 | 95.6 |
| Top K/1 | K=2 | 75.6 | 91.3 | 79.5 | 88.8 | 93.4 |
|  | K=4 | 82.8 | 94.0 | 85.5 | 91.8 | 94.9 |
|  | K=5 | 86.4 | 95.2 | 87.2 | 92.9 | 95.5 |
| Top 1/K | K=2 | 75.6 | 91.5 | 78.3 | 89.3 | 93.9 |
|  | K=4 | 84.9 | 96.9 | 87.1 | 94.8 | 98.0 |
|  | K=5 | 88.0 | 98.0 | 89.5 | 96.1 | 98.9 |

It can be observed than the number of set B is at least a quarter of set A to reach the predict accuracy higher than 90% in most cases. Thus, the number of set A and set B in the test system could be considered together as: [(8,32), (16,32), (16,64), (32,64), (32,128), (64,128)], where the maximum could be (64,128).

Proposal 4: RAN4 to consider the maximum number of set B and set A Tx beams that test system should be able to emulate: (64,128).

However, the test system framework is not decided yet, it is too early to discuss the number the AoAs. For MIMO OTA, the target is to reconstruct the channel environment and all the Tx beams can be emulated, but the cost and complexity is high. For multi-Rx test system, one probe to emulate one beam and the number of AoAs is restricted by the number of probes. We can discuss this issue after more progress about the chosen of the test system are made.

Proposal 5: RAN4 to first study the test system framework and come back to discuss the number of AoAs after more progress are made.

For the UE rotation during the test, it also depends on the AI BM cases and the test framework. For the spatial domain beam prediction, it is not necessary to rotate the DUT in one test case. However, when consider the case for the generalization tests that many tests need to be performed during a continues testing time, maybe DUT rotation is needed in this dynamic testing. For time domain prediction, since UE trajectory needs to be emulated, DUT rotation is needed in tests and how to emulate and rotate is still FFS. In the next section, we provide an example in Fig. 6 and Fig. 7 to show how to rotate the UE to emulate the time domain dynamic testing.

Proposal 6: UE rotation is needed for time domain beam prediction. For spatial domain beam prediction case, there is no need to rotate the DUT in every single test. RAN4 to consider whether and how to rotate the UE for generalization tests.

Issue 2-4: Channel models - Proposals - Option 1: CDL - Option 2: TDL - Option 3: AWGN - Option 4: others - Recommended WF - Option 1

Choosing a flat TDL or AWGN channel as the propagation conditions is meaningless for AI model. The legacy predict algorithms can also obtain a good performance compare to AI under the channel without spatial characteristics and whether the AI is tested or not is unknown. For evaluating the real performance of AI prediction, using CDL model as baseline to define the test is necessary.

Proposal 7: RAN4 to use CDL channel model as the baseline for the AI/ML based BM test.

- 3.2 AI/ML based BM OTA test framework

In the previous meetings, the testability aspects have been discussed and at least the following 2 existing test frameworks could be used as the baseline:

1. Based on multi-Rx chain test system

Fig. 2. An example of testing methodology for multi-Rx chain

2. Based on FR2 MIMO OTA test system

Fig. 3. An illustration of the framework of the FR2 MIMO OTA

For the multi-Rx test system, one probe is to emulate a single DL Tx beam and the aim of the test is for emulating the beams of mTRPs, which is different from purpose of the AI/ML based beam management. The FR2 MIMO OTA test is for emulating a realistic channel environment by setup a certain number of OTA probes and channel coefficients in channel emulator in the anechoic chamber. The base station emulator could transmit signals through the reconstructed channel in the sector multi-probe anechoic chamber (S-MPAC), and the signal go through the reconstructed channel environment will be received at the DUT fixed in center of the test zone. The power weight mapped in the OTA probes and the channel coefficients running in the channel emulator are the key points to reconstruct the target channel model in the anechoic chamber, and the probe power weight can be calculated by some optimization algorithms, e.g., convex optimization, and the target function is the spatial correlation or the power angular spectrum (PAS), etc. For the AI/ML based beam management, the BS emulator could transmit Tx beams through the channel and multi-beams can also be emulated if the number of probes and the range of the probe location is enough. Thus, RAN4 could use the framework of FR2 MIMO OTA test as the test framework for AI/ML based BM, by at least considering the following aspects:

- The number of DL Tx beams that need to be emulated

- The number of OTA probes

- The range of OTA probes deployment

- The channel needs to be reconstructed

- The rotation of DUT

- The changing of the selected probe/probe power weight/coefficients running in the channel emulator

Proposal 8: RAN4 to consider the framework of FR2 MIMO OTA test as the test framework for AI/ML based BM, by at least considering the following aspects:

- The number of DL Tx beams that need to be emulated

- The number of OTA probes

- The range of OTA probes deployment

- The channel needs to be reconstructed

- The rotation of DUT for emulating movements

- The changing of the selected probe/probe power weight/coefficients running in the channel emulator

Here we give a detailed analysis of the reconstruction of the multi-beams for the FR2 MIMO OTA test system.

3.2.1 Introduction of MIMO channel modelling

A MIMO communication system can be denoted as

Where  is the received signal vector,  is the number the Rx antenna elements,  is the transmit signal vector,  is the number of Tx antenna number,  denotes the noise,  is the channel matrix. Obviously, the multi-beam signals transmitted from the BS is , the purpose of the test system is to reconstruct the spatial and time domain characteristics of the channel, which is .

According to the 3GPP TR 38.901, the channel coefficient of s-th Tx element to u-th Rx element of the n-th cluster for the NLOS channel can be written as

Where  is the polarization matrix

Fig. 3. illustrates an example of 3D MIMO channel modelling

Fig. 4. The illustration of the 3D MIMO channel modelling

For LOS channel, we can have the similar analysis and details are not explained here.

For a conducted test, it can be observed that if the ZOA, ZOD of DL Tx beams are different, the channel parameters (e.g., the field pattern of Tx, the power of the clusters, etc) will be different. Besides, if UE movements are considered, it can be also observed that the AOA/ZOA of clusters will be changed. Also, the number and direction of clusters will be different during the UE movement. Note that velocity  can also be a time-varying factor that impact the Doppler.

However, for MIMO OTA tests, there are many different aspects needs be considered.

3.2.2 MIMO OTA channel modelling

One of the mainstream methods for MIMO OTA channel modelling is the prefaded signal synthesis (PFS) method. The PFS method allocates different power weight (real number, not complex number) at different probes in different locations, so as to reconstruct the target channel in the center of the test zone. The power weight and the time-domain characteristics are both realized in the channel emulator, every single cluster are synthesized by probes and all clusters are independent identically distributed. For an OTA MPAC system with K probes, the channel  can be denoted as product of the matrix  which denotes the characteristics from the probes to the DUT Rx elements in the air and the matrix  realized in channel emulator

Assume there are U Rx elements in DUT side, then , and  can be written as

Where  and  are the field pattern of the k-th OTA probe in the polarization direction,  and  respectively.  is the sphere unit vector from k-th OTA probe to the u-th DUT Rx element,  is the distance between k-th OTA probe to the u-th DUT Rx element,  is the path loss between them.

From the analysis, it can be observed that the matrix  is decided by the DUT, OTA and the transmission space between them, different DUT, OTA probe and the space lead to different . For  needs to be realized in the channel emulator, it can be written as the following for the Tx side with S antennal elements.

Where  is the power weight of k-th probe for n-th cluster,  is the polarization matrix for OTA probe.

The purpose of the test system is to reconstruct the  similar as as much as possible. However,  is always unknown or time varying and we need to use some method to derive the number of probes, location of probes and the power weight  to emulate the spatial environment. Note that the power weights of probes are obtain by some optimization algorithms e.g., convex optimization, by setting some good target function.

3.2.3 AI/ML based BM scenarios analysis

For the BM case 1, if a certain number of beams of set B, e.g., NB, are to be emulated in the anechoic chamber, different AOD/ZOD in Tx side needs to be considered in and same AOA/ZOA of clusters can be considered for the power weight of probes can be written as . The channel coefficient can be written as . For k-th probe, it can be denoted as

Therefore, for BM case 1, channel emulator to realize the coefficients  for NB beams of set B during the total testing time T as

Where K is the number of probes, S is the number of the Tx antenna elements, N is the number of clusters, NB is the number of the DL Tx beams.  is the power weight of k-th probe for the n-th cluster when nB-th DL Tx beam is transmitted in Tx side.  is the polarization matrix.

For the BM case 2, the UE movement and the time domain non-stationary characteristics are considered for test. The straight forward thinking is that almost every parameter in the above channel coefficients are time-varing (add the independent variable t in the corresponding parameters). Besides, as we mentioned before, the AOA/ZOA and UE velocity needs to be considered. However, it is not easy to consider a continuous time domain procedure. Here we given a simple way to deal with the time-domain issue [2].

When UE is moving, the channel environment is changing all the time and the related parameters includes the AOA/ZOA, AOD/ZOD, LOS/NLOS, number of clusters, path loss, etc. Fig. 4. illustrates an example of the channel parameters changing during the UE movement.

Fig. 5. An illustration of the channel parameters changing during the UE movement

Thus, some non-static factors need to be considered in the BM case 2 test. We can consider to reconstruct the non-static channel in different moments or time-domain segments. Fig. 5. shows an example of the mapping of the non-static channel and the FR2 MIMO OTA test system. Note that the LOS in Fig. 5 is only for the LOS direction, it does not mean that the channel model under test is a LOS channel.

Fig. 6. An illustration of the mapping between of MIMO channel model and the FR2 MIMO OTA test system

RAN4 could define a UE trajectory with its channel environments, then separate the duration of UE movement into multiple NB segments. In every segment, the Tx side needs to emulate a fixed DL Tx beam and the total number of segments equals to the number of set B. For example, if the total testing time is T, it needs to be separated into NB segments as T1, T2, …, TNB, where T=T1+T2+…+TNB. Fig. 6. shows the illustration of test framework of BM case 2 when UE is moving.

Fig. 7. An illustration of test framework of BM case 2

Note that different or same probe deployment (number and position of probes) for different DL Tx beams could be considered for this test system. It depends on different implementation and cost of the test system. Fig .6. is a different probe deployment implementation for different DL Tx beam.

Therefore, for BM case 2, the total testing time T could be divided into NB segments as [T1, T2, …, TNB], where NB is the number of beams of set B. The power weight of probes, channel coefficients and the UE direction need to be considered in every single segment.

For BM case 2, channel emulator to realize the coefficients  for NB beams of set B remain the same as proposal 4 for BM case 1 except velocity becomes vnB. and testing time is TnB

Where K is the number of probes, S is the number of the Tx antenna elements, N is the number of clusters, NB is the number of the DL Tx beams.  is the power weight of k-th probe for the n-th cluster when nB-th DL Tx beam is transmitted in Tx side.  is the polarization matrix.

Proposal 9: UE rotation coordinates with the changing of the channel coefficient needs to be considered for BM case 2.

Proposal 10: For BM case 2, the probe power weight can be derived by considering:

- Different AOA/ZOA of clusters and different number of clusters

- Different or same probe deployment (number and position of probes) for different DL Tx beams

- Different probe power weight for different DL Tx beams

- 3.3 Probe number reduction method

Many concerns about the number of probes for emulating a great number of Tx beams are raised by considering the complexity and cost of the test system. Therefore, how to reduce the number of probes so that the cost of test system can be reduced is an importance issue need to be studied. Here we give an example to show the way to reduce the probe number in multi-probe anechoic chamber test system.

UE always recognize the AoA direction of the strongest power path as the reference path for beamforming. When some certain different Tx beams are sweeping and transmitted in the channel environment, the AoA of the strongest power path will not change while the power is still changing. Fig. 8 shows an example of the barlett power spectrum for AoA estimation with different Tx beams in fading channel environment, where the UE is assumed as a 4×4 antenna array. It can be observed that at least for beam sets 2-3, 5-10, 13-14-… the AoAs of the strongest path in UE side are the same.

Fig. 8. Bartlett power spectrum for AoA estimation with different Tx beams

Based on the above understanding, some Tx beams can be grouped together (e.g., beam 5-10) if the strongest receive power AoA are the same. Besides, there are different ways to group the Tx beams and needs to be studied. For example, setting a range for AoA of the strongest path and using this range to group Tx beams. Strongest path AoA based grouping is just an example for introducing this theory. An illustration of the Tx beam grouping is shown in Fig. 9.

Fig. 9. Tx beams grouping

Since only AoA of the signal needs to be emulated in the anechoic chamber while different Tx beams in one group are corresponding to only one AoA, which means that only one probe with one AoA to emulate multiple Tx beams in one group is feasible, and the number of groups equals to the number of probes. TE only needs to change the power of the specific probe in the specific time to emulate the power of the different Tx beams transmitted through fading channel, which makes the test system much easier. Fig. 10. shows an illustration of multi-probe anechoic chamber test system with the Tx beam grouping method. In this MPAC system, only power and direction need to be emulated which is applicable for AI/ML based BM case.

In Fig. 10., the probe 1, 2, …, L is for emulating the Tx beams in group 1, 2, …, L, respectively. Only one probe is allowed to be active at the same time for transmitting the beams in the corresponding group. The Tx beams are realized in BS emulator and the fading channel is realized in channel emulator (CE). When the probe 1 is active and probe 2 to L are deactive, probe 1 could emulate the same AoA and different power of different beams in group 1, which are the basic elements for AI/ML based BM testing (L1-RSRPs and spatial characteristics).

Fig. 10. Illustration of MPAC test system with Tx beam grouping for emulating AoA and power

Besides, if other channel characteristics needs to be reconstructed in quiet zoom, the test system in Fig. 11 can be referred to reconstruct the channel environment, e.g., spatial correlation, PAS, etc. However, more probes will be needed in this MPAC system.

In Fig. 11., the probe set [1, 2, …,K] is for emulating the Tx beams in group 1, and another set […, i, j,…] is for emulating the Tx beams in group 2, and so on. Only one set of probes is allowed to be active at the same time for transmitting the beams in the corresponding group. When the probes in probe set [1, 2, …,K] are active and other probe sets are deactive, probe set [1, 2, …,K] could emulate the spatial correlation, PAS. This test system could emulate a more accurate environment that the test system in Fig. 10., but more probes are needed.

Fig. 11. Illustration of MPAC test system with Tx beam grouping for reconstructing channel environment

Observation 9: The number of probes can be greatly reduced by grouping different Tx beams together. Only one probe or one set of probes can emulate different Tx beams in one group.

Observation 10: Different test system with reduced cost and complexity can be introduced by using the Tx beam grouping method.

Proposal 11: RAN4 to consider methods to group Tx beams together to reduce the number of probes in the test system.

- Summary

In this contribution, we further provided our views related issues on beam management. Based on above analysis, following proposals are present.

Observation 1: There is a non-negligible performance degradation on beam prediction accuracy when considering measurement error modelling in training/validation data.

Observation 2: 1dB margin could bring a noticeable gain on the prediction accuracy.

Observation 3: Training/validation dataset inconsistency due to the measurement error slightly impact the prediction accuracy.

Observation 4: There is no large performance gap on the RSRP accuracy between the following two definitions of RSRP difference

- Alt1: RSRP difference only pertains to the case if AI/ML predicted beam ID is consistent with corresponding genie beam, or

- Alt2: RSRP difference between predicted Top-K beam with corresponding Top-K genie beam

Observation 5: It can be observed that when beam prediction accuracy is high, the corresponding RSRP accuracy is also within reasonable range

Proposal 1: RAN4 to use beam prediction accuracy as the baseline of KPI and whether RSRP accuracy can be the KPI depends on if the RSRP is mandatory to be reported in the report content

Observation 6: Whether a high SNR is needed or not depends on the KPI selection and the requirement.

Proposal 2: Before determining the certain SNR level, the simulations of whether high SNR level is necessary and the impact due to Rx beamforming gain may be needed.

Observation 7: The multiple-AoA test setup depends on the test framework progress.

Observation 8: “Same” channel condition meaning is diverse, e.g., same channel coefficient, same channel large/small scale parameter, etc.

Proposal 3: Same channel conditions are needed when testing to ensure the RSRP values are unchanging and comparable.

Proposal 4: RAN4 to consider the maximum number of set B and set A Tx beams that test system should be able to emulate: (64,128).

Proposal 5: RAN4 to first study the test system framework and come back to discuss the number of AoAs after more progress are made.

Proposal 6: UE rotation is needed for time domain beam prediction. For spatial domain beam prediction case, there is no need to rotate the DUT in every single test. RAN4 to consider whether and how to rotate the UE for generalization tests.

Proposal 7: RAN4 to use CDL channel model as the baseline for the AI/ML based BM test.

Proposal 8: RAN4 to consider the framework of FR2 MIMO OTA test as the test framework for AI/ML based BM, by at least considering the following aspects:

- The number of DL Tx beams that need to be emulated

- The number of OTA probes

- The range of OTA probes deployment

- The channel needs to be reconstructed

- The rotation of DUT for emulating movements

- The changing of the selected probe/probe power weight/coefficients running in the channel emulator

Proposal 9: UE rotation coordinates with the changing of the channel coefficient needs to be considered for BM case 2.

Proposal 10: For BM case 2, the probe power weight can be derived by considering:

- Different AOA/ZOA of clusters and different number of clusters

- Different or same probe deployment (number and position of probes) for different DL Tx beams

- Different probe power weight for different DL Tx beams

Observation 9: The number of probes can be greatly reduced by grouping different Tx beams together. Only one probe or one set of probes can emulate different Tx beams in one group.

Observation 10: Different test system with reduced cost and complexity can be introduced by using the Tx beam grouping method.

Proposal 11: RAN4 to consider methods to group Tx beams together to reduce the number of probes in the test system.

- References

- R4-2410570	WF on AI/ML, Qualcomm

- Z. Jiang, Z. Wang, C. Guo, Z. -C. Hao and W. Hong, Time-Domain Nonstationary Channel Emulation in Multiprobe Anechoic Chamber Setups for Over-the-Air Testing[J]. IEEE Antennas and Wireless Propagation Letters, 2021, 20(12):2511-2515.