3GPP TSG RAN WG4 Meeting #112	R4-2412251

Maastricht, NL, 19 Aug - 23 Aug, 2024

Agenda item:			8.17.3

Source:	vivo

Title:	Discussion on testability and interoperability issues for positioning accuracy enhancement

Document for:	Discussion

- Introduction

RAN4 has studied many aspects about AI/ML use case aspects in Rel-18 SI stage, including CSI feedback enhancement, beam management enhancement and positioning accuracy enhancement. The outcomes of the study for positioning are captured in TR 38.843 [1]. In the RAN4#111 meeting, the WF [2] for AI/ML based positioning is captured as

Issue 3-2: Requirements for case 2a Agreement: No requirements on LMF for positioning accuracy FFS on RAN4 requirements for any UE reported measurements defined by other groups Issue 3-3: Requirements for case 2b Agreement: No requirements on LMF for positioning accuracy FFS on RAN4 requirements for any UE reported measurements defined by other groups

In this contribution, we further provide our views on testability and interoperability aspects for positioning accuracy enhancement.

- Discussion

For AI/ML positioning cases, 5 sub-use cases are identified and summarized in Table 1.

- A detailed list of various positioning cases

| Cases | Priority | Model deployment | Positioning methods | Measured by which entity | Model output | Position calculation |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1 | 1st priority | UE-side | Direct AI/ML positioning | UE (Downlink positioning) | Position | UE-side |
| Case 2a | 2nd priority | UE-side | AI/ML assisted positioning | UE (Downlink positioning) | Intermediate feature | LMF |
| Case 2b | 2nd priority | LMF-side | Direct AI/ML positioning | UE (Downlink positioning) | Position | LMF |
| Case 3a | 1st priority | gNB-side | AI/ML assisted positioning | gNB (uplink positioning) | Intermediate feature | LMF |
| Case 3b | 1st priority | LMF-side | Direct AI/ML positioning | gNB (uplink positioning) | Position | LMF |

- 2.1	Potential RAN4 requirement

In RAN1#116bis meeting, following agreements about performance monitoring for case 1 are reached as

Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded.

For the performance monitoring mechanisms that still under discussion in RAN1, there are many options under discussion that may have RAN4 impacts for delay and accuracy requirements.

For option A-1, the label-based monitoring metrics calculation is performed at UE side and the ground truth label is generated by LMF and provided to the target UE. In that case, LMF may indicate UE to perform monitoring procedure through signaling and then the target UE may inform LMF the monitoring decision or monitoring metrics calculation results (NW decision) through report. These procedures imply a delay requirement and the details are depending on RAN1/2 progress. Besides, as the agreement mentioned, the target UE could send measurement or monitoring metrics to LMF so that LMF can derive the information on ground truth label or make a decision, which means that it may have potential accuracy requirement.

For option A-2 and 3, the label-based monitoring metrics calculation is performed at UE side and some assistance data can be transferred from LMF to UE side. These procedures may also have delay requirement and more progress of RAN1/2 are needed for discussing. RAN4 may also needs to consider the measurement accuracy requirement for the assistance data, e.g., PRU measurement accuracy, if necessary.

For option A-4, the PRU measurement are sent from PRU to the target UE, it is more like implementations and vendors could use different ways for sending information, e.g., OTT server, sidelink. More progress is needed for discussing this option.

For option B-1, the label-based monitoring metrics calculation is performed at LMF side and the model inference needs to be sent from target UE to LMF. In that case, it is very clear for RAN4 to identify the delay and accuracy requirement. For delay requirement, the LMF may indicate UE to perform AI operations, and then UE reports the inference results (e.g. AI estimated UE position) to LMF. For accuracy requirement, it depends on what kind of metrics UE reports. For case 1, the accuracy requirement for UE position needs to be defined in that case. Here we give an example for accuracy of UE position. The reported contents may be a coordinate (x1, y1, z1) for P1, x1, y1 and z1 are the related location compared with the local coordinate system, and the real coordinate of the target UE is (x0, y0, z0) for P0, then the accuracy metric can be the Euclidean distance of these two coordinates:

In these methods, the accuracy requirement can be simply defined and the unit is meter. If the reported content is other types, it can be simplified as . Dmin represents the straight-line distance of two position points.

For option B-2, the target UE uses the PRU channel measurements as the model input and outputs the estimated PRU position as the model inference result to report to LMF for performing monitoring procedure. It is also very clear that the accuracy requirement for the reported KPI needs to be defined, e.g., the accuracy requirement for UE (PRU) position as we mentioned in option B-1. These procedures may also have the delay requirement from RAN4 perspective,

Observation 1: Potential report schemes need to be introduced in performance monitoring procedures according to RAN1 agreements and RAN4 requirements for delay/accuracy may need to be considered in different options.

Observation 2: Delay/accuracy requirement for performance monitoring procedure in case 1 may need to be defined and more progress from RAN1/2 are needed for detailed further discussing.

Observation 3: More progress for the monitoring procedure from other WGs are needed to consider whether and how to define requirement/test for the potential metrics in case 1.

In RAN2#126 meeting, following agreements about NW side model are reached as

Agreements for NWside model 1 RAN2 to await RAN1 progress to determine need for any specification work on AI/ML positioning Case 3a and Case 3b.

Agreements about UE side model are reached as

Agreements - The LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) are used to indicate supported AI/ML positioning capabilities. FFS how to handle dynamic capabilities, depending on further RAN1 progress and understanding of the functionality. - wait for RAN1 for associate ID discussion - At least for Case 1, existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing the results of the UE sided model inference operation. FFS further details on signaling enhancements

In the current stage, RAN1 has not reached enough agreement about the need of the specification work for case 3a and 3b. RAN4 can wait the progress of RAN1/2 to decide whether to define the requirement for case 3a and 3b.

For case 1, RAN2 agreed to use the existing LPP procedures related to Location Information Transfer to provide the UE sided model inference results, which is the UE position, to the NW side. It means that accuracy requirements for UE position may need to be defined for case 1. As we mention before, the straight-line distance (unit: m) between the real position and the estimated position could be considered as the metric for accuracy requirement. However, there are no legacy requirement for the UE position and hard to test in the lab. Perhaps using PRUs location as the ground truth value and PRUs channel measurement as model input is a feasible test method according to RAN1’s monitoring mechanism. RAN4 needs to discuss whether and how to define requirements/tests for the case 1.

Observation 4: Existing LPP procedures related to Location Information Transfer are used for case 1 and the UE position, which is the output for case 1, is supported for reporting.

Observation 5: PRUs input and ground truth location could be used for testing the AI model for case 1.

Proposal 1: RAN4 to consider whether and how to define accuracy requirement and test for UE position in case 1.

- Summary

In this contribution, we provided our views on AI/ML based positioning. Based on above analysis, following observations and proposals are present.

Observation 1: Potential report schemes need to be introduced in performance monitoring procedures according to RAN1 agreements and RAN4 requirements for delay/accuracy may need to be considered in different options.

Observation 2: Delay/accuracy requirement for performance monitoring procedure in case 1 may need to be defined and more progress from RAN1/2 are needed for detailed further discussing.

Observation 3: More progress for the monitoring procedure from other WGs are needed to consider whether and how to define requirement/test for the potential metrics in case 1.

Observation 4: Existing LPP procedures related to Location Information Transfer are used for case 1 and the UE position, which is the output for case 1, is supported for reporting.

Observation 5: PRUs input and ground truth location could be used for testing the AI model for case 1.

Proposal 1: RAN4 to consider whether and how to define accuracy requirement and test for UE position in case 1.

- References

- TR 38.842 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)

- R4-2410570 WF on AI/ML Qualcomm Incorporated

- R4-2400617 WF on AI/ML Qualcomm Incorporated