3GPP TSG RAN WG4 Meeting #112	R4-2412256

Maastricht, NL, 19 Aug - 23 Aug, 2024

Agenda item:			8.15.2.1

Source:	vivo

Title:	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Document for:	Discussion

- Introduction

In the RAN#103 meeting, a new WID on enhancements for NR and MR-DC RRM requirements was approved [1]. The objectives for RAN4 in the WID are as following

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

In the last RAN4 meeting, the topic of FR2-1 SSB based L3 measurement delay reduction for connected model is discussed and the way forwards are captured in [2]. In this contribution, we provided our views on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor.

- Discussion

- 2.1 L3 measurement delay reduction by optimizing Rx BSF for UE supporting multiple-Rx simultaneous reception

In general, the FR2 L3 measurement delay is much longer than the delay in FR1 due to the Rx beam sweeping factor and the enhancement is needed. There are so many works have been already done in Rel-18 for reducing the FR2 measurement delay, e.g., introducing UE capability for FR2 L1 measurement delay reduction with reduced Rx beam sweeping factor (currently under discussion). For UE supporting multiple-Rx simultaneous reception on single carrier, RAN4 could consider to extend the capability to apply for L3 measurement for reducing the delay.

Proposal 1: RAN4 to consider introduce multi-Rx UE capability of reduced Rx beam sweeping factor for FR2 L3 measurement delay reduction.

In the last meeting, following WF is reached

UE Power class: FFS the following options and moderator option: - Option 1 (Apple, OPPO, LGE, vivo, HW, Samsung): - RAN4 to consider UE supporting FR2-1 power class 3 as first priority. - Option 1a (LGE): - but RAN4 should consider if other power classes could apply the outcome of the WI discussion - Option 2 (CATT): - It is proposed not to restrict the number of configured carriers and applied power class. - Option 3 (Ericsson): - We can have generic requirements for all power classes, maybe PC6 can be precluded, if no specific use cases for some power classes are pursued. - Option 4 (Moderator) - Baseline: RAN4 to consider UE supporting FR2-1 power class 3 as first priority. - Note: whether other power classes could apply the outcome of the WI discussion can be FFS after concluding on PC3. These extra FFS parts will NOT delay the WI completion. Other clarification on WID: - FFS: - “For UE supporting multiple-Rx simultaneous reception for L3 delay enhancement” means UE supporting “simultaneous reception of multiple SSBs from different directions of the same target frequency layer inside a SMTC window. But it does not mean “UE can process multiple SSBs from different directions of the target frequency in parallel.”

For the 1st issue about the UE power class, it is OK to treat the PC3 as the 1st priority and whether other power classes could apply depends on the conclusion of PC3. For the 2nd issue, it is obvious that whether UE can process multiple SSBs from different directions of the target frequency in parallel depends on implementation and RAN4 only needs to focus on the scenario of simultaneous reception. Therefore, the clarification on WID does make sense and the FFS can be removed.

Proposal 2: RAN4 to consider UE supporting FR2-1 power class 3 as first priority and whether other power classes could apply the outcome of the WI discussion can be FFS after concluding on PC3.

Proposal 3: For the issue of clarification on “for UE supporting multiple-Rx simultaneous reception for L3 delay enhancement”, there is no need to consider whether UE can process multiple SSBs from different directions of the target frequency in parallel and the FFS can be removed.

Issue 2-1-2: Conditions to apply L3 measurement delay reduction by optimizing Rx BSF [Moderator note]: The condition here means in which case/condition/use-case/mode UE can apply the L3 measurement delay reduction by optimizing Rx BSF. - FFS: multi-Rx simultaneous reception of UE is in active mode, which is expected to follow the one specified in Rel-18 for multi-Rx simultaneous reception feature - FFS: UE’s mobility status, e.g., whether HST is precluded or not - FFS: RRM measurement with two panels activated, two searchers are occupied by this single carrier - FFS: SSB processing delay/time for processing multiple beams received in a SMTC - FFS: Power consumption issue - FFS: UE has prior knowledge on the cell to be measured - FFS: Rel-19 L3 measurement with multi-Rx DL reception is irrelevant to multi-TRP operation deployment - FFS: cell-centre UE or cell-edge UE - FFS: DRX is configured or not (newly added issue in this meeting based on Xiaomi’s proposal) - FFS: Simultaneous operation between L3 and L1 measurements

First of all, we think the last bullet that simultaneous operation between multi-Rx simultaneous reception (L1) and L3 measurement delay reduction by optimizing Rx BSF is out of scope and no need to consider for RAN4.

Condition 1: multi-Rx simultaneous reception of UE is in active mode, which is expected to follow the one specified in Rel-18 for multi-Rx simultaneous reception features. In fact, multi-Rx simultaneous reception of UE is in active mode in Rel-18 is mainly for data scheduling. In Rel-19, mobility is what we are focusing on and the conditions in Rel-18 may not be applicable. RAN4 needs to consider new conditions for Rel-19 L3 measurement delay reduction.

Condition 2: UE’s mobility status, e.g., whether HST is precluded or not. We think RAN4 needs to treat the low mobility status as the baseline to find out the optimization ways and no need to consider the HST in Rel-19.

Condition 3: RRM measurement with two panels activated, two searchers are occupied by this single carrier. We think this condition is make sense and since 2 searchers are occupied by single carrier, it may have impacts to other CCs.

Condition 4: SSB processing delay/time for processing multiple beams received in a SMTC. Since the interval between two SMTC is long enough for buffering and processing, there is no need to consider such issue for L3 measurement delay reduction.

Condition 5: Power consumption issue. It is a very important issue for BSF optimization based RRM measurement since it is a long-term activity and there is no reason to active the multi-Rx all the time which will cause the overheating issues.

Condition 6: UE has prior knowledge on the cell to be measured. Different prior knowledges may have different impacts for BSF optimization based RRM measurements and more study is needed.

Condition 8: Rel-19 L3 measurement with multi-Rx DL reception is irrelevant to multi-TRP operation deployment. Rel-18 discussion is based on mTRP from NW perspective. For Rel-19 BSF optimization, RRM measurements are more important in cell-edge which is different from the typical scenario of the mTRP.

Condition 9: Cell-centre UE or cell-edge UE. As we mentioned before, cell-edge RRM measurement may be more important than the cell-centre RRM measurement in Rel-19, and it needs to be studied.

Condition 10: Whether DRX is configure or not has no impact on the discussion for the BSF based L3 measurement delay reduction and there is no need to consider this issue at the current stage.

Condition 11: The issue of simultaneous operation between L3 and L1 measurements is out of scope and no need to discussion. However, another issue needs to be raised for discussion: when the UE in multi-panel mode for measuring L1-RSRP is deactived, can multi-panel based L3 measurement still performing?

Proposal 4: RAN4 to discuss the following conditions for UE to apply L3 measurement delay reduction by optimizing Rx BSF.

  - RAN4 to consider new conditions for Rel-19 multi-Rx simultaneous reception of UE is in active mode.

  - Low mobility status

  - RRM measurement with two panels activated, two searchers are occupied by this single carrier

  - Time for buffering and processing multiple beams received in a SMTC is long enough and no need to considered the condition.

  - Power consumption issue is important and BSF reduction of L3 measurement will not trigger UE to activate multi-Rx

  - UE has prior knowledge on the cell to be measured

  - Rel-19 L3 measurement with multi-Rx DL reception is irrelevant to multi-TRP operation deployment

  - Consider cell-edge UE for L3 FBS

  - The issue of simultaneous operation between L3 and L1 measurements is out of scope and no need to discussion

  - RAN4 to consider whether the multi-panel based L3 measurement still performing or not when the multi-panel based L1 measurement is deactived.

Issue 2-1-3: Scenarios to use L3 measurement delay reduction by optimizing Rx BSF [Moderator note]: The scenarios here means which UE behavior/activity/procedure(s) would be improved with this feature or which corresponding requirements in the existing RRM spec will be enhanced to accommodate this feature. FFS on the justifications to decide which of the following scenarios shall be considered. Encourage companies to discuss the expected benefit to considering BSF reduction for the following scenarios. FFS on following scenarios to decide which ones are considered to use L3 measurement delay reduction by optimizing Rx BSF: - Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Scenario 5: Handover - Scenario 6: PSCell addition - Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection - Scenario 8: SCell activation - Scenario 9: SCG activation - Scenario 10: CGI identification - Scenario 11: CSI-RS based intra-/inter-frequency measurements, the CSI-RS is configured associatedSSB. The discussion on CSI-RS configured with associatedSSB could be revisited if SSB based L3 measurement delay reduction is concluded.

From our perspective, at least the SSB based inter/intra frequency with/without MG needs to be considered (scenario 1-4). The straightforward way is to reduce the value M for scenarios 1-4.

Mpss/sss_sync_w/o_gaps: For a UE supporting FR2-1 power class 1 or 5, Mpss/sss_sync_w/o_gaps =40. For a UE supporting power class 2, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-1 power class 3, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-1 power class 4, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-2 power class 1, Mpss/sss_sync_w/o_gaps = 60. For a UE supporting FR2-2 power class 2, Mpss/sss_sync_w/o_gaps = 36. For a UE supporting FR2-2 power class 3, Mpss/sss_sync_w/o_gaps = 36. Mmeas_period_w/o_gaps: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_w/o_gaps =40. For a UE supporting FR2-1 power class 2, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-1 power class 3, Mmeas_period_w/o_gaps =24. For a UE supporting power class 4, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-2 power class 1, Mmeas_period_w/o_gaps = 60. For a UE supporting FR2-2 power class 2, Mmeas_period_w/o_gaps = 36. For a UE supporting FR2-2 power class 3, Mmeas_period_w/o_gaps = 36. MSSB_index_intra: For a UE supporting FR2-2 power class 1, MSSB_index_intra = 72 samples. For a UE supporting FR2-2 power class 2, MSSB_index_intra = 48 samples. For a UE supporting FR2 power class 3, MSSB_index_intra = 48 samples.

Proposal 5: RAN4 to considered the SSB based inter/intra frequency with/without MG as the scenarios to use L3 measurement delay reduction by optimizing Rx BSF and find solutions to reduce the M value.

We also think the following aspects are out of scope:

    - Handover

    - PSCell addition

    - RRC Re-establishment/RRC Connection Release with Redirection

    - SCell activation

    - SCG activation

    - CGI identification

    - CSI-RS based intra-/inter-frequency measurements, the CSI-RS is configured associatedSSB

      - The discussion on CSI-RS configured with associatedSSB could be revisited if SSB based L3 measurement delay reduction is concluded.

Proposal 6: Following aspects are out of scope:

    - Handover

    - PSCell addition

    - RRC Re-establishment/RRC Connection Release with Redirection

    - SCell activation

    - SCG activation

    - CGI identification

    - CSI-RS based intra-/inter-frequency measurements, the CSI-RS is configured associatedSSB

      - The discussion on CSI-RS configured with associatedSSB could be revisited if SSB based L3 measurement delay reduction is concluded.

- Summary

In this contribution, we provided our views on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor. Based on above analysis, following proposals are present.

Proposal 1: RAN4 to consider introduce multi-Rx UE capability of reduced Rx beam sweeping factor for FR2 L3 measurement delay reduction.

Proposal 2: RAN4 to consider UE supporting FR2-1 power class 3 as first priority and whether other power classes could apply the outcome of the WI discussion can be FFS after concluding on PC3.

Proposal 3: For the issue of clarification on “for UE supporting multiple-Rx simultaneous reception for L3 delay enhancement”, there is no need to consider whether UE can process multiple SSBs from different directions of the target frequency in parallel and the FFS can be removed.

Proposal 4: RAN4 to discuss the following conditions for UE to apply L3 measurement delay reduction by optimizing Rx BSF.

  - Multi-Rx simultaneous reception of UE is in active mode, which is expected to follow the one specified in Rel-18 for multi-Rx simultaneous reception features

  - Low mobility status

  - RRM measurement with two panels activated, two searchers are occupied by this single carrier

  - Time for buffering and processing multiple beams received in a SMTC is long enough and no need to considered the condition.

  - Power consumption issue is important and BSF reduction of L3 measurement will not trigger UE to activate multi-Rx

  - UE has prior knowledge on the cell to be measured

  - Rel-19 L3 measurement with multi-Rx DL reception is irrelevant to multi-TRP operation deployment

  - Consider cell-edge UE for L3 FBS

  - The issue of simultaneous operation between L3 and L1 measurements is out of scope and no need to discussion

Proposal 5: RAN4 to considered the SSB based inter/intra frequency with/without MG as the scenarios to use L3 measurement delay reduction by optimizing Rx BSF and find solutions to reduce the M value.

Proposal 6: Following aspects are out of scope:

    - Handover

    - PSCell addition

    - RRC Re-establishment/RRC Connection Release with Redirection

    - SCell activation

    - SCG activation

    - CGI identification

    - CSI-RS based intra-/inter-frequency measurements, the CSI-RS is configured associatedSSB

      - The discussion on CSI-RS configured with associatedSSB could be revisited if SSB based L3 measurement delay reduction is concluded.

- References

- R4-240830 New WID: WI resulting from discussion on RRM enhancements

- R4-2410260 WF for [111][228] NR_RRM_Ph5 Apple