3GPP TSG RAN WG4 Meeting #112	R4-2412257

Maastricht, NL, 19 Aug - 23 Aug, 2024

Agenda item:			8.15.2.2

Source:	vivo

Title:	Discussion on FR 2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Document for:	Discussion

- Introduction

In the RAN#103 meeting, a new WID on enhancements for NR and MR-DC RRM requirements was approved [1]. The objectives for RAN4 in the WID are as following

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

In the last RAN4 meeting, the topic of FR2-1 SSB based L3 measurement delay reduction for connected model is discussed and the way forwards are captured in [2]. In this contribution, we provided our views on on FR 2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC.

- Discussion

- 2.1 L3 measurement delay reduction by optimizing CSSF for UE not in multiple-Rx simultaneous reception mode

Issue 2-2-1: Clarification on the bullets in WID for this CSSF optimization FFS: - Option 1 (Apple): Rel-19 CSSF optimization applies for the both cases: (1)UE is not capable of Rel-18 multi-Rx simultaneous reception, (2)UE is capable of Rel-18 multi-Rx simultaneous reception but work in single-Rx currently. - Option 2 (ZTE): Besides the case UE is not capable of R18 multi-Rx simultaneous reception, R19 CSSF optimization is also applied to the case that UE is capable of multi-Rx but not configured with GBBR report. - Option 3 (Ericsson): Clarify whether ‘the case UE is capable of Rel-18 multi-Rx simultaneous reception but work in single-Rx currently’. - Does it indicate that the UE only can use a single panel (and subsequent receiver including baseband) out of multiple panels for reception and measurement? If so, is there any degradation of reception and measurement from measuring on a single panel? - Option 4 (Nokia): Rel-19 discussion on the scenarios for CSSF optimization will be considered in CA/DC scenarios, independently of the UE support of multi-Rx capabilities.

For the first issue of clarification on the bullets in WID for this CSSF optimization “for UE not in multiple-Rx simultaneous reception mode”, it has been already discussed in RAN#103 and the following two cases are considered as the common understanding:

Case 1: UE is not capable of multiple-Rx simultaneous reception

Case 2: UE is capable of multiple-Rx simultaneous reception but the UE is not in multi-Rx operation.

For the 2 cases, the enhanced solutions for CSSF optimization are different and both cases need to be considered. Therefore, option 1 is OK for us.

Propose 1: Following two cases applies:

- Case 1: UE is not capable of multiple-Rx simultaneous reception

- Case 2: UE is capable of multiple-Rx simultaneous reception but the UE is not in multi-Rx operation.

For UE not in multiple-Rx simultaneous reception mode in CA/DC scenario, the current delay requirements for measurement outside gap are defined based on 2 searchers assumption for total CCs. For FR2, more resources are needed and another independent searcher may be needed. In the current specification, a UE support 2-searcher is assumed as a PCC use one searcher, and SCC share the other searcher. However, consider the scenario that a UE is measuring a FR1 PCC and a FR2 SCC outside gap (CSSF=1), while a FR2 SCC is within the gap. In this scenario, a UE support 3-searchers measurement is perhaps implied to be considered to meet the requirements. However, changing the number of searchers is a very challenged job for vendors and whether the 3 searcher is a missing piece of current spec needs to be discussed.

Observation 1: It is a very challenged job to change the number of searchers and 3 searcher-implementation may impacts a lot of aspects of UE architecture.

Issue 2-2-2: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF FFS: - Option 1: the following scenarios in CA/DC to use L3 measurement delay reduction by optimizing CSSF shall be prioritized: - SSB based Intra-frequency measurement without MG (Apple, CATT, CMCC, vivo, CTC, HW, Nokia, MTK) - Alt1: including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Alt2: including TPSS/SSS_sync_intra, TSSB_time_index_intra and TSSB_measurement_period_intra - SSB based Inter-frequency measurement without MG, (Apple, CATT, CMCC, vivo, CTC, HW, Nokia, MTK) - Alt1: including TPSS/SSS_sync_inter and TSSB_measurement_period_inter - Alt2: including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Inter-RAT SSB measurement without MG (CMCC, Ericsson) - NeedForGaps measurement without MG, including both with and without interruption (Ericsson) - NCSG measurement without MG without interruption (Ericsson)

It is OK for the scenario 1 that the measurement is based on the SSB based Intra-frequency measurement without MG, For the SSB based inter-frequency measurement without MG scenario 2, we believe that there are also some impacts on scenario 2 if we optimize CSSF for scenario 1, since the 2 scenarios use the common CSSF table.

Proposal 2: RAN4 to consider the SSB based intra and inter frequency measurement without MG as the scenarios to use L3 measurement delay reduction by optimizing CSSF.

Issue 2-2-3: Applicability requirement of L3 measurement delay reduction by optimizing CSSF FFS: - Option 1 (Apple, CATT, OPPO, CMCC, ZTE): RAN4 to consider following CA/DC mode for L3 measurement delay reduction by optimizing CSSFoutside_gap,i - EN-DC (Apple, CATT, OPPO, CMCC, ZTE, Ericsson): - Intel proposed to deprioritize EN-DC - NE-DC (Apple, CATT, OPPO, CMCC, ZTE, Intel, Ericsson): - SA (Apple, CATT, OPPO, CMCC, ZTE, CTC, Intel, HW): - FR2+FR2 CA (HW), FR1+FR2 CA (HW, Ericsson), FR2 only intra band CA (Ericsson), FR2 only inter band CA (Ericsson) - NR-DC (Apple, CATT, OPPO, CMCC, ZTE, CTC, Intel) - FR1+FR2 NR-DC (HW, Ericsson)

According to the WID and RAN#103 discussion, any CA and DC modes are not precluded and can be discussion. In the current spec, EN-DC, SA, NR-DC, NE-DC modes are all covered and can also be considered in Rel-19.

Proposal 3: RAN4 to consider EN-DC, SA, NR-DC, NE-DC modes to define requirements of L3 measurement delay reduction by optimizing CSSF.

- Summary

In this contribution, we provided our views on FR2-1 SSB based L3 measurement delay reduction for connected mode. Based on above analysis, following observations and proposals are present.

Propose 1: Following two cases applies:

- Case 1: UE is not capable of multiple-Rx simultaneous reception

- Case 2: UE is capable of multiple-Rx simultaneous reception but the UE is not in multi-Rx operation.

Observation 1: It is a very challenged job to change the number of searchers and 3 searcher-implementation may impacts a lot of aspects of UE architecture.

Proposal 2: RAN4 to consider the SSB based intra and inter frequency measurement without MG as the scenarios to use L3 measurement delay reduction by optimizing CSSF.

Proposal 3: RAN4 to consider EN-DC, SA, NR-DC, NE-DC modes to define requirements of L3 measurement delay reduction by optimizing CSSF.

- References

- R4-240830 New WID: WI resulting from discussion on RRM enhancements

- R4-2410260 WF for [111][228] NR_RRM_Ph5 Apple