3GPP TSG-RAN4 Meeting #112 	R4-2412269

# Maastricht, Netherlands, 19- 23 August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | DraftCR | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 38.101-1 to add PC support for CA_n28A-n41A-n77A with 2UL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | HPUE_NR_CADC_SUL_R19 |  |  |  |  |  | Date: |  |  | 2024-07-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In WI HPUE_FR1_TDD_NR_CADC_SUL_R18, PC2 support of CA_n28A-n41A-n77A with the following 2UL has been completed in TP R4-2305019 and covered by bigCR R4-2307971.But the changes are not included in the TS. CA_n28A-n77A7 CA_n41A-n77A7 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding PC support for CA_n28A-n41A-n77A with 2UL |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PC2 cannot be used for this combination. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.5A.3	Configurations for inter-band CA

<Unchanged Contents omitted>

START OF CHANGE

##### Table 5.5A.3.2-1b

Table 5.5A.3.2-1b: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n8A-n20A-n28A | CA_n8A-n20A | n8 | 5, 10, 15, 20 | 0 |
|  | CA_n8A-n28A | n20 | 5, 10, 15, 20 |  |
|  | CA_n20A-n28A | n28 | 5, 10, 15, 20, 25, 30 |  |
| CA_n8A-n20A-n75A | - | n8 | n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n20 | n20 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
| CA_n8A-n28A-n40A | CA_n8A-n28A CA_n8A-n40A CA_n28A-n40A | n8 | n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n40 | n40 channel bandwidths in Table 5.3.5-1 |  |
| CA_n8A-n28A-n75A | - | n8 | n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
| CA_n8A-n28A-n77A | CA_n8A-n28A CA_n8A-n77A CA_n28A-n77A | n8 | n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n8A-n28A-n77(2A) | CA_n8A-n28A CA_n8A-n77A CA_n28A-n77A | n8 | n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n8A-n28A-n78A | - | n8 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n8A-n38A-n40A | - | n8 | 5, 10, 15, 20 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n8A-n39A-n40A | CA_n8A-n39A CA_n8A-n40A CA_n39A-n40A | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
| CA_n8A-n39A-n41A | CA_n8A-n39A CA_n8A-n41A CA_n39A-n41A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 100 |  |
|  | - | n8 | 5, 10, 15, 20 | 1 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60 |  |
|  | CA_n8A-n39A CA_n8A-n41A CA_n39A-n41A | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n8A-n39A-n41C | CA_n8A-n39A CA_n8A-n41A CA_n39A-n41A | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
| CA_n8A-n39A-n79A | CA_n8A-n39A CA_n8A-n79A CA_n39A-n79A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n8A-n40A-n41A | CA_n8A-n40A CA_n8A-n41A CA_n40A-n41A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n8A-n40A-n41C | CA_n41C CA_n8A-n40A CA_n8A-n41A CA_n40A-n41A | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | CA_n41C_BCS4 and 5 |  |
| CA_n8A-n40A-n78A | CA_n8A-n40A CA_n8A-n78A CA_n40A-n78A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n40 | 5, 10, 15, 20, 30, 40, 50, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n8A-n40A-n77A | CA_n8A-n40A CA_n8A-n77A CA_n40A-n77A | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | See n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n8A-n40A-n77(2A) | CA_n8A-n40A CA_n8A-n77A CA_n40A-n77A | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n8A-n40A-n79A | CA_n8A-n40A CA_n8A-n79A CA_n40A-n79A | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n8A-n41A-n79A | CA_n8A-n41A CA_n8A-n79A CA_n41A-n79A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n8 | 5, 10, 15, 20 | 1 |
|  |  | n41 | 10, 15, 20, 40, 50, 60 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n8A-n41C-n79A | CA_n41C CA_n8A-n41A CA_n8A-n79A CA_n41A-n79A | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n39 | CA_n41C_BCS 4 and 5 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n8A-n78A-n79A | - | n8 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n8A-n78(2A)-n79A | - | n8 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n12A-n25A-n41A | - | n12 | 5, 10, 15 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n12A-n25A-n66A | - | n12 | 5, 10, 15 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
| CA_n12A-n30A-n66A | CA_n12A-n30A CA_n12A-n66A CA_n30A-n66A | n12 | 5, 10, 15 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n12A-n30A-n66(2A) | CA_n12A-n30A CA_n12A-n66A CA_n30A-n66A | n12 | 5, 10, 15 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n12A-n30A-n66(3A) | CA_n12A-n30A CA_n12A-n66A CA_n30A-n66A | n12 | 5, 10, 15 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n12A-n30A-n77A | n777,9 CA_n12A-n30A, CA_n12A-n77A7 CA_n30A-n77A7 | n12 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n12A-n30A-n77(2A) | n777,9 CA_n12A-n30A CA_n12A-n77A7 CA_n30A-n77A7 | n12 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n12A-n41A-n66A | - | n12 | 5, 10, 15 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
| CA_n12A-n41A-n77A | - | n12 | 5, 10, 15 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n12A-n66A-n77A | n777,9 CA_n12A-n66A CA_n12A-n77A7 CA_n66A-n77A7 | n12 | 5, 10, 15 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n12A-n66(2A)-n77A | n777,9 CA_n12A-n66A CA_n12A-n77A7 CA_n66A-n77A7 | n12 | 5, 10, 15 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n12A-n66A-n77(2A) | n777,9 CA_n12A-n66A CA_n12A-n77A7 CA_n66A-n77A7 | n12 | 5, 10, 15 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n12A-n66(2A)-n77(2A) | n777,9 CA_n12A-n66A CA_n12A-n77A7 CA_n66A-n77A7 | n12 | 5, 10, 15 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n12A-n66(3A)-n77A | n777,9 CA_n12A-n66A CA_n12A-n77A7 CA_n66A-n77A7 | n12 | 5, 10, 15 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n12A-n66(3A)-n77(2A) | n777 CA_n12A-n66A CA_n12A-n77A7 CA_n66A-n77A7 | n12 | 5, 10, 15 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n12A-n71A-n77A | CA_n12A-n77A CA_n71A-n77A | n12 | 5, 10, 15 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n13A-n25A-n66A | CA_n13A-n25A CA_n13A-n66A CA_n25A-n66A | n13 | 5, 10 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n13A-n25A-n77A | n777,9 CA_n13A-n25A CA_n13A-n77A7 CA_n25A-n77A7 | n13 | 5, 10 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n13A-n25A-n77(2A) | n777,9 CA_n77(2A)7 CA_n13A-n25A CA_n13A-n77A7 CA_n25A-n77A7 | n13 | 5, 10 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n13A-n66A-n77A | n777, 9 CA_n13A-n66A CA_n13A-n77A7 CA_n66A-n77A7 | n13 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n13A-n66A-n77(2A) | n777,9 CA_n77(2A)7 CA_n13A-n66A CA_n13A-n77A7 CA_n66A-n77A7 | n13 | 5, 10 | 0 |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n14A-n30A-n66A | CA_n14A-n30A CA_n14A-n66A CA_n30A-n66A | n14 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n14A-n30A-n66(2A) | CA_n14A-n30A CA_n14A-n66A CA_n30A-n66A | n14 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n14A-n30A-n66(3A) | CA_n14A-n30A CA_n14A-n66A CA_n30A-n66A | n14 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n14A-n30A-n77A | n777,9 CA_n14A-n30A CA_n14A-n77A7 CA_n30A-n77A7 | n14 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n14A-n30A-n77(2A) | n777,9 CA_n14A-n30A CA_n14A-n77A7 CA_n30A-n77A7 | n14 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n14A-n66A-n77A | n777,9 CA_n14A-n66A CA_n14A-n77A7 CA_n66A-n77A7 | n14 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n14A-n66(2A)-n77A | n777,9 CA_n14A-n66A CA_n14A-n77A7 CA_n66A-n77A7 | n14 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n14A-n66A-n77(2A) | n777,9 CA_n14A-n66A CA_n14A-n77A7 CA_n66A-n77A7 | n14 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n14A-n66(2A)-n77(2A) | n777,9 CA_n14A-n66A CA_n14A-n77A7 CA_n66A-n77A7 | n14 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n14A-n66(3A)-n77A | n777,9 CA_n14A-n66A CA_n14A-n77A7 CA_n66A-n77A7 | n14 | 5, 10 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n14A-n66(3A)-n77(2A) | n777 CA_n14A-n66A CA_n14A-n77A7 CA_n66A-n77A7 | n14 | 5, 10 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n18A-n28A-n41A | CA_n18A-n28A CA_n18A-n41A CA_n28A-n41A | n18 | 5, 10, 15 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n18A-n28A-n77A | n777 CA_n18A-n28A CA_n18A-n77A7 CA_n28A-n77A7 | n18 | 5, 10, 15 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n18A-n28A-n77(2A) | n777 CA_n18A-n28A CA_n18A-n77A CA_n28A-n77A | n18 | 5, 10, 15 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n18A-n41A-n77A | n417 n777 CA_n18A-n41A7 CA_n18A-n77A7 CA_n41A-n77A7 | n18 | 5, 10, 15 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n18A-n41A-n77(2A) | n777 CA_n18A-n41A7 CA_n18A-n77A7 CA_n41A-n77A7 | n18 | 5, 10, 15 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n20A-n28A-n75A | - | n20 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n75 | 5, 10, 15, 20 |  |
| CA_n20A-n28A-n78A | - | n20 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n20A-n28A-n78C | - | n20 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n20A-n67A-n78A | CA_n20A-n78A | n20 | See n20 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n67 | See n67 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n20A-n67A-n78(2A) | CA_n20A-n78A CA_n78(2A) | n20 | See n20 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n67 | See n67 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n24A-n41A-n48A | CA_n24A-n41A CA_n24A-n48A CA_n41A-n48A | n24 | 5, 10 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
| CA_n24A-n41(2A)-n48A | CA_n24A-n41A CA_n24A-n48A CA_n41A-n48A | n24 | 5, 10 | 0 |
|  |  | n41 | CA_n41(2A) BCS1 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
| CA_n24A-n41A-n48(2A) | CA_n24A-n41A CA_n24A-n48A CA_n41A-n48A | n24 | 5, 10 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n48 | CA_n48(2A) BCS0 |  |
| CA_n24A-n41(2A)-n48(2A) | CA_n24A-n41A CA_n24A-n48A CA_n41A-n48A | n24 | 5, 10 | 0 |
|  |  | n41 | CA_n41(2A) BCS1 |  |
|  |  | n48 | CA_n48(2A) BCS0 |  |
| CA_n24A-n41A-n77A | CA_n24A-n41A CA_n24A-n77A CA_n41A-n77A | n24 | 5, 10 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n24 | See n24 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | See n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n24A-n41(2A)-n77A | CA_n24A-n41A CA_n24A-n77A CA_n41A-n77A | n24 | 5, 10 | 0 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n24 | 5, 10 | 1 |
|  |  | n41 | CA_n41(2A) BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n24 | See n24 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A) BCS4 and 5 |  |
|  |  | n77 | See n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n24A-n41A-n77(2A) | CA_n24A-n41A CA_n24A-n77A CA_n41A-n77A | n24 | 5, 10 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n24 | 5, 10 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A) BCS0 |  |
|  |  | n24 | See n24 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A) BCS4 and 5 |  |
| CA_n24A-n41(2A)-n77(2A) | CA_n24A-n41A CA_n24A-n77A CA_n41A-n77A | n24 | 5, 10 | 0 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n24 | 5, 10 | 1 |
|  |  | n41 | CA_n41(2A) BCS1 |  |
|  |  | n77 | CA_n77(2A) BCS0 |  |
|  |  | n24 | See n24 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A) BCS4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS4 and 5 |  |
| CA_n24A-n48A-n77A | - | n24 | 5, 10 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 5012, 6012, 8012, 9012, 10012 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n24A-n48(2A)-n77A | - | n24 | 5, 10 | 0 |
|  |  | n48 | CA_n48(2A) BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n24A-n48A-n77(2A) | - | n24 | 5, 10 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 5012, 6012, 7012, 8012, 9012, 10012 |  |
|  |  | n77 | CA_n77(2A) BCS0 |  |
| CA_n24A-n48(2A)-n77(2A) | - | n24 | 5, 10 | 0 |
|  |  | n48 | CA_n48(2A) BCS0 |  |
|  |  | n77 | CA_n77(2A) BCS0 |  |
| CA_n25A-n29A-n66A | CA_n25A-n66A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25A-n38A-n66A | CA_n25A-n38A CA_n25A-n66A CA_n38A-n66A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25(2A)-n38A-n66A | CA_n25A-n38A CA_n25A-n66A CA_n38A-n66A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25(2A)-n38A-n66(2A) | CA_n25A-n38A CA_n25A-n66A CA_n38A-n66A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n25A-n38A-n66(2A) | CA_n25A-n38A CA_n25A-n66A CA_n38A-n66A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n38 | 5, 10, 15, 20 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n25A-n38A-n78A | CA_n25A-n38A CA_n25A-n78A CA_n38A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n38A-n78(2A) | CA_n25A-n38A CA_n25A-n78A CA_n38A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25(2A)-n38A-n78A | CA_n25A-n38A CA_n25A-n78A CA_n38A-n78A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25(2A)-n38A-n78(2A) | CA_n25A-n38A CA_n25A-n78A CA_n38A-n78A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25A-n41A-n66A | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41A-n66(2A) | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
| CA_n25A-n41C-n66A | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 CA_n41C7 CA_n25A-n41C CA_n41C-n66A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | CA_n41C_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(2A)-n66A | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(2A)-n66(2A) | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 |  |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
| CA_n25A-n41(3A)-n66(2A) | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(3A) BCS 4 and 5 |  |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
| CA_n25A-n41(3A)-n66A | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(3A) BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41C-n66(2A) | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 CA_n41C7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C BCS 4 and 5 |  |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
| CA_n25A-n41(A-C)-n66A | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 CA_n41C7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(A-C) BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(A-C)-n66(2A) | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A CA_n41C | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(A-C) BCS 4 and 5 |  |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
| CA_n25(2A)-n41A-n66A | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n25 | CA_n25(2A)_BCS1 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 30, 40 |  |
|  |  | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41A-n66(2A) | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
| CA_n25(2A)-n41(2A)-n66A | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41(3A)-n66A | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(3A) BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41(2A)-n66(2A) | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 |  |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
| CA_n25(2A)-n41C-n66A | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 CA_n41C7 | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41C BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41C-n66(2A) | n417,9 CA_n25A-n41A7 CA_n25A-n66A CA_n41A-n66A7 CA_n41C7 | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41C BCS 4 and 5 |  |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
| CA_n25(2A)-n41(A-C)-n66A | CA_n25A-n41A CA_n25A-n66A CA_n41A-n66A CA_n41C | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(A-C) BCS 4 and 5 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41A-n71A | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41A-n71B | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n25 | 5, 10, 15, 20, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
| CA_n25A-n41A-n71(2A) | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n25 | 5, 10, 15, 20, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
| CA_n25A-n41(2A)-n71A | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(2A)-n71B | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 |  |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
| CA_n25A-n41(2A)-n71(2A) | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 |  |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
| CA_n25A-n41(3A)-n71A | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(3A) BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(3A)-n71B | CA_n25A-n41A CA_n25A-n71A CA_n41A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(3A) BCS 4 and 5 |  |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
| CA_n25A-n41(3A)-n71(2A) | CA_n25A-n41A CA_n25A-n71A CA_n41A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(3A) BCS 4 and 5 |  |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
| CA_n25A-n41C-n71A | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 CA_n41C7 CA_n25A-n41C CA_n41C-n71A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | CA_n41C_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41C-n71B | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 CA_n41C7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C BCS 4 and 5 |  |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
| CA_n25A-n41C-n71(2A) | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 CA_n41C7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C BCS 4 and 5 |  |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
| CA_n25A-n41(A-C)-n71A | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 CA_n41C7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(A-C) BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(A-C)-n71B | CA_n25A-n41A CA_n25A-n71A CA_n41A-n71A CA_n41C | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(A-C) BCS 4 and 5 |  |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
| CA_n25A-n41(A-C)-n71(2A) | CA_n25A-n41A CA_n25A-n71A CA_n41A-n71A CA_n41C | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(A-C) BCS 4 and 5 |  |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
| CA_n25(2A)-n41A-n71A | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n25 | CA_n25(2A)_BCS1 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41A-n71B | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n25(2A)-n41A-n71(2A) | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n25(2A)-n41(2A)-n71A | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41(2A)-n71(2A) | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 |  |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
| CA_n25(2A)-n41(2A)-n71B | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 |  |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n25(2A)-n41(3A)-n71A | CA_n25A-n41A CA_n25A-n71A CA_n41A-n71A | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(3A) BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41C-n71A | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 CA_n41C7 | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41C BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41C-n71(2A) | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 CA_n41C7 | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41C BCS 4 and 5 |  |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
| CA_n25(2A)-n41C-n71B | n417,9 CA_n25A-n41A7 CA_n25A-n71A CA_n41A-n71A7 CA_n41C7 | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41C BCS 4 and 5 |  |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n25(2A)-n41(A-C)-n71A | CA_n25A-n41A CA_n25A-n71A CA_n41A-n71A CA_n41C | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(A-C) BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41A-n77A | n417,9 n777,9 CA_n25A-n41A7 CA_n25A-n77A7 CA_n41A-n77A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(2A)-n77A | n417,9 n777,9 CA_n25A-n41A7 CA_n25A-n77A7 CA_n41A-n77A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(3A)-n77A | n417,9 n777.9 CA_n25A-n41A7 CA_n25A-n77A7 CA_n41A-n77A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(3A) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41A-n77(2A) | n417,9 n777,9 CA_n25A-n41A7 CA_n25A-n77A7 CA_n41A-n77A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n25A-n41(2A)-n77(2A) | n417,9 n777.9 CA_n25A-n41A7 CA_n25A-n77A7 CA_n41A-n77A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n25(2A)-n41A-n77A | n417,9 n777.9 CA_n25A-n41A7 CA_n25A-n77A7 CA_n41A-n77A7 | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41A-n77(2A) | n417,9 n777,9 CA_n25A-n41A7 CA_n25A-n77A7 CA_n41A-n77A7 | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n25(2A)-n41C-n77A | n417,9 n777,9 CA_n25A-n41A7 CA_n25A-n77A7 CA_n41A-n77A7 CA_n41C7 | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41C BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41(2A)-n77A | n417,9 n777,9 CA_n25A-n41A7 CA_n25A-n77A7 CA_n41A-n77A7 | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41C-n77A | n417,9 n777,9 CA_n25A-n41A7 CA_n25A-n77A7 CA_n41A-n77A7 CA_n41C7 CA_n25A-n41C CA_n41C-n77A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | CA_n41C_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(A-C)-n77A | n417,9 n777.9 CA_n25A-n41A7 CA_n25A-n77A7 CA_n41A-n77A7 CA_n41C7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(A-C) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41C-n77(2A) | n417,9 n777.9 CA_n25A-n41A7 CA_n25A-n77A7 CA_n41A-n77A7 CA_n41C7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n25(2A)-n41C-n77(2A) | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A CA_n41C | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41C BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n25(2A)-n41(2A)-n77(2A) | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n25(2A)-n41(3A)-n77A | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(3A) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41(A-C)-n77A | CA_n25A-n41A CA_n25A-n77A CA_n41A-n77A CA_n41C | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(A-C) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41A-n78A | CA_n25A-n41A CA_n25A-n78A CA_n41A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n41A-n78(2A) | CA_n25A-n41A CA_n25A-n78A CA_n41A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25A-n41A-n85A | CA_n25A-n41A CA_n25A-n85A CA_n41A-n85A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41C-n85A | CA_n25A-n41A CA_n25A-n85A CA_n41A-n85A CA_n41C | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C BCS 4 and 5 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n41(2A)-n85A | CA_n25A-n41A CA_n25A-n85A CA_n41A-n85A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41A-n85A | CA_n25A-n41A CA_n25A-n85A CA_n41A-n85A | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n48A-n66A | CA_n25A-n48A CA_n25A-n66A CA_n48A-n66A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 5012 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | 5, 10, 15, 20, 40, 5012, 6012, 8012, 9012, 10012 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25A-n48(2A)-n66A | CA_n25A-n48A CA_n25A-n66A CA_n48A-n66A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25A-n48C-n66A | CA_n25A-n48A CA_n25A-n66A CA_n48A-n66A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25A-n66A-n71A | - | n25 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n66A-n71B | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | CA_n71B_BCS2 |  |
|  | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
| CA_n25A-n66A-n71(2A) | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
| CA_n25A-n66(2A)-n71A | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n66(2A)-n71B | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n25A-n66(2A)-n71(2A) | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n25(2A)-n66A-n71A | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n66(2A)-n71A | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n66A-n71B | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n25(2A)-n66(2A)-n71B | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
| CA_n25(2A)-n66A-n71(2A) | CA_n25A-n66A CA_n25A-n71A CA_n66A-n71A | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n25A-n66A-n77A | n777,9 CA_n25A-n66A CA_n25A-n77A7 CA_n66A-n77A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n66(2A)-n77A | n777,9 CA_n25A-n66A CA_n25A-n77A7 CA_n66A-n77A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n66A-n77(2A) | n777,9 CA_n77(2A)7 CA_n25A-n66A CA_n25A-n77A7 CA_n66A-n77A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n25A-n66A-n77(3A) | n777,9 CA_n77(2A)7 CA_n25A-n66A CA_n25A-n77A7 CA_n66A-n77A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(3A) BCS 4 and 5 |  |
| CA_n25A-n66(2A)-n77(2A) | n777,9 CA_n25A-n66A CA_n25A-n77A7 CA_n66A-n77A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n25(2A)-n66A-n77A | n777,9 CA_n25A-n66A CA_n25A-n77A7 CA_n66A-n77A7 | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n66(2A)-n77A | n777,9 CA_n25A-n66A CA_n25A-n77A7 CA_n66A-n77A7 | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n66A-n77(2A) | n777,9 CA_n25A-n66A CA_n25A-n77A7 CA_n66A-n77A7 | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n25(2A)-n66(2A)-n77(2A) | n777,9 CA_n25A-n66A CA_n25A-n77A7 CA_n66A-n77A7 | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n25A-n66A-n78A | n787,9 CA_n25A-n66A CA_n25A-n78A7 CA_n66A-n78A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25(2A)-n66A-n78A | n787,9 CA_n25A-n66A CA_n25A-n78A7 CA_n66A-n78A7 | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n66(2A)-n78A | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n66A-n78(2A) | n787 CA_n78(2A)7 CA_n25A-n66A CA_n25A-n78A7 CA_n66A-n78A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25(2A)-n66(2A)-n78A | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25(2A)-n66A-n78(2A) | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25A-n66(2A)-n78(2A) | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25(2A)-n66(2A)-n78(2A) | CA_n25A-n66A CA_n25A-n78A CA_n66A-n78A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25A-n66A-n85A | CA_n25A-n66A CA_n25A-n85A CA_n66A-n85A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n66(2A)-n85A | CA_n25A-n66A CA_n25A-n85A CA_n66A-n85A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n66A-n85A | CA_n25A-n66A CA_n25A-n85A CA_n66A-n85A | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n71A-n77A | n777,9 CA_n25A-n71A CA_n25A-n77A7 CA_n71A-n77A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n71A-n77(2A) | n777,9 CA_n77(2A)7 CA_n25A-n71A CA_n25A-n77A7 CA_n71A-n77A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A) BCS1 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n25A-n71A-n77(3A) | n777,9 CA_n77(2A)7 CA_n25A-n71A CA_n25A-n77A7 CA_n71A-n77A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(3A) BCS1 |  |
| CA_n25A-n71B-n77A | n777,9 CA_n25A-n71A CA_n25A-n77A7 CA_n71A-n77A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n71B-n77(2A) | n777,9 CA_n25A-n71A CA_n25A-n77A7 CA_n71A-n77A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n25A-n71(2A)-n77A | n777,9 CA_n25A-n71A CA_n25A-n77A7 CA_n71A-n77A7 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n71(2A)-n77(2A) | n777,9 CA_n25A-n71A CA_n25A-n77A7 CA_n71A-n77A7 | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n25(2A)-n71A-n77A | n777,9 CA_n25A-n71A CA_n25A-n77A7 CA_n71A-n77A7 | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n71A-n77(2A) | n777,9 CA_n25A-n71A CA_n25A-n77A7 CA_n71A-n77A7 | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n25(2A)-n71B-n77A | n777,9 CA_n25A-n71A CA_n25A-n77A7 CA_n71A-n77A7 | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n71B-n77(2A) | n777,9 CA_n25A-n71A CA_n25A-n77A7 CA_n71A-n77A7 | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n25(2A)-n71(2A)-n77A | n777,9 CA_n25A-n71A CA_n25A-n77A7 CA_n71A-n77A7 | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n71(2A)-n77(2A) | n777,9 CA_n25A-n71A CA_n25A-n77A7 CA_n71A-n77A7 | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n25A-n71A-n78A | CA_n25A-n71A CA_n25A-n78A CA_n71A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n25A-n71A-n78(2A) | CA_n25A-n71A CA_n25A-n78A CA_n71A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n25A-n71A-n85A | CA_n25A-n71A CA_n25A-n85A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n71B-n85A | CA_n25A-n71A CA_n25A-n85A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n71(2A)-n85A | CA_n25A-n71A CA_n25A-n85A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n71A-n85A | CA_n25A-n71A CA_n25A-n85A | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n77A-n85A | CA_n25A-n77A CA_n25A-n85A CA_n77A-n85A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n77(2A)-n85A | CA_n25A-n77A CA_n25A-n85A CA_n77A-n85A | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n77A-n85A | CA_n25A-n77A CA_n25A-n85A CA_n77A-n85A | n25 | CA_n25(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n26A-n29A-n66A | CA_n26A-n66A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n26A-n29A-n66(2A) | CA_n26A-n66A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n26A-n29A-n66(3A) | - | n26 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n26A-n29A-n70A | CA_n26A-n70A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
|  |  | n70 | 5, 10, 15, 20, 25 |  |
| CA_n26A-n48A-n66A | CA_n26A-n48A CA_n26A-n66A CA_n48A-n66A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 5012, 6012, 8012, 9012, 10012 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n26A-n48(2A)-n66A | CA_n26A-n48A CA_n26A-n66A CA_n48A-n66A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n26A-n48A-n66(2A) | CA_n26A-n48A CA_n26A-n66A CA_n48A-n66A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 5012, 6012, 8012, 9012, 10012 |  |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
| CA_n26A-n48(2A)-n66(2A) | CA_n26A-n48A CA_n26A-n66A CA_n48A-n66A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
| CA_n26A-n48A-n70A | CA_n26A-n48A CA_n26A-n70A CA_n48A-n70A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 5012, 6012, 8012, 9012, 10012 |  |
|  |  | n70 | 5, 10, 15, 20, 25 |  |
| CA_n26A-n66A-n70A | CA_n26A-n66A CA_n26A-n70A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n26A-n66(2A)-n70A | CA_n26A-n66A CA_n26A-n70A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n26A-n66(3A)-n70A | - | n26 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n26A-n66A-n71A | CA_n26A-n66A CA_n66A-n71A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n26A-n66(2A)-n71A | CA_n26A-n66A CA_n66A-n71A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n26A-n66(3A)-n71A | CA_n66A-n71A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n26A-n66A-n77A | CA_n26A-n66A CA_n26A-n77A CA_n66A-n77A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40 |  |
| CA_n26A-n70A-n71A | CA_n26A-n70A CA_n70A-n71A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n70 | 5, 10, 15, 20, 25 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n26A-n70A-n77A | CA_n26A-n70A CA_n26A-n77A CA_n70A-n77A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n70 | 5, 10, 15, 20, 25 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40 |  |
| CA_n28A-n38A-n78A | - | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n39A-n40A | CA_n28A-n39A CA_n28A-n40A CA_n39A-n40A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100 |  |
| CA_n28A-n39A-n41A | CA_n28A-n39A CA_n28A-n41A CA_n39A-n41A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n39 | n39 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n28A-n39A-n41C | CA_n28A-n39A CA_n28A-n41A CA_n39A-n41A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | CA_n41C_BCS1 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n39 | n39 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
| CA_n28A-n39A-n79A | CA_n28A-n39A CA_n28A-n79A CA_n39A-n79A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n28A-n40A-n41A | CA_n28A-n40A CA_n28A-n41A CA_n40A-n41A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n40A-n41C | CA_n28A-n40A CA_n28A-n41A CA_n40A-n41A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n41 | CA_n41C_BCS0 |  |
| CA_n28A-n40A-n78A | CA_n28A-n40A CA_n28A-n78A CA_n40A-n78A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n28A-n40A CA_n28A-n78A CA_n40A-n78A | n28 | 5, 10, 15, 20 | 1 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n28A-n40A-n77A | CA_n28A-n40A CA_n28A-n77A CA_n40A-n77A | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n40A-n77(2A) | CA_n28A-n40A CA_n28A-n77A CA_n40A-n77A | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n28A-n40B-n78A | - | n28 | 5, 10, 15, 20 | 0 |
|  |  | n40 | CA_n40B_BCS0 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | CA_n40B_BCS4 and 5 |  |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n28A-n40A-n79A | CA_n28A-n40A CA_n28A-n79A CA_n40A-n79A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n28A-n41A-n77A | n417,9 n777,9 CA_n28A-n41A7 | n28 | 5, 10, 15, 20, 30 | 0 |
|  | CA_n28A-n77A7 | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n41A-n77A7 | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n41B-n77A | CA_n28A-n41A CA_n28A-n77A CA_n41A-n77A | n28 | 5, 10 | 0 |
|  |  | n41 | CA_n41B_BCS0 |  |
|  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n41A-n77(2A) | n417,9 n777,9 CA_n28A-n41A7 | n28 | 5, 10, 15, 20, 30 | 0 |
|  | CA_n28A-n77A7 | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n41A-n77A7 CA_n77(2A)7 | n77 | CA_n77(2A)_BCS0 |  |
| CA_n28A-n41A-n77(3A) | CA_n28A-n41A CA_n28A-n77A CA_n41A-n77A | n28 | 5, 10 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n28A-n41A-n78A | CA_n28A-n41A CA_n41A-n78A CA_n28A-n78A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 90, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n28A-n41A-n78(2A) | CA_n78(2A) | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n28A-n41A-n79A | CA_n28A-n41A CA_n28A-n79A CA_n41A-n79A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n28A-n41A-n79C | - | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | CA_n79C_BCS 4 and 5 |  |
| CA_n28A-n41C-n79A | CA_n28A-n41A CA_n28A-n79A CA_n41A-n79A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | CA_n41C_BCS1 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n28A-n41C-n79C | - | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | CA_n41C_BCS1 |  |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n79 | CA_n79C_BCS 4 and 5 |  |
| CA_n28A-n46A-n78A | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | 20, 40, 60, 80 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n46C-n78A | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | CA_n46C_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n46D-n78A | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | CA_n46D_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n46(2A)-n78A | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | CA_n46(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n46(2A)-n78(2A) | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A CA_n78(2A) | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | CA_n46(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n28A-n46A-n78(2A) | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A CA_n78(2A) | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | 20, 40, 60, 80 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n28A-n46C-n78(2A) | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A CA_n78(2A) | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | CA_n46C_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n28A-n46D-n78(2A) | CA_n28A-n46A CA_n28A-n78A CA_n46A-n78A CA_n78(2A) | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | CA_n46D_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n28A-n75A-n78A | - | n28 | 5, 10, 15, 20 | 0 |
|  |  | n75 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n28A-n77A-n79A4 | n777,9 n797,9 CA_n28A-n77A7 CA_n28A-n79A7 CA_n77A-n79A7 | n28 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n28A-n77(2A)-n79A4 | n777,9 n797,9 CA_n28A-n77A7 CA_n28A-n79A7 CA_n77A-n79A7 | n28 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n28A-n77(3A)-n79A4 | CA_n28A-n77A CA_n28A-n79A CA_n77A-n79A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n28A-n78A-n79A | n787,9 n797,9 CA_n28A-n78A CA_n28A-n79A CA_n78A-n79A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n28A-n78A-n102A | CA_n28A-n78A CA_n28A-n102A CA_n78A-n102A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n28A-n78A-n102B | CA_n28A-n78A CA_n28A-n102A CA_n28A-n102B CA_n78A-n102A CA_n78A-n102B | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n28A-n78A-n102C | CA_n28A-n78A CA_n28A-n102A CA_n28A-n102C CA_n78A-n102A CA_n78A-n102C | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n28A-n78A-n102D | CA_n28A-n78A CA_n28A-n102A CA_n78A-n102A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n28A-n78A-n102E | CA_n28A-n78A CA_n28A-n102A CA_n78A-n102A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n28A-n78A-n102(2A) | CA_n28A-n78A CA_n28A-n102A CA_n78A-n102A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n28A-n78(2A)-n102A | CA_n28A-n78A CA_n28A-n102A CA_n78A-n102A CA_n78(2A) | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n28A-n78(2A)-n102B | CA_n28A-n78A CA_n28A-n102A CA_n28A-n102B CA_n78A-n102A CA_n78A-n102B CA_n78(2A) | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n28A-n78(2A)-n102C | CA_n28A-n78A CA_n28A-n102A CA_n28A-n102C CA_n78A-n102A CA_n78A-n102C CA_n78(2A) | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n28A-n78(2A)-n102D | CA_n28A-n78A CA_n28A-n102A CA_n78A-n102A CA_n78(2A) | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n28A-n78(2A)-n102E | CA_n28A-n78A CA_n28A-n102A CA_n78A-n102A CA_n78(2A) | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n28A-n78(2A)-n102(2A) | CA_n28A-n78A CA_n28A-n102A CA_n78A-n102A CA_n78(2A) | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n29A-n30A-n66A | CA_n30A-n66A | n29 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n29A-n30A-n66(2A) | CA_n30A-n66A | n29 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n29A-n30A-n77A | n777,9 CA_n30A-n77A7 | n29 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n29A-n30A-n77(2A) | n777,9 CA_n30A-n77A7 | n29 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n29A-n66A-n70A | - | n29 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n29A-n66B-n70A | - | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66B_BCS0 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n29A-n66(2A)-n70A | - | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n29A-n66(3A)-n70A | - | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n29A-n66A-n71A | CA_n66A-n71A | n29 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n29A-n66(2A)-n71A | CA_n66A-n71A | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n29A-n66(3A)-n71A | CA_n66A-n71A | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n29A-n66A-n77A | n777,9 CA_n66A-n77A7 | n29 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n29A-n66(2A)-n77A | n777,9 CA_n66A-n77A7 | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n29A-n66A-n77(2A) | n777,9 CA_n66A-n77A7 | n29 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n29A-n66(3A)-n77A | n777,9 CA_n66A-n77A7 | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n29A-n66(2A)-n77(2A) | n777,9 CA_n66A-n77A7 | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n29A-n66(3A)-n77(2A) | n777 CA_n66A-n77A7 | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n29A-n70A-n71A | CA_n70A-n71A | n29 | 5, 10 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n30A-n66A-n77A | n777,9 CA_n30A-n66A CA_n30A-n77A7 CA_n66A-n77A7 | n30 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n30A-n66(2A)-n77A | n777,9 CA_n30A-n66A CA_n30A-n77A7 CA_n66A-n77A7 | n30 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n30A-n66A-n77(2A) | n777,9 CA_n30A-n66A CA_n30A-n77A7 CA_n66A-n77A7 | n30 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n30A-n66(2A)-n77(2A) | n777,9 CA_n30A-n66A CA_n30A-n77A7 CA_n66A-n77A7 | n30 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n30A-n66(3A)-n77A | n777,9 CA_n30A-n66A CA_n30A-n77A7 CA_n66A-n77A7 | n30 | 5, 10 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n30A-n66(3A)-n77(2A) | n777 CA_n30A-n66A CA_n30A-n77A7 CA_n66A-n77A7 | n30 | 5, 10 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n34A-n39A-n40A | CA_n34A-n39A CA_n34A-n40A CA_n39A-n40A | n34 | 5, 10, 15 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100 |  |
|  |  | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
| CA_n34A-n39A-n41A | CA_n34A-n39A CA_n34A-n41A CA_n39A-n41A | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n34A-n39A-n41C | CA_n34A-n39A CA_n34A-n41A CA_n39A-n41A | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
| CA_n34A-n40A-n41A | CA_n34A-n40A CA_n34A-n41A CA_n40A-n41A | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n34A-n40A-n41C | CA_n34A-n40A CA_n34A-n41A CA_n40A-n41A | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
| CA_n34A-n41A-n79A | CA_n34A-n41A CA_n34A-n79A CA_n41A-n79A | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n34A-n41A-n79C | CA_n34A-n41A CA_n34A-n79A CA_n41A-n79A | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | CA_n79C_BCS 4 and 5 |  |
| CA_n34A-n41C-n79A | CA_n34A-n41A CA_n34A-n79A CA_n41A-n79A | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n34A-n41C-n79C | CA_n34A-n41A CA_n34A-n79A CA_n41A-n79A | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
|  |  | n79 | CA_n79C_BCS 4 and 5 |  |
| CA_n38A-n66A-n78A | CA_n38A-n66A CA_n38A-n78A CA_n66A-n78A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n38A-n66A-n78(2A) | CA_n38A-n66A CA_n38A-n78A CA_n66A-n78A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n38A-n66(2A)-n78A | CA_n38A-n66A CA_n38A-n78A CA_n66A-n78A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n38A-n66(2A)-n78(2A) | CA_n38A-n66A CA_n38A-n78A CA_n66A-n78A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n39A-n40A-n41A | CA_n39A-n40A CA_n39A-n41A CA_n40A-n41A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n39A-n40A-n41C | CA_n39A-n40A CA_n39A-n41A CA_n40A-n41A | n39 | See n39 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
| CA_n39A-n40A-n79A | CA_n39A-n40A CA_n40A-n79A CA_n39A-n79A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n39A-n41A-n79C | CA_n39A-n41A CA_n39A-n79A CA_n41A-n79A | n39 | See n39 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | CA_n79C_BCS 4 and 5 |  |
| CA_n39A-n41A-n79A | CA_n39A-n41A CA_n39A-n79A CA_n41A-n79A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 40, 50, 60 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n40A-n41A-n79A | CA_n40A-n41A CA_n40A-n79A CA_n41A-n79A | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 100 |  |
|  |  | n79 | , 40, 50, 60, 80, 100 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 40, 50, 60 |  |
|  |  | n79 | , 40, 50, 60, 80, 100 |  |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n40A-n41C-n79A | CA_n41C CA_n41A-n79A CA_n40A-n41A CA_n40A-n79A | n40 | See n40 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS4 and 5 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n40A-n78A-n105A | CA_n40A-n78A CA_n40A-n105A CA_n78A-n105A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n41A-n66A-n70A | CA_n41A-n66A CA_n41A-n70A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n41A-n66A-n71A | n417,9 CA_n41A-n71A7 CA_n41A-n66A7 CA_n66A-n71A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n66A-n71B | n417,9 CA_n41A-n66A7 CA_n41A-n71A7 CA_n66A-n71A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
| CA_n41A-n66A-n71(2A) | n417,9 CA_n41A-n66A7 CA_n41A-n71A7 CA_n66A-n71A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
| CA_n41A-n66(2A)-n71A | n417,9 CA_n41A-n66A7 CA_n66A-n71A CA_n41A-n71A7 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n66(2A)-n71B | n417,9 CA_n41A-n66A7 CA_n66A-n71A CA_n41A-n71A7 | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n41A-n66(2A)-n71(2A) | n417,9 CA_n41A-n66A7 CA_n66A-n71A CA_n41A-n71A7 | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n41(2A)-n66A-n71A | n417,9 CA_n41A-n71A7 CA_n41A-n66A7 CA_n66A-n71A | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | CA_n41(2A)_BCS1 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n66A-n71B | n417,9 CA_n41A-n66A7 CA_n41A-n71A7 CA_n66A-n71A | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
| CA_n41(2A)-n66A-n71(2A) | n417,9 CA_n41A-n66A7 CA_n41A-n71A7 CA_n66A-n71A | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
| CA_n41(2A)-n66(2A)-n71A | n417,9 CA_n41A-n71A7 CA_n41A-n66A7 CA_n66A-n71A | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n66(2A)-n71(2A) | n417,9 CA_n41A-n71A7 CA_n41A-n66A7 CA_n66A-n71A | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
| CA_n41(2A)-n66(2A)-n71B | n417,9 CA_n41A-n71A7 CA_n41A-n66A7 CA_n66A-n71A | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
| CA_n41(3A)-n66A-n71A | n417,9 CA_n41A-n71A7 CA_n41A-n66A7 CA_n66A-n71A | n41 | CA_n41(3A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(3A)-n66(2A)-n71A | CA_n41A-n71A CA_n41A-n66A CA_n66A-n71A | n41 | CA_n41(3A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(3A)-n66A-n71B | CA_n41A-n71A CA_n41A-n66A CA_n66A-n71A | n41 | CA_n41(3A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
| CA_n41(3A)-n66A-n71(2A) | CA_n41A-n71A CA_n41A-n66A CA_n66A-n71A | n41 | CA_n41(3A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
| CA_n41C-n66A-n71A | n417,9 CA_n41A-n71A7 CA_n41A-n66A7 CA_n41C7 CA_n66A-n71A | n41 | CA_n41C_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | CA_n41C_BCS1 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n66A-n71B | n417,9 CA_n41A-n66A7 CA_n41A-n71A7 CA_n41C7 CA_n66A-n71A | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
| CA_n41C-n66A-n71(2A) | n417,9 CA_n41A-n66A7 CA_n41A-n71A7 CA_n41C7 CA_n66A-n71A | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
| CA_n41C-n66(2A)-n71A | n417,9 CA_n41A-n71A7 CA_n41A-n66A7 CA_n41C7 CA_n66A-n71A | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n66(2A)-n71(2A) | n417,9 CA_n41A-n71A7 CA_n41A-n66A7 CA_n41C7 CA_n66A-n71A | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
| CA_n41C-n66(2A)-n71B | n417,9 CA_n41A-n71A7 CA_n41A-n66A7 CA_n41C7 CA_n66A-n71A | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
| CA_n41(A-C)-n66A-n71A | n417,9 CA_n41C7 CA_n41A-n71A7 CA_n41A-n66A7 CA_n66A-n71A | n41 | CA_n41(A-C) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(A-C)-n66A-n71B | CA_n41A-n66A CA_n41A-n71A CA_n41C CA_n66A-n71A | n41 | CA_n41(A-C) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
| CA_n41(A-C)-n66A-n71(2A) | CA_n41A-n71A CA_n41A-n66A CA_n66A-n71A CA_n41C | n41 | CA_n41(A-C) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
| CA_n41(A-C)-n66(2A)-n71A | CA_n41A-n71A CA_n41A-n66A CA_n66A-n71A CA_n41C | n41 | CA_n41(A-C) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n66A-n77A | n417,9 n777,9 CA_n41A-n66A7 CA_n41A-n77A7 CA_n66A-n77A7 | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n66A-n77(2A) | n417,9 n777,9 CA_n41A-n77A7 CA_n41A-n66A7 CA_n66A-n77A7 | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n41A-n66(2A)-n77A | n417,9 n777,9 CA_n41A-n66A7 CA_n41A-n77A CA_n66A-n77A7 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n66(2A)-n77(2A) | n417,9 n777,9 CA_n41A-n66A7 CA_n41A-n77A7 CA_n66A-n77A7 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n41(2A)-n66A-n77A | n417,9 n777,9 CA_n41A-n66A7 CA_n41A-n77A7 CA_n66A-n77A7 | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n66A-n77(2A) | n417,9 n777,9 CA_n41A-n66A7 CA_n41A-n77A7 CA_n66A-n77A7 | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n41(2A)-n66(2A)-n77A | n417,9 n777,9 CA_n41A-n66A7 CA_n41A-n77A7 CA_n66A-n77A7 | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n66(2A)-n77(2A) | CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n41(3A)-n66A-n77A | n417,9 n777,9 CA_n41A-n66A7 CA_n41A-n77A7 CA_n66A-n77A7 | n41 | CA_n41(3A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(3A)-n66(2A)-n77A | CA_n41A-n66A CA_n41A-n77A CA_n66A-n77A | n41 | CA_n41(3A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n66A-n77A | n417,9 n777,9 CA_n41A-n66A7 CA_n41A-n77A7 CA_n41C7 CA_n66A-n77A7 CA_n41C-n66A CA_n41C-n77A | n41 | CA_n41C_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n66A-n77(2A) | n417,9 n777,9 CA_n41A-n66A7 CA_n41A-n77A7 CA_n41C7 CA_n66A-n77A7 | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n41C-n66(2A)-n77A | n417,9 n777,9 CA_n41A-n66A7 CA_n41A-n77A7 CA_n41C7 CA_n66A-n77A7 | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n66(2A)-n77(2A) | CA_n41A-n66A CA_n41A-n77A CA_n41C CA_n66A-n77A | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n41(A-C)-n66A-n77A | n417,9 n777,9 CA_n41A-n66A7 CA_n41A-n77A7 CA_n41C7 CA_n66A-n77A7 | n41 | CA_n41(A-C) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(A-C)-n66(2A)-n77A | CA_n41A-n66A CA_n41A-n77A CA_n41C CA_n66A-n77A | n41 | CA_n41(A-C) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n66A-n78A | CA_n41A-n66A CA_n41A-n78A CA_n66A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n66A-n78(2A) | CA_n41A-n66A CA_n41A-n78A CA_n66A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n41A-n66(2A)-n78A | CA_n41A-n66A CA_n41A-n78A CA_n66A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n66(2A)-n78(2A) | CA_n41A-n66A CA_n41A-n78A CA_n66A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n41A-n66A-n85A | CA_n41A-n66A CA_n41A-n85A CA_n66A-n85A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n66(2A)-n85A | CA_n41A-n66A CA_n41A-n85A CA_n66A-n85A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A) BCS 4 and 5 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n66A-n85A | CA_n41A-n66A CA_n41A-n85A CA_n66A-n85A | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n66A-n85A | CA_n41A-n66A CA_n41A-n85A CA_n41C CA_n66A-n85A | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n70A-n78A | CA_n41A-n70A CA_n41A-n78A CA_n70A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n70 | 5, 10, 15, 20, 25 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n71A-n77A | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n71A-n77A7 | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n71B-n77A | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n71A-n77A7 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n71B-n77(2A) | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n71A-n77A7 | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n41A-n71(2A)-n77A | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n71A-n77A7 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n71A-n77(2A) | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n71A-n77A7 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n41(2A)-n71A-n77A | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n71A-n77A7 | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n71B-n77A | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n71A-n77A7 | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n71(2A)-n77A | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n71A-n77A7 | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n71A-n77(2A) | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n71A-n77A7 | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n41(3A)-n71A-n77A | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n71A-n77A7 | n41 | CA_n41(3A) BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n71(2A)-n77(2A) | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n71A-n77A7 | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n41(2A)-n71B-n77(2A) | CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n41(2A)-n71(2A)-n77(2A) | CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n41(3A)-n71B-n77A | CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n41 | CA_n41(3A) BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(3A)-n71(2A)-n77A | CA_n41A-n71A CA_n41A-n77A CA_n71A-n77A | n41 | CA_n41(3A) BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n71A-n77A | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n41C7 CA_n71A-n77A7 CA_n41C-n71A CA_n41C-n77A | n41 | CA_n41C_BCS0 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n71B-n77A | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n41C7 CA_n71A-n77A7 | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n71(2A)-n77A | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n41C7 CA_n71A-n77A7 | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n71A-n77(2A) | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n41C7 CA_n71A-n77A7 | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n41C-n71B-n77(2A) | CA_n41A-n71A CA_n41A-n77A CA_n41C CA_n71A-n77A | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n41C-n71(2A)-n77(2A) | CA_n41A-n71A CA_n41A-n77A CA_n41C CA_n71A-n77A | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n41(A-C)-n71A-n77A | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n41C7 CA_n71A-n77A7 | n41 | CA_n41(A-C) BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(A-C)-n71B-n77A | CA_n41A-n71A CA_n41A-n77A CA_n41C CA_n71A-n77A | n41 | CA_n41(A-C) BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(A-C)-n71(2A)-n77A | CA_n41A-n71A CA_n41A-n77A CA_n41C CA_n71A-n77A | n41 | CA_n41(A-C) BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n71A-n78A | CA_n41A-n71A CA_n41A-n78A CA_n71A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41A-n71A-n78(2A) | CA_n41A-n71A CA_n41A-n78A CA_n71A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n41A-n71A-n85A | CA_n41A-n71A CA_n41A-n85A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n71B-n85A | CA_n41A-n71A CA_n41A-n85A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71B BCS 4 and 5 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n71(2A)-n85A | CA_n41A-n71A CA_n41A-n85A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71(2A) BCS 4 and 5 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n77A-n79A | CA_n41A-n77A CA_n41A-n79A CA_n77A-n79A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n41A-n77(2A)-n79A | CA_n41A-n77A CA_n41A-n79A CA_n77A-n79A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n41A-n77(3A)-n79A | CA_n41A-n77A CA_n41A-n79A CA_n77A-n79A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n41A-n77A-n85A | CA_n41A-n77A CA_n41A-n85A CA_n77A-n85A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n77A-n85A | CA_n41A-n77A CA_n41A-n85A CA_n77A-n85A | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n77(2A)-n85A | CA_n41A-n77A CA_n41A-n85A CA_n77A-n85A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n77A-n85A | CA_n41A-n77A CA_n41A-n85A CA_n41C CA_n77A-n85A | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |

END OF CHANGE