# 3GPP TSG RAN WG4 Meeting #112	R4-2412272

Maastricht, Netherlands, August 19 – 23, 2024

Source: 	Union Inter. Chemin de Fer

Title: 					CA/DC using PC1 in bands n100 and n101.

Agenda Item:	8.4.1

Document for:	Discussion

# Introduction

The current Rel-19 WI (RP-240832 [1]) on NR channel BW less than 5MHz for FR1 Phase 2 addresses the use of bands n100 and n101 in CA/DC with PC3 UEs. For railways - FRMCS - it is essential that PC1 is supported for CA/DC by FRMCS UEs to enable fully benefitting from the 5G NR capabilities, especially to maximize the available throughput in the entire radio cells, including at cell edge.

This contribution provides a motivation to enable the simultaneous use of PC1 in bands n100 and n101 for NR CA/DC.

# Discussion

The objective of the RP-240832 WI [1] is:

4.1	Objective of SI or Core part WI or Testing part WI

•	Define common co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band

•	Example band combination: CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101

•	Define RRM requirements for inter-band CA and DC for combinations introduced in RF part

Note: other band combinations than example band combinations can be specified in basket WIs after the above generic requirements are specified.

The current work on this WI is assuming the use of PC3 UEs in bands n100 and n101.

In the contribution to RAN#103 RP-240108 3 MHz NR CA/DC request for Release 19 [2], the additional need has been identified for CA and DC with PC1 HPUE conducted output power of 31dBm per component carrier.

For the deployment of FRMCS in bands n100 and n101 in European railways, it is essential to use UEs with PC1 31dBm output power to optimize network coverage and available data throughput. The use of PC1 will significantly reduce the required number of basestations compared to using PC3, hence resulting in significant cost savings. It is anticipated that most, if not all, FRMCS radio networks will be designed assuming PC1 UE. To optimize the available data throughput, also CA/DC using PC1 at 31dBm per component carrier is necessary.

Observation 1: For railways it is essential to use PC1 UEs for CA/DC to enable high throughput while limiting the number of basestations and, as a result, reducing the network implementation cost.

In ECC DEC(20)02 [3] and EC Decision 2021/1730 [4] it has been defined that cabradios (i.e. vehicular PC1 UEs) can  use a maximum output power of 31dBm in band n100 and in band n101. This has been reflected in TS38.101-1 [5] Table 6.2.1-1: UE Power Class.

Observation 2: PC1 support in band n100 and band n101 is supported by ECC and EC regulation as well as 3GPP technical specification.

In RAN4#111 it has been discussed if the simultaneous transmission in bands n100 and n101 should be considered as “multiple carrier”. RAN4 has concluded that the use of one carrier in band n100 and one carrier in band n101 is not considered to be a multiple carriers case, as reflected in R4-2410001 [6]. Hence the ECC and EC regulation does not require additional coordination procedure or other mitigation measures. This view has subsequently been supported by RAN#104 in RP-240889 [7].

Observation 3: Simultaneous transmission in bands n100 and n101 is not considered to be multiple carrier.

As part of the current RAN4 work on the CA/DC WI [1], several contributions have been submitted to RAN4#111 that all conclude that there is no need to specify MSD requirements due to harmonic and IMD products for CA_n100-n101 and NR DC_n100-n101, refer to e.g. the WF in R4-2410602 [8], and R4-2407548 [9], R4-2408797 [10].

Observation 4: There are no harmonics nor IMD issues foreseen between using n100 and n101 in CA/DC.

PC1 in bands n100 and n101 is only allowed for vehicle-mounted FRMCS cabradios. For FRMCS cabradios no physical implementation limitations are foreseen. Different from regular handset UE devices, cabradios have no specific limitations on size, weight or power consumption. This allows using individual PAs per frequency band, with the transmit signal available via 1 or 2 antenna connector(s). Also, due to the HPUE antenna being on the rooftop of a train at approximately 4m above ground, no limitations related to SAR apply.

Observation 4: There are no physical implementation limitations for FRMCS cabradios in bands n100 and n101.

In TS38.101-1 [5] section 6.2A.1.3 it is stated that “For inter-band uplink carrier aggregation with uplink assigned to two NR bands, UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is defined as the sum of maximum output power from each UE antenna connector”. As, unlike handheld smartphone UE devices, FRMCS cabradios have no physical implementation limitations, UIC is of the view that this maximum output power definition should not be applicable to cabradio HPUE in bands n100 and n101.

Observation 5: TS38.101-1 [5] section 6.2A.1.3 should not be applicable to FRMCS cabradio HPUE in bands n100 and n101.

Based on the discussion above, the following proposal has been formulated:

Proposal 1: The European spectrum regulation allows simultaneous transmission of 31dBm in bands n100 and n101. There are no technical nor implementation related reasons that should block simultaneous transmission with 31dBm per component carrier in 5G NR CA/DC. 
RAN4 should discuss how to include in the 3GPP specifications inter-band CA/DC in bands n100 and n101 using 31dBm component carriers for FRMCS cabradios.

# References

[1]	RP-240832 NR channel BW less than 5MHz for FR1 Phase 2

[2]	RP-240108 3 MHz NR CA/DC request for Release 19

[3] 	ECC DEC(20)02 Harmonised use of the paired frequency bands 874.4-880.0 MHz and 919.4-925.0 MHz and of the unpaired frequency band 1900-1910 MHz for Railway Mobile Radio (RMR)

[4] 	EC COMMISSION IMPLEMENTING DECISION (EU) 2021/1730 2021/1730

[5]	TS38.101-1

[6]	R4-2410001 LS on multiple carrier operation for n100/n101 bands

[7]	RP-240889 LS on multiple carrier operation for n100/n101 bands

[8]	R4-2410602 WF on UE RF requirements for Rel-19 NR channel BW less than 5MHz for FR1 Phase 2

[9]	R4-2407548 Further discussion on UE RF requirements for inter-band NR CA/DC with 3MHz CBW

[10]	R4-2408797 Discussion on UE RF requirements for inter-band NR CA/DC with 3MHz CBW