3GPP TSG-RAN WG4 Meeting#112		R4-2412274

Maastricht, the Netherlands, Aug 19th -23th, 2024

Agenda item:	8.6.2

Source: 	Spreadtrum Communications

Title: 	Discussion on methods for reducing the number of UE RX chains

Document for:	Discussion

Introduction

In RAN#104-P meeting, a new SID [1] on study on NR FR1 DL Fragmented Carriers focusing on the use of fewer (shared) RX chains than the number of carrier fragments in a band. In this contribution, we want to provide some initial views on methods for reducing the number of UE RX chains.

The objective of this study is as follows: - Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering: - Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4]; - Other requirements are not precluded, if identified - The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2]; - Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4]; - Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4] - Impacts on DL performance [RAN4] ; - Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2]. NOTE 1: No RAN1 impact is foreseen NOTE 2: RAN2 work, if necessary, will be triggered by RAN4 LS NOTE 3: This study starts from single DL band. Sharing RF chain is not considered among inter-band DL carriers. - Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

Discussion

UE architectures assumption

According to the SID, the one of main objective is identifying methods for reducing the number of UE RX chains, such as from separate RF chains per CC to shared RF chains. In our understanding, common understanding for UE architectures assumption is necessary. A unified architecture is helpful to the definition of modified requirements because existing requirements about non-contiguous are based on separate RF chain assumptions.

Shared RF chain+Separate Analog BB chain

Figure 1: shared LNA and separate LO

Figure 2: shared LNA, shared LO and separate LPF and ADC

Shared RF chain+Shared Analog BB chain

Figure 3: shared LNA, shared LO, shared LPF and shared ADC

As we all known that, different architectures have different performance and different complexity. As shown in Figure1, shared LNA means that we could not adjust RF front-end gain for each cc and will cause other intermodulation spurious and unwanted emissions. As shown in Figure 2, shared LNA and shared LO. Shared LO is a challenge for channel design and wider channel bandwidth. As shown in Figure 3, shared RF chain and shared LPF and ADC. Shared LPF cannot suppress blocker and interference. Different received power spectral density imbalance between the two non-contiguous CCs must be limited within a certain range, other it will exceed the dynamic range of ADC and lead to the deterioration of SNR when shared ADC. Therefore, a common understanding for UE architectures assumption is essential. In our understanding, shared LNA or shared LNA and shared LO architecture assumption could as a starting point for UE architectures assumption.

Proposal 1:  UE architectures assumption needs to be further studied.

Proposal 2: Shared LNA or shared LNA and shared LO architecture assumption could be a starting point for UE architectures assumption.

Methods for reducing the number of UE Rx chains

Inter-operator co-ordination

According to the SID, the scenario was limited as inter-operator co-located. If operators can coordinate with each other, there will be no network deployment within the gap between non-contiguous 2cc. Thus, no additional interference and blocker will fall into this gap. Shared RF chains could fulfil existing requirements. However, if inter-operator co-ordination cannot be achieved, the relaxation of existing requirements is needed.

Observation 1: If inter-operator co-ordination is practical, shared RF chain for non-contiguous 2cc is feasible.

Proposal 3: Whether inter-operator co-ordination is feasible should be further studied.

Measurement

We can measure the gap between non-contiguous 2cc, measure the noise and interference of the gap and distribution of outside interference. When we measure the interference power and frequency domain position, UE informs that network to select proper PRBs place when gNB configures non-contiguous 2cc.

Proposal 4: Noise and interference measurement for gap to select proper PRBs could be considered.

Semi-statically switch hardware resources

The new SID mentioned that the ability to semi-statically switch hardware resources means that a single RX chain support non-contiguous 2cc isn’t a static and inherent UE ability. It can be supported at time A, but it may not be supported at time B. How to confirm time A and time B? Similarly, when gNB needs to configure the UE with multiple CCs, trigger the UE to measure the gap, if the RSRP/RSSI for the gap fulfil the requirements then UE could inform gNB that this time UE could support 2cc with a single RX chain UE switch from 2RX chains to 1 RX chains so that gNB could schedule more CCs.

Proposal 5: gNB trigger UE to measure gap could be considered.

Conclusions

In this paper, we provide our views on RX requirements of REFSENS, we have made the following proposals.

Proposal 1:  UE architectures assumption is needed to be studied further.

Proposal 2: Shared LNA or shared LNA and shared LO architecture assumption could be a starting point for UE architectures assumption.

Observation 1: If inter-operator co-ordination is practical, shared RF chain for non-contiguous 2cc is feasible.

Proposal 3: Whether inter-operator co-ordination is feasible should be further studied.

Proposal 4: Noise and interference measurement of gap to select proper PRBs could be considered.

Proposal 5: gNB trigger UE to measure gap could be considered.

- Reference

- RP-241669, New SID: Study on NR FR1 DL Fragmented Carries, Media Tek Inc. TSG-RAN Plenary 104 meeting