3GPP TSG-RAN4 Meeting #112	R4-2412275

# Maastricht, NL, 19th Aug 2024 – 23rd Aug 2024

Source:	LG Electronics

Title:	Discussion on IBB and OBB requirements for LP-WUS

Agenda item:	8.22.2.3

Document for:	Discussion

Introduction

In the RAN4#111 meeting,  the WF for LU-WUS_UERF was approved. In this discussion, we share our point of view for LP-WUS issues based on the approved WF[1]

Discussion

Table 1 Agreed WF on IBB and OBB requirements of LP-WUS in the RAN4#111 meeting.

Issue 3-1-1: IBB and OBB assumption Agreement: - RAN4 further check and discuss whether same interference level of IBB and OBB as MR is assumed for LP-WUR. - With assumption that Rx dynamic range for LR may not be as good as MR Issue 3-1-2: IBB and OBB requirements Agreement: - RAN4 further discuss detailed test condition for IBB and OBB, e.g., location of LP-WUS in-band, MR channel bandwidth, whether guard RBs are allocated, power level. - FFS IBB and OBB requirements

The LP-WUR architecture is different with main receiver, and LP-WUR architecture has a relatively high NF with low power consumption, which means that the RF requirements of the LP-WUR may be different from the legacy main receiver. However, the communication environment in which the LP-WUR operates is not different from the environment in which the main receiver operates, which means that the interference level experienced by the LP-WUR is the same as the legacy main receiver. Therefore, the interferer power in IBB and OBB requirements should be the same as in legacy MR. Currently, the REFSENS requirement of LP-WUR is being discussed and the REFSENS of LP-WUR may be specified differently from MR. Therefore, REFSENS in IBB and OOB requirements can be replaced by LP-WUR REFSENS (LP-WUR REFSENS = REFSENS + ΔRLP-WUS relaxation)

Proposal: For LP-WUR IBB and OBB requirements, the interferer power can be the same as MR and the REFSENS can be replaced by LP-WUR REFSENS (LP-WUR REFSENS = REFSENS + ΔRLP-WUS relaxation)

Conclusion

Proposal: For LP-WUR IBB and OBB requirements, the interferer power can be the same as legacy MR and the REFSENS can be replaced by LP-WUR REFSENS (LP-WUR REFSENS = REFSENS + ΔRLP-WUS relaxation)

- References

# [1] R4-2410569, WF for [111][136] NR_LPWUS_UERF (VIVO)