3GPP TSG-RAN4 Meeting #112	R4-2412276

# Maastricht, NL, 19th Aug 2024 – 23rd Aug 2024

Source:	LG Electronics

Title:	Discussion on REFSENS and ASCS for the LP-WUS/WUR

Agenda item:	8.22.2.2

Document for:	Discussion

Introduction

In the RAN4#111 meeting,  the WF for LU-WUS_UERF was approved. In this discussion, we share our point of view for the LP-WUS/WUR based on the approved WF[1]

Discussion

REFSENS requirements

Table 1 Agreements for LP-WUR REFSENS requirements in the RAN4#111 meeting.

- Sub-topic 2-2 REFSENS requirements Issue 2-2-3: SNR value (not requirement) for REFSENS Agreement: - Stick to last meeting agreements: Wait for RAN1 SNR progress Issue 2-2-4: NF Gap between LR and MR for REFSENS (assume MR as 9dB) Agreement: - RAN4 further discuss potential NF for OOK-based and OFDM-based receiver. - Linked to resolution of Issue 2-2-8 Issue 2-2-6: IM value for REFSENS Agreement: - RAN4 further discuss potential IM value for OOK-based and OFDM-based receiver. Issue 2-2-8: Whether a baseline architecture is needed for LP-WUS receiver Agreement: - RAN4 will further discuss and decide baseline architecture(s) for OOK-based receiver and OFMD-based receiver.

The equation format of the REFSENS requirement on legacy LTE/NR can be reused to specify the REFSENS requirement of LP-WUS/WUR.

- REFSENS = -174 dBm + 10 *log(BW) + NF +SNR+IM

It is necessary to study how to decide the values of NF, SNR and IM for the LP-WUS/WUR REFSEMS.

As agreed in the last meeting, the SNR value can be decided by RAN1 and the NF and IM values can be determined by RAN4.

Observation 1: The equation format of the REFSENS requirement on legacy LTE/NR can be reused to specify the REFSENS requirement of LP-WUS/WUR.

Proposal 1: Use the same REFSENS equation for LP-WUS/WUR as for the legacy LTE/NR MR.

NF and IM can be determined according to the architecture of the LP-WUR, and architectures are described in detail in TR 38.869 clause 7.1. The power consumption and noise figure of each architecture can be significantly different and are summarized below. [2]

RF envelope detection

power consumption:0.01~0.2

Noise figure(dB):12~22

Heterodyne architecture with IF envelope detection

power consumption: 0.1~4

Noise figure(dB): 9~15

Homodyne/zero-IF architecture with baseband envelope detection

power consumption: 0.09~4

Noise figure(dB): 10~16

The RF envelope detection architecture can be expected to have very low power consumption but a relatively high noise figure, while the heterodyne/homodyne architecture is characterized by a low noise figure at the cost of a relatively high power consumption. The characteristics of each structure can be applied differently to different LP-WUS/WUR use cases. Therefore, it is necessary to consider the different REFSENS depending on the architecture of the LP-WUR.

Observation 2: The power consumption and noise figure can be significantly different according to the LP-WUR architecture. The characteristics of each structure can be applied differently to different LP-WUS/WUR use cases. Therefore, it is necessary to consider more than one set of REFSENS requirement depending on the architecture of the LP-WUR.

Proposal 2: It is necessary to consider more than one set of REFSENS requirement depending on the architecture of the LP-WUR.

In general, LP-WUR crchitecture has a simpler structure than legacy LTE/NR MR and there is no reason to use higher IM than legacy LTE/NR MR. Therefore, the IM of the legacy LTE/NR can be reused for the LP-WUS/WUR REFSENS requirement.

Proposal 3: Consider the same IM value for LP-WUS/WUR as for the legacy LTE/NR MR.

ASCS requirements

Table 2 Agreed WF for ASCS in the RAN4#111 meeting

Issue 2-1-10: The overall simulation parameters for LLS simulation Agreement: - Wait for RAN1 conclusion on FR2 to decide the parameters for evaluations - Adopt the following parameters in table below for FR1 simulation evaluations Attributes; Assumptions. Case name (waveform); OOK-1 waveform; OOK-4 waveform. Center frequency; 900MHz, 2.6GHz and 3.5GHz. Channel structure; Total 8/16 bits. Chip rate; M=1; M=1/2/4. WUS duration; FFS. Coding; 1/2 rate Manchester coding. Time error; 0. Residual Frequency error; 0/10/20 ppm. SCS; 30kHz. UE Channel BW; 20MHz (51 RB)-case 1 10MHz (24 RB)-case 2 5MHz (11 RB)-case 3. WUS RB; - Fixed 11RB ~ 3.96MHz for 10MHz and 20MHz cases. Position within channel; - For 10/20MHz CBW, Center for ASCS, edge for ACS [assume no ASCS impact] - For 5MHz CBW, fixed center of channel. Guardband of NR channel, both wanted cell and interfer cell (ACS); - For wanted signal: 505kHz for 5MHz, 665kHz for 10MH, 805kHz for 20Mhz - For interference cell2 5MHz: fixed 505kHz. Guard RB; - For ASCS: 0 or 1RB on each side of LP-WUS bandwidth - For ACS: 1/2/3/4 RB. Filter; - 3th/5th Order lowpass Butterworth matching fixed 3.96MHz RF bandwidth for 10MHz/20MHz case - Other order lowpass filter is not precluded - The filter bandwidth is adapted with actual WUS RBs, for 5MHz case. ASCS; PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same PSD with WUS signal. ACS; PDSCH mapped on interference RBs (11RB for 5MHz CBW), one side; EPRE of PDSCH /EPRE of in-band LP-WUS = [20~33] dB NOTE: decide the interference level depending on SNR. Wanted signal level; For ACS, REFSENS + 14 dB for LP-WUS. Sampling rate; 7.68MHz. ADC bit width; 4/8 bits ADC for ASCS/ACS Encourage companies to provide simulation results with both options for comparison. Phase noise; Not modelled. Non-linearities; Not modelled. Power boosting; EPRE ratio: 0dB/3dB for OOK-1/OOK-4 NOTE: 3dB is optional for simulation. Channel Model; Option 1: TDL-C 300 Option 2: AWGN Note: encourage companies to provide simulation results with both options. Performance metric; - 1% MDR/BLER as baseline and 5% MDR/BLER as optional - The following false alarm rate can be considered - 1% - 5% Providing the information whether the false alarm rate is considered or not. Issue 2-3-1: Align the definition of ASCS requirements Agreement: - RAN4 can use the following ASCS definition as starting point. - Adjacent SubCarrier Selectivity (ASCS) is a measure of a receiver's ability to receive an LP-WUS signal at its assigned channel frequency in the presence of adjacent subcarrier NR signal(s) at a given frequency offset (guard RB) between LP-WUS and NR. ASCS is the ratio of the receive filter attenuation on the assigned LP-WUR channel frequency to the receive filter attenuation on the adjacent NR subcarrier Issue 2-3-2: ASCS requirements value Agreement: - RAN4 further discuss whether ASCS requirements value should be specified, or just the test parameter are specified, i.e., LP_WUS and NR signal bandwidths and power levels, and guard RBs. Issue 2-3-3: Required number of guard RB for ASCS Agreement: - RAN4 further discuss required number of guard RB for ASCS based on LLS simulation outcome next meeting. Issue 2-3-4: Test case for ASCS Agreement: - RAN4 further discuss detailed test parameters for ASCS.

Figure 1 MDR LLS simulation results on OOK-1, AWGN

Figure 2 MDR LLS simulation results on OOK-1, TDLC-300

Figure 3 MDR LLS simulation results on OOK-4, AWGN

Figure 4 MDR LLS simulation results on OOK-4, TDLC-300

Observation 3: Assigning an ASCS guard band of 1 RB resulted in an SNR improvement of less than 0.5 dB over unassigned.

Proposal 4: Do not specify the guard RB for ASCS.

Conclusion

Observation 1: The equation format of the REFSENS requirement on legacy LTE/NR can be reused to specify the REFSENS requirement of LP-WUS/WUR.

Observation 2: The power consumption and noise figure can be significantly different according to the LP-WUR architecture. The characteristics of each structure can be applied differently to different LP-WUS/WUR use cases. Therefore, it is necessary to consider more than one set of REFSENS requirement depending on the architecture of the LP-WUR.

Observation 3: Assigning an ASCS guard band of 1 Rb resulted in an SNR improvement of less than 0.5 dB over unassigned.

Proposal 1: Use the same REFSENS equation for LP-WUS/WUR as for the legacy LTE/NR MR.

Proposal 2: It is necessary to consider more than one set of REFSENS requirement depending on the architecture of the LP-WUR.

Proposal 3: Consider the same IM value for LP-WUS/WUR as for the legacy LTE/NR MR.

Proposal 4: Do not specify the guard RB for ASCS.

- References

# [1] R4-2410569, WF for [111][136] NR_LPWUS_UERF (VIVO)

# [2] TR 38.869