3GPP TSG-RAN WG4 Meeting#112							         			R4-2412277

Maastricht, Netherland, 19 – 23 August 2024

Title: 	On PC1.5 intra-band non-contiguous ULCA with dualPA

Source: 	Skyworks Solutions, Inc.

Agenda Item:	8.1.1.1.1	Intra-band contiguous and non-contiguous UL CA with PC1.5	[NR_ENDC_RF_Ph4-Core]

Document for:	Approval

# 1	Background

In RAN4#111, a way forward [2] captured agreements related to the Release 19 power domain enhancement work item that has been further detailed in RAN#104 in [3]. Notably, it covered the PC1.5 intra-band ULCA feature for both contiguous and non-contiguous cases. In this contribution, further adavancing from our contribution [1] in RAN4#111, we specifically address, the MPR requirement for PC1.5 non-contiguous ULCA based on DualPA architecture.

# Discussion

## 2.1 Way forward and plenary inputs

As described below, we extracted the agreements and way forwards relative to PC1.5 non-contiguous ULCA as captured in [2].

Agreement on Architecture:

- For intra-band NC ULCA w/o UL MIMO, the Architecture#1 is prioritized for MPR/A-MPR evaluation.

  - FFS on Architecture #2

| Architecture | Description | Indicated capability | Applicable cases |
| --- | --- | --- | --- |
| #1 | 2x26 dBm PA + 2 LO with 100MHz BW | dualPA-Architecture | NC CA w/o UL MIMO |
| #2 | 2x26 dBm PA + 1 LO with 200MHz BW | TxD | NC CA w/o UL MIMO |

Observation: Note that some of the captured architecture attributes are either incomplete or incorrect:

- For the non-contiguous case, the Architecture#1 (dualPA-Architecture) cannot support UL MIMO, but Architecture#2 can.

- For Architecture#2, the issue is instead the limitation in the gap size and total BW size.

- The power limitation per carrier to 26dBm of the dualPA-Architecture is not captured, this will impact the SNR at the BS of one of the carriers in case of an asymmetric active RB allocation between the CCs.

- The conditions for one CC PC1.5 fallback are not captured.

Proposal on the architecture description for the PC1.5 non-contiguous intra-band ULCA: The following table applies, and priority is to develop the MPR/A-MPR requirement for Architecture #1. In our view, Architecture#2 has too much limitations, in terms of the channel configuration supported, and especially cannot support n77(2A) for most operators and regions.

| Architecture | Implementation | Indicated capability | UL MIMO support | Power limitation | Support of 1CC PC1.5 fallback | Separation BW limitations |
| --- | --- | --- | --- | --- | --- | --- |
| #1 | 2x26 dBm PA + 2 LO with 100MHz BW | dualPA-Architecture | No | Each carrier is limited to 26dBm | Requires LO switching and thus interruption | No limitations other than the currently defined largest separation BW of 600MHz |
| #2 | 2x26 dBm PA + 1 LO with 200MHz BW | TxD and/or UL MIMO | Yes | One carrier can reach 29dBm | Can support without any switching | Max separation BW of 200MHz, Gap size <aggregated BW |

Other aspects were also captured in [2]:

Way forward on antenna isolation:  Yes, and assuming 20dB minimum antenna isolation for FWA and 10dB for handheld UE

Way forward on additional power reduction:

- A-MPR requirements are not needed for PC1.5 n77/n78/n79 intra-band ULCA

- Evaluate A-MPR requirements for PC1.5 n41 intra-band UL CA

Way forward on PCmax: FFS in future meetings

Proposals:

  - Proposal 1: Define PCMAX,c limitation for each component carrier considering PC1.5 UE architecture of intra-band UL CA. (LGE)

- For dualPA-architecture

  - PCMAX,C limitation  for each component carrier is 26dBm

- For TxD (dualTx)

  - PCMAX,CC1 = 26+10*log10( LCRB1*SCS1LCRB1*SCS1+LCRB2*SCS2)+3

  - PCMAX,CC2 = 26+10*log10( LCRB2*SCS2LCRB1*SCS1+LCRB2*SCS2)+3

  - Proposal 2: Changes to TS 38.101-1 for dual-PA architecture, inset new equation for PCMAX,c (Ericsson, more details refer to R4-2407721)

In this contribution, we address the initial backoff assessment based on a dualPA architecture, and applying the usual equal PSD assumption. The aspects related to PCmax, equal PWR/PSD assumption are covered in [4].

## 2.2 Existing non-contiguous intra-band ULCA MPR

As an initial assessment for PC1.5 MPR, we first analyze the currently specified MPR for PC2 and PC3 for the two IMD3 limitations at -13dBm/MHz and -30dBm/MHz as collected in Table 1.

Especially, the architectures for DualPA (2LO) and single PA for both CCs (1LO) are relevant for PC1.5 as:

- The PC2 DualPA architecture is already implemented with at least one 26dBm PA: Same power capability per CC than PC1.5 DualPA case.

- The PC2 single PA for both CCs also uses one 26dBm PA and a PC1.5 2Tx architecture would be based on two of those PAs.

- The difference between PC3 and PC2 may provide valuable insight on the PC2 to PC1.5 MPR increase.

The following PA implementations were used to derive the non-contiguous ULCA MPR:

- One 23dBm PA for PC3 MPR for UE not indicating dualPA

- Two 23dBm PA for PC3 MPR for UE indicating dualPA (it may be sufficient to use only one 23dBm PA. However,  in fact there are no 20dBm PAs for NR bands, except for NR-U)

- One 26dBm PA for PC2 MPR for UE not indicating dualPA

- Two 26dBm PAs for PC3 MPR for UE indicating dualPA, but not requiring LO/CC switching

- One 26dBm PA + one 23dBm PA for UE indicating dualPA, but requiring LO/CC switching.

Table 1: PC3 and PC2 MPR for non-contiguous ULCA for 1LO or 2LO architectures

| SEM limit | -30dBm/MHz |  |  |  | -13dBm/MHz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Architecture | DualPA (2LO) |  | Single PA for both CCs (1LO) |  | DualPA (2LO) |  | Single PA for both CCs (1LO) |  |  |
| Parameter | MA | RB BW | MA | RB BW | MA | RB BW | MA | RB BW |
| PC3 MPR | 15 | 0 ≤ B < 1.08 | 17.5 | 0 ≤ B < 1.08 | 9 | 0 ≤ B < 0.54 | 11 | 0 ≤ B < 1.08 |
|  | 14.5 | 1.08 ≤ B < 2.16 | 17 | 1.08 ≤ B < 2.16 | 8 | 0.54 ≤ B < 1.08 | 10.5 | 1.08 ≤ B < 2.16 |
|  | 13.5 | 2.16 ≤ B < 3.24 | 16.5 | 2.16 ≤ B < 3.24 | 7 | 1.08 ≤ B < 2.16 | 10 | 2.16 ≤ B < 3.24 |
|  | 12.5 | 3.24 ≤ B < 5.04 | 16 | 3.24 ≤ B < 5.04 | 6.5 | 2.16 ≤ B < 3.24 | 9.5 | 3.24≤ B < 5.04 |
|  | 11.5 | 5.04≤ B < 10.08 | 15 | 5.04≤ B < 10.08 | 5.5 | 3.24 ≤ B < 5.4 | 8.5 | 5.04 ≤ B < 10.08 |
|  | 10.5 | 10.08 ≤ B < 16.38 | 14.5 | 10.08 ≤ B < 36 | 4 | 5.4 ≤ B | 7.5 | 10.08 ≤ B < 36 |
|  | 10 | 16.38 ≤ B < 21.78 | 10 | 36 ≤ B < 56.88 |  |  | 7 | 36 ≤ B |
|  | 9 | 21.78 ≤ B | 9 | 56.88 ≤ B |  |  |  |  |
| PC2 MPR | 15.5 | 0 ≤ B < 1.44 | 19.5 | 0 ≤ B < 1.08 | 9 | 0 ≤ B < 0.54 | 14 | 0 ≤ B < 1.08 |
|  | 15 | 1.44 ≤ B < 2.88 | 19 | 1.08 ≤ B < 2.16 | 8 | 0.54 ≤ B < 1.08 | 12 | 1.08 ≤ B < 2.16 |
|  | 14 | 2.88 ≤ B < 5.76 | 18 | 2.16 ≤ B < 5.04 | 7 | 1.08 ≤ B < 2.16 | 11.5 | 2.16 ≤ B < 3.24 |
|  | 12 | 5.76 ≤ B < 10.8 | 16.5 | 5.04≤ B < 10.08 | 6.5 | 2.16 ≤ B < 3.24 | 11 | 3.24≤ B < 5.04 |
|  | 10.5 | 10.8 ≤ B < 23.04 | 16 | 10.08 ≤ B < 36 | 6 | 3.24 ≤ B < 5.4 | 9.5 | 5.04 ≤ B < 10.08 |
|  | 9 | 23.04 ≤ B | 12 | 36 ≤ B < 56.88 | 5.5 | 5.4 ≤ B ≤ 10.8 | 8.5 | 10.08 ≤ B < 36 |
|  |  |  | 10.5 | 56.88 ≤ B | 4 | 10.8 < B | 6.5 | 36 ≤ B |

Observations:

- From PC3 to PC2, and for the worst-case allocations, MPR exhibits the following increases:

  - For dualPA (2LO): 0.5dB for -30dBm/MHz range, 0dB for -13dBm/MHz range

  - For single PA for both CCs (2LO): 2dB for -30dBm/MHz range, 3dB for -13dBm/MHz range

  - This means that the PC2 dualPA approach is already optimized, as compared to PC3.

- Beyond its intrinsic bandwidth limitations, the 1LO (single PA for both CCs) has 2.5dB to 5dB worse MPR as compared to the dualPA approach. This would only be worse for PC1.5 and 2Tx.

- Note that there is no MPR defined for non-contiguous ULCA + UL MIMO or TxD

- The dualPA PC2 MPR being that already implement a PA with 26dBm power capability per CC, similar to PC1.5 dualPA case:

  - For the SEM limited cases, one can expect the PC1.5 MPR level to be just 3dB higher, as there is only a change in the PCmax reference.

  - However, the PC2 DualPA MPR being implementable with one 26dBm PA and one 23dBm PA, the reverse IMD is worse than the case where two 26dBm PAs are used as in PC1.5.

  - Additionally, the MPR for higher RB BWs is often limited by ACLR, and since this is a relative requirement, the MPR may not need to be increased

- Overall, a new evaluation is needed for PC1.5, but for small RB allocation based on PC2 Dual PA specification, the PC1.5 dual PA MPR could be as high as:

  - 18.5dB for IMD3 falling beyond the ACLR1 range (-30dBm/MHz limit)

  - 12dB for IMD3 falling within the ACLR1 range (-13dBm/MHz).

## 2.3 Evaluation assumption for PC1.5 intra-band ULCA MPR with dualPA

To evaluate the PC1.5 intra-band non-contiguous ULCA MPR for dualPA architecture, we suggest simulating or measuring the back-off difference of two coupled 26dBm PAs for PC2 and PC1.5.

Proposal on MPR evaluation parameters:

- Post PA loss: 4dB, antenna isolation: 10dB for smartphone and 20dB for FWA

- PA calibration for DFT-s-OFDM QPSK 20MHz 100RB0 allocation: 31dB ACLR at 29dBm for the two “26dBm” PA used in both Dual PA PC2 and PC1.5 cases.

- Measuring the back-off needed for SEM and ACLR (for QPSK), and EVM for higher order modulations

- Equal PSD power sharing as a starting point. However, companies are encouraged to also conduct evaluation with a 6dB PSD imbalance.

- Both DFT-s-OFDM and CP-OFDM waveforms starting with QPSK

- When the same allocation is used in each CC the waveforms should be uncorrelated (using different data)

- Waveforms used in each Tx Path should use TxD processing to have some level of correlation (worst case)

- Emissions are calculated based on the mathematical sum of each path under the assumed coupling.

- Cases where power limitation per CC at 26dBm occurs must be reported.

- Both actual back-off and back-off difference between PC2 and PC1.5, may be reported.

# Conclusion

In this contribution, we examined the intra-band non-contiguous ULCA architectures for PC1.5 and made an initial assessment at back-off needed to meet emissions. This allowed to formulate the following proposals.

Proposal on the architecture description for the PC1.5 non-contiguous intra-band ULCA: The following table applies, and priority is to develop the MPR/A-MPR requirement for Architecture #1. In our view, Architecture#2 has too much limitations, in terms of the channel configuration supported, and especially cannot support n77(2A) for most operators and regions.

| Architecture | Implementation | Indicated capability | UL MIMO support | Power limitation | Support of 1CC PC1.5 fallback | Separation BW limitations |
| --- | --- | --- | --- | --- | --- | --- |
| #1 | 2x26 dBm PA + 2 LO with 100MHz BW | dualPA-Architecture | No | Each carrier is limited to 26dBm | Requires LO switching and thus interruption | No limitations other than the currently defined largest separation BW of 600MHz |
| #2 | 2x26 dBm PA + 1 LO with 200MHz BW | TxD and/or UL MIMO | Yes | One carrier can reach 29dBm | Can support without any switching | Max separation BW of 200MHz, Gap size <aggregated BW |

Proposal on MPR evaluation parameters:

- Post PA loss: 4dB, antenna isolation: 10dB for smartphone and 20dB for FWA

- PA calibration for DFT-s-OFDM QPSK 20MHz 100RB0 allocation: 31dB ACLR at 29dBm for the two “26dBm” PA used in both Dual PA PC2 and PC1.5 cases.

- Measuring the back-off needed for SEM and ACLR (for QPSK), and EVM for higher order modulations

- Equal PSD power sharing as a starting point. However, companies are encouraged to also conduct evaluation with a 6dB PSD imbalance.

- Both DFT-s-OFDM and CP-OFDM waveforms starting with QPSK

- When the same allocation is used in each CC the waveforms should be uncorrelated (using different data)

- Waveforms used in each Tx Path should use TxD processing to have some level of correlation (worst case)

- Emissions are calculated based on the mathematical sum of each path under the assumed coupling.

- Cases where power limitation per CC at 26dBm occurs must be reported.

- Both actual back-off and back-off difference between PC2 and PC1.5, may be reported.

# References

[1] R4-2407371 PC1p5 intra-band ULCA MOP for 2Tx and Dual-PA cases, Skyworks Solutions Inc., RAN4#111

[2] R4-2410750 WF on high power UE, Samsung, RAN4#111

[3] RP‑241656 UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, Huawei, HiSilicon, RAN#104

[4] R4‑2411302 On equal PSD vs equal power spectral regrowth, Skyworks Solutions Inc., RAN4#112