3GPP TSG RAN WG4 Meeting #112	R4-2412279

Maastricht, Netherlands, August 19th – August 23rd, 2024

Agenda Item:	8.19.3

Source: 	Ericsson

Title:	Initial discussion on RRM core requirements for Rel-19 Evolution of NR duplex operation: Sub-band full duplex (SBFD)

Document for:	Discussion

# Introduction

At RAN #114 meeting, WID on Rel-19 Evolution of NR duplex operation: Sub-band full duplex (SBFD) was revised, in this contribution the WID is listed in [1].

In this paper we provide our view on RRM core requirement within related Objectives.

WID objective related to RRM core requirement:

- Specify applicable RRM core requirements for CLI handling mechanisms [RAN4]

- Specify other RRM core requirements for SBFD operation, if identified [RAN4]

# Discussion

## CLI handling mechanism related RRM core requirements

Objective for CLI handling in WID description - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource. - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement

Within the 3 bullets of the CLI enhancement for SBFD, there are 2 bullets requiring RAN4 work.

  - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4]

  - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4]

The UL resource muting for PUSCH is a mitigation scheme mainly for gNB to gNB CLI handling which to our perspective shall not have impact on the RRM UE requirement.

The L1 based UE-to-UE CLI measurement and reporting is within the RRM core requirement scope.

Measurement Resource and Capability

Based on current RAN1 agreement, we understand that the CLI measurement will be performed within the active DL BWP.

Comparing with the Rel-16 CLI measurement, the current RAN1 agreement is aligned with the Rel-16 CLI measurement applicability due to active BWP for SBFD aware UE can be within DL subband, UL subband or guardband.

RAN1#116 agreement: For SBFD aware UEs, CLI measurements is performed within the active DL BWP and the following can be considered: - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - Method#3: UE measures RSSI within UL subband - Method#4: UE measures RSSI within guard band, if guard band exists Note: If DL subband, UL subband or guard band is outside the active DL BWP, the above methods does not apply. Note: Method#4 does not imply that guard band is explicitly configured. TS 38.133 Clause 9.7.1 CLI measurements are only applicable for RRC_CONNECTED intra-frequency: - when SRS-RSRP measurement resource is fully confined within BW of DL active BWP - when CLI-RSSI measurement resource is configured within active BWP

The difference between Method 1 and Method 3 is that the measurement RS being configured can be different in frequency location, which may result in different measurement results. For RAN4 to specify harmonized core requirement, the current RAN1 supported configurations will have complexity impact.

Observation 1: Current RAN1 agreed CLI measurement methods may result in different measurement requirement.

The CSI based measurement capability will be dependent on the maximum number of RS being configured and the maximum number of RS UE can be monitored.

For SRS-RSRP:

- Maximum number of SRS network can be configured per slot

- Maximum number of SRS UE can monitor per slot

- Total number of SRSs to be monitored by UE

For CLI-RSSI:

- Maximum number of total measurement resource network can configure for CLI-RSSI measurement.

- Maximum number of total measurement resource UE can monitor for CLI-RSSI

We think same ways or working of Rel-16 CLI can be applied here, RAN1 and RAN2 will specify the configuration, RAN4 will define the capability to monitor based on consensus within RAN4 group.

Since only co-channel UE-to-UE CLI handling schemes will be specified in RAN1, so we think measurements are not performed in gaps.

Proposal 1:  The measurement capability for L1-CLI measurement and report shall wait and follow the RAN1 specific agreement and RAN4 can start the discussion of what type of requirements shall be specify and the capability can be based on the maximum configured resource.

For SRS-RSRP, network will provide the measurement resources by SRS configuration.

An SRS resource determines:

- Time and frequency resource where UE should measure.

- The comb that UE should measure.

- The root sequence and cyclic shift UE should measure.

To our view, if UE has this information from the higher layer configuration, the SRS-RSRP measurement shall be performed accordingly.

For CLI-RSSI measurement, network will also provide the measurement resource for the CLI-RSSI RS configuration.

Based on current RAN1 agreement, only the SRS-RSRP legacy RS patterns are agreed to re-use, for CLI-RSSI measurement, certain information is needed for periodical, semi-persistence and aperiodic measurement resources:

- Number of PRBs and start PRB index for subband indication, configured PRBs are contiguous.

- Number of OFDM symbols and first OFDM symbol index in a slot, configured OFDM symbols are contiguous.

The legacy L3 CLI-RSSI measurement resources were configured by the network with slot duration correspond with one value of periodicity of the SSB. As the WID indicate the Rel-19 L1-CLI measurement will be based on existing CSI framework with Periodic, semi-persistent, or aperiodic measurement resource (set) being configured. This is different comparing with legacy Rel CLI-RSSI which the CLI-RSSI resources is being configured with rssi-PeriodicityAndOffset periodically.

Observation 2: The SBFD UE-to-UE CLI resources configuration is different compare with legacy CLI measurement. Legacy CLI-RSSI resource is configured periodically while SBFD CLI resource support Periodic, semi-persistent, or aperiodic measurement resource configuration.

Proposal 2: Measurement resources configuration for SBFD UE for both SRS-RSRP and CLI-RSSI will upon network configuration based on RAN1 and RAN2 agreement, RAN4 will define the measurement requirement based on future agreement of exact number of PRBs to be used and exact number of symbols to be used within one set of requirement.

Measurement timing, period, and side condition

For Rel-16 legacy CLI measurement, the exact measurement timing is decided up to UE implementation for both SRS-RSRP and CLI-RSSI. The timing error which has impact on measurement accuracy was specified as side condition that the constant offset relative to the current serving DL reference timing shall be applied. We think similar issue occurs for SBFD UE as well, and potential side condition for legacy CLI measurement can be used as a starting baseline.

In regarding the SBFD CLI measurement resource is more flexible than legacy CLI measurement, we think current CSI-RS based L1-RSRP measurement can be used as another baseline for future discussion of measurement period.

In regarding the measurement performance of accuracy, we list certain bullet points that to our knowledge may have impact.

CLI-RSSI

SBFD is within co-channel UE-to-UE CLI, here we read as supporting main 2 scenarios.

- Intra-frequency, Intra-cell

- Intra-frequency, inter-cell

Due to the reason that CLI-RSSI measurement is linear average of the total power, the configuration is upon network for different usage. The main use case for network to configure the RSSI measurement is to quantify or identify whether there is CLI issue for the UE. As the CLI-RSSI resources can be configured in any active BWP, the targeted measurement results will vary from UL subband to DL subband.

Also, to notice that measurement performance has high dependency on the sampling rate, averaging method and L1 filtering, which is up to UE implementation, we see a balance needed for the effort to achieve high measurement performance and the value of high measurement accuracy for CLI-RSSI measurement.

Comparing with the legacy CLI-RSSI measurement, another difference is the L1-CLI-RSSI reporting can be limited based on current RAN1 agreement.

The use case for SRS-RSRP is different. When network configures L1-SRS-RSRP measurement to the UE, the purpose is to identify the strongest aggressor among all. Measurement accuracy is important to serve that purpose. We list some factor may impact L1-SRS-RSRP measurement performance.

SRS-RSRP

- BW of the SRS, for example the legacy SRS-RSRP limits the SRS BW to 48 PRBs

- Propagation delay on the cyclic shift when multiple aggressors exist

- Multiple SRS multiplexed in comb, root sequence or cyclic shift.

- Measurement period will limit the number of samples for UE to take into averaging is depending on the required accuracy

- Other side conditions e.g., Ês/Iot for measurement efficiency since we only want to measure strong aggressor.

Observation 3: CLI-RSSI and SRS-RSRP serve different purposes and have different dependency factor for measurement performance.

Since the SBFD L1-based CLI measurement will follow the existing CSI framework, we see the L1-SRS-RSRP as an extension to add measurement quantity to existing L1-RSRP reporting. The main different here to notice is that L1-RSRP measures CSI-RS while L1-SRS-RSRP measures the SRS.

The CSI-RS and the SRS considered as reference symbols are designed differently, however from UE measurement point of view, what impact the measurement performances is how many reference symbol samples can be taken within the measurement window. Since there are no measurement gaps or sharing involved in SBFD CLI measurement, then the difference of the CSI-RS and SRS can be compared in detail for example root sequence, transmit density, power level etc for further analysis.

Proposal 3: RAN4 shall specify L1- CLI-RSSI and L1-SRS-RSRP measurement requirement.  RAN4 shall start the initial discussion for simulation assumption for setting up the requirements. The Legacy Rel-16 CLI-RSSI requirement can be used as L1-CLI-RSSI baseline and legacy L1-RSRP requirement can be used as L1-SRS-RSRP baseline.

## Other RRM core requirements for SBFD operation

WID objective Specify other RRM core requirements for SBFD operation, if identified [RAN4] - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2] - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2] - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol

In this section, we will list the RRM specification will potential be impacted by RAN1&RAN2 SBFD operation supporting Random Access, transmission, reception, and measurement behavior in the SBFD symbols.

Random Access Procedure in SBFD symbols

The existing random access UE requirement were defined in the TS 38.133 for correct UE behavior with each step for random access procedure.

Considering the SBFD UE only support half-duplex mode in compare with legacy UEs, within the SBFD symbols, UE can not support simultaneous transmission and reception.

Currently, RAN 1 discussion is still open in regarding of the RACH configuration, RO validation and RO prioritization etc. which may have impact on the RAN4 RRM specification for correct Random-Access UE behavior.

There are several legacy RAN4 requirements may be impacted by the final agreement of the RAN1 configuration and validation rules.

For example, UE needs to receive the SSB for DL synchronization and associate this SSB for the preamble which is mapping to the only RACH Occasion. The correct UE behavior is based on that at least one SSB received which associates the PRACH occasion. However, the HD SBFD UE may have issue to guarantee that presumption.

Another example is the existing legacy Random Access correct UE behavior for RA preamble selection request UE to randomly select RA preamble with equal probability based on the SS-RSRP threshold of the SSBUE selected which is based on the configured parameters like sb-perRACH-OccasionAndCB-PreamblesPerSSB etc.

Also, currently there are certain RA preamble transmission restriction based on the RRC parameters like ra-ssb-OccasionMaskIndex. Whether there will be transmission restriction specified for SBFD aware UE still needs to wait for RAN1 to conclude.

Observation 4: SBFD aware UE Random Access configuration, RO validation still has many open issues within RAN1 and RAN2 discussion, certain configurations and priority rules will have impact on RAN4 to specify the correct UE behavior for Random Access procedure.

Observation 5: SBFD aware UE only can operate in half duplex mode within the SBFD symbol which is different in compare with the legacy NR UE.

Proposal 4: RAN4 can wait for RAN1 and RAN2 agreement for Random Access procedure stable then define the correct UE behavior based on the agreements. Certain side conditions may need to be specified on top of the UE behavior due to the SBFD UE only support HD mode.

UE transmission, reception and measurement behavior and procedures in SBFD symbols

One issue to notice is the collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol.

RAN1# 117 agreement Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission. FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs

Based on the latest RAN1 #117 agreement the SSBs will always be prioritized over the UL transmission, the impact to RAN4 RRM specification is being minimized.

However, the SSB here to our understanding is only the SSBs within the active DL BWP which can only be within the current serving cell. Another scenario has not been ruled out from this agreement is that when a neighboring cell SSB in the same frequency location and time location collide with the UL transmission which is within the SBFD symbol. This will have potential impact for the mobility related RRM measurements if the SSBs for the neighboring cell are not being prioritized.

There are two options to resolve this potential issue, one option is to have a clarification for the SSBs being defined in this agreement. Then the question remains can RAN1 guarantee the neighboring cell configured SSBs being prioritized from configuration and restriction aspect?

Another option is that RAN4 introduce scheduling restriction on UL transmission for this SBFD symbol to prioritize the SSB. However, based on current RAN1 agreement, this symbol-based scheduling restriction is not optimized in frequency domain, the DL receptions within the DL usable PRBs are allowed within this SBFD symbols which is still needs for further study in RAN1.

Observation 6: Current RAN1 agreement has not clearly defined which SSB is being considered for the priority rules, only current serving cell SSB or any SSB within the SBFD symbol.

Observation 7: Typically, RAN4 specified scheduling restriction for handling collision between SSB and other transmissions, however the RAN4 entire symbol-based restriction is not optimized if RAN1 agree to use the PRBs that is not occupied by SSBs in frequency domain.

Proposal 5: For collision case RAN 4 need clarification from RAN1 which SSB is being agreed, only current serving cell or any SSBs collide with the SBFD symbol. RAN4 shall wait for RAN1 final agreement for the frequency domain usage of that SBFD symbol to decide how to address the collision case with RRM requirement.

# Summary and Conclusion

In this contribution we have provided our views on performance part of Mobility enhancement. The following proposals are made:

Observation 1: Current RAN1 agreed CLI measurement methods may result in different measurement requirement.

Observation 2: The SBFD UE-to-UE CLI resources configuration is different compare with legacy CLI measurement. Legacy CLI-RSSI resource is configured periodically while SBFD CLI resource support Periodic, semi-persistent, or aperiodic measurement resource configuration.

Observation 3: CLI-RSSI and SRS-RSRP serve different purposes and have different dependency factor for measurement performance.

Observation 4: SBFD aware UE Random Access configuration, RO validation still has many open issues within RAN1 and RAN2 discussion, certain configurations and priority rules will have impact on RAN4 to specify the correct UE behavior for Random Access procedure.

Observation 5: SBFD aware UE only can operate in half duplex mode within the SBFD symbol which is different in compare with the legacy NR UE.

Observation 6: Current RAN1 agreement has not clearly defined which SSB is being considered for the priority rules, only current serving cell SSB or any SSB within the SBFD symbol.

Observation 7: Typically, RAN4 specified scheduling restriction for handling collision between SSB and other transmissions, however the RAN4 entire symbol-based restriction is not optimized if RAN1 agree to use the PRBs that is not occupied by SSBs in frequency domain.

Proposal 1:  The measurement capability for L1-CLI measurement and report shall wait and follow the RAN1 specific agreement and RAN4 can start the discussion of what type of requirements shall be specify and the capability can be based on the maximum configured resource.

Proposal 2: Measurement resources configuration for SBFD UE for both SRS-RSRP and CLI-RSSI will upon network configuration based on RAN1 and RAN2 agreement, RAN4 will define the measurement requirement based on future agreement of exact number of PRBs to be used and exact number of symbols to be used within one set of requirement.

Proposal 3: RAN4 shall specify L1- CLI-RSSI and L1-SRS-RSRP measurement requirement.  RAN4 shall start the initial discussion for simulation assumption for setting up the requirements. The Legacy Rel-16 CLI-RSSI requirement can be used as L1-CLI-RSSI baseline and legacy L1-RSRP requirement can be used as L1-SRS-RSRP baseline.

Proposal 4: RAN4 can wait for RAN1 and RAN2 agreement for Random Access procedure stable then define the correct UE behavior based on the agreements. Certain side conditions may need to be specified on top of the UE behavior due to the SBFD UE only support HD mode.

Proposal 5: For collision case RAN 4 need clarification from RAN1 which SSB is being agreed, only current serving cell or any SSBs collide with the SBFD symbol. RAN4 shall wait for RAN1 final agreement for the frequency domain usage of that SBFD symbol to decide how to address the collision case with RRM requirement.

# References

- RP-241614 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, June ,2024