3GPP TSG RAN WG4 Meeting #112	R4-2412280

Maastricht, Netherlands, August 19th – August 23rd, 2024

Agenda Item:	8.15.3

Source: 	Ericsson

Title:	Initial discussion on Fast SCell activation for UE supporting Rel-18 EMR Document for:	Discussion

# Introduction

From RAN #103 meeting the WID for RRM enhancement phase 5 is listed in [1]. In this paper we provide our view on initial discussion for the second objective.

Background: WID objective #2

- Fast SCell activation for UE supporting Rel-18 EMR

- Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting

- Apply fast scell activation in FR1 and FR2-1

- Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

# Discussion

## Fast SCell activation for UE supporting Rel-18 EMR

Applicable Scenarios

Firstly, we would like to clarify to our view on what is Fast Scell activation within this WI scope since carrier aggregation is a feature with long legacy and continuous enhancement.

The latest enhancement from RAN4 RRM point of view is the Rel-18 WI on RRM enhancement to improve the Scell activation delay for the unknown target cell when there is no active serving cell in the same band. When UE supports l3-MeasUnknownSCellActivation-r18 and reports valid L3 measurement results after receiving the SCell activation command for unknown SCell, new requirement has been defined. Based on different UE capability on cell detection and L1-RSRP measurement delay reduction, different Scell activation delay will be applied.

The fast scell activation was a feature from Rel-17 MR-DC work item where aperiodic CSI-RS resources can be configured and introduced for Scell activation delay reduction since UE can use this CSI-RS based RS for the AGC retuning and reduce the waiting time for the SSB. However, the Rel-17 fast scell activation requirement only defined for the scenario that target cell is known to the UE or for the unknown target cell, and there is an active serving cell in the same band as the target cell.

Observation 1: Fast Scell activation from the WID objective can be interpreted as Rel-17 Fast Scell activation or Rel-18 Scell activation delay reduction enhancement.

Another thing to notice is for both FR1 and FR2, the Scell activation target cell known condition is always based on whether the UE has sent a valid measurement report. This was inherited from Rel-15 Scell activation, when the network configures the UE with a set of candidate Scells, UE may not be in the coverage of all of them. When a measurement report being triggered can be a direct indication to the network that the UE is within the coverage of certain Scell, then the network can trigger the activation command to add frequency resource for this UE.

This is the main motivation to introduce the Rel-18 EMR enhancement to combine with the Scell activation. For the UE transitioning from Idle/Inactive to the Connected mode, an earlier triggered measurement report can be an early indication to the network for activate extra resources for this UE. From this point of view, both Rel-17 A-TRS based Scell activation and Rel-18 Scell activation shall be considered for the WI scope.

Proposal 1: Both Rel-17 fast Scell activation and Rel-18 Scell activation delay reduction shall be considered as the baseline scenario for this Rel-19 RRM enhancement.

For UE supporting the Rel-18 EMR from our point of view also needs clarification.

The Rel-18 work for UE supporting Idle/Inactive measurement for CA/DC setup include two sets of measurement with 2 sets of UE behavior.

The measurement within the Rel-18 Idle/Inactive measurement for CA/DC capability are:

- UE supports Rel-16 early measurement for CA/DC setup aka EMR which have specific measurement configuration and report framework with a network configured validity timer

- UE supports reporting of valid cell reselection measurement results based on the network configured validity timer.

Observation 2: Valid EMR reporting can be interpreted as based on two sets of different measurements.

The UE behavior within the Rel-18 Idle/Inactive measurement for CA/DC capability are:

- UE has valid measurement results and reports those results

- UE does not have valid measurement results and does not report those

Since the WID objective limits the scope within UE with valid EMR reporting, which to our point of view excludes the UE behavior when there are no valid measurement results and does not report. When UE does not report measurement results, however, can be because of two different reasons and we don’t think this can be treated as the same when it comes whether it is applicable for fast Scell activation.

First situation for UE does not report is due to the measurement results have not reach the target accuracy. This behavior could have different causes for example UE measurement window is too short that UE does not have enough sample to reach the target accuracy. Another reason is simply the Idle/Inactive mode is so short that the UE did not even have chance to take one shot of measurement.

Another potential situation is that UE do not report due to the reason that measurement results is done certain time ago due to the network configured validity timer. Since this scenario can be controlled by the network configuration of the timer, we think it is clear if UE has invalid measurement results that is too old to report, the scenario shall not be included for the fast Scell activation enhancement.

For the UE that has not reached the target accuracy especially due to the reason that UE stays too short time in the Idle/Inactive mode, the UE behavior is not to report. However, Rel-18 agreement and conclusion is that UE is allowed to use the previous connected measurement if the Idle/Inactive mode is short that the validity timer covers the previous connected mode. And to our knowledge since the variables to store the Idle/Inactive measurement results is not being flashed, these results can be useful to be combined as the connected mode L3-report which could be beneficial for the Scell activation delay reduction as earlier L3 report can be sent.

RAN4# 111 meeting agreement Issue 1-2: applicability of validity check based on measurement obtained in CONNECTED mode before UE enters IDLE/INACTIVE mode For the validity check, it is up to UE implementation whether/how to utilize the measurement which was performed during CONNECTED mode before UE enters IDLE/INACTIVE mode.

Observation 3: Valid EMR reporting can be interpreted as only have UE behavior that UE reports measurement results during Idle/Inactive mode. However, the scenario where UE stays too short in Idle/Inactive mode and do not report cannot be fully ruled out for the scope.

Regarding the scenario when Idle/Inactive mode duration is shorter than the validity time being configured, we think it is a very common scenario since the traffic is always bursty, for the power saving purpose, UE is quite often toggle between at least Inactive and Connected mode. The validity timer can be configured by the network. However, the duration of the Idle/Inactive mode is purely based on how the traffic will be.

To our knowledge the purpose of this work item is to see where this Rel-18 EMR enhancement can be applied since the typical known condition for Scell activation requires a L3 measurement report to be sent.

We would like to think the EMR reporting for Scell activation enhancement as a side condition update instead of discussing whether it is feasible. Since UE sent early measurement report during the transition period, for example in Inactive mode the report will be sent in msg3 and in Idle the report can be request from msg3. When the early report set early can be used equivalent as a L3 measurement report for the side conditions defined for known target cell, we think there is an update for the side condition.

For example, both the Rel-17 and Rel-18 Scell activation delay enhancement starting the delay requirement timeline according to TS 38.133 is

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length

The known condition for both Rel-17 and Rel-18 in regarding period to send L3 report is define in the TS38.133 as

For FR1 During the period equal to max (5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of the SCell activation command For FR2 During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable)

The difference between Rel-17 and Rel-18 is that Rel-18 introduced a delay component TL3, report to to allow the L3 report timeline parallel with the L1 report timeline which enable UE with different capabilities to reduce the activation delay.

were, TL3, report is delay of acquiring the first available UL resource for L3 reporting from 7ms +THARQ after receiving SCell activation command. - The L3 measurement reporting requirement is defined in clause 9.2.4.4 - UE is ready to report the L3 measurement result no later than 7ms + THARQ ms from receiving the SCell activation command.

Meanwhile the Rel-18 EMR reporting timeline defined can be seen from figure 1 for both Idle and Inactive mode

Figure 1

For FR1, the EMR report sent within the period max (5*measCycleSCell, 5*DRX cycles) and for FR2 EMR report shall be sent within 3s or 4s depending on the UE power class.

The only difference to notice here is that typical Scell activation timeline is within connected mode, the scell being activated and deactivated, however for UE supporting the EMR report, the Scell will always be the 1st time being activated.

Observation 4: From timing wise, the EMR report timeline can be within the Scell activation L3 measurement report timeline for the target cell to be considered as known based on current legacy specification.

The Rel-16 EMR original purpose is provide Idle/Inactive measurement for network to setup the frequency to add CA/DC for the UE. The step right after the setup is the activation command. The only action UE may take from the time UE sent out the EMR report to the time UE receive the MAC CE activation command is that UE may or may not sent a valid L3 report. The time to process and compile L3 measurement results is within x ms, and the L3 report need UL resource for UE to transmit. The UL resource to transmit L3 report can be the only reason that valid measurement results cannot be sent. Typically, if event triggered L3 measurement report is being configured, the L3 measurement report needs to qualify the event and then wait for the UL resource, this can be beneficial when UE stays in connected mode due to the flow of scheduling is faster. The situation may be different for the UE stays in Idle/Inactive mode.

Since the measurement frequency, validity timer etc. are upon network configuration, the focus here we think should be the UE behavior can be observed.

For UE transit from idle to connected mode:

2 reports being sent

- UE sent EMR report at UE information response and UE sent L3 report before the MAC CE activation command

1 report being sent

- UE sent EMR report at UE information response and UE has not sent the L3 report before the MAC CE activation command

For UE transit from inactive to connected mode:

2 reports being sent

- UE sent EMR report at msg3 and UE sent L3 report before the MAC CE activation command

1 report being sent

- UE sent EMR report at msg3 and UE has not sent the L3 report before the MAC CE activation command

One very important thing to notice here is that the known and unknown condition is totally transparent to the network, which means after setup a list of candidate Scells, network has no knowledge which scell will be activated fast and which scell will be activated slow due to the known and unknown condition. For network, this is an uncertain situation that may cause resource being wasted by network waiting for some UE to pop up at e.g. PRACH while

And if we investigate the UE behavior closely, we will see that when UE has only EMR report, that target cell will still being considered as unknown. The minimum requirement for unknown target cell is different, however if UE only has EMR report while the SNR of this target cell SSB remains very good, we don’t see the reason that UE should activate this target cell slower than 2 reports being sent.

Observation 5: The RAN4 known and unknown condition is only UE knowledge while the network is not even aware during the activation.

The second question is whether the EMR report can be used as a valid L3 measurement report. We think in general, the EMR report is not as different as a normal L3 report except the measurement accuracy has been compromised due to the reason that the measurement sample is taken during Idle/Inactive mode. The measurement accuracy for EMR report was defined according to T338.133 Clause 4.4

…… For inter-frequency carriers configured for idle mode CA/DC measurements, the UE shall be capable of performing SS-RSRP and SS-RSRQ measurements of the carriers, and the UE physical layer shall be capable of reporting SS-RSRP and SS-RSRQ measurements of the carriers configured for idle mode CA/DC measurements to higher layers, with measurement accuracy as specified in clauses 10.1.4B and 10.1.5B and 10.1.9B and 10.1.10B, respectively. … #### 4.4.2.3 Measurements on serving cell. The UE shall measure the RSRP and RSRQ level of the serving cell and evaluate the cell selection criterion S defined in clause 4.2.2.2 and the UE physical layer shall be capable of reporting RSRP and RSRQ measurements of the serving cell to higher layers, with measurement accuracy as specified in 10.1.2B, 10.1.3B, 10.1.7B and 10.1.8B.

The main difference of the measurement accuracy requirements set for EMR vs normal L3 measurement reporting.

For both intra-frequency and inter-frequency measurement, there is only absolute accuracy defined for SS-RSRP and SS-RSRQ both for FR1 and FR2 with EMR reporting. With normal L3 report, the inter-frequency SS-RSRP and SS-RSRQ also requires the relative measurement accuracy. The relative measurement accuracy was defined as the RSRP or RSRQ measured from one cell in one frequency to another cell in another frequency.

Also to notice the absolute measurement accuracy for inter-frequency SS-RSRP and SS-RSRQ is roughly about 1.5dB relaxed in comparing with the normal inter-frequency measurement.

The measurement accuracy serves for purpose when the measurement report being sent to network, typically, network taken the measurements being reported and pick a list of candidate cells to be configured for the UE. In another words, the measurement results being sent more accurate have no or very little impact for the UE to activate this candidate cell. Since the measurement results is for network decision, the UE activation delay still heavily being impacted by the simultaneous radio condition which cannot be report directly.

Observation 6:The measurement accuracy defined for Rel-18 EMR is different in comparing with normal L3 measurement report. However, measurement accuracy will only impact the activation success rate, not the activation delay.

Proposal 2: The Rel-19 RRM enhancement in fast scell activation for UE support Rel-18 EMR shall not only be RAN4 side condition update, certain activation fast or slow indication to the network is needed.

# Summary and Conclusion

In this contribution we have provided our views on performance part of Mobility enhancement. The following proposals are made:

Observation 1: Fast Scell activation from the WID objective can be interpreted as Rel-17 Fast Scell activation or Rel-18 Scell activation delay reduction enhancement.

Observation 2: Valid EMR reporting can be interpreted as based on two sets of different measurements.

Observation 3: Valid EMR reporting can be interpreted as only have UE behavior that UE reports measurement results during Idle/Inactive mode. However, the scenario where UE stays too short in Idle/Inactive mode and do not report cannot be fully ruled out for the scope.

Observation 4: From timing wise, the EMR report timeline can be within the Scell activation L3 measurement report timeline for the target cell to be considered as known based on current legacy specification.

Observation 5: The RAN4 known and unknown condition is only UE knowledge while the network is not even aware during the activation.

Observation 6:The measurement accuracy defined for Rel-18 EMR is different in comparing with normal L3 measurement report. However, measurement accuracy will only impact the activation success rate, not the activation delay.

Proposal 1: Both Rel-17 fast Scell activation and Rel-18 Scell activation delay reduction shall be considered as the baseline scenario for this Rel-19 RRM enhancement.

Proposal 2: The Rel-19 RRM enhancement in fast scell activation for UE support Rel-18 EMR shall not only be RAN4 side condition update, certain activation fast or slow indication to the network is needed.

# References

- RP-240830 “New WID: WI resulting from discussion on RRM enhancements”, VIVO, March 2024