3GPP TSG-RAN WG4 Meeting #112	R4-2412304

# Maastricht, Netherlands, August 19th – 23th 2024

Source:	Ericsson

Title:	Discussion on Rel-18 NR NTN enhancement manufactory declaration and applicability rule for PUSCH DM-RS bundling

Agenda item:	5.23.8.1

Document for:	Discussion

# Introduction

In previous meeting, FR1-NTN PUSCH DM-RS bundling demodulation requirements were introduced and captured in the latest specification. But the corresponding manufactory declaration and applicability rules are missing. It would cause the confusion on how to use these requirements for conformance tests.

In this contribution, manufactory declaration and applicability rules based on TN specification are proposed for further discussion. The relevant CR [2] is also delivered.

# Discussion

Demodulating DM-RS bundling depends on SAN capability on joint DM-RS channel estimation and also depends on available consecutive UL slots duration. In that case, two manufactory declarations are introduced for TN demodulation specification 38.141 for TDD (with specified SCS) and FDD separately.

The corresponding applicability rules indicate how to apply requirements for different SCS, relevant TDD patterns and antenna connectors.

#### 8.1.2.9	Applicability of performance requirements for PUSCH with DM-RS bundling

##### 8.1.2.9.1	Applicability of requirements for TDD with different subcarrier spacings

Unless otherwise stated, PUSCH with DM-RS bundling requirement tests shall apply only for each subcarrier spcaing declared to be supported (see D.119 and D.120 in table 4.6-1).

##### 8.1.2.9.2	Applicability of requirements for TDD with different UL-DL patterns

Unless otherwise stated, for each subcarrier spacing declared to be supported, if BS supports multiple TDD UL-DL patterns, only one of the supported TDD UL-DL patterns with more than one concecutive UL slots shall be used for the PUSCH with DM-RS bundling performance tests.

##### 8.1.2.9.3	Applicability of requirements for different receiver antenna connectors

Unless otherwise stated, for a BS supporting different numbers of antenna connectors (for BS type 1-C) or TAB connectors (for BS type 1-H) (see D.37 in table 4.6-1), the PUSCH with DM-RS bundling performance tests with low MIMO correlation level shall apply only for the highest numbers of supported connectors, and the specific connectors used for testing are based on manufacturer declaration.

For NR NTN, similar approach could be applied but only PUSCH under FDD scenario should be considered. In that case, one declaration for FDD scenario and two applicability rules for SCS and antenna connectors could be feasible for SAN PUSCH DM-RS bundling requirements.

Proposal 1	Take following manufactory declaration for SAN PUSCH DM-RS bundling requirements.

| D.xxx | Supported FDD PUSCH DM-RS bundling | Declaration of supporting FDD PUSCH DM-RS bundling | c | x | n/a |
| --- | --- | --- | --- | --- | --- |

Proposal 2	Take following applicability rule for SAN PUSCH DM-RS bundling requirements (example for conducted tests).

#### 8.1.2.5	Applicability of performance requirements for PUSCH with DM-RS bundling

##### 8.1.2.5.1	Applicability of requirements for different subcarrier spacings

Unless otherwise stated, PUSCH with DM-RS bundling requirement tests shall apply only for each subcarrier spacings declared to be supported (see D.xxx in table 4.6-1).

##### 8.1.2.5.2	Applicability of requirements for different receiver antenna connectors

Unless otherwise stated, for a SAN supporting different numbers of TAB connectors (for SAN type 1-H) (see D.37 in table 4.6-1), the PUSCH with DM-RS bundling performance tests with low MIMO correlation level shall apply only for the highest numbers of supported connectors, and the specific connectors used for testing are based on manufacturer declaration.

# Conclusions

In the previous sections we made the following observations:

No table of figures entries found.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Take following manufactory declaration for SAN PUSCH DM-RS bundling requirements.

| D.xxx | Supported FDD PUSCH DM-RS bundling | Declaration of supporting FDD PUSCH DM-RS bundling | c | x | n/a |
| --- | --- | --- | --- | --- | --- |

Proposal 2	Take following applicability rule for SAN PUSCH DM-RS bundling requirements.

#### 8.1.2.5	Applicability of performance requirements for PUSCH with DM-RS bundling

##### 8.1.2.5.1	Applicability of requirements for different subcarrier spacings

Unless otherwise stated, PUSCH with DM-RS bundling requirement tests shall apply only for each subcarrier spacings declared to be supported (see D.xxx in table 4.6-1).

##### 8.1.2.5.2	Applicability of requirements for different receiver antenna connectors

Unless otherwise stated, for a SAN supporting different numbers of TAB connectors (for SAN type 1-H) (see D.37 in table 4.6-1), the PUSCH with DM-RS bundling performance tests with low MIMO correlation level shall apply only for the highest numbers of supported connectors, and the specific connectors used for testing are based on manufacturer declaration.

# References

[1] 	38.141-1 v18.6.0

[2] 	R4-2412305, CR for 38.181 on manufactory declaration and applicability rule for PUSCH DM-RS bundling.