3GPP TSG-RAN WG4 Meeting #112	R4-2412307

# Maastricht, Netherlands, August 19th – 23th 2024

Source:	Ericsson

Title:	Discussion on PRACH format 1 demodulation requirements for HAPS

Agenda item:	5.35

Document for:	Discussion

# Introduction

In RAN4#111 meeting, there are operators mentioned that there is no demodulation requirement for HAPS scenario and suggest considering at least PRACH format 1 demodulation requirement.  In this contribution, the general view on it will be delivered.

# Discussion

The HAPS (High Altitude Platform Station) is that the wireless access service is served by a BS located on an air vehicle and UEs located on the ground. Its altitude is smaller than satellite and similar as airplane. The typical altitude is 20km and the cell range is around 100km. HAPS is considered as semi-static respective to the earth. Currently, the typical deployment band is on FDD bands <2GHz.

As the HAPS could possibly serve normal handheld UE in rural scenario as GSO but with much smaller delay and Doppler, the UE capability for UL timing and frequency pre-compensation is not mandatory. In that case, the normal PRACH format could be used, especially format 1 whose cover range could reach ~108km. In current NR PRACH demodulation requirements, there is no PRACH format 1 requirement, and it is necessary to add it to cover HAPS scenario as discussed in RAN4#111.

HAPS cell range could reach over 100km and NR PRACH format 1 could be typical configuration for this scenario.

Proposal 1	Adding NR PRACH format 1 demodulation requirement to cover HAPS scenario.

NR PRACH format 1 is designed similar as LTE PRACH format 3. LTE PRACH demodulation requirements include PRACH format 3 under AWGN (with 0 frequency offset) and fading channel EPA [1]. For NR PRACH format 1, one possible way is to reuse LTE PRACH format 3 requirements to avoid simulation campaign. The other way is to collect new simulations from interested companies with new channel model for NR.

Table 2-1 comparison between LTE format 3 and NR format 1

| Preamble format | T_CP [Ts] | T_CP [ms] | T_SEQ [Ts] | T_SEQ [ms] | T_GP [Ts] | T_GP [ms] | Total Length [ms] | Cell Radius |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LTE format 3 | 21024 | 0.685 | 2x24576 | 1.6 | 21984 | 0.7162 | 3 | ~108 km |
| NR format 1 | 21024 | 0.685 | 2x24576 | 1.6 | 21984 | 0.7162 | 3 | ~108 km |

Channel model

The typical channel model between HAPS and UE on the ground could be LOS channel with very low Doppler shift. In ATG discussion, AWGN with limited Doppler shift (200Hz for FDD band) is agreed to be used as channel model for demodulation regarding to ATG UE could do pre-compensation on Doppler and timing.

As for HAPS, single tap channel model could also be considered for demodulation requirement. Different from ATG deployment, HAPS could be considered as static or semi-static to the ground, and the Doppler is caused mainly by UE on the ground. If we consider ~100km/h UE speed, the corresponding Doppler shift is 200Hz at 2GHz. However, the Doppler shift value won’t cause too much performance difference with single tap channel. In that case, if companies want to reduce simulation effort, one possible option is to reuse LTE PRACH format 3 AWGN channel requirement value. Otherwise, companies could consider run new simulations with new channel model as AWGN + 200 Hz with same configurations as LTE PRACH format 3.

Table 2-2 Parameters configuration for NR PRACH format 1

| Parameters |  | Value |
| --- | --- | --- |
| PRACH preamble |  | 1 |
| PRACH SCS [kHz] |  | 1.25 |
| Channel model |  | AWGN + [200] Hz Doppler |
| Time error tolerance | AWGN | 1.04 us |
| Ncs |  | 0 |
| Logical sequence index |  | 22 |
| v |  | 0 |

LOS channel with very low Doppler shift could be a typical channel model for HAPS deployment.

Proposal 2	Only consider single tap channel model requirement for NR PRACH format 1 if introduced.

Proposal 3 	Take Table 2-2 for PRACH format 1 parameters configuration.

Proposal 4	Companies to discuss following options for NR PRACH format 1 if it is agreed to be introduced.

Option 1:  To avoid new simulations, reuse LTE PRACH format 3 AWGN channel requirement values.

Option 2: Collect new simulations for NR PRACH format 1 with AWGN + [200]Hz Doppler shift.

In our contribution [2], we deliver simulation results with AWGN + 0 Doppler shift and 200Hz Doppler shift, and the performance are similar, but it is a bit different from LTE values if impairment is added.

# Conclusions

In the previous sections we made the following observations:

Observation 1	HAPS cell range could reach over 100km and NR PRACH format 1 could be typical configuration for this scenario.

Observation 2	LOS channel with very low Doppler shift could be a typical channel model for HAPS deployment.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Adding NR PRACH format 1 demodulation requirement to cover HAPS scenario.

Proposal 2	Only consider single tap channel model requirement for NR PRACH format 1 if introduced.

Proposal 3 	Take Table 2-2 for PRACH format 1 parameters configuration.

Table 2-1 Parameters configuration for NR PRACH format 1

| Parameters |  | Value |
| --- | --- | --- |
| PRACH preamble |  | 1 |
| PRACH SCS [kHz] |  | 1.25 |
| Channel model |  | AWGN + 200 Hz Doppler |
| Time error tolerance | AWGN | 1.04 us |
| Ncs |  | 0 |
| Logical sequence index |  | 22 |
| v |  | 0 |

Proposal 4	Companies to discuss following options for NR PRACH format 1 if it is agreed to be introduced.

Option 1:  To avoid new simulations, reuse LTE PRACH format 3 AWGN channel requirement values.

Option 2: Run new simulations for NR PRACH format 1 with AWGN + 200Hz Doppler shift.

# References

[1] 	TS 36.104

[2] 	R4-2412308, Simulation results for NR PRACH format 1 for HAPS, Ericsson