3GPP TSG-RAN WG4 Meeting #112 	R4-2412316

# Maastricht, Netherlands  19th  – 23th August, 2024

Source:	Ericsson

Title:	Simulation results for Rel-18 NR PRACH repetitions demodulation requirements

Agenda item:	5.21.2

Document for:	Information

# Introduction

In RAN4#111bis meeting, companies agreed to define PRACH repetition within 1 slot for TDD 120 KHz SCS for format A2 and C2. However, this requirement will have no limitation for PRACH configuration index selection for BS conformance testing. Based on the WF [1], further results are delivered in this meeting.

# Simulation result

## 2.1	Simulation assumptions

Table 2-1: Test parameters for PRACH repetition demodulation requirement for FR2-1

| PRACH | PRACH SCS | Time error tolerance |
| --- | --- | --- |
| preamble | (kHz) | TDLA30-300 |
| A2, C2 | 120 | 0.22 us |

| Burst format | SCS (kHz) | Ncs | Repetition | Logical sequence index | v |
| --- | --- | --- | --- | --- | --- |
| A2, C2 | 120 | 69 | Within 1 slot | 0 | 0 |

## 2.2	Simulation results

Table 2-1: Missed detection requirements for multiple PRACH transmission, 120 kHz SCS in FR2-1

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix (Annex G) | Frequency offset | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format A2 | Burst format C2 |
| 1 | 2 | TDLA30-300 Low | 4000 Hz | -10.4 | -10.5 |

# Conclusions

Simulation results for Rel-18 NR PRACH enhancement demodulation requirements are delivered.

# References

[1] R4-2410018, “WF for NR_cov_enh2_demod WI”, China Telecom, RAN4#111, Fukuoka, Japan