3GPP TSG-RAN WG4 Meeting #112	R4-2412319

# Maastricht, Netherlands, August 19th – 23th 2024

Source:	Ericsson

Title:	Discussion on Rel-19 MMSE-IRC BS demodulation requirements

Agenda item:	8.16.3

Document for:	Discussion

# Introduction

The WID of NR performance enhancement phase 5 was approved in RAN#103 meeting [1], and the following justifications and scope are agreed for MMSE-IRC BS demodulation.

Justification:

- BS receiver with MMSE-IRC can efficiently suppress the uplink interference from UEs in neighbouring cells and thus improve uplink coverage and capability. In addition, the widely use of HPUE makes UL co-channel interference from the neighbouring cell UE severer than that in LTE system. There is necessity to introduce BS performance requirements for MMSE-IRC advanced receiver under inter-cell interference.

Objective:

- Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions:

  - Receiver type: MMSE-IRC receiver

  - Scenarios:

    - FDD synchronous deployments

    - TDD synchronous deployments with aligned UL:DL configurations among cells

    - Slot-based transmission and aligned SCS among cells

  - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

In this contribution, the general view on MMSE-IRC BS demodulation requirement, corresponding interference profile and general assumptions are discussed.

# Discussion

In our contribution [3], we propose companies to check if artificial interference profile can be applied to avoid system level simulation. If companies still think the system level simulations are necessary to derive interference profile, we should discuss about parameters for simulation.

## 2.1	Typical deployment and system level simulation

Considering current network deployment, NR FDD bands are normally <2GHz and NR TDD bands are normally >3GHz. Corresponding ISD (inter site distance), BS antenna configurations and UE configurations would also be different.

For NR FDD bands, frequency, BS antenna configuration and UE configurations are normally same as LTE deployment. In that case, the system level simulation configuration in Table 5.1-1 and Table 5.1-2 in 36.883 could be reused for NR FDD homogenous and heterogenous deployment, and simulation results for SINR and DIP model could be similar. As for HPUE PC1 in FDD, it could be possible but not typical.

LTE BS MMSE-IRC profile is feasible for NR FDD band deployment.

For NR TDD bands, higher frequency bands and smaller ISD are used compared to FDD bands, and higher BS antenna branches and high-power UE might be used to achieve sufficient SINR regarding higher pathloss and larger channel bandwidth are typical in TDD. If system level simulation campaign is agreed for NR BS MMSE-IRC discussion, it is possible only to check the SINR and interference distribution for NR TDD deployment.

Proposal 1: 	Only setup NR TDD deployment to check the interference distribution if system level simulation is agreed.

### System level simulation parameters

Corresponding configurations could refer to Table A.2.1-1 in 38.802 and take “Urban Macro” configurations for homogenous deployment and “Dense Urban” configurations for heterogenous deployment as start point. Some configurations of TDD bands could be furtherly discussed here.

- ISD (Inter Site Distance): For heterogenous deployment, the ISD of macro layer is assumed as 200m in Table A.2.1-1 in 38.802, but it could be a bit larger regarding current typical value in real network such as 350m for 4GHz. The corresponding BS Tx power could be also higher than 44 dBm, such as 49dBm, to compensate the pathloss caused by distance increasement.

- Number of Micro BS per Macro cell: Use 3 Micro BS randomly drop per Macro cell to simplify the simulations.

- BS antenna model: Regarding Urban and Dense Urban scenarios would be considered, the typical BS antenna is 64TxRx AAS and the number of antenna elements could start from 128. The element deployment is as below figure. The HPBW (Half Power Bandwidth) of antenna elements could take 90o for both vertical and horizontal. The antenna radiated pattern model could refer to TR 36.873.

Figure 2.1-1 BS antenna array deployment

- UE Tx power: There are 3 power class UE (PC1 30dBm, PC2 26dBm and PC3 23dBm).  According to the utilization of each UE type, the deployment percentage of these 3 types of UE should be PC1 % < PC2 %< PC3 %. It could be difficult to configure different power and corresponding distribution in general simulator. Thus, it is proposed to use 26 dBm as the start point to check the interference impact by high power UE.

- UE distribution: 20% outdoor and 80% indoor for both homogenous and heterogenous deployments. 10 users per TRP.

- UE antenna model: Use omnidirectional antenna elements, and the number of antenna elements could start from 4. For UL transmission, only 1 Tx can be considered to estimate the inter-cell interference situation which could be typically from the cell edge.

- Penetration loss: Take low loss model for 80% users and high loss model for 20% users.

- UE scheduling method: there are two options mentioned in LTE system. One is RR (Round-Robin) as the baseline and the other is PF (Proportional Fairness) as optional. RR method have the best fairness for all UEs and no UE throughput is considered. It might not be the best and typical scheduling method in real network, but it is simple and could let all UE have possibility to transmit. It could be a better choice to have statistical interference distribution in whole network. It is also mentioned in the study report 36.884 that the simulation results are similar between RR and PF. The final LTE requirements are defined based on system simulations with RR method. We propose only consider RR scheduling method for NR system simulation either.

Proposal 2	Take PC2 UE (26dBm) as the start point for NR TDD SLS and further discuss how to consider 3 types of UE distribution in SLS.

If companies agree to use system level simulation to derive interference profile for NR deployment, we propose to consider following configurations as the start point based on analysis above.

Table 2.1-1: System-level assumptions for NR TDD homogeneous scenarios

| Parameters | configurations |
| --- | --- |
| Layout | Single layer Macro layer: Hex. Grid |
| ISD [m] | 500 |
| Carrier frequency [GHz] | 4 |
| Simulation bandwidth [MHz] | 20 |
| Channel model | 3D Uma |
| BS Tx power [dBm] | 49 |
| UE Tx power [dBm] | 26 as start point |
| BS antenna height [m] | 25 |
| BS antenna configurations | See table 2.2-3 |
| BS receiver noise figure [dB] | 5 |
| UE distribution | 20% outdoor 30km/h and 80% indoor 3km/h. |
| UE antenna height [m] | Follow TR36.873 |
| UE antenna configurations | See table 2.2-3 |
| UE receiver noise figure [dB] | 9 |
| UE receiver | MMSE-IRC |
| Traffic model | Full buffer |
| UE scheduling | Round robin |

Table 2.1-2: System-level assumptions for NR TDD heterogeneous scenarios

| Parameters | configurations |
| --- | --- |
| Layout | Two layers Macro layer: Hex. Grid Micro layer: Random drop (All micro BSs are all outdoor). 3 micro BSs per macro BS. |
| ISD [m] | Macro layer: 350 |
| Carrier frequency [GHz] | 4 |
| Simulation bandwidth [MHz] | 20 |
| Channel model | Macro layer: 3D Uma Micro layer: 3D Umi |
| BS Tx power [dBm] | Macro layer: 49 Micro layer: 33 |
| UE Tx power [dBm] | 26 as start point |
| BS antenna height [m] | Macro cell: 25 Micro cell: 10 |
| BS antenna configurations | See table 2.2-3 |
| BS receiver noise figure [dB] | 5 |
| UE distribution | 20% outdoor 30km/h and 80% indoor 3km/h. |
| UE antenna height [m] | Follow TR36.873 |
| UE antenna configurations | See table 2.2-3 |
| UE receiver noise figure [dB] | 9 |
| UE receiver | MMSE-IRC |
| Traffic model | Full buffer |
| UE scheduling | Round robin |

Table 2.1-3 Antenna configurations for NR TDD homogenous and heterogenous scenarios

| Parameters | Configurations |
| --- | --- |
| TxRU mapping | Per panel, reuse models in TR 36.897 |
| Number of BS antenna elements across all panels | 128 Tx/Rx elements |
| Number of UE antenna elements | 4 Tx/Rx antenna elements |
| BS (M, N, P, Mg, Ng) | (2, 1, 2, 4, 8) |
| BS (dH, dV, dH,g, dV,g) | (dH, dV) = (dH,g, dV,g) = (0.5, 0.8)λ |
| UE antenna model parameters | Mg = 1, Ng = 1, P = 2, dH = 0.5 |
| BS antenna element HPBW (HPBWh, HPBWv) [deg] | (90, 90) |
| BS antenna element gain pattern | According to TR36.873 |
| UE antenna gain [dBi] | -3 |
| UE antenna element gain pattern | Omnidirectional |

Proposal 3 	Take configurations in Table 2.1-1 to 2.1-3 as the start point for NR TDD system simulations if SLS is agreed.

## 2.2	Interference profile

From the performance checking perspective, there should be no differentiation between FDD and TDD and demodulation requirements are band agnostic, so only one set of profile should be defined to cover both whole bands.

If system level simulation is agreed for NR BS MMSE-IRC interference profile evaluation, companies could choose worse case from LTE FDD profile and outcome of NR TDD SLS.

If artificial values are agreed for interference profile evaluation, companies could propose some practical INR combinations for link level simulation and furtherly discuss it based on simulation results.

Proposal 4	If SLS is agreed for NR MMSE-IRC BS interference profile evaluation, take worse case from LTE profile and NR profile.

### Modulation level for interference

In practical network, any modulation level could be possible for interferences. Regarding the discussion scope in Rel-19, only inter-cell interference is considered, then relative low modulation level could be typical. In LTE requirements, only 16QAM is used for interference, and it could be reused for NR to reduce simulation and test effort.

Proposal 5	Only take 16QAM for interference profile of NR MMSE-IRC BS demodulation requirements.

## 2.3	IRC general assumptions

Once the interference profile is defined, the link level simulations for demodulation requirement could start. Some general assumptions are discussed here.

### Number of Rx antenna

Different Rx antenna numbers could handle different interference sources by different spatial combination gain. In legacy demodulation requirements, 2/4/8 Rx antennas are considered, and LTE MMSE-IRC BS demodulation requirements also use same Rx antenna number configurations as normal requirements. To have same coverage and check the product performance handling different interferers, 2/4/8 Rx antennas are proposed for NR MMSE-IRC BS demodulation requirements.

Proposal 6	Consider 2/4/8 Rx antenna configurations for NR MMSE-IRC BS demodulation requirements.

### Number of interferers

In LTE BS MMSE-IRC demodulation requirements, one and two interferences are considered. As the different spatial combination gain, one interferer is considered for 2 Rx and two interferers are considered for 4/8 Rx. Similar approach could be followed in NR discussion.

More interferers would highly increase the test complexity and could be avoided. Up to two interferers can have enough performance gain compared to non-IRC receiver.

Proposal 7	Consider 1 interferer case for 2 Rx antenna configuration and 2 interferers case for 4/8 Rx antenna configuration.

### MCS

The modulation of the wanted signal could be different in real network. To have a better coverage, one MCS per modulation level could be considered. Furthermore, the modulation level could be limited to 64QAM to cover typical interference limit scenarios. UL 256QAM transmission would happen in the cell center with very good channel condition and its inter-cell interference should be very small.

The specific MCS could start from typical values used in normal PUSCH demodulation requirements, e.g., MCS index 2, 12, 20 in MCS table for QPSK, 16QAM and 64QAM separately. Lower MCS could be also considered if simulation results show feasibility issue.

Proposal 8	Do not consider 256QAM requirements for NR MMSE-IRC PUSCH demodulation requirements.

Proposal 9	Start from MCS index 2, 12, 20 in MCS table for QPSK, 16QAM and 64QAM simulations separately.

### Channel model

Following aspects could be considered in channel model discussion here:

- Different deployments

- Serving signal and interferer

- Different MCS

Regarding both homogenous and heterogenous deployments are considered, LTE MMSE-IRC BS demodulation requirements consider two channel models for each deployment. Low Doppler models for heterogenous (EPA5 and ETU5) and high Doppler models for homogenous (EVA70 and ETU70) due to different cell ranges and typical UE speed. The corresponding interference profiles are also a bit different in LTE requirements. EPA and EVA are normally used to model low velocity scenario and ETU is normally used to model higher velocity scenario. It should be noted that, LTE MMSE-IRC PUSCH requirements apply same channel model for different modulation order under same deployment scenario which is different from normal PUSCH requirements. See table 2.4-1 below.

Table 2.3-1 Channel model in LTE MMSE-IRC PUSCH requirement

| Deployment scenario | MCS | Propagation condition (Serving, interferers) | Antenna configuration for serving and interferers |
| --- | --- | --- | --- |
| Heterogenous | 6 | (EPA5, ETU5) | 1x2 Low |
|  | 15 | (EPA5, ETU5) | 1x4 Low |
|  | 20 | (EPA5, ETU5) | 1x8 Low |
| Homogenous | 6 | (EVA70, ETU70) | 1x2 Low |
|  | 15 | (EVA70, ETU70) | 1x4 Low |
|  | 20 | (EVA70, ETU70) | 1x8 Low |

In NR, only TDLA30, TDLB100 and TDLC300 are used for demodulation requirements for NLOS propagation conditions where TDLA30 is normally used to model small cell scenario but there is no clear boundary between these models. Regarding only inter-cell interference is considered, large delay spread could be typical for interference and median delay spread might be practical for serving signal.

On the other hand, legacy NR normal PUSCH demodulation requirements apply different TDL channel models for different MCS, for example, TDLB100-400 for QPSK, TDLC300-100 for 16QAM and TDLA30-10 for 64QAM and 256QAM.

In summary, if NR follow similar approach as LTE here, following methodologies could be taken:

- same channel model for all modulation orders of serving signal under same deployment scenario;

- Use same propagation channel model for interferer signal beside of Doppler shift.

- larger delay spread and Doppler shift could be applied for homogenous deployment than heterogenous deployment;

The table 2.3-2 capture some possible channel model configuration based on summary above and could be the start point for further discussion.

Table 2.3-2 Channel model for NR MMSE-IRC PUSCH requirement

| Deployment scenario | Modulation level | Propagation condition (Serving, interferers) | Antenna configuration for serving and interferers |
| --- | --- | --- | --- |
| Heterogenous | QPSK | (TDLA30-10, TDLC300-10) | 1x2 Low |
|  | 16QAM | (TDLA30-10, TDLC300-10) | 1x4 Low |
|  | 64QAM | (TDLA30-10, TDLC300-10) | 1x8 Low |
| Homogenous | QPSK | (TDLB100-100, TDLC300-100) | 1x2 Low |
|  | 16QAM | (TDLB100-100, TDLC300-100) | 1x4 Low |
|  | 64QAM | (TDLB100-100, TDLC300-100) | 1x8 Low |

Proposal 10	Use channel model configuration in Table 2.4-2 for serving signal and interferer as the start point for NR MMSE-IRC BS demodulation requirement discussion.

### Channel bandwidth

As almost all NR enhanced features only consider the minimum channel bandwidth for each SCS, NR MMSE-IRC BS demodulation requirements could also start with the minimum channel bandwidth. Companies could furtherly check if clear performance difference is seen in other channel bandwidths. Normally, the better performance would be expected for larger channel bandwidth due to more DM-RS are captured. By observing LTE simulation results, the performance difference is small between 3MHz and 20MHz CBW.

Proposal 11	Take the minimum channel bandwidth for the NR MMSE-IRC requirements as the start point.

# Conclusions

In the previous sections we made the following observations:

Observation 1	LTE BS MMSE-IRC profile is feasible for NR FDD band deployment.

Based on the discussion in the previous sections we propose the following:

Proposal 1: 	Only setup NR TDD deployment to check the interference distribution if system level simulation is agreed.

Proposal 2	Take PC2 UE (26dBm) as the start point for NR TDD SLS and further discuss how to consider 3 types of UE distribution in SLS.

Proposal 3 	Take configurations in Table 2.1-1 to 2.1-3 as the start point for NR TDD system simulations if SLS is agreed.

Proposal 4	If SLS is agreed for NR MMSE-IRC BS interference profile evaluation, take worse case from LTE profile and NR profile.

Proposal 5	Only take 16QAM for interference profile of NR MMSE-IRC BS demodulation requirements.

Proposal 6	Consider 2/4/8 Rx antenna configurations for NR MMSE-IRC BS demodulation requirements.

Proposal 7	Consider 1 interferer case for 2 Rx antenna configuration and 2 interferers case for 4/8 Rx antenna configuration.

Proposal 8	Do not consider 256QAM requirements for NR MMSE-IRC PUSCH demodulation requirements.

Proposal 9	Start from MCS index 2, 12, 20 in MCS table for QPSK, 16QAM and 64QAM simulations separately.

Proposal 10	Use channel model configuration in Table 2.4-2 for serving signal and interferer as the start point for NR MMSE-IRC BS demodulation requirement discussion.

Proposal 11	Take the minimum channel bandwidth for the NR MMSE-IRC requirements as the start point.

# References

[1] 	RP-240831, New WID: WI resulting from discussion on Demodulation performance requirements enhancements, China Telecomm

[2]	TR36.884

[3]	TR38.802