3GPP TSG-RAN WG4 Meeting #112	R4-2412320

# Maastricht, Netherlands, August 19th – 23th 2024

Source:	Ericsson

Title:	Discussion on Rel-19 spatial channel model for demodulation

Agenda item:	8.14.1

Document for:	Discussion

# Introduction

In RAN#104, the SID RP-241610 was approved for Rel-19 scope. The corresponding objectives are as following:

The study item includes the following objectives:

- Study practical spatial channel modelling methodology for both SU- and MU-MIMO demodulation requirements and CSI reporting requirements:

  - Identify the limitations of the current (i.e. up to and including Release 18) channel models and corresponding scenarios and how they relate to UE MIMO performance

  - Consider both Clustered Delay Line (CDL)-based and TDL-based channel modelling approaches

    - For CDL-based channel modelling, use the tuned repeatable spatial channel model of TR38.827 as the starting point and identify any necessary changes.

  - Study and verify test methodology feasibility including test complexity and achievable results uncertainty. The test complexity shall not be significantly increased.

  - The methodology shall include both FR1 (conducted) and FR2 (wireless cable), with first priority for FR1.

According to TU allocated by plenary meeting, this study item will start from RAN4#112. In this contribution, general view on some key aspects of spatial channel model are delivered.

# Discussion

The massive MIMO is one of the most important features for beyond 4G wireless communication system. The spatial multiplexing significantly increases the channel capacity and break the Shannon limitation at some level. From the beginning of the study of MIMO features, spatial channel statistical impact was considered for system level and link level simulations [2, 3]. The modeling for AoD/AoA/ZoD/ZoA and ASD/ASA/ZSD/ZSA for clusters and rays are evaluated according to different deployment scenarios such as Uma, Rma etc. The CDL models play a very important role to get an insight of benefits and limitations for MIMO relevant features. These CDL models are widely used in RAN1 simulations.

TDL models are evaluated from CDL models by filtering the angle characters to delay taps and achieve a desired RMS delay spread, see figure 7.7.4-1 in [3]. This simplification would be easier for simulations, result alignment and test setup especially for demodulation requirement derivation. In that case, all existing demodulation requirements are defined by TDL channel model.

But it should be noted that, existing TDL models in [3] are derived from isotropic antenna pattern for both Tx and Rx sides which is not practical for real deployment. At least on BS side, directional antenna or array antenna would be much more typical. By non-isotropic antenna pattern, the propagation power by different departure/arrival angles would be quite different. Furthermore, the spatial filtering used in current TDL model is also simplified, then different layers have same received power. The spatial fading impact will be more significant in some directions. However, existing TDL models can only reflect fading in delay and Doppler domains not in spatial domain. It is also pointed out by [3] that “The TDL models for simplified evaluations, e.g., for non-MIMO evaluations, are defined for the full frequency range from 0.5 GHz to 100 GHz with a maximum bandwidth of 2 GHz.”

The existing TDL models are derived from isotropic antenna model on both sides.

The existing TDL models can’t directly reflect spatial channel fading in the real network.

As some operators providing the performance in field tests [4], it proves that different layer could have quite different received power. In that case, how to handle this power difference in receiver is highly relevant to the whole performance. This performance impact can’t be showed by current demodulation requirement since TDL model is used.

Some performance relevant field test results can’t be tested by existing performance tests using TDL model.

The existing CDL models in TR38.901 was tried for demodulation requirements at the early stage of NR release. But it has to be replaced by TDL model due to large span among companies’ simulation results and time pressure.  It leaves to people an impression that CDL model is more complex and harder to get alignment. So, all following demodulation requirements use TDL models rather than CDL. However, more and more enhanced MIMO features are developed based on multiple beams and corresponding performance partly depends on beam selection and estimation. Limited performance gain can be showed by TDL model in RAN4 while obvious gain can be achieved and showed in RAN1 evaluation by CDL model. This misalignment weakened persuasiveness of conformance test in RAN4 and feature benefit in RAN1 at some level.

As pointed out by 38.901, the CDL models in it are not very suitable for MIMO simulations due to following reasons:

- The angle values in these models are fixed and PMI statistics can become biased.

- A fixed precoder may perform better than open-loop and on par with close-loop or reciprocity beamforming.

- A CDL model could only represents a single channel.

Existing CDL models in 38.901 are not very suitable for MIMO simulations due to fixed angle values.

There are two extension methods are mentioned in TR38.901. One is on CDL models by applying angular translation and scaling to have desired cluster angle. The other method is to use TDL model and the spatial filtered TDL models with correlation matrix.

In TR 38.827, the first method is implemented for UE multi-antenna reception performance tests. The second method is partially applied in some existing UE demodulation requirements based on simple antenna assumption and different correlation level matrix. Both methods have their own pros and cons. Detailed analysis is needed to check the feasibility of two methods [5].

There are two extension methods mentioned in TR38.901 on legacy CDL and TDL model to evaluate more practical channel model for MIMO simulations.

As the complexity of corresponding conformance tests, TR38.827 have delivered some measurement procedures for FR1 and FR2 OTA tests which can be referred, but conducted test and wireless cable tests are not included. As the start point of the study, conducted test is more suitable to simplify the discussion. TE vendors should be fully involved in the whole discussion to make sure the channel model can be implemented properly.

Conducted conformance test methodology is more suitable for the start point of discussion.

Regarding observations above, RAN4 should investigate a set of proper spatial channel models based on available CDL/TDL models and methodologies for demodulation to secure the performance of product’s MIMO feature and potential future AI features under spatial fading channel environment. Following issues could be considered for the study report according to scope in WID.

- Investigate the feasibility of CDL models in TR38.827.

  - Difference from legacy CDL models.

  - Comparison to existing TDL models in requirements.

  - Trial simulation on one model to check performance and implementation difference between companies.

  - Modification if necessary.

- Investigate the feasibility of extension method by TDL with antenna correlation matrix mentioned by TR38.901.

  - Feasibility of existing correlation matrix.

  - Comparison to existing TDL models in requirements and CDL models in 38.827. Especially on complexity and feasibility of implementation.

  - Derive proper spatial channel models for MIMO simulation if necessary.

  - Trial simulation on one model to check performance and implementation difference between companies.

- Choose one set of models from modified CDL models and/or TDL models from item 1 and 2.

  - Align all parameter configurations.

  - Trial simulations and result comparison.

  - Potential enhancement in the future.

- Investigate the complexity of conformance tests.

  - Test setup and procedure. Check if current setup can be reused or not.

  - Channel implementation.

  - Test uncertainty and tolerance analysis.

  - Trial tests if possible.

- Investigate the potential test cases based on the consensus of SCM methodology if necessary.

  - Deliver a list of suitable cases from existing UE demodulation requirements with suggested priority for new requirements.

  - Proper test metric per test cases.

Proposal 1	RAN4 consider following content for the study report of SCM.

- Investigate the feasibility of CDL models in TR38.827.

  - Difference from legacy CDL models.

  - Comparison to existing TDL models in requirements.

  - Trial simulation on one model to check performance and implementation difference between companies.

  - Modification if necessary.

- Investigate the feasibility of extension method by TDL with antenna correlation matrix mentioned by TR38.901.

  - Feasibility of existing correlation matrix.

  - Comparison to existing TDL models in requirements and CDL models in 38.827. Especially on complexity and feasibility of implementation.

  - Derive proper spatial channel models for MIMO simulation if necessary.

  - Trial simulation on one model to check performance and implementation difference between companies.

- Choose one set of models from modified CDL models and/or TDL models from item 1 and 2.

  - Align all parameter configurations.

  - Trial simulations and result comparison.

  - Potential enhancement in the future.

- Investigate the complexity of conformance tests.

  - Test setup and procedure. Check if current setup can be reused or not.

  - Channel implementation.

  - Test uncertainty and tolerance analysis.

  - Trial tests if possible.

- Investigate the potential test cases based on the consensus of SCM methodology if necessary.

  - Deliver a list of suitable cases from existing UE demodulation requirements with suggested priority for new requirements.

  - Proper test metric per test cases.

# Conclusions

In the previous sections we made the following observations:

Observation 1	The existing TDL models are derived from isotropic antenna model on both sides.

Observation 2	The existing TDL models can’t directly reflect spatial channel fading in the real network.

Observation 3	Some performance relevant field test results can’t be tested by existing performance tests using TDL model.

Observation 4	Existing CDL models in 38.901 are not very suitable for MIMO simulations due to fixed angle values.

Observation 5	There are two extension methods mentioned in TR38.901 on legacy CDL and TDL model to evaluate more practical channel model for MIMO simulations.

Observation 6	Conducted conformance test methodology is more suitable for the start point of discussion.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN4 consider following content for the study report of SCM.

- Investigate the feasibility of CDL models in TR38.827.

  - Difference from legacy CDL models.

  - Comparison to existing TDL models in requirements.

  - Trial simulation on one model to check performance and implementation difference between companies.

  - Modification if necessary.

- Investigate the feasibility of extension method by TDL with antenna correlation matrix mentioned by TR38.901.

  - Feasibility of existing correlation matrix.

  - Comparison to existing TDL models in requirements and CDL models in 38.827. Especially on complexity and feasibility of implementation.

  - Derive proper spatial channel models for MIMO simulation if necessary.

  - Trial simulation on one model to check performance and implementation difference between companies.

- Choose one set of models from modified CDL models and/or TDL models from item 1 and 2.

  - Align all parameter configurations.

  - Trial simulations and result comparison.

  - Potential enhancement in the future.

- Investigate the complexity of conformance tests.

  - Test setup and procedure. Check if current setup can be reused or not.

  - Channel implementation.

  - Test uncertainty and tolerance analysis.

  - Trial tests if possible.

- Investigate the potential test cases based on the consensus of SCM methodology if necessary.

  - Deliver a list of suitable cases from existing UE demodulation requirements with suggested priority for new requirements.

  - Proper test metric per test cases.

# References

[1] 	RP-241610, “Study on spatial channel model for demodulation performance requirements”, Nokia, BT Plc, etc.

[2] 	TR 36.873

[3] 	TR 38.901

[4]	TR 38.827

[5] 	R4-2412321, Discussion on methodology of spatial channel model, Ericsson