3GPP TSG-RAN WG4 Meeting #112	R4-2412321

# Maastricht, Netherlands, August 19th – 23th 2024

Source:	Ericsson

Title:	Discussion on Rel-19 spatial channel model for demodulation

Agenda item:	8.14.2

Document for:	Discussion

# Introduction

In RAN#104, the SID RP-241610 was approved for Rel-19 scope. The corresponding objectives are as following:

The study item includes the following objectives:

- Study practical spatial channel modelling methodology for both SU- and MU-MIMO demodulation requirements and CSI reporting requirements:

  - Identify the limitations of the current (i.e. up to and including Release 18) channel models and corresponding scenarios and how they relate to UE MIMO performance

  - Consider both Clustered Delay Line (CDL)-based and TDL-based channel modelling approaches

    - For CDL-based channel modelling, use the tuned repeatable spatial channel model of TR38.827 as the starting point and identify any necessary changes.

  - Study and verify test methodology feasibility including test complexity and achievable results uncertainty. The test complexity shall not be significantly increased.

  - The methodology shall include both FR1 (conducted) and FR2 (wireless cable), with first priority for FR1.

According to TU allocated by plenary meeting, this study item will start from RAN4#112. In this contribution, the extension methods for legacy CDL and TDL model are analyzed, and some trial simulation results based on CDL model in 38.827 are delivered and discussed.  Common thinking on test complexity is also mentioned.

# Discussion

In our contribution for general view on SCM [2], we propose some study items according to WID scope including investigation on existing CDL model and TDL model, test complexity and potential test cases can be applied with SCM. Here we would give some detailed analysis.

## 2.1	Extension method on CDL models

In 38.901 7.7.5.1, a method to extend current CDL models for better MIMO simulation is delivered.

The predefined angle values in the CDL models can be generalized by introducing angular translation and scaling. By translation, mean angle can be changed to  and angular spread can be changed by scaling. The translated and scaled ray angles can be obtained according to the following equation:

(7.7-5)

in which:

is the tabulated CDL ray angle

is the rms angular spread of the tabulated CDL including the offset ray angles, calculated using the angular spread definition in Annex A

is the mean angle of the tabulated CDL, calculated using the definition in Annex A

is the desired mean angle

is the desired rms angular spread

is the resulting scaled ray angle.

The angular scaling is applied on the ray angles including offsets from the tabulated cluster angles. Typical angular spreads for different scenarios can be obtained from the system-level model.

This scaling method is investigated in TR38.827 and a new set angle spread value per deployment scenario are delivered.

Besides the angle values, following parameters are also scaled.

- The cluster delay is scaled from relative value to fixed value per scenario.

- The handing the two strongest clusters basically follow 38.901 method but don’t use same intra-cluster delay spread.

- Use fixed coupling ray angles rather than random values.

- BS antenna model use array antenna not isotropic antenna respective to frequency range. In that case, Tx antenna far field pattern is different from 38.901, and beamforming weight with random initial phase for sub-paths are also included in the channel coefficients.

- Use fixed initial phase of polarization matrix (with scalar under uniform distribution) rather than random value.

Comparing final CDL-C model table in 38.901 and 38.827 as an example, we can see that 38.827 expand CDL-C model to multiple models for different scenarios, such as Umi/Uma/InO, and also for different frequency bands, such as 3.5GHz and 28GHz. The power per cluster is similar, but delay per cluster and angle values become scenarios dependent in 38.827, not like 38.901 CDL model with fixed configuration for all scenarios. It could be noted that, all ZoA value in 38.827 CDL models for 3.5GHz are fixed to 90o.

TR38.827 also give indication on the code book definition for BS antenna pattern and the selection method of strongest beam from pre-defined grid for the test. The final selected beams would align with cluster characters defined in CDL models. Then the initial transmission direction is defined and could check the feedback from UE.

With all methodology instructions in TR 38.827, it could be possible to align CDL model implementation for link level simulations for DL direction. Taking it as the start point for further CDL model optimization in demodulation, such as higher layer performance, should be a practical way.

The original CDL models in TR38.901 are modified in TR38.827 on angle values with different scaling factors and extended to fit for different deployment scenarios.

The methodology described by TR 38.827 are detailed and instructive which could be a good start point for further optimization on model profile and implementation.

But the disadvantages of this method could also be noticed that scenario dependent CDL models would need more effort on parameters’ feasibility checking once a new scenario test case is considered. Based on the methodology in 38.827, the consideration of scaling focus mostly on test Different combinations of delay, AS and per-cluster parameter setting would cause a large set of model profiles used for demodulation requirements. It would lead to inconvenience for TE vendors and specification maintenance.

Scenario dependent CDL model would cause more effort on parameters’ feasibility checking for a new test case, and also might lead to a large set of channel model profile in specifications.

## 2.2	Extension method on TDL models

In 38.901 7.7.5.2, a TDL extension method by applying a correlation matrix is mentioned.

The TDLs and the spatial-filtered TDLs can be used with the correlation matrices for MIMO link-level simulations. Typical correlation parameters can be derived from 1) delay & angular scaled CDLs with antenna array assumptions, 2) system-level model with antenna array assumptions, or 3) by selecting extreme cases, e.g. uncorrelated, highly correlated etc. For example, these following options can be considered:

1)	Zero correlation (IID channel coefficients) can be used for any number of antenna elements

2)	The correlation matrix construction method from TS36.101/104 [15][16] can be used for linear and planar (single- or dual-polarized) arrays.

-	Other correlation parameters α, β, γ than those specified in TS36.101/104 [15][16] and extensions to larger antenna arrays can be considered. For typical scenarios, α and β will be in the range 0-1

-	A representative set of values is {0,0.7,0.9,0.99}

Note:	This approach can be applied to TDLs derived from spatially filtered CDLs to emulate hybrid BF system

Note:	Other methodologies could also be developed, e.g.

-	extending the TS36.101/104 [15][16] procedure to planar arrays or more elements

-	using CDLs in combination with array assumptions to derive per-tap correlation matrices as in [17].

-	using the system-level model in combination with array assumptions to derive per-tap or per-channel correlation matrices

In some existing demodulation requirements for LTE and NR, a simplified approach is applied for some MIMO relevant test cases. In 36.101 Annex B, there are 3 types of correlation matrix are defined, such as ULA, cross polarization 1D antennas and cross polarization 2D antennas. The different correlation level factors and beam steering methods are also defined to control beam directions during the tests. Only limited test cases in CQI reporting and PMI reporting would use correlation matrix for cross polarization antennas, for example PMI tests with large Tx number use 2D X-pol antenna correlation matrix. All other test cases only use ULA correlation matrix. In 38.101-4, only correlation matrix for ULA and cross polarization 1D antennas are defined for all test cases for now.

As noted in our contribution [2], the TDL model in TR 38.901 only use tapped delay-line for isotropic antenna model for both BS and UE side. Even applying correlation matrix on these TDL models, all taps would use same correlation matrix which also can’t reflect the spatial fading on different clusters caused by MIMO. The benefit of using TDL model would be less parameters and easier for model implementation in TE and could possibly get alignment between companies.

If RAN4 would use this extension method to have more practical channel model for future MIMO evaluation, the other methodologies mentioned in 38.901 could be considered, such as “using CDLs in combination with array assumptions to derive per-tap correlation matrices”. It would need following evaluations:

- New derived TDL models based on CDL model and proper array antenna assumptions.

- Define different correlation matrix per-tap based on different deployment scenarios, such as the per-tap polarization power matrix.

- The last step could use classical Doppler spectrum for each tap rather than using different Doppler spectrum to simplify the model. However, the feasibility of this simplification should be furtherly investigated.

Furthermore, model profile by this method would be much more complex if the number of both antenna ports and taps are high.

Existing demodulation requirements only use TDL model with same correlation matrix on all taps.

To have TDL model with per-tap correlation matrix, detailed evaluation would be needed, and the output model profile would be more complex if the number of both antenna ports and taps are high.

## 2.3	Comparison between two extension methods

As discussed in 2.1 and 2.2, general principles for channel model extension are described in 38.901. Both methods could be possible to give more practical spatial channel model for demodulation, but only TR 38.827 delivered complete parameters for CDL extension and also calibrated by measurements on low antenna branches. Regarding the extra effort needed for extension on TDL model, the extension on CDL described in 38.827 should be prioritized.

As the example or start point, CDL-C FR1 Uma model could be used for initial alignment on model implementation and further performance analysis because some detailed background information and methodologies have been provided.

Proposal 1	Prioritize CDL model extension method described in TR38.827 compared to TDL model extension method.

Proposal 2	Align CDL channel model implementation starting from CDL-C FR1 Uma model in 38.827.

## 2.4	Test setup

Regarding the WID scope and proposal in our contribution [2], the FR1 conducted tests should be prioritized. The test setup and procedures described in 38.827 is for OTA tests, but it could also be referred for conducted test setup, for example Conducted Two Stages (CTS) by using same BS and channel emulators.

In legacy test setup, very simple BS/UE antenna assumption is used in TDL models. Now the difference is the antenna assumption should be more practical, then the impact on channel should be considered.  For demodulation tests, the RF and antenna impact are normally “bypassed” to just compare the performance of receiver demodulator. Following the similar way here, we could only consider BS/UE antenna impact in the channel model by practical assumptions, and the antenna pattern measurement (as the first stage of general CTS) can be avoided. In that case, the measurements would be aligned no matter what kind of UE antenna is under the test. The test setup would be the same as current demodulation test setup. One of the benefits is the performance then can be directly compared between legacy channel model and new implemented model. The test uncertainties and tolerance could reuse legacy value since the setup is basically same.

The current demodulation test setup could be reused if only consider antenna pattern impact in channel model.

## 2.5	Preliminary simulation assumptions and results

Since 38.827 have delivered some modified CDL channel models for different scenario and frequency bands, it could be possible to run some simulations based on these models.

Considering the performance diverse per layer observed by some companies, we also choose 8 layers test cases. As for channel model, CDL-C for FR1 Uma 3.5GHz could be the candidate since its delay spread is 365ns and UE speed is 30km/h which is close to the delay spread and Doppler shift in TDLC300-100. It would be a bit more convenient to compare the performance difference between CDL model and TDL model.

Furthermore, CDL-C for FR1 Uma 3.5GHz is chosen as the example for beam selection in 38.827. It is also easier to follow similar way if we extend the 4x4 MIMO simulation to 8x8 scenario.

It should be noted that, Uma CDL-C model here might not be suitable for 8 layers simulations regarding to high fading loss caused by delay and Doppler. The purpose here is just to do comparison between CDL and TDL model and trying to use one model to get alignment on model implementation between companies. After that, smaller delay and Doppler model should be applied if we continue 8 layers performance simulations with CDL model. By then, more evaluation is needed based on available models.

In our contribution [5], we can see that:

- No much performance difference (<=1dB) between 2 CW under TDL model.

- There is 4~8dB performance difference between two codewords under CDL-C model in 38.900 and 2~3dB difference with CDL-C model in 38.827 while using random PMI. Following PMI results would also have obvious performance difference between CWs.

- The performance is much degraded under CDL comparing to TDL model.

The performance difference between two codewords are clear under 8x8 scenario with FR1 CDL-C Uma model and legacy CDL-C model.

The performance degradation by using FR1 CDL-C Uma model compared to TDLC300-100 is obvious.

# Conclusions

In the previous sections we made the following observations:

Observation 1	The original CDL models in TR38.901 are modified in TR38.827 on angle values with different scaling factors and extended to fit for different deployment scenarios.

Observation 2	The methodology described by TR 38.827 are detailed and instructive which could be a good start point for further optimization on model profile and implementation.

Observation 3	Scenario dependent CDL model would cause more effort on parameters’ feasibility checking for a new test case, and also might lead to a large set of channel model profile in specifications.

Observation 4	Existing demodulation requirements only use TDL model with same correlation matrix on all taps.

Observation 5	To have TDL model with per-tap correlation matrix, detailed evaluation would be needed, and the output model profile would be more complex if the number of both antenna ports and taps are high.

Observation 6	The current demodulation test setup could be reused if only consider antenna pattern impact in channel model.

Observation 7	The performance difference between two codewords are clear under 8x8 scenario with FR1 CDL-C Uma model and legacy CDL-C model.

Observation 8	The performance degradation by using FR1 CDL-C Uma model compared to TDLC300-100 is obvious.

Based on the discussion in the previous sections we propose the following:

_Toc174113186

Proposal 1	Prioritize CDL model extension method described in TR38.827 compared to TDL model extension method.

Proposal 2	Align CDL channel model implementation starting from CDL-C FR1 Uma model in 38.827.

# References

[1] 	RP-241610, “Study on spatial channel model for demodulation performance requirements”, Nokia, BT Plc, etc.

[2] 	TR 36.873

[3] 	TR 38.901

[4]	TR 38.827

[5] 	R4-2412320, General view on SCM for demodulation, Ericsson

[6] 	R4-2412322, Simulation results for SCM, Ericsson