3GPP TSG-RAN WG4 Meeting #112	R4-2412322

# Maastricht, Netherlands, August 19th – 23th 2024

Source:	Ericsson

Title:	Simulation results for Rel-19 spatial channel model for demodulation

Agenda item:	8.14.2

Document for:	Information

# Introduction

The Rel-19 SCM study start from RAN4#112 meeting according to plenary meeting. The modified CDL model is derived in TR38.827 which could be referred as the start point for further discussion.

In this contribution, simulation results for 8 layers PDSCH performance based on CDL-C FR1 Uma model in 38.827 are delivered.

# Discussion

### Simulation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Simulated Channel |  | PDSCH |
| Duplex mode |  | FDD DL only |
| FR / Carrier frequency |  | FR1 / 3.5 GHz |
| UE receiver type |  | SU MMSE |
| (BB) Antenna configuration |  | 8 Tx / 8 Rx |
| Waveform |  | CP-OFDM with normal CP |
| Channel Bandwidth/SCS |  | 40MHz/30kHz |
| MCS |  | 17 (for both CWs) |
| Channel model |  | CDL-C UMa in TR38827 TDLC300-100 |
| Active DL BWP index |  | 1 |
| PDSCH configuration | Mapping type | Type A |
|  | k0 | 0 |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 12 |
|  | PDSCH aggregation factor | 1 |
|  | PRB bundling type | N/A |
|  | PRB bundling size | N/A |
|  | Resource allocation type | Type 0 |
|  | VRB-to-PRB mapping interleaver bundle size | N/A |
|  | Transmission Rank | Fixed to 8 (2CW) |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 2 (for rank > 4) |
|  | DMRS ports indexes | {1000-1007} for Rank8 |
| Codebook configuration | CodebookType | typeI-SinglePanel |
|  | CodebookMode | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) | (4,1) for 8Tx |
|  | (CodebookConfig-O1,CodebookConfig-O2) | (4,1) |
|  | RI Restriction | 10000000 rank 8 |
| PDSCH & PDSCH DMRS Precoding | Common parameters | Wideband |
|  | Random | Random precoder |
| Analog impairments |  | None (No TO, no FO, no PA model, no TxEVM, no RxEVM, no IQ imbalance, etc.) |
| Number of HARQ Processes |  | 4 |
| Maximum HARQ transmissions |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  | 2 |
| UE speed |  | 30 km/h |

### Beam directions

The selection of strongest beam direction is based on 128Tx 8Rx configuration which is a bit different from TR38.827 but have quite similar directions.

Strongest beam: AoD: -6.99°, ZoD: 99.99°

2nd strongest beam: AoD: -21.99°, ZoD: 99.99°

3rd strongest beam: AoD: -35.98°, ZoD: 99.99°

4th strongest beam: AoD: -50.97°, ZoD: 99.99°

### Simulation results

### 2.1 TDL_C_300_100 8Tx8Rx rank8

### 2.2 CDL_C_TR38900 8Tx8Rx rank8

### 2.3 CDL_C_TR38827 8Tx8Rx rank8

### 2.4 TDL vs CDL TR38900 8Tx8Rx rank8

### 2.5 CDL_C TR38827 vs TR38900 8Tx8Rx rank8

# Conclusions

In the previous sections we made the following observations:

No table of figures entries found.

Based on the discussion in the previous sections we propose the following:

No table of figures entries found.

# References

[1] 	R4-2412321, Discussion on Rel-19 spatial channel model for demodulation, Ericsson