3GPP TSG-RAN WG4 Meeting #112	R4-2412331

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	8.17.1

Source:	Intel Corporation

Title:	Views on general aspects of AI/ML testability and interoperability

Document for:	Discussion

# Introduction

In Rel-18 RAN WG4 conducted detailed studies on the testability and interoperability aspects of AI/ML enabled techniques for NR air interface as a part of FS_NR_AIML_Air SI [1] and the conclusions are summarized in TR 38.843 [2]. A new WI on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved in RAN #102 meeting [3] with the following RAN4 objectives (the study stage objectives are marked in yellow):

### Objective of SI or Core part WI or Testing part WI Provide specification support for the following aspects: - … - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above two use cases. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring. … Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - … - Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis NOTE: offline training is assumed for the purpose of this project. NOTE: the outcome of the study objectives should be captured in TR 38.843 for future reference. ### Objective of Performance part WI - For Beam Management and Positioning Accuracy enhancement use cases, specify performance requirements and test cases for AI/ML LCM procedures (including performance monitoring) and UE features enabled by UE-sided models - Specify necessary performance requirements and tests (including metrics) for the above-mentioned use cases - Specify necessary test cases and performance requirements for LCM procedure, including performance monitoring.

The work item stage discussions took place in RAN4 #110, #110bis and #111 with agreements summarized in [4-6].

In this contribution we provide views and proposals on the remaining issues of general aspects of AI/ML testability.

# Discussion

## Post-deployment validation due to model change/drift

The topic of post-deployment functionality/drift validation was discussed during both SI and WI stages. The following was captured in RAN4 #108bis - RAN4 #111 meetings [4-9].

RAN4 #108bis (moderator summary) Issue 1-9: Post deployment testing - Proposals - Proposal 1: RAN4 should study a framework to enable post deployment tests for model updates and/or drift validation (and possible other use cases) - Following options can be taken for reference in further discussion: - Option 1- a: The changes/updates to the ML-enabled Functionality/Feature are tested and declared by the device vendor against RAN4 requirements before any deployment to the UE is performed. - Option 1- b: After deployment to the UE and before changed/updated ML-enabled Functionality/Feature is activated in the UE, a post-deployment validation is performed, e.g., a sanity check test loop is run, e.g., using the functionality performance monitoring and LCM activation/deactivation/switching procedures, - Option 1- c: At least one fallback/default Functionality/Feature that passed conformance testing must always be present in the device. - Other options can also be discussed - Proposal 2: RAN4 does not need to study such framework - Proposal 3: others, please provide some proposals RAN4 #109 (moderator summary) Issue 1-7: Tests post-deployment - Proposals - Proposal 1: The post deployment testing should be based on the model monitoring framework Postpone the discussion to a future release, possible as a study part of Rel-19 WI - Proposal 2: RAN4 should study the ways to validate performance after model updates and/or detected drift and discuss at least the following non-mutually exclusive options: - The changes/updates to the ML-enabled Functionality/Feature are tested and declared by the device vendor against RAN4 requirements before any deployment to the UE is performed. - After deployment to the UE and before changed/updated ML-enabled Functionality/Feature is activated in the UE, a post-deployment validation is performed, e.g., a sanity check test loop is run, e.g., using the functionality performance monitoring and LCM activation/deactivation/switching procedures, - At least one fallback/default Functionality/Feature that passed conformance testing must always be present in the device. - Proposal 3: There is no need for post-deployment testing - Proposal 4: other RAN4 #110 (WF) Issue 1-2: Post deployment handling Agreement: - To ensure the AI performance after device deployment, discuss the following options further - Option 1: Conduct the conformance testing for AI model/functionality before deployment - FFS on the feasibility - Option 2: Design the test to verify the performance monitoring - Depend on the other WG progress - Monitoring can be used for managing fallback, model update/model switching/model transfer, if applicable - Other options are not precluded RAN4 #110bis (moderator summary) Issue 1-1: Post deployment testing options: - Proposals - Proposal 1: Add Option 3 (R4-240495 – vivo) Option 3: Define the test to verify the performance validation together with model Proposal /update - validation scheme can be designed by RAN1 - Proposal 2: Add Option 4 (R4-2405737 – Nokia) Proposal 4: RAN4 to test the procedure when updated/new model/functionality stays inactive in the device before is has passed assessment/verification and can substitute currently active model/functionality. - Proposal 3: include some mandatory fallback (e.g. to a “baseline” model or older model already tested/validate) as an option - Proposal 4: As a further option relating to post deployment testing, consider the possibility of capturing model input during testing for later testing of new models: Capture model input during conformance testing for later testing of new models. (FFS the captured data needs to be held completely by the UE vendor) (Ericsson – R4-2405610) - Proposal 5: other RAN4 #111 (moderator summary and WF) Issue 1-1: Post deployment testing - Proposals - Proposal 1: Option 1 is feasible only if model updates are infrequent - Proposal 2: Option 2 is feasible only if the validation can be performed by UE vendors individually (no need to test in a certified lab) - FFS whether some validation information needs to be shared with gNB - Proposal 3: Add an option in which the conformance test inputs are stored and used for post-deployment testing/validation - Proposal 4: Drop Option 1, only discuss post deployment handling as part of LCM - Proposal 5: RAN4 should discuss a framework to ensure that at least a validated model exists at the UE - Proposal 6: RAN4 should discuss a framework to ensure that non-validated/non-tested models are not used by the UE - Proposal 7: Others - Agreement - Interested companies are invited to bring analysis on the feasibility of the options proposed so far.

Managing the performance of AI/ML model (ML-enabled Functionality/Feature) updates or changes over time, particularly for UE-side models is one of the key challenges of AI/ML framework. Per study item agreements, AI/ML models may originate from a UE, UE-side OTT server and there are two general alternatives considered: 1) model updates or changes that are transparent to the network, and 2) model updates are non-transparent to the network. One of the specific characteristics of AI/ML air interface is that AI/ML models (features/functionality) can change over the lifetime of UE via (re)configurations, models updates (e.g. due to additional training) and other reasons. In practice, the changes in the AI/ML models used at the UE side may lead to alterations in the performance of the UE, which in turn can impact the overall network performance. Therefore, it is crucial to establish a robust framework for post-deployment model verification.

While regular conformance tests are expected to be performed for certain default AI/ML models, the existing 3GPP conformance framework may not be directly suitable for post-deployment verification. In particular, the fact that UE has passed the conformance tests, does not guarantee that the performance will be maintained in case of AI/ML model updates or changes over UE lifetime.

Considering the discussion in previous meetings we observe the following generic options on how to handle post-deployment AI/ML model performance verification, which are eventually not mutually exclusive.

Option 1: Conformance testing for AI model/functionality before deployment to the UE.

The first option is to test AI/ML model/feature updates against RAN4 requirements and declare the changes or updates to the AI/ML model by the device vendor before any deployment to the UE is performed. This comprehensive approach guarantees that models meet the minimum performance requirements. One of benefits of such approach is that the minimum performance is guaranteed, which may be helpful for the network as it may not need to extensively rely on the performance monitoring framework (e.g., not use frequent monitoring or use a subset of metrics depending on RAN1 design).

Under an assumption that the AI/ML model update comes from the UE-side OTT server it is reasonable to assume that device vendors have ability to perform conformance testing of respective models, before the model can be updated (i.e., the testing is considered as feasible). The conformance testing does not necessarily imply lab testing and a simplified procedure can be considered including UE vendor testing & declaration (no certification lab). The respective validation information can be shared with gNB.

Same time we acknowledge that this option may limit the flexibility of UE implementation and is not compatible with more adaptive approaches: 1) training/updates performed directly at the UE side, 2) use of localized AI/ML models optimized for specific conditions (i.e., models are not generalized for a wide set of conditions and may eventually have degraded performance under conformance test conditions).

Observation #1: Conformance testing for AI model/functionality before deployment at the UE

- Can guarantee the minimum performance of models under various conditions.

- May be feasible at least for the case when AI/ML model update comes from the UE-side OTT server.

- More suitable for generalized AI-ML models with infrequent updates

- Not applicable for 1) training/updates performed directly at the UE side, 2) localized AI/ML models optimized for specific scenarios.

Option 2: Performance monitoring framework/requirements (incl. mandatory fallback and performance validation)

The second option is to not explicitly test the changes or updates to the models during UE lifetime (post-deployment), and instead, the network and/or UE would rely on performance monitoring mechanisms. We agree that the AI/ML performance monitoring framework is an integral part of the whole AI/ML feature, and it is important to define respective requirements to guarantee the control of performance. However, it is currently unclear whether the performance monitoring mechanism can provide reliable performance all the time and impose limited overhead at the same time. Also, it is yet unclear whether such test cases can fully replace conformance testing.

Observation #2: The details of performance monitoring framework are not yet defined, and it is unclear whether the mechanism can guarantee minimum performance in the field.

In previous meetings it was also proposed to include specific procedure for performance validation during model transfer/update, which we assume can be considered as part general performance monitoring framework. If such approach is defined, then the performance validation can be performed in a dynamic manner and the procedure of testing UE performance can be effectively built-in into the AI/ML framework. The details of such approach candidate approach are unclear and need to be further discussed.

Observation #3: The specific procedure for performance validation during model transfer/update can be further discussed with the goal to ensure minimum performance.

Similarly, there were proposals to require UE to have a mandatory support of model(s) fulfilling conformance requirements, which can be used as a fallback subject to decisions made via performance monitoring framework (“procedure when updated/new model/functionality stays inactive in the device before is has passed assessment/verification and can substitute currently active model/functionality”). The respective approach can be considered as a complimentary method to ensure that UE can guarantee proper performance.

Observation #4: The support of mandatory support of model(s) fulfilling conformance requirements which can be used as a fallback as a part of performance monitoring framework may be considered as a complimentary method to ensure that UE can guarantee proper performance.

Conclusions

In our view neither approach is flexible enough, and the eventual design should take the pros/cons of both into account. The following proposals are made:

- UE shall support at least some default/baseline AI/ML models/features that pass conformance requirements. The respective models can be used as a fallback and can substitute currently active models/features.

- RAN4 to support both approaches including conformance testing and performance monitoring. Further discuss conditions/scenario when each approach is applied.

- Changes or updates to the AI/ML models/features are tested by the device vendors against RAN4 requirements before the deployment to the UE is performed. The information on whether AI/ML model/feature update has passed conformance test (and potentially associated data) shall be conveyed to the network, and based on this, the network may adjust the model monitoring framework accordingly.

- Further discuss a potential specific performance validation procedure during AI/ML model transfer/update as a part of performance monitoring framework (involving RAN1).

Proposal #1:	Consider the following framework for post-deployment model (feature/functionality) verification:

  - UE shall support at least some default/baseline AI/ML models/features that passed conformance requirements. The respective models can be used as a fallback mechanism.

  - RAN4 to support both approaches including conformance testing and performance monitoring. Further discuss conditions/scenario when each approach is applied.

  - Changes or updates to the AI/ML models/features are tested by the device vendors against RAN4 requirements before the deployment to the UE is performed. The information on whether AI/ML model/feature update has passed conformance test (and potentially associated data) shall be conveyed to the network, and based on this, the network may adjust the model monitoring framework accordingly.

  - Further discuss a potential specific performance validation procedure during AI/ML model transfer/update as a part of performance monitoring framework (involving RAN1).

## Reference AI/ML models for performance requirements

Due to a diverse nature of AI/ML models implementations, there may be a significant variation in their complexity and performance, which is illustrated by RAN1 studies [2]. From RAN4 requirements perspective it is important to establish a common baseline for performance and complexity (for both 1-sided and 2-side models) and a big divergence in models’ complexity/performance may become a problem. To overcome the challenge, it is recommended to define reference AI/ML models for minimum performance requirements definition. Many existing demodulation requirements are defined under an assumption of a certain reference receiver (e.g., MMSE, R-ML, etc.) and a similar concept can be taken for 1-sided and 2-side AI/ML models. The models, whether partially or fully defined, would be utilized during the performance requirements definition stage to achieve a certain minimum level of performance under an assumption of a specific complexity level. The reference model concept is not necessarily meant to be standardized in detail, but rather should serve as a general guideline. Given the complexity of discussions on reference models and assumptions, it is recommended that these discussions be initiated early in the WI process.

In previous meetings RAN4 discussed feasibility of testing 2-sided AI/ML models for CSI compression and several candidate options on how to test encoder/decoder implementation at the test equipment side were discussed. During the RAN4 #110 meeting it was decided to narrow the testing options for two-sided AI/ML models with the focus shifted towards defining a common reference decoder for 2-sided AI/ML models - Option 3 with fully standardized reference decoder and Option 4 with partially standardized reference decoder (dataset + key parameters). If either of respective approaches is identified to be feasible, a similar method can be applied to define reference models for requirements definition.

Proposal #2:	Define reference AI/ML models for one-sided and two-sided models (both encoder/decoder) for performance requirements definition taking into account performance/complexity tradeoffs.

## LCM Requirements

In RAN4 #111 the following agreements on LCM requirements were reached [6]:

Issue 1-5: LCM Requirements Agreement: - RAN4 considers defining RRM requirements for LCM procedures based on the specific use cases (beam management and positioning), and candidates are: - identification - selection - activation - deactivation - switching - fallback to non-AI operation - performance monitoring - Others are not precluded

It is important to note that discussions in RAN1/2 regarding these topics are ongoing, including the decision on whether to adopt functionality- and model-based LCM approaches. Consequently, finalizing the set of requirements at this stage may be premature. Considering that RAN4 aims to finalize the feasibility of testing for the CSI compression use case, it is recommended that deliberations on LCM requirements be deferred until Q4 2024. These discussions should take place during the RRM session, as the requirements are expected to be incorporated into the RRM specification.

Moreover, RAN4 should strive to establish requirements in a generic framework to facilitate broad applicability. This framework should be adaptable for various use cases, allowing for the customization of parameters as necessary.

Proposal #3:	Postpone discussion on LCM requirements to Q4’24 and handle it in the RRM session.

## Channel models for testing datasets

The question of data collection/generation for testing was discussed since the SI stage, where the following agreements were made [2]:

Different generating methods of test dataset can be used for different tests. The following candidate methods are to be considered: - Dataset based on TR 38.901, e.g. UMa channel, UMi channel, CDL channel, “legacy approach”, etc. - “Legacy approach” refers legacy test in which a channel model is used - Field dataset (data collected directly from field measurements) - TE generates dataset for test based on assumptions/parameters defined by RAN4 (e.g. by defining some rules/function to generate data) - Other methods are not precluded

In RAN4 #11the following agreements on related aspects for inference were made [4]

Issue 1-7: Test data handling Agreement: - For inference test, use synthetic channels as baseline, and check whether it can be used for the individual use case

In RAN4 #111 the following proposals on channels used for training/testing dataset were discussed [7]:

Issue 1-9: Training/Test data handling - Proposals - Option 1: For each use case, RAN4 needs to study whether using synthetic channels for test data will reliably test models trained on real data. - Option 2: Synthetic channels (RAN4 channels) should be enough for testing - Option 3: RAN4 should discuss whether/how field data can be used for testing - Option 4: Other

In our view further discussion on the relevant aspect can be helpful taking into account the progress for individual use cases. The following principles are suggested in terms of channel models to be used for testing:

- Synthetic channels as baseline: Utilize synthetic channel models as the default assumption for both training and inference phases. Assess their suitability for each specific use case.

- Field datasets with lower priority: Consider field datasets as a secondary option, given their potential impact on the standardization progress and timelines. Field datasets should only be used if significant issues arise with synthetic models.

- Generic synthetic channels for training of AI-ML Models: For training AI-ML models, start with a generic synthetic channel model datasets based on TR 38.901, e.g. UMa channel, UMi channel, CDL channel, “legacy approach”, etc.

- Simplified synthetic channels for Inference/Testing Phase: For the inference (i.e., testing phase), a simplified set of synthetic channel models, such as TDL (Tapped Delay Line) models or other channels used in legacy RAN4 tests, may be used. This approach aims to minimize the impact on test feasibility and promote the reuse of existing methodologies.

Proposal #4:	Adopt the following channel modelling framework for AI-ML requirements

  - Use synthetic channels as the default assumption for both training and inference phases. Assess their suitability for each specific use case.

  - Field datasets should only be used if significant issues are identified with synthetic models.

  - For training AI-ML models use generic synthetic channel model datasets based on TR 38.901, e.g. UMa channel, UMi channel, CDL channel, “legacy approach”, etc.

  - For the inference (i.e., testing phase) use a simplified set of synthetic channel models, such as TDL (Tapped Delay Line) models or other channels used in legacy RAN4 tests.

# Conclusion

In this paper we provide our views on general aspects of AI/ML testability and interoperability aspects, and, in summary, make the following proposals:

Proposal #1:	Consider the following framework for post-deployment model (feature/functionality) verification:

  - UE shall support at least some default/baseline AI/ML models/features that passed conformance requirements. The respective models can be used as a fallback mechanism.

  - RAN4 to support both approaches including conformance testing and performance monitoring. Further discuss conditions/scenario when each approach is applied.

  - Changes or updates to the AI/ML models/features are tested by the device vendors against RAN4 requirements before the deployment to the UE is performed. The information on whether AI/ML model/feature update has passed conformance test (and potentially associated data) shall be conveyed to the network, and based on this, the network may adjust the model monitoring framework accordingly.

  - Further discuss a potential specific performance validation procedure during AI/ML model transfer/update as a part of performance monitoring framework (involving RAN1).

Proposal #2:	Define reference AI/ML models for one-sided and two-sided models (both encoder/decoder) for performance requirements definition taking into account performance/complexity tradeoffs.

Proposal #3:	Postpone discussion on LCM requirements to Q4’24 and handle it in the RRM session.

Proposal #4:	Adopt the following channel modelling framework for AI-ML requirements

  - Use synthetic channels as the default assumption for both training and inference phases. Assess their suitability for each specific use case.

  - Field datasets should only be used if significant issues are identified with synthetic models.

  - For training AI-ML models use generic synthetic channel model datasets based on TR 38.901, e.g. UMa channel, UMi channel, CDL channel, “legacy approach”, etc.

  - For the inference (i.e., testing phase) use a simplified set of synthetic channel models, such as TDL (Tapped Delay Line) models or other channels used in legacy RAN4 tests.

# References

- RP-221348 SID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #96, June 2022

- 3GPP TR 38.843 Study on Artificial Intelligence (AI) / Machine Learning (ML) for NR air interface

- RP-234039 WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #102, December 2023

- R4-2403712 WF on AI/ML, Qualcomm, RAN4 #110, February 2024

- R4-2406617 WF on AI/ML, Qualcomm, RAN4 #110bis, April 2024

- R4-2401570 WF on AI/ML, Qualcomm, RAN4 #111, May 2024

- R4-2408944 Topic summary for [111][133] NR_AIML_air , Qualcomm, RAN4 #111, May 2024

- R4-2317258 Topic summary for [108bis][135] FS_NR_AIML_air, Qualcomm, RAN4 #108bis, October 2023

- R4-2318142 Topic summary for [109][136] FS_NR_AIML_air, Qualcomm, RAN4 #109, November 2023