3GPP TSG-RAN WG4 Meeting #112	R4-2412332

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	8.17.4

Source:	Intel Corporation

Title:	Views on AI/ML testability for CSI compression and prediction

Document for:	Discussion

# Introduction

In Rel-18 RAN WG4 conducted detailed studies on the testability and interoperability aspects of AI/ML enabled techniques for NR air interface as a part of FS_NR_AIML_Air SI [1] and the conclusions are summarized in TR 38.843 [2]. A new WI on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved in RAN #102 meeting [3] with the following RAN4 objectives (the study stage objectives are marked in yellow):

### Objective of SI or Core part WI or Testing part WI Provide specification support for the following aspects: - … - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above two use cases. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring. … Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - … - Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis NOTE: offline training is assumed for the purpose of this project. NOTE: the outcome of the study objectives should be captured in TR 38.843 for future reference. ### Objective of Performance part WI - For Beam Management and Positioning Accuracy enhancement use cases, specify performance requirements and test cases for AI/ML LCM procedures (including performance monitoring) and UE features enabled by UE-sided models - Specify necessary performance requirements and tests (including metrics) for the above-mentioned use cases - Specify necessary test cases and performance requirements for LCM procedure, including performance monitoring.

The work item stage discussions took place in RAN4 #110, #110bis and #111 with agreements summarized in [4-6]. In this contribution we provide views and proposals on the AI/ML testability and interoperability aspects for two-sided AI/ML models for CSI compression and CSI compression use case.

# Discussion

## Background

In previous meetings RAN4 discussed feasibility of testing 2-sided AI/ML models for CSI compression use case and several candidate options on how to test encoder/decoder implementation at the test equipment side were identified [2].

Following the above principles, the considered options of test decoder are listed below - Option 1: DUT provides the decoder - Option 2: Infra vendor provides the decoder - Option 3: Full decoder specification in standard - Option 4: TE vendor provides the decoder

In the RAN4 #110 meeting, a decision was made to focus on establishing a standardized reference decoder for two-sided AI/ML models, effectively narrowing the scope to options 3 and 4 [4].

Issue 4-2: Testing options for 2-sided model - RAN4 to further discuss only options 3 and 4

In RAN4#110bis a discussion on the potential next steps took place and a high-level procedure was presented in [5] although not much technical progress was made.

In RAN4 #111 meeting and a subsequent email discussion the following progress was reached regarding the analysis of feasible testing options for two-sided AI-ML models:

- It was agreed that companies should present results for a pair of encoder/decoders under Option 3. From these, one decoder will be selected for further evaluation to determine if UE-vendor-specific encoders can be effectively trained to achieve comparable performance and complexity (or other evaluation criteria).

- The simulation parameters for assessing the feasibility of defining harmonized models across companies were established based on the assumptions made by RAN1 during the study item phase.

- For the initial feasibility analysis, RAN4 agreed to utilize simple AI/ML models (CNNs ResNet). However, for assessing performance requirement stage, the development of a new Test Decoder model is anticipated.

- The specific parameters for the simplified AI/ML model to be used for feasibility analysis were determined through an email discussion in June 2024.

## Inter-vendor training collaboration framework

The concurrent discussion with respect to inter-vendor training collaboration of AI/ML-based CSI compression is taking place in RAN4. In RAN1 #116 (Feb 2024) the following agreements were reached:

Agreement To alleviate / resolve the issues related to inter-vendor training collaboration of AI/ML-based CSI compression using two-sided model, study the following options: - Option 1: Fully standardized reference model (structure + parameters) - Option 2: Standardized dataset - Option 3: Standardized reference model structure + Parameter exchange between NW-side and UE-side - Option 4: Standardized data / dataset format + Dataset exchange between NW-side and UE-side - Option 5: Standardized model format + Reference model exchange between NW-side and UE-side Note 1: The above options may not be mutually exclusive and may be used together. Note 2: Other options are not precluded. Note 3: The study should consider how different methods of exchanging the parameters / dataset / reference model would affect the feasibility and collaboration complexity of options 3 / 4 / 5 respectively, e.g., over the air-interface, offline delivery, etc. Note 4: “Dataset” refers to a set of data samples of CSI feedback and associated target CSI.

In RAN1 #116bis (Apr 2024) and RAN1 #117 (May 2024) additional agreements on details of respective options were made, but no final conclusions were reached and almost all options are under consideration. As acknowledged in RAN1 discussion the RAN1 Option 1 closely resembles RAN4 Options 3/4, with the primary distinction being the potential inclusion of a reference encoder in RAN1 approach. If RAN1 Option 1 is adopted, then the respective models can be potentially reused for RAN4 testing. Meanwhile, RAN1 options 3-5 are reflecting a broader range of considerations and if any of the options are adopted, then additional RAN4 discussions may be required. RAN1 discussions on reference models may have significant implications for RAN4 work on defining reference decoder options and decisions made by RAN1 will influence the direction of RAN4 standardization efforts. Given the interdependency of RAN1/4 discussions, it is recommended for RAN4 to continue analyzing both Options 3 and 4 concurrently and wait for RAN1 conclusions on how to facilitate inter-vendor collaboration before deriving final conclusions. The respective proposal is in-line with the common understanding in RAN4 #110bis (“RAN4 continues to discuss Option 3 and 4 and can review/discuss RAN1 agreements on interoperability if there will be any”).

Proposal #1:	Further align the AI/ML test methodology with RAN1's conclusions on inter-vendor training collaboration for AI/ML-based CSI compression.

## Option 3: Fully standardized reference/test decoder

### Feasibility analysis procedure

The WFs in the previous RAN4 meetings do not describe the eventual procedure for Option 3 feasibility analysis, and we recommend to clearly outline it. For instance, the following steps are being considered:

- Agree on the simulation assumptions for dataset generation (completed)

- Agree on the AI-ML model assumptions for feasibility analysis (completed)

- Agree on the procedure for vendor-specific encoder training

- Agree on the framework/format for AI-ML model sharing among companies

- Agree on the framework for dataset sharing among companies including specific set of datasets to be used for encoder training

- Select one or more decoder models for further analysis

  - Companies bring simulation results for the pair of trained Encoder/Decoder models

    - Check and confirm simulation results alignment

    - If alignment is not achieved, agree on modified/additional simulation assumptions and/or model assumptions for the next round.

  - Select one or more decoder models for further analysis

  - Agreed models are shared with other companies based on the agreed framework/format

- Collect datasets for model training

- Check feasibility of Option 3 (i.e. to design/train UE-specific encoder based on the agreed common Decoder)

  - Agree on the criteria to confirm feasibility of Option 3

  - Companies bring simulation results for the pair of trained company-specific Encoder and common Decoder models

  - Check feasibility based on the agreed criteria

Proposal #2: 	Discuss and agree on the detailed procedure for Option 3 feasibility analysis.

### Simulation assumptions

In RAN4#111 and the subsequent email discussion the set of simulation assumptions were agreed. To facilitate further alignment of models it is recommended to focus on a limited subset of scenarios instead of proceeding with a multitude of scenarios:

- Number of MIMO layers: 1

- Frequency: 2GHz

- SCS/BW: 15kHz SCS + 10MHz BW

- Traffic load: 50%

Also, the selected parameters for CSI feedback and UE receiver are redundant and do not have impact on final results and suggested to be removed.

Table 1. Simulation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1 only, [2GHz, 4GHz] |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx power |  | 44dBm for 20MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | Baseline: 15kHz for 2GHz; Optional: 30kHz for 4GHz |
| Simulation bandwidth |  | Baseline: 10 MHz for 15kHz Optional: 20 MHz for 30kHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU-MIMO |
| MIMO layers |  | 1 layer (2 layers optional) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback): 5 ms (baseline) - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Traffic load (Resource utilization) |  | Baseline: 50% Optional: 20/70% |
| UE distribution |  | CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Ideal channel estimation |
| Sub-bands |  | 13 |
| Metrics |  | SGCS of AI CSI feedback and eTypeII CSI feedback, both relative to ideal CSI. |

Proposal #3: 	Update the simulation assumptions for Option 3 feasibility analysis based on Table 1.

### AI-ML model and training assumptions

The simplified model agreed during the June RAN4 email discussion is clear enough, although a few clarifications are suggested to ensure model compatibility across companies:

- Quantization/dequantization: It is important to align on the specifics of the scalar quantization and dequantization methods to achieve consistent models across companies. We recommend adopting scalar quantization/dequantization with predetermined levels within the [0.0, 1.0] range. Additionally, a sigmoid function is suggested to be applied prior to the quantization and an inverse sigmoid after the dequantization.

Based on our estimates the agreed models have the following complexity:

|  | Number of trainable parameters | MFLOPs |
| --- | --- | --- |
| Encoder | 125 793 | 46.62 |
| Decoder | 1 804 482 | 738.58 |

We acknowledge that the decoder model's complexity appears excessively high. Nevertheless, this should not have impact on the feasibility analysis, while a more detailed discussion on target complexity shall take place during the WI performance stage for the Test Decoder.

The previous meeting discussions briefly touched on training hyperparameters and assumptions, but not many agreements were reached. Further alignment may be required if a divergence of results is observed. For reference, the following training parameters were chosen for the analysis presented in this paper:

- Optimization algorithm: Adam

- Learning rate: 0.001 (fixed)

- Batch size: 10

- Regularization techniques: No

- Training metrics: SGCS

- Training dataset size: 200 000

- Validation dataset size: 40 000

- Layer-specific training is performed for each MIMO layer

### Simulation results

The modelling results for Mean SGCS for the agreed model for the test dataset are provided below. The results include:

- AI CSI feedback with RAN4 CNN model w/ 64 bits per layer

- eTypeII CSI feedback with 62 bits for 1 layer case and 113 bits for 2 layers

The table below provided the results for the average SGCS across all datasets and all subbands within the dataset, while the figures below provide a more details CDF statistics of the SGCS. The results show that SGCS for AI CSI feedback with RAN4 models is slightly better than eType II CSI feedback for layer 1 and layer 2. Given that average SGCS metric may not provide full insights on the SGCS statistics, it is recommended to additionally use CDF statistics (e.g., 5/50/95% points) for the model alignment and criteria for the feasibility analysis.

| Scenario | Mean SGCS |  |
| --- | --- | --- |
|  | AI CSI feedback (RAN4 model) | eTypeII CSI feedback |
| 2GHz / 15kHz SCS / 10MHz BW / layer 1 | 0.689 | 0.658 |
| 2GHz / 15kHz SCS / 10MHz BW / layer 2 | 0.517 | 0.512 |

### Training procedure for vendor-specific encoder

To advance the feasibility analysis for Option 3, once a common decoder model is selected, RAN4 must further clarify the assumptions regarding vendor-specific encoder training, as this could significantly influence the convergence process. The methodology employed by RAN4 for Option 3 mirrors the approach of independent training at the NW and UE sides, with a sequential training starting with NW side training (NW-first training) as described [2].

For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following procedure is considered for the sequential training starting with NW side training (NW-first training): - Step1: NW side trains the NW side CSI generation part (which is not used for inference) and the NW side CSI reconstruction part jointly - Step2: After NW side training is finished, NW side shares UE side with a set of information (e.g., dataset) that is used by the UE side to be able to train the UE side CSI generation part - Companies to report Dataset construction, e.g., the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only, or other information if applicable. Also report the Quantization behaviour, e.g., whether the shared output of the Network side CSI generation part is before or after quantization. - Step3: UE side trains the UE side CSI generation part based on the received set of information - Other Type 3 NW-first training approaches are not precluded

The following options are recommended for further discussion in RAN4:

- Option 1: Encoder training based on the encoder output / decoder input dataset. This method replicates the NW-first independent training protocol at NW side and UE side as described in [2] and would require further alignment on the corresponding dataset. The training process should aim to minimize the loss between the encoder outputs and the predetermined dataset (generated during the joint encoder/decoder training). The agreed Test Decoder may be excluded from the training process and is anticipated to be utilized for evaluating the effective SCGS metric after the Encoder training is concluded.

- Option 2: Encoder training based on the decoder output dataset. This training process includes both the Encoder and Decoder (without weight refinement), optimizing the loss function between the encoder inputs and decoder outputs. This approach might impose certain limitations on the model sharing methodology, and it is preferable that full model details be disclosed to guarantee an efficient training procedure.

Proposal #4: 	Discuss and agree on the detailed training procedure for vendor-specific encoder.

### Dataset and model sharing

For the feasibility analysis for Option 3, it is anticipated that companies will need to exchange specific trained decoder models and potentially the associated datasets. Therefore, further RAN4 agreements on the format of the shared information is required to facilitate further studies.

During RAN#104, preliminary discussions addressed the logistical considerations for model and dataset sharing among companies, including the potential use of MCC infrastructure or tools. These discussions, which continued informally via email reflector, are expected to progress at RAN4#112. The outcomes of these discussions are expected to be reported to RANP to support further establishing a common infrastructure for data sharing.

AI/ML models sharing

For the feasibility analysis to progress, it is essential for companies to be able to exchange and utilize pre-trained decoder models. To facilitate this, there must be an agreement on the procedure and format for sharing AI-ML models. This agreement should prioritize the ease of model sharing and, critically, the usability of the models by other companies. These approaches should be compatible with PyTorch and/or TensorFlow, which are the commonly used development tools among companies. The suggested approaches for further discussion are as follows (note that this list is not exhaustive, and additional approaches may be considered).

- Option 1: Share model parameters (weights/biases) in PyTorch or TensorFlow formats and source codes

  - Feasible with both PyTorch and TensorFlow. PyTorch allows the saving of a state dictionary containing the model parameters (PyTorch-specific checkpoint format), while TensorFlow permits the saving of model parameters as checkpoint files (HDF5 format).

  - Users will require the model's code to reconstruct the architecture before loading the weights, necessitating the sharing of the original code.

  - It is recommended to agree on a single development tool (PyTorch or TensorFlow) and align on the version of libraries to minimize compatibility issues.

  - This approach may reduce cross-platform portability issues.

- Option 2: Share entire models in PyTorch or TensorFlow formats (no source codes)

  - Entire models can be shared and used directly in their respective PyTorch or TensorFlow (SavedModel) formats, which include both the model architecture and the weights.

  - This method is the simplest for sharing models and does not require source code sharing. However, there may be portability issues between different platforms or versions, which may be difficult to overcome.

  - It is recommended to agree on a single development tool (PyTorch or TensorFlow) and align on the version of libraries to minimize compatibility issues.

- Option 3: Share models in ONNX (Open Neural Network Exchange) format (no source codes)

  - ONNX allows for the sharing of models across different frameworks that support ONNX, without the need for the original source codes.

  - This approach is applicable to both PyTorch and TensorFlow, and the models can be used in other deep learning frameworks that support ONNX (e.g., PyTorch, TensorFlow, MATLAB, OpenVINO).

  - Potential compatibility issues with ONNX may still arise and require further investigation.

  - ONNX models are typically used for inference only and do not support backpropagation directly.

The respective approaches may have certain implications on the vendor-specific encoder model training procedure and should be further assessed for feasibility.

Proposal #5:	Further investigate and select the approach to share AI-ML models for CSI Decoder including

  - Option 1: Share model parameters in PyTorch or TensorFlow formats and source codes

  - Option 2: Share entire models in PyTorch or TensorFlow formats (no source codes)

  - Option 3: Share models in ONNX format (no source codes)

Dataset sharing/alignment

Dataset alignment for AI-ML models training and testing to assess Option 3 feasibility and to define Test Decoder may be required. The following aspects may need further alignments:

- Simulation assumptions (preferable subset of agreed parameters)

- Set of shared datasets depending on the agreed encoder training procedure

- Data format

- Logistics of dataset sharing (file formats, storage)

- Principles to mix / select datasets

Proposal #6:	Further discuss and agree on the format for dataset sharing/alignment for Option 3 feasibility analysis.

## Models complexity

Due to a diverse nature of AI/ML models implementations, there may be a significant variation in their complexity and performance, which is illustrated by Rel-18/19 RAN1 studies. The summary AI/ML models used for RAN1 evaluations for 2-sided AI/ML models for CSI compressions is provided in TR 38.843 [2] with the complexity in the range from 10M to 1100M FLOPs and with 1M to 17M parameters for the decoder (CSI reconstruction) part:

The complexity metric in terms of FLOPs and number of parameters of AI/ML models adopted in the evaluations of CSI compression are summarized in Figure 6.2.2.1-1, where the complexity for the CSI generation part and the complexity for the CSI reconstruction part are illustrated separately. - A majority of 25 sources adopt the CSI generation model subject to the FLOPs from 10M to 800M, and 26 sources adopt the CSI reconstruction model subject to the FLOPs from 10M to 1100M. - A majority of 21 sources adopt the CSI generation model subject to the number of parameters from 1M to 13M, and 22 sources adopt the CSI reconstruction model subject to the number of parameters from 1M to 17M. - Results refer to Table 1 of clause 7.3, R1-2310450. Figure 6.2.2.1-1: Complexity of AI/ML models from evaluation results in terms of FLOPs and number of parameters for CSI compression

In the figure below we illustrate the dependency of SGCS performance depending on the AI/ML model complexity based on results in [7]. It can be observed that AI/ML model complexity has a direct impact on the overall performance and that the complexity of the methods may substantially exceed the complexity of legacy PMI-based CSI feedback schemes.

Figure 1: SGCS and complexity for eType II PMI codebook and different transformer-based AI/ML models.

The RAN4 reference models for testing and performance requirements definition are expected to set the basis for both UE and NW implementations and serve as reference decoders for performance requirements definition. Therefore, RAN4 needs to further discuss on associated complexity for Options 3/4 considering RAN1 analysis on the performance as well as practical considerations on the algorithm implementation complexity including computational complexity and memory consumption. Additional RAN4 analysis on complexity should be performed to ensure that reference models represent practical implementations in terms of complexity and power consumption. The respective bounds on the maximum complexity shall be considered for both Option 3 and Option 4 methods.

Proposal #7:	Further discuss and define the upper bound complexity including the number of computations and number of parameters for test/reference encoders/decoders for the purpose of performance requirements definition.

# Conclusion

In this paper we provide our views on general aspects of AI/ML testability and interoperability aspects, and, in summary, make the following proposals:

Proposal #1:	Further align the AI/ML test methodology with RAN1's conclusions on inter-vendor training collaboration for AI/ML-based CSI compression.

Proposal #2: 	Discuss and agree on the detailed procedure for Option 3 feasibility analysis.

Proposal #3: 	Update the simulation assumptions for Option 3 feasibility analysis based on Table 1.

Proposal #4: 	Discuss and agree on the detailed training procedure for vendor-specific encoder.

Proposal #5:	Further investigate and select the approach to share AI-ML models for CSI Decoder including

  - Option 1: Share model parameters in PyTorch or TensorFlow formats and source codes

  - Option 2: Share entire models in PyTorch or TensorFlow formats (no source codes)

  - Option 3: Share models in ONNX format (no source codes)

Proposal #6:	Further discuss and agree on the format for dataset sharing/alignment for Option 3 feasibility analysis.

Proposal #7:	Further discuss and define the upper bound complexity including the number of computations and number of parameters for test/reference encoders/decoders for the purpose of performance requirements definition.

# References

- RP-221348, “SID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN #96, June 2022

- 3GPP TR 38.843 “Study on Artificial Intelligence (AI) / Machine Learning (ML) for NR air interface”

- RP-234039, “WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN #102, December 2023

- R4-2403712, “WF on AI/ML”, Qualcomm, RAN4 #110, February 2024

- R4-2406617, “WF on AI/ML”, Qualcomm, RAN4 #110bis, April 2024

- R4-2410570, “WF on AI/ML”, Qualcomm, RAN4 #111, May 2024

- R1-2402147, “AI/ML for CSI compression”, Intel Corporation, RAN1 #116bis, April 2024