3GPP TSG-WG4 Meeting #112	R4-2412334

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	NEC

Title: 	Discussion on ACLR requirements for NCR

Agenda Item:		5.31.1	RF core requirements 	[NR_netcon_repeater-Core]

Document for:	discussion

Introduction

This contribution discusses the issues on ACLR requirements for NCR and provides proposals to solve the issues.

Discussion

ACLR requirements for repeater are specified using nominal channel bandwidth BWNominal, and transmission bandwidth configuration BWConfig.

Example of the ACLR limit is shown in table 1 below.

Table 1: ACLR limit for DL (all repeater classes) and for UL for Wide Area class (Table 6.5.2.2-1 in TS 38.106)

| nominal channel bandwidth BWNominal (MHz) (NOTE 5) | Repeater type 1-C adjacent channel centre frequency offset below or above the passband edge | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | BWNominal/2 | NR of same BW (Note 2) | Square (BWConfig) | 45 dB 38 dB (Note 4) |
|  | 1.5 x BWNominal | NR of same BW (Note 2) | Square (BWConfig) | 45 dB 38 dB (Note 4) |
|  | 2.5 MHz | 5 MHz E-UTRA | Square (4.5 MHz) | 45 dB (Note 3) |
|  | 7.5 MHz | 5 MHz E-UTRA | Square (4.5 MHz) | 45 dB (Note 3) |
| NOTE 1: BWNominal is the nominal channel bandwidth. BWConfig is the transmission bandwidth configuration assumed for the adjacent channel. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: The requirements are applicable when the band is also defined for E-UTRA or UTRA. NOTE 4: For repeater operating in band n104, ACLR requirement 38 dB applies. For repeater operating in other bands, ACLR requirement 45 dB applies. NOTE 5: For simultaneous NCR-Fwd and NCR-MT transmission, if the NCR-MT carrier is within the NCR-Fwd then the nominal bandwidth shall be the NCR-Fwd passband. If the NCT-MT carrier is adjacent to the NCR-Fwd passband then the nominal bandwidth shall be the combined bandwidth of NCR-Fwd passband and NCR-MT carrier bandwidth. If the NCR-MT carrier is not adjacent to the passband then CACLR shall be applied in the gap between the NCR-Fwd passband and the NCR-MT carrier. |  |  |  |  |

NOTE 5 in table 1 specifies the nominal channel bandwidth for the simultaneous NCR-Fwd and NCR-MT transmission. Three cases of the simultaneous NCR-Fwd and NCR-MT transmission are considered. (“passband” is indicated as “NCR-Fwd”, “NCR-Fwd passband”, or “passband” in NOTE 5)

- The NCR-MT carrier is within the passband

- The NCR-MT carrier is adjacent to the passband

- The NCR-MT carrier is not adjacent to the passband

Case 1, NCR-MT carrier is within the passband: The nominal channel bandwidth is defined as the passband bandwidth by the NOTE. We do not think this definition works when the passband bandwidth is not same as a bandwidth defined as a BS channel bandwidth. As there is no restriction on the passband bandwidth, passband bandwidth can have any value including ones not defined as a BS channel bandwidth. If the passband bandwidth is not defined for the BS channel bandwidth, the nominal channel bandwidth is also not defined as the BS channel bandwidth. For example, if the passband bandwidth is 75 MHz, the nominal channel bandwidth is also 75 MHz. In table 1, we cannot find the ACLR limit for the nominal channel bandwidth of 75 MHz. It means ACLR requirement is not defined in this case.

Even if the passband bandwidth is defined as a BS channel bandwidth, we still see an issue to apply the passband bandwidth as the nominal channel bandwidth. If the passband bandwidth is adopted as the nominal channel bandwidth, it means a carrier occupying the passband is assumed. However, we do not think it is a realistic assumption. As we are considering the nominal channel bandwidth under the simultaneous NCR-Fwd and NCR-MT transmission, if a carrier with its channel bandwidth equal to the passband exists the NCR-Fwd carrier and the NCR-MT carrier interfere with each other regardless of the position of the NCR-MT carrier in the passband. When NCR-MT carrier is located within the passband, we think NCR-Fwd carrier shall be located where NCR-MT carrier does not exist.

For ACLR requirement, in general, bandwidth of the outermost carrier is taken in account for multicarrier cases, not the sum of the bandwidths. It shall be same for ACLR requirement for the simultaneous transmission case. In this case, the nominal channel bandwidth shall be the possible widest channel bandwidth at the edge of the passband. For this reason, the nominal channel bandwidth for upper side and lower side of the passband may be different.

Now, NCR knows the position of the NCR-MT carrier because NCR-MT carrier is allocated through negotiation between BS and NCR.

In case the NCR-MT carrier is located at the lower edge of the passband, the nominal channel bandwidth for the lower side shall be same as the NCR-MT carrier bandwidth, and the nominal channel bandwidth for the upper side shall be determined based on the remaining bandwidth of the higher frequency region above the NCR-MT carrier in the passband where NCR-Fwd carrier can be allocated (=BWpassband_high).

Similarly, in case the NCR-MT carrier is located at the middle of the passband, the nominal channel bandwidth for the lower side shall be determined based on the passband bandwidth below the NCR-MT carrier (=BWpassband_low), and the nominal channel bandwidth for the higher side shall be determined based on the remaining passband bandwidth above the NCR-MT carrier (=BWpassband_high). The way to derive the nominal channel bandwidth is same as the general case.

In general cases, the nominal channel bandwidth is defined as,

Nominal channel bandwidth: Bandwidth calculated as min(100MHz, BWpassband) in FR1 or min(400MHz, BWpassband) in FR2. If this bandwidth is not defined for BS channel bandwidth for the operating band, nominal channel bandwidth shall be defined as the widest BS channel bandwidth for the operating band which is narrower than BWpassband.

We could derive the nominal channel bandwidth for the simultaneous transmission by replacing BWpassband above with BWpassband_low or BWpassband_high.

Fig 1 shows an example, where passband bandwidth is 75 MHz, NCR-MT carrier channel bandwidth is 20 MHz, passband bandwidth below the NCR-MT carrier (BWpassband_low) is 35 MHz, passband bandwidth above the NCR-MT carrier (BWpassband_high) is 20 MHz. NCR-Fwd carrier can exist only in the frequency region below or above the NCR-MT carrier, coloured yellow in fig.1. The nominal channel bandwidth shall be determine based on the bandwidth of these regions. For the lower frequency side, the bandwidth of the region where NCR-Fwd carrier can exist  (BWpassband_low) is 35 MHz. The nominal channel bandwidth for the lower frequency side can be derived by regarding BWpassband_low as BWpassband in the nominal channel bandwidth calculation. If we assume operating band n1, because 35 MHz is not defined as a BS channel bandwidth and the widest BS channel bandwidth narrower than 35 MHz in n1 is 30 MHz, the nominal channel bandwidth for the lower frequency side is calculated as 30 MHz. Similarly, for the upper frequency side, BWpassband_high is 20MHz and this bandwidth is defined as a BS channel bandwidth in n1, the nominal channel bandwidth for the upper side can be calculated as 20 MHz.

Fig. 1 Example of BWnominal when NCR-MT carrier is within the passband

In case 2, the NCR-MT carrier is adjacent to the passband when the NCR-MT carrier is adjacent to the passband: The nominal channel bandwidth is the combined bandwidth of passband and NCR-MT carrier bandwidth by the NOTE. Same issue as for case 1 may happen. That is, combined bandwidth may not be defined as the BS channel bandwidth. It means ACLR requirement may not be specified. This case is relatively simpler. The outermost carrier of the NCR-MT carrier side is the NCR-MT carrier. Therefore, ACLR requirements for NCR-MT shall be adopted for this side. For the other side exists the passband. For this side, we do not see any difference from the non-simultaneous transmission case. The nominal channel bandwidth calculated with the passband bandwidth shall be used. Fig.2 below shows an example where NCR-MT carrier of 20 MHz is located adjacent to the passband of 75 MHz. For the NCR-MT carrier side, assumed bandwidth of the adjacent carrier shall be same as NCR-MT carrier bandwidth, 20 MHz. For the passband side, the nominal channel bandwidth shall be 50 MHz if we assume operating band n1 because the widest channel bandwidth defined for band n1 is 50 MHz.

Fig. 2 Example of BWnominal when NCR-MT carrier is adjacent to the passband

In case 3, the NCR-MT carrier is not adjacent to the passband: CACLR is applied in the gap between the passband and the NCR-MT carrier by the NOTE. We are ok to apply CACLR in the gap between the passband and the NCR-MT carrier.

In summary, ACLR requirement may not be defined for simultaneous NCR-Fwd and NCR-MT transmission cases when NCR-MT carrier is within the passband or NCR-MT carrier is adjacent to the passband. To combat with this issue, the way to derive the nominal channel bandwidth for the simultaneous NCR-Fwd and NCR-MT transmission shall be modified. The nominal channel bandwidth shall be determined by the possible widest channel bandwidth of the NCR-Fwd carrier. Therefore, following proposal is formulated.

Proposal 1:

To modify the NOTE for the simultaneous NCR-Fwd and NCR-MT transmission in the ACLR requirement tables as blow.

“For simultaneous NCR-Fwd and NCR-MT transmission, if the NCR-MT carrier is within the passband then the nominal channel bandwidth shall be calculated based on the BWpassband_low or BWpassband_high, where BWpassband_low and BWpassband_high are the passband bandwidth below and above the NCR-MT carrier, respectively. If the NCT-MT carrier is adjacent to the passband then ACLR requirement for NCR-MT shall be applied for the NCR-MT carrier side and the nominal channel bandwidth calculated with the passband bandwidth shall be used for the passband side. If the NCR-MT carrier is not adjacent to the passband then CACLR shall be applied in the gap between the passband and the NCR-MT carrier.”

Conclusions

We discussed the issues of the NCR ACLR requirement for the simultaneous NCR-Fwd and NCR-MT transmission and provided the following proposal.

Proposal 1:

To modify the NOTE for the simultaneous NCR-Fwd and NCR-MT transmission in the ACLR requirement tables as blow.

“For simultaneous NCR-Fwd and NCR-MT transmission, if the NCR-MT carrier is within the passband then the nominal channel bandwidth shall be calculated based on the BWpassband_low or BWpassband_high, where BWpassband_low and BWpassband_high are the passband bandwidth below and above the NCR-MT carrier, respectively. If the NCT-MT carrier is adjacent to the passband then ACLR requirement for NCR-MT shall be applied for the NCR-MT carrier side and the nominal channel bandwidth calculated with the passband bandwidth shall be used for the passband side. If the NCR-MT carrier is not adjacent to the passband then CACLR shall be applied in the gap between the passband and the NCR-MT carrier.”