3GPP TSG-WG4 Meeting #112	R4-2412342

# Maastricht, Netherlands, 19th – 23rd August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-1 | CR | 0464 | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_bands_R17_BWs-Core) CR to 38.141-1: Correction of regional requirement table |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_bands_R17_BWs-Core |  |  |  |  |  | Date: |  |  | 2024-8-9 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the previous meeting, CRs for Tx intremodulation requirements in band n79 were agreed. However, cat-A CRs were not identical to cat-F CRs and they contained errors. Regional requirement table in rel-18 says tx inter modulation requirements for n79 in Japan apply when the narrowest channel bandwidth supported by the BS is 40MHz or wider. However, this condition cannot be seen in the main contents. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove “when the narrowest channel bandwidth supported by the BS is 40MHz or wider”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Comment in the regional requirements table remains not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.104. |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.141-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------Start of change-------------

4.4	Regional requirements

Some requirements in the present document may only apply in certain regions either as optional requirements, or as mandatory requirements set by local and regional regulation. It is normally not stated in the 3GPP specifications under what exact circumstances the regional requirements apply, since this is defined by local or regional regulation.

Table 4.4-1 lists all requirements in the present specification that may be applied differently in different regions.

Table 4.4-1: List of regional requirements

| Clause number | Requirement | Comments |
| --- | --- | --- |
| 5 | Operating bands | Some NR operating bands may be applied regionally. |
| 6.2.1 | Base station output power | Additional output power limits may be applied regionally. |
| 6.6.2 | Occupied bandwidth | The requirement may be applied regionally. There may also be regional requirements to declare the occupied bandwidth according to the definition in present specification. |
| 6.6.3.5.3 | Adjacent Channel Leakage Power Ratio | For Band n41 and n90 operation in Japan, absolute ACLR limits shall be applied to the sum of the absolute ACLR power over all antenna connectors for BS type 1-C. |
| 6.6.4.5 | Operating band unwanted emission | Category A or Category B operating band unwanted emission limits may be applied regionally. For operation with shared spectrum channel access, the BS may have to comply with the applicable BS power limits established regionally, when deployed in regions where those limits apply and under the conditions declared by the manufacturer. |
| 6.6.4.5.6.1 | Operating band unwanted emissions: Limits in FCC Title 47 | The BS may have to comply with the additional requirements, when deployed in regions where those limits are applied, and under the conditions declared by the manufacturer. |
| 6.6.4.5.6.2 | Operating band unwanted emission Protection of DTT | The BS operating in Band n20 may have to comply with the additional requirements for protection of DTT, when deployed in certain regions. |
| 6.6.4.5.6.5 | Operating band unwanted emissions Additional requirements for n24 | The BS operating in Band n24 may have to comply with the additional requirements when deployed in regions where FCC regulation applies. |
| 6.6.4.5.7 | Operating band unwanted emission, | For Band n41 and n90 operation in Japan, the operating band unwanted emissions limits shall be applied to the sum of the emission power over all antenna connectors for BS type 1-C. |
| 6.6.5.5.1.1 | Transmitter spurious emissions | Category A or Category B spurious emission limits, as defined in ITU-R Recommendation SM.329 [5], may apply regionally. The emission limits for BS type 1-H specified as the basic limit + X (dB) are applicable, unless stated differently in regional regulation. In addition, for operation with shared spectrum channel access, the BS may have to comply with the applicable spurious emission limits established regionally, when deployed in regions where those limits apply and under the conditions declared by the manufacturer. |
| 6.6.5.5.1.3 | Transmitter spurious emissions: additional requirements | These requirements may be applied for the protection of system operating in frequency ranges other than the BS operating band. |
| 6.6.5.5.3 | Transmitter spurious emissions | For Band n41 and n90 operation in Japan, the sum of the spurious emissions over all antenna connectors for BS type 1-C shall not exceed the basic limits. |
| 6.7.5.1.1, 6.7.5.2.1 | Transmitter intermodulation | Interfering signal positions that are partially or completely outside of any downlink operating band of the base station are not excluded from the requirement in Japan in Band n77, n78, n79. |
| 6.7.5.1.2, 6.7.5.2.3 | Transmitter intermodulation | The BS may have to comply with the additional requirements, when deployed in certain regions. |
| 6.7.5.1.2, 6.7.5.2.3 | Transmitter intermodulation | For Band n79 operation in Japan, the BS shall comply with the additional requirements. |
| 6.7.5.1.2, 6.7.5.2.3 | Transmitter intermodulation | For Band n26 and n28 operation in Japan, the BS shall comply with the additional requirements when the narrowest channel bandwidth supported by the BS is 5MHz or wider, |
| 7.6.5.2, 7.6.5.3 | Receiver spurious emissions | The emission limits for BS type 1-H specified as the basic limit + X (dB) are applicable, unless stated differently in regional regulation. For Band n41 and n90 operation in Japan, the sum of RX spurious emissions over all antenna connectors for BS type 1-C shall not exceed basic limits |

--------------End of change-------------