3GPP TSG-WG4 Meeting #112	R4-2412344

# Maastricht, Netherlands, 19th – 23rd August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.176-2 | CR | 0060 | rev |  | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_IAB-Perf) CR to 38.176-2: Correction of ACLR absolute limit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Perf |  |  |  |  |  | Date: |  |  | 2024-8-9 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Note in ACLR absolute limit table that tells scaling factor and test tolerant applied in test requirements is not required. Similar note is not seen in other requirement tables. Notation of IAB-MT ACLR absolute limit is derived from (basic limit + 9) -9 +10log10(NTXU,OTApercell). Simpler notation is required. MR IAB-DU limit is not applicable to IAB-MT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the note 1 in ACLR absolute limit tables. Modify the ACLR absolute limit notation and the scaling factor. Change the MR IAB-DU ACLR absolute limit for IAB-MT to “NA”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Notation of ACLR absolute limit is confusing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.3.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------Start of change-------------

6.7.3.5.1	IAB-DU type 1-O and IAB-MT type 1-O

For the OTA ACLR requirement either the OTA ACLR limits in tables 6.7.3.5.1-1/3 or the OTA ACLR absolute limits in table 6.7.3.5.1-2 shall apply, whichever is less stringent. The OTA CACLR limits in table 6.7.3.5.1-4 or the OTA CACLR absolute limits in table 6.7.3.5.1-5 shall apply, whichever is less stringent.

The CACLR in a sub-block gap and Inter RF Bandwidth gap is the ratio of:

a)	the sum of the filtered mean power centred on the assigned channel frequencies for the two carriers adjacent to each side of the sub-block gap or the Inter RF Bandwidth gap, and

b)	the filtered mean power centred on a frequency channel adjacent to one of the respective sub-block edges or Base Station RF Bandwidth edges.

The assumed filter for the adjacent channel frequency is defined in table 6.7.3.5.1-4 and the filters on the assigned channels are defined in table 6.7.3.5.1-6.

For operation in paired and unpaired spectrum, the OTA ACLR measurement result shall not be less than the OTA ACLR limit specified in table 6.7.3.5.1-1.

Table 6.7.3.5.1-1: IAB-DU and IAB-MT type 1-O ACLR limit

| IAB channel bandwidth of lowest/highest NR carrier transmitted BWChannel (MHz) | IAB adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA ACLR limit (0 – 3 GHz) | OTA ACLR limit (3 – 6 GHz) |
| --- | --- | --- | --- | --- | --- |
| 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90,100 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
|  | 2 x BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
|  | BWChannel /2 + 2.5 MHz | 5 MHz E-UTRA | Square (4.5 MHz) | 44 dB (Note 3) | 43.8 dB (Note 3) |
|  | BWChannel /2 + 7.5 MHz | 5 MHz E-UTRA | Square (4.5 MHz) | 44 dB (Note 3) | 43.8 dB (Note 3) |
| NOTE 1: BWChannel and BWConfig are the IAB channel bandwidth and transmission bandwidth configuration of the lowest/highest NR carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: The requirements are applicable when the band is also defined for E-UTRA or UTRA. |  |  |  |  |  |

The absolute total power measurement shall not exceed the OTA ACLR absolute limit specified in table 6.7.3.5.1-2.

Table 6.7.3.5.1-2: IAB-DU and IAB-MT type 1-O ACLR absolute limit

| IAB category / IAB class | IAB-DU type 1-O OTA ACLR absolute limit | IAB-MT type 1-O OTA ACLR absolute limit (Note 2) |
| --- | --- | --- |
| Category A Wide Area IAB-DU and Category A Wide Area IAB-MT | -4 dBm/MHz | -13 + Y dBm/MHz |
| Category B Wide Area IAB-DU and Category B Wide Area IAB-MT | -6 dBm/MHz | -15 + Y dBm/MHz |
| Medium Range IAB-DU | -16 dBm/MHz | NA |
| Local Area IAB-DU and Local Area IAB-MT | -23 dBm/MHz | -32 + Y dBm/MHz |
| NOTE 1: Void NOTE 2: Y = 10log10(NTXU,OTApercell) dB |  |  |

--------------Next change-------------

The absolute total power measurement shall not exceed the OTA CACLR absolute limit specified in table 6.7.3.5.1-5.

Table 6.7.3.5.1-5: IAB-DU and IAB-MT type 1-O CACLR absolute limit

| IAB category / IAB class | IAB-DU type 1-O OTA CACLR absolute limit | IAB-MT type 1-O OTA CACLR absolute limit (Note 2) |
| --- | --- | --- |
| Category A Wide Area IAB-DU and Category A Wide Area IAB-MT | -4 dBm/MHz | -13 + Y dBm/MHz |
| Category B Wide Area IAB-DU and Category B Wide Area IAB-MT | -6 dBm/MHz | -15 + Y dBm/MHz |
| Medium Range IAB-DU | -16 dBm/MHz | NA |
| Local Area IAB-DU and Local Area IAB-MT | -23 dBm/MHz | -32 + Y dBm/MHz |
| NOTE 1: Void NOTE 2: Y = 10log10(NTXU,OTApercell) dB |  |  |

--------------End of change-------------