3GPP TSG-RAN WG4 Meeting #112	R4-2412348
Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	R19 3Tx single band

Source:	OPPO

Agenda item:	8.18.2

Document for:	Approval

- Introduction

3Tx for inter-band CA/ENDC has been introduced in Rel-18, and 3Layer UL MIMO is part of the MIMO enhancements in Rel-19 [1]. RAN4 need to consider specify requirements for single band 3Tx in Rel-19, and this paper share views on this aspect.

- Discussion

2.1 Objectives of 3Tx

In the WID [1], it mainly focuses on specifying 3port codebook in RAN1, and not enhance the ULFPTx and SRS antenna switching.

However, in RAN4#110bis meeting, RAN4 has sent LS [2] to RAN1 to ask them to consider introducing 4T6R and 3T6R SRS antenna switching in Rel-19 since this is part of the objectives in RAN4 lead UE RF enhancement WI.

Then in June RAN plenary, this was jointly discussed in RAN1 and RAN4, and it was agreed to study and specify 3T6R SRS antenna switching in RAN1 [3].

- RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. Note 1: SRS antenna switching to support 4T6R is precluded. Note 2: RAN1 discussion starts from RAN1#118bis (Oct. 2024)

Therefore, for RAN4, at least 3Layer UL MIMO and 3Tx based SRS antenna switching need to be specified, but considering 3T6R has been part of UE RF enhancement WI, then in this MIMO WI, it can focus on specifying 3Layer UL MIMO.

Observation 1:   The functionality of 3port codebook-based UL MIMO and 3T6R SRS antenna switching will be specified in RAN1.

Observation 2:   3T6R SRS antenna switching has been covered in UE RF enhancement and this WI can focus on specifying 3Layer UL MIMO related requirements.

Proposal 1:         Specify 3Layer UL MIMO related requirements in this WI, leave SRS antenna switching requirements to UE RF enhancement WI, and not specifying 3Tx based ULFPTx requirements.

2.2 Single band 3Tx

Single band 3Tx can have two operating modes, one is 3Layer, the other is 3TxD. Usually these two modes are needed in the spec to keep the feature as a whole. And how UL MIMO and TxD works also connected with the targeted power class, especially PC2 or PC1.5.

Based on the status of Rel-18 inter-band 3Tx, it seems PC1.5 is the most interested power class, then PC2, and PC3 is keep jut to make the spec looks complete. For this single band 3Tx, it can consider PC1.5 as the high priority and see whether there is interest for PC2 and PC3.

Observation 3:   During Rel-18 inter-band 3Tx, PC1.5 is the most interested power class, then PC2, and PC3 is keep jut to make the spec looks complete.

Proposal 2:         Specify 3Layer UL MIMO with PC1.5 as high priority in this WI, and check the interests for other power classes like PC2 and PC3.

- Conclusion

This paper shared general considerations for single band 3Tx in MIMO enhancement WI and got below observations and proposals.

Observation 1:   The functionality of 3port codebook-based UL MIMO and 3T6R SRS antenna switching will be specified in RAN1.

Observation 2:   3T6R SRS antenna switching has been covered in UE RF enhancement and this WI can focus on specifying 3Layer UL MIMO related requirements.

Proposal 1:         Specify 3Layer UL MIMO related requirements in this WI, leave SRS antenna switching requirements to UE RF enhancement WI, and not specifying 3Tx based ULFPTx requirements.

Observation 3:   During Rel-18 inter-band 3Tx, PC1.5 is the most interested power class, then PC2, and PC3 is keep jut to make the spec looks complete.

Proposal 2:         Specify 3Layer UL MIMO with PC1.5 as high priority in this WI, and check the interests for other power classes like PC2 and PC3.

- Reference

[1] RP-234007 New WID: NR MIMO Phase 5, Samsung, RAN#102 2023-12

[2] R4-2406718 LS on 3T6R and 4T6R antenna switching SRS, R4->R1, RAN4#110bis 2024-04

[3] RP-241621 Summary of offline discussion on 3T6R, CMCC, RAN#104 2024-06