3GPP TSG-RAN WG4 Meeting #112	R4-2412349
Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	R19 MPR for PC1.5 contiguous CA

Source:	OPPO

Agenda item:	8.1.1.1.1

Document for:	Approval

- Introduction

This paper discusses the Rel-19 PC1.5 contiguous CA MPR. It clarifies the applicable emission requirements like EVM/IBE and would like to align the assumptions in the simulation/measurements, then it shares some MPR simulation results with two PC2 PAs and TxD capability.

- Discussion

2.1 Clarification of requirements

2.1.1 CA EVM

In 38.101-1, the EVM is defined as below figure 1 for contiguous UL CA, which says “Requirements only apply with PRB allocation in one of the component carriers”. If understood correctly, it means when PRBs are configured in both CCs then EVM requirements will not be verified. Similar situation is in NC CA of 38.101-1.

Figure 1 EVM of contiguous UL CA

Observation 1:   For contiguous CA, the EVM requirements only defined for the case of PRB configured in one of the CCs. And no EVM requirements apply when both CCs are configured with PRBs.

2.1.2 CA IBE

In 38.101-1, the IBE is defined as below figure 2 for contiguous UL CA, which says “both component carriers active and one single contiguous PRB allocation of bandwidth LCRB at the edge of aggregated transmission BW configuration”.  And the LCRB is defined in NOTE 6/7, which seems the CA IBE is only verified when PRB is configured in one CC as illustrated in figure 3.

If this is correct understanding, then it means when PRBs are configured in both CCs the IBE requirements will not be verified.

Figure 2 Contiguous CA IBE requirements

Figure 3 PRB configurations for Contiguous CA IBE requirements

Observation 2:   For contiguous CA, the IBE requirements only defined for the case of PRB configured in one of the CCs. And no IBE requirements apply when both CCs are configured with PRBs.

2.1.3 CA MPR

If we look at the CA MPR tables for PC2 (figure 4), the MPR for different modulations are different in the inner region where the IBE/EVM are usually gating factors. It seems the EVM/IBE requirements are verified when defining MPRs for “contiguous RB allocation” and “non-contiguous RB allocation”.

Figure 4 CA MPR tables

However, according to the definition of non-Contiguous RB allocation, both CCs are allocated with PRBs. If the EVM/IBEs are verified when defining MPRs, it seems this is contradicting with the CA EVM/IBE requirement definition?

Figure 5 Non-Contiguous RB allocation definition

Observation 3:   It is unclear when defining MPR requirements for CA (especially non-contiguous RB allocations), the EVM/IBE are verified or not with current spec.

And if the MPR difference among modulations in the contiguous RB allocation comes from the singe CC RB allocation case, then make it clear that IBE/EVM are not verified in the MPR simulations/tests.

Proposal 1:         To align the intra-band CA MPR simulation/test configurations, clarify whether the EVM/IBE requirements are verified when both CCs are configured with PRBs?

  - Non-Contig RB allocation MRP

2.2.1 Initial MRP results

With the understanding of EVM / IBE requirements are NOT verified when both CCs are configured with PRBs, below tables give the initial simulation results for alignment purpose. It includes CP-OFDM and DFT-S-OFDM with Class B and Class C and PRB configured in both CCs.

And it should be noticed that in the simulation TxD/1LO is used to achieve PC1.5, but the reverse IMD impacts are not accounted in the simulation, and some additional values need to be added to cope with it in the final MPR results.

The configurations are:

- CA class B or C

- CA power class PC1.5

- Two PC2 PA

- CP-OFDM /DFT-S-OFDM

- scs is 15khz or 30khz

- LO number is 1LO+TxD

- Modulations are same in both CCs

Table 1 CP-OFDM non-contig RB allocation: CA Class B and Class C

| CP-OFDM: CA Class B |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CBW1 | CBW2 | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | Region | MPR (dB) |  |  |  |  |
|  |  |  |  |  |  |  |  | pi/2 bpsk | qpsk | 16qam | 64qam | 256qam |
| 20 | 20 | 1 | 73 | 1 | 32 | 15khz | inner | 1.8 | 1.9 | 1.7 | 1.7 | 1.6 |
| 20 | 20 | 1 | 72 | 1 | 33 | 15khz | inner | 1.8 | 1.9 | 1.7 | 1.8 | 1.6 |
| 20 | 20 | 1 | 56 | 1 | 1 | 15khz | inner | 2.2 | 2.3 | 2.2 | 2.2 | 2.0 |
| 20 | 20 | 1 | 55 | 1 | 1 | 15khz | inner | 1.9 | 2.0 | 1.8 | 1.9 | 1.7 |
| 50 | 50 | 1 | 183 | 1 | 86 | 15khz | inner | 1.8 | 1.9 | 1.7 | 1.7 | 1.6 |
| 50 | 50 | 1 | 139 | 1 | 1 | 15khz | inner | 2.0 | 2.1 | 1.9 | 2.0 | 1.7 |
| 50 | 50 | 1 | 138 | 1 | 1 | 15khz | inner | 1.0 | 0.9 | 0.8 | 1.3 | 1.0 |
| Inner Max MPR |  |  |  |  |  |  |  | 2.2 | 2.3 | 2.2 | 2.2 | 2.0 |
| 20 | 20 | 1 | 72 | 1 | 50 | 15khz | outer1 | 4.1 | 4.2 | 4.0 | 4.4 | 4.2 |
| 20 | 20 | 1 | 54 | 1 | 50 | 15khz | outer1 | 4.9 | 5.0 | 4.6 | 5.2 | 4.9 |
| 20 | 20 | 1 | 42 | 1 | 63 | 15khz | outer1 | 4.9 | 5.0 | 4.6 | 5.2 | 4.9 |
| 20 | 20 | 1 | 1 | 1 | 1 | 15khz | outer1 | 4.7 | 4.8 | 4.5 | 5.2 | 4.9 |
| 50 | 50 | 1 | 137 | 1 | 60 | 15khz | outer1 | 4.9 | 4.8 | 4.6 | 5.2 | 4.9 |
| 50 | 50 | 1 | 137 | 1 | 20 | 15khz | outer1 | 4.1 | 4.1 | 4.0 | 4.4 | 4.2 |
| 50 | 50 | 1 | 109 | 1 | 160 | 15khz | outer1 | 4.9 | 5.0 | 4.6 | 5.2 | 4.9 |
| 50 | 50 | 1 | 108 | 1 | 161 | 15khz | outer1 | 4.6 | 4.7 | 4.3 | 5.0 | 4.7 |
| 50 | 50 | 1 | 1 | 1 | 1 | 15khz | outer1 | 5.1 | 5.0 | 4.8 | 5.4 | 5.2 |
| Outer1 Max MPR |  |  |  |  |  |  |  | 5.1 | 5.0 | 4.8 | 5.4 | 5.2 |
| 20 | 20 | 1 | 40 | 1 | 65 | 15khz | outer2 | 6.8 | 6.8 | 6.6 | 7.3 | 6.8 |
| 20 | 20 | 1 | 1 | 1 | 105 | 15khz | outer2 | 4.9 | 4.8 | 4.7 | 5.2 | 5.1 |
| 20 | 20 | 1 | 1 | 1 | 80 | 15khz | outer2 | 4.8 | 4.7 | 4.5 | 5.1 | 4.8 |
| 20 | 20 | 1 | 41 | 1 | 64 | 15khz | outer2 | 6.8 | 6.6 | 6.6 | 7.1 | 6.8 |
| 50 | 50 | 1 | 50 | 1 | 162 | 15khz | outer2 | 4.9 | 5.0 | 4.6 | 5.2 | 4.9 |
| 50 | 50 | 1 | 1 | 1 | 269 | 15khz | outer2 | 4.5 | 4.6 | 4.4 | 5.0 | 4.7 |
| 50 | 50 | 1 | 1 | 1 | 80 | 15khz | outer2 | 4.5 | 4.6 | 4.4 | 5.0 | 4.7 |
| Outer2 Max MPR |  |  |  |  |  |  |  | 6.8 | 6.8 | 6.6 | 7.3 | 6.8 |
| CP-OFDM: CA Class C |  |  |  |  |  |  |  |  |  |  |  |  |
| CBW1 | CBW2 | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | Region | MPR (dB) |  |  |  |  |
|  |  |  |  |  |  |  |  | pi/2 bpsk | qpsk | 16qam | 64qam | 256qam |
| 60 | 100 | 1 | 110 | 1 | 51 | 30khz | inner | 0.6 | 0.5 | 0.3 | 0.9 | 0.7 |
| 60 | 100 | 1 | 109 | 1 | 52 | 30khz | inner | 1.3 | 1.2 | 1.0 | 1.4 | 1.2 |
| 60 | 100 | 1 | 84 | 1 | 1 | 30khz | inner | 0.9 | 0.8 | 0.8 | 0.9 | 0.7 |
| 60 | 100 | 1 | 83 | 1 | 1 | 30khz | inner | 0.8 | 0.8 | 0.8 | 0.8 | 0.7 |
| 100 | 100 | 1 | 184 | 1 | 51 | 30khz | inner | 0.5 | 0.5 | 0.3 | 0.9 | 0.7 |
| 100 | 100 | 1 | 183 | 1 | 52 | 30khz | inner | 1.3 | 1.2 | 1.0 | 1.4 | 1.2 |
| 100 | 100 | 1 | 140 | 1 | 1 | 30khz | inner | 1.7 | 1.9 | 1.6 | 1.7 | 1.4 |
| 100 | 100 | 1 | 139 | 1 | 1 | 30khz | inner | 1.7 | 1.8 | 1.5 | 1.7 | 1.5 |
| Inner Max MPR |  |  |  |  |  |  |  | 1.7 | 1.9 | 1.6 | 1.7 | 1.5 |
| 60 | 100 | 1 | 120 | 1 | 118 | 30khz | outer1 | 4.4 | 4.4 | 4.2 | 4.8 | 4.5 |
| 60 | 100 | 1 | 82 | 1 | 118 | 30khz | outer1 | 4.7 | 4.7 | 4.5 | 5.1 | 4.8 |
| 60 | 100 | 1 | 43 | 1 | 118 | 30khz | outer1 | 4.7 | 4.7 | 4.6 | 5.2 | 4.9 |
| 60 | 100 | 1 | 42 | 1 | 118 | 30khz | outer1 | 4.5 | 4.6 | 4.3 | 4.9 | 4.5 |
| 60 | 100 | 1 | 1 | 1 | 1 | 30khz | outer1 | 5.1 | 5.0 | 4.8 | 5.4 | 5.1 |
| 100 | 100 | 1 | 182 | 1 | 182 | 30khz | outer1 | 4.7 | 4.7 | 4.5 | 5.1 | 4.8 |
| 100 | 100 | 1 | 138 | 1 | 182 | 30khz | outer1 | 4.7 | 4.8 | 4.6 | 5.2 | 4.9 |
| 100 | 100 | 1 | 110 | 1 | 150 | 30khz | outer1 | 4.7 | 4.8 | 4.6 | 5.2 | 4.9 |
| 100 | 100 | 1 | 109 | 1 | 163 | 30khz | outer1 | 4.7 | 4.8 | 4.6 | 5.2 | 4.9 |
| 100 | 100 | 1 | 1 | 1 | 1 | 30khz | outer1 | 5.0 | 5.0 | 4.8 | 5.4 | 5.2 |
| Outer1 Max MPR |  |  |  |  |  |  |  | 5.1 | 5.0 | 4.8 | 5.4 | 5.2 |
| 60 | 100 | 1 | 41 | 1 | 120 | 30khz | outer2 | 6.0 | 6.0 | 5.8 | 6.1 | 5.9 |
| 60 | 100 | 1 | 1 | 1 | 272 | 30khz | outer2 | 4.6 | 4.7 | 4.4 | 5.0 | 4.7 |
| 60 | 100 | 1 | 1 | 1 | 180 | 30khz | outer2 | 4.8 | 4.8 | 4.6 | 5.2 | 5.0 |
| 100 | 100 | 1 | 108 | 1 | 180 | 30khz | outer2 | 4.6 | 4.6 | 4.3 | 5.0 | 4.7 |
| 100 | 100 | 1 | 1 | 1 | 272 | 30khz | outer2 | 4.8 | 4.7 | 4.5 | 5.1 | 4.8 |
| 100 | 100 | 1 | 1 | 1 | 100 | 30khz | outer2 | 4.4 | 4.5 | 4.2 | 4.8 | 4.4 |
| Outer2 Max MPR |  |  |  |  |  |  |  | 6.0 | 6.0 | 5.8 | 6.1 | 5.9 |

Table 2 DFT-S-OFDM non-contig RB allocation: CA Class B and Class C

| DFT-S-OFDM: CA Class B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CBW1 | CBW2 | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | Region | MPR (dB) |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | pi/2 bpsk | qpsk | 16qam |  | 64qam |  | 256qam |
| 20 | 20 | 1 | 73 | 1 | 32 | 15khz | inner | 2.1 | 2.2 | 2.1 | 2.1 |  | 2.1 |  |
| 20 | 20 | 1 | 72 | 1 | 33 | 15khz | inner | 2.2 | 2.2 | 2.1 | 2.1 |  | 2.2 |  |
| 20 | 20 | 1 | 56 | 1 | 1 | 15khz | inner | 2.2 | 2.3 | 2.3 | 2.4 |  | 2.5 |  |
| 20 | 20 | 1 | 55 | 1 | 1 | 15khz | inner | 2.3 | 2.3 | 2.3 | 2.3 |  | 2.3 |  |
| 50 | 50 | 1 | 183 | 1 | 86 | 15khz | inner | 2.2 | 2.2 | 2.1 | 2.1 |  | 2.2 |  |
| 50 | 50 | 1 | 139 | 1 | 1 | 15khz | inner | 2.3 | 2.3 | 2.3 | 2.3 |  | 2.4 |  |
| 50 | 50 | 1 | 138 | 1 | 1 | 15khz | inner | 1.5 | 1.3 | 1.1 | 1.1 |  | 1.0 |  |
| Inner Max MPR |  |  |  |  |  |  |  | 2.3 | 2.3 | 2.3 | 2.4 |  | 2.5 |  |
| 20 | 20 | 1 | 72 | 1 | 50 | 15khz | outer1 | 2.5 | 3.1 | 3.5 | 3.6 |  | 3.7 |  |
| 20 | 20 | 1 | 54 | 1 | 50 | 15khz | outer1 | 3.4 | 4.0 | 4.3 | 4.4 |  | 4.4 |  |
| 20 | 20 | 1 | 42 | 1 | 63 | 15khz | outer1 | 3.4 | 4.1 | 4.3 | 4.4 |  | 4.4 |  |
| 20 | 20 | 1 | 1 | 1 | 1 | 15khz | outer1 | 3.0 | 3.7 | 4.0 | 4.1 |  | 4.3 |  |
| 50 | 50 | 1 | 137 | 1 | 60 | 15khz | outer1 | 3.3 | 4.1 | 4.1 | 4.4 |  | 4.4 |  |
| 50 | 50 | 1 | 137 | 1 | 20 | 15khz | outer1 | 2.4 | 3.0 | 3.4 | 3.5 |  | 3.7 |  |
| 50 | 50 | 1 | 109 | 1 | 160 | 15khz | outer1 | 3.4 | 4.1 | 4.3 | 4.4 |  | 4.6 |  |
| 50 | 50 | 1 | 108 | 1 | 161 | 15khz | outer1 | 2.9 | 3.5 | 3.9 | 4.0 |  | 4.2 |  |
| 50 | 50 | 1 | 1 | 1 | 1 | 15khz | outer1 | 3.5 | 4.2 | 4.4 | 4.6 |  | 4.7 |  |
| Outer1 Max MPR |  |  |  |  |  |  |  | 3.5 | 4.2 | 4.4 | 4.6 |  | 4.7 |  |
| 20 | 20 | 1 | 40 | 1 | 65 | 15khz | outer2 | 5.4 | 5.8 | 6.1 | 6.2 |  | 6.5 |  |
| 20 | 20 | 1 | 1 | 1 | 105 | 15khz | outer2 | 3.5 | 4.1 | 4.3 | 4.4 |  | 4.6 |  |
| 20 | 20 | 1 | 1 | 1 | 80 | 15khz | outer2 | 3.1 | 3.8 | 4.1 | 4.1 |  | 4.3 |  |
| 20 | 20 | 1 | 41 | 1 | 64 | 15khz | outer2 | 5.4 | 5.8 | 6.1 | 6.2 |  | 6.5 |  |
| 50 | 50 | 1 | 50 | 1 | 162 | 15khz | outer2 | 3.3 | 4.0 | 4.2 | 4.4 |  | 4.4 |  |
| 50 | 50 | 1 | 1 | 1 | 269 | 15khz | outer2 | 2.8 | 3.6 | 3.8 | 3.9 |  | 4.1 |  |
| 50 | 50 | 1 | 1 | 1 | 80 | 15khz | outer2 | 2.8 | 3.6 | 3.8 | 4.0 |  | 4.1 |  |
| Outer2 Max MPR |  |  |  |  |  |  |  | 5.4 | 5.8 | 6.1 | 6.2 |  | 6.5 |  |
| DFT-S-OFDM: CA Class C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CBW1 | CBW2 | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | Region | MPR (dB) |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | pi/2 bpsk | qpsk | 16qam |  | 64qam |  | 256qam |
| 60 | 100 | 1 | 110 | 1 | 51 | 30khz | inner | 1.4 | 1.4 | 1.0 | 1.0 |  | 0.8 |  |
| 60 | 100 | 1 | 109 | 1 | 52 | 30khz | inner | 1.8 | 1.9 | 1.8 | 1.8 |  | 1.8 |  |
| 60 | 100 | 1 | 84 | 1 | 1 | 30khz | inner | 1.3 | 1.2 | 0.7 | 0.7 |  | 0.8 |  |
| 60 | 100 | 1 | 83 | 1 | 1 | 30khz | inner | 1.6 | 1.3 | 1.0 | 0.9 |  | 0.8 |  |
| 100 | 100 | 1 | 184 | 1 | 51 | 30khz | inner | 1.4 | 1.4 | 0.9 | 0.8 |  | 0.7 |  |
| 100 | 100 | 1 | 183 | 1 | 52 | 30khz | inner | 1.8 | 1.9 | 1.8 | 1.8 |  | 1.8 |  |
| 100 | 100 | 1 | 140 | 1 | 1 | 30khz | inner | 2.1 | 2.2 | 2.2 | 2.2 |  | 2.2 |  |
| 100 | 100 | 1 | 139 | 1 | 1 | 30khz | inner | 2.1 | 2.2 | 2.2 | 2.1 |  | 2.2 |  |
| Inner Max MPR |  |  |  |  |  |  |  | 2.1 | 2.2 | 2.2 | 2.2 |  | 2.2 |  |
| 60 | 100 | 1 | 120 | 1 | 118 | 30khz | outer1 | 2.6 | 3.4 | 3.7 | 3.7 |  | 4.0 |  |
| 60 | 100 | 1 | 82 | 1 | 118 | 30khz | outer1 | 3.0 | 3.8 | 4.0 | 4.1 |  | 4.3 |  |
| 60 | 100 | 1 | 43 | 1 | 118 | 30khz | outer1 | 3.2 | 3.9 | 4.0 | 4.2 |  | 4.3 |  |
| 60 | 100 | 1 | 42 | 1 | 118 | 30khz | outer1 | 2.8 | 3.4 | 3.8 | 4.0 |  | 4.1 |  |
| 60 | 100 | 1 | 1 | 1 | 1 | 30khz | outer1 | 3.4 | 4.2 | 4.4 | 4.5 |  | 4.7 |  |
| 100 | 100 | 1 | 182 | 1 | 182 | 30khz | outer1 | 3.1 | 3.8 | 3.9 | 4.1 |  | 4.3 |  |
| 100 | 100 | 1 | 138 | 1 | 182 | 30khz | outer1 | 3.2 | 3.9 | 4.2 | 4.2 |  | 4.4 |  |
| 100 | 100 | 1 | 110 | 1 | 150 | 30khz | outer1 | 3.2 | 4.0 | 4.2 | 4.2 |  | 4.4 |  |
| 100 | 100 | 1 | 109 | 1 | 163 | 30khz | outer1 | 3.2 | 3.9 | 4.2 | 4.2 |  | 4.4 |  |
| 100 | 100 | 1 | 1 | 1 | 1 | 30khz | outer1 | 3.4 | 4.2 | 4.4 | 4.5 |  | 4.7 |  |
| Outer1 Max MPR |  |  |  |  |  |  |  | 3.4 | 4.2 | 4.4 | 4.5 |  | 4.7 |  |
| 60 | 100 | 1 | 41 | 1 | 120 | 30khz | outer2 | 4.9 | 5.0 | 5.3 | 5.6 |  | 5.8 |  |
| 60 | 100 | 1 | 1 | 1 | 272 | 30khz | outer2 | 2.9 | 3.7 | 3.9 | 4.1 |  | 4.2 |  |
| 60 | 100 | 1 | 1 | 1 | 180 | 30khz | outer2 | 3.2 | 3.9 | 4.2 | 4.2 |  | 4.5 |  |
| 100 | 100 | 1 | 108 | 1 | 180 | 30khz | outer2 | 2.8 | 3.5 | 3.8 | 4.0 |  | 4.1 |  |
| 100 | 100 | 1 | 1 | 1 | 272 | 30khz | outer2 | 3.1 | 3.8 | 4.1 | 4.1 |  | 4.3 |  |
| 100 | 100 | 1 | 1 | 1 | 100 | 30khz | outer2 | 2.7 | 3.3 | 3.7 | 3.8 |  | 4.0 |  |
| Outer2 Max MPR |  |  |  |  |  |  |  | 4.9 | 5.0 | 5.3 | 5.6 |  | 5.8 |  |

2.2.2 Summary of MRP results for non-contiguous RB allocations

Above MPR results can be summarized as below table 3 with 1dB added to take account the 2Tx reverse IMD impacts. Additional implementation margin is needed, however, to facilitate alignment among companies the values without implementation margin are given in below table.

Table 3 MPR for non-contiguous RB allocation

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | Outer2 | Inner | Outer1 | Outer2 |
| DFT-s-OFDM | Pi/2 BPSK | 3.3 | 4.5 | 6.4 | 3.1 | 4.4 | 5.9 |
|  | QPSK | 3.3 | 5.2 | 6.8 | 3.2 | 5.2 | 6 |
|  | 16QAM | 3.3 | 5.4 | 7.1 | 3.2 | 5.4 | 6.3 |
|  | 64QAM | 3.4 | 5.6 | 7.2 | 3.2 | 5.5 | 6.6 |
|  | 256QAM | 3.5 | 5.7 | 7.5 | 3.2 | 5.7 | 6.8 |
| CP-OFDM | QPSK | 3.3 | 6 | 7.8 | 2.9 | 6 | 7 |
|  | 16QAM | 3.2 | 5.8 | 7.6 | 2.6 | 5.8 | 6.8 |
|  | 64QAM | 3.2 | 6.4 | 8.3 | 2.7 | 6.4 | 7.1 |
|  | 256QAM | 3 | 6.2 | 7.8 | 2.5 | 6.2 | 6.9 |

Proposal 2:         Consider initial MPR results for non-contiguous RB allocations in two CCs in table 3 (additional implementation margin is needed in additional to these results).

  - Contiguous RB allocation MRP

2.2.1 Initial MRP results with RB in both CCs

The MPR results for contiguous RB allocation for CP-OFDM/DFT-S-OFDM are given in below tables. And the RBs are configured in both CCs which means the EVM/IBE are not verified in the simulation.

The configurations are:

- CA class B or C

- CA power class PC1.5

- Two PC2 PA

- CP-OFDM /DFT-S-OFDM

- scs is 15khz or 30khz

- LO number is 1LO+TxD

- Modulations are same in both CCs

Table 4 CP-OFDM contig RB allocation in two CCs: CA Class B and Class C

| CP-OFDM: CA class B |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CBW1 | CBW2 | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | Inner/Outer | MPR (dB) |  |  |  |  |
|  |  |  |  |  |  |  |  | pi/2 BPSK | QPSK | 16QAM | 64QAM | 256QAM |
| 20 | 20 | 54 | 52 | 54 | 1 | 15 | inner | 1.2 | 1.4 | 1.3 | 1.4 | 1.3 |
| 20 | 20 | 53 | 53 | 53 | 1 | 15 | inner | 1.3 | 1.3 | 1.3 | 1.3 | 1.2 |
| 20 | 20 | 52 | 52 | 53 | 1 | 15 | inner | 1.4 | 1.4 | 1.4 | 1.3 | 1.4 |
| 50 | 50 | 136 | 134 | 136 | 1 | 15 | inner | 0.7 | 0.7 | 0.8 | 0.7 | 0.7 |
| 50 | 50 | 135 | 135 | 135 | 1 | 15 | inner | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 |
| 50 | 50 | 134 | 136 | 135 | 1 | 15 | inner | 0.7 | 0.7 | 0.6 | 0.6 | 0.6 |
| Inner Max MPR |  |  |  |  |  |  |  | 1.4 | 1.4 | 1.4 | 1.4 | 1.4 |
| 20 | 20 | 106 | 1 | 106 | 1 | 15 | outer | 2.5 | 2.5 | 2.4 | 2.4 | 2.5 |
| 20 | 20 | 81 | 25 | 80 | 1 | 15 | outer | 2.2 | 2.3 | 2.3 | 2.3 | 2.3 |
| 20 | 20 | 80 | 26 | 80 | 1 | 15 | outer | 2.3 | 2.3 | 2.2 | 2.3 | 2.3 |
| 20 | 20 | 79 | 27 | 80 | 1 | 15 | outer | 2.3 | 2.2 | 2.3 | 2.3 | 2.2 |
| 50 | 50 | 270 | 1 | 270 | 1 | 15 | outer | 2.3 | 2.3 | 2.3 | 2.3 | 2.3 |
| 50 | 50 | 203 | 67 | 202 | 1 | 15 | outer | 1.8 | 1.8 | 1.8 | 1.8 | 1.8 |
| 50 | 50 | 202 | 68 | 202 | 1 | 15 | outer | 1.8 | 1.8 | 1.8 | 1.8 | 1.8 |
| 50 | 50 | 201 | 69 | 202 | 1 | 15 | outer | 1.8 | 1.8 | 1.8 | 1.8 | 1.8 |
| Outer Max MPR |  |  |  |  |  |  |  | 2.5 | 2.5 | 2.4 | 2.4 | 2.5 |
| CP-OFDM: CA class C |  |  |  |  |  |  |  |  |  |  |  |  |
| CBW1 | CBW2 | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | Inner/Outer | MPR (dB) |  |  |  |  |
|  |  |  |  |  |  |  |  | pi/2 BPSK | QPSK | 16QAM | 64QAM | 256QAM |
| 60 | 100 | 54 | 108 | 162 | 1 | 30 | inner | 0.3 | 0.4 | 0.4 | 0.3 | 0.3 |
| 60 | 100 | 53 | 109 | 162 | 1 | 30 | inner | 0.3 | 0.3 | 0.2 | 0.3 | 0.3 |
| 100 | 100 | 137 | 136 | 137 | 1 | 30 | inner | 0.2 | 0.3 | 0.2 | 0.2 | 0.2 |
| 100 | 100 | 136 | 137 | 136 | 1 | 30 | inner | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 |
| 100 | 100 | 135 | 138 | 135 | 1 | 30 | inner | 0.1 | 0.2 | 0.1 | 0.1 | 0.2 |
| Inner Max MPR |  |  |  |  |  |  |  | 0.3 | 0.4 | 0.4 | 0.3 | 0.3 |
| 60 | 100 | 162 | 1 | 273 | 1 | 30 | outer | 2.3 | 2.3 | 2.3 | 2.3 | 2.3 |
| 60 | 100 | 125 | 37 | 209 | 1 | 30 | outer | 2.0 | 2.1 | 2.0 | 2.1 | 2.0 |
| 60 | 100 | 125 | 37 | 208 | 1 | 30 | outer | 2.0 | 2.1 | 2.0 | 2.1 | 2.0 |
| 60 | 100 | 125 | 37 | 207 | 1 | 30 | outer | 2.0 | 2.1 | 2.0 | 2.1 | 2.0 |
| 100 | 100 | 273 | 1 | 273 | 1 | 30 | outer | 2.3 | 2.3 | 2.4 | 2.4 | 2.3 |
| 100 | 100 | 205 | 68 | 204 | 1 | 30 | outer | 1.8 | 1.8 | 1.8 | 1.8 | 1.8 |
| 100 | 100 | 204 | 69 | 204 | 1 | 30 | outer | 1.8 | 1.8 | 1.8 | 1.8 | 1.8 |
| 100 | 100 | 203 | 70 | 204 | 1 | 30 | outer | 1.8 | 1.8 | 1.8 | 1.8 | 1.8 |
| 60 | 100 | 60 | 102 | 162 | 1 | 30 | outer | 0.2 | 0.5 | 0.7 | 0.8 | 0.8 |
| Outer Max MPR |  |  |  |  |  |  |  | 2.3 | 2.3 | 2.4 | 2.4 | 2.3 |

Table 5 DFT-S-OFDM contig RB allocation in two CCs: CA Class B and Class C

| DFT-S-OFDM: CA class B |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CBW1 | CBW2 | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | Inner/Outer | MPR (dB) |  |  |  |  |
|  |  |  |  |  |  |  |  | pi/2 BPSK | QPSK | 16QAM | 64QAM | 256QAM |
| 20 | 20 | 54 | 52 | 54 | 1 | 15 | inner | 0.6 | 0.6 | 0.9 | 1.0 | 0.9 |
| 20 | 20 | 54 | 52 | 50 | 1 | 15 | inner | 0.4 | 0.6 | 0.9 | 0.9 | 0.9 |
| 20 | 20 | 50 | 56 | 50 | 1 | 15 | inner | 0.4 | 0.5 | 0.7 | 0.8 | 0.8 |
| 50 | 50 | 144 | 126 | 128 | 1 | 15 | inner | 0.1 | 0.3 | 0.5 | 0.6 | 0.5 |
| 50 | 50 | 144 | 126 | 125 | 1 | 15 | inner | 0.2 | 0.3 | 0.5 | 0.5 | 0.5 |
| 50 | 50 | 135 | 135 | 135 | 1 | 15 | inner | 0.1 | 0.2 | 0.4 | 0.5 | 0.5 |
| Inner Max MPR |  |  |  |  |  |  |  | 0.6 | 0.6 | 0.9 | 1 | 0.9 |
| 20 | 20 | 100 | 6 | 100 | 1 | 15 | outer | 0.8 | 1.4 | 1.9 | 1.9 | 1.9 |
| 20 | 20 | 90 | 16 | 72 | 1 | 15 | outer | 0.8 | 1.3 | 1.7 | 1.8 | 1.9 |
| 20 | 20 | 80 | 26 | 80 | 1 | 15 | outer | 0.8 | 1.2 | 1.7 | 1.7 | 1.8 |
| 20 | 20 | 80 | 26 | 72 | 1 | 15 | outer | 0.8 | 1.2 | 1.6 | 1.7 | 1.7 |
| 50 | 50 | 270 | 0 | 270 | 1 | 15 | outer | 0.4 | 0.9 | 1.5 | 1.6 | 1.7 |
| 50 | 50 | 216 | 54 | 192 | 1 | 15 | outer | 0.4 | 0.8 | 1.2 | 1.2 | 1.2 |
| 50 | 50 | 216 | 54 | 180 | 1 | 15 | outer | 0.4 | 0.8 | 1.2 | 1.2 | 1.2 |
| 50 | 50 | 200 | 70 | 200 | 1 | 15 | outer | 0.4 | 0.7 | 1.1 | 1.2 | 1.2 |
| 60 | 100 | 55 | 107 | 162 | 1 | 30 | outer | 0.3 | 0.4 | 0.3 | 0.3 | 0.3 |
| Outer Max MPR |  |  |  |  |  |  |  | 0.8 | 1.4 | 1.9 | 1.9 | 1.9 |
| DFT-S-OFDM: CA class C |  |  |  |  |  |  |  |  |  |  |  |  |
| CBW1 | CBW2 | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | Inner/Outer | MPR (dB) |  |  |  |  |
|  |  |  |  |  |  |  |  | pi/2 BPSK | QPSK | 16QAM | 64QAM | 256QAM |
| 60 | 100 | 54 | 108 | 162 | 1 | 30 | inner | 0.2 | 0.4 | 0.7 | 0.7 | 0.7 |
| 60 | 100 | 50 | 112 | 162 | 1 | 30 | inner | 0.2 | 0.4 | 0.7 | 0.7 | 0.7 |
| 100 | 100 | 144 | 129 | 135 | 1 | 30 | inner | 0.0 | 0.0 | 0.1 | 0.1 | 0.1 |
| 100 | 100 | 135 | 138 | 135 | 1 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 100 | 100 | 135 | 138 | 128 | 1 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Inner Max MPR |  |  |  |  |  |  |  | 0.2 | 0.4 | 0.7 | 0.7 | 0.7 |
| 60 | 100 | 162 | 1 | 270 | 1 | 30 | outer | 0.5 | 1.1 | 1.6 | 1.8 | 1.8 |
| 60 | 100 | 135 | 27 | 200 | 1 | 30 | outer | 0.5 | 1.1 | 1.6 | 1.6 | 1.7 |
| 60 | 100 | 128 | 34 | 200 | 1 | 30 | outer | 0.5 | 1.1 | 1.6 | 1.6 | 1.6 |
| 60 | 100 | 128 | 34 | 192 | 1 | 30 | outer | 0.5 | 1.0 | 1.6 | 1.6 | 1.6 |
| 100 | 100 | 270 | 3 | 270 | 1 | 30 | outer | 0.4 | 0.9 | 1.5 | 1.6 | 1.6 |
| 100 | 100 | 216 | 57 | 200 | 1 | 30 | outer | 0.3 | 0.8 | 1.2 | 1.2 | 1.3 |
| 100 | 100 | 216 | 57 | 192 | 1 | 30 | outer | 0.3 | 0.8 | 1.2 | 1.2 | 1.2 |
| 100 | 100 | 200 | 73 | 192 | 1 | 30 | outer | 0.3 | 0.7 | 1.1 | 1.1 | 1.1 |
| Outer Max MPR |  |  |  |  |  |  |  | 0.5 | 1.1 | 1.6 | 1.8 | 1.8 |

2.2.1 Initial MRP results with RB in one CC

To further check the EVM /IBE impacts in contiguous CA, below Table 6 and 7 give the simulation results for CP-OFDM and DFT-S-OFDM respectively when only CC1 is configured with PRBs. The assumptions are similar as two CC cases.

Table 5 CP-OFDM contig RB allocation in one CC: CA Class B and Class C

| CP-OFDM: CA class B |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CBW1 | CBW2 | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | Inner/Outer | MPR (dB) |  |  |  |  |
|  |  |  |  |  |  |  |  | pi/2 BPSK | QPSK | 16QAM | 64QAM | 256QAM |
| 20 | 20 | 54 | 52 | 0 | 1 | 15 | inner | 0.0 | 0.4 | 0.6 | 1.8 | 2.6 |
| 20 | 20 | 52 | 52 | 0 | 1 | 15 | inner | 0.0 | 0.4 | 0.6 | 1.2 | 2.6 |
| 20 | 20 | 50 | 56 | 0 | 1 | 15 | inner | 0.0 | 0.4 | 0.6 | 1.2 | 2.7 |
| 50 | 50 | 136 | 134 | 0 | 1 | 15 | inner | 0.0 | 0.4 | 0.6 | 1.8 | 2.6 |
| 50 | 50 | 135 | 135 | 0 | 1 | 15 | inner | 0.0 | 0.4 | 0.6 | 1.2 | 2.6 |
| 50 | 50 | 144 | 126 | 0 | 1 | 15 | inner | 0.0 | 0.4 | 0.6 | 1.2 | 2.6 |
| Inner Max MPR |  |  |  |  |  |  |  | 0.0 | 0.4 | 0.6 | 1.8 | 2.7 |
| 20 | 20 | 106 | 1 | 0 | 1 | 15 | outer | 3.1 | 3.1 | 3.1 | 3.6 | 3.6 |
| 20 | 20 | 100 | 6 | 0 | 1 | 15 | outer | 2.6 | 3.1 | 3.1 | 3.6 | 3.6 |
| 20 | 20 | 90 | 16 | 0 | 1 | 15 | outer | 2.6 | 2.6 | 2.6 | 2.6 | 3.6 |
| 20 | 20 | 81 | 25 | 0 | 1 | 15 | outer | 2.2 | 2.2 | 2.2 | 2.6 | 3.1 |
| 20 | 20 | 80 | 26 | 0 | 1 | 15 | outer | 2.2 | 2.2 | 2.2 | 2.2 | 3.1 |
| 50 | 50 | 270 | 1 | 0 | 1 | 15 | outer | 2.6 | 2.6 | 2.6 | 2.6 | 3.6 |
| 50 | 50 | 216 | 54 | 0 | 1 | 15 | outer | 1.8 | 1.8 | 2.2 | 2.2 | 3.1 |
| 50 | 50 | 203 | 67 | 0 | 1 | 15 | outer | 1.8 | 1.8 | 1.8 | 2.2 | 3.1 |
| Outer Max MPR |  |  |  |  |  |  |  | 3.1 | 3.1 | 3.1 | 3.6 | 3.6 |
| CP-OFDM: CA class C |  |  |  |  |  |  |  |  |  |  |  |  |
| CBW1 | CBW2 | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | Inner/Outer | MPR (dB) |  |  |  |  |
|  |  |  |  |  |  |  |  | pi/2 BPSK | QPSK | 16QAM | 64QAM | 256QAM |
| 60 | 100 | 55 | 107 | 0 | 1 | 30 | inner | 0.0 | 0.4 | 0.6 | 1.2 | 2.6 |
| 60 | 100 | 60 | 102 | 0 | 1 | 30 | inner | 0.0 | 0.4 | 0.6 | 1.2 | 2.6 |
| 60 | 100 | 50 | 112 | 0 | 1 | 30 | inner | 0.0 | 0.4 | 0.6 | 1.2 | 2.6 |
| 100 | 100 | 136 | 137 | 0 | 1 | 30 | inner | 0.0 | 0.4 | 0.6 | 1.8 | 2.6 |
| 100 | 100 | 144 | 129 | 0 | 1 | 30 | inner | 0.0 | 0.4 | 0.6 | 1.2 | 2.6 |
| 100 | 100 | 135 | 138 | 0 | 1 | 30 | inner | 0.0 | 0.4 | 0.6 | 1.2 | 2.6 |
| Inner Max MPR |  |  |  |  |  |  |  | 0.0 | 0.4 | 0.6 | 1.8 | 2.6 |
| 60 | 100 | 162 | 1 | 0 | 1 | 30 | outer | 2.6 | 2.6 | 2.6 | 2.6 | 3.6 |
| 60 | 100 | 125 | 37 | 0 | 1 | 30 | outer | 1.8 | 1.8 | 1.8 | 1.8 | 3.1 |
| 60 | 100 | 135 | 27 | 0 | 1 | 30 | outer | 1.8 | 1.8 | 2.2 | 1.8 | 3.1 |
| 100 | 100 | 273 | 1 | 0 | 1 | 30 | outer | 2.6 | 2.6 | 2.6 | 2.6 | 3.6 |
| 100 | 100 | 270 | 3 | 0 | 1 | 30 | outer | 2.6 | 2.6 | 2.6 | 2.6 | 3.6 |
| 100 | 100 | 205 | 68 | 0 | 1 | 30 | outer | 0.9 | 0.9 | 0.9 | 1.2 | 2.6 |
| 100 | 100 | 216 | 57 | 0 | 1 | 30 | outer | 1.2 | 1.2 | 1.2 | 1.8 | 2.6 |
| Outer Max MPR |  |  |  |  |  |  |  | 2.6 | 2.6 | 2.6 | 2.6 | 3.6 |

Table 6 DFT-S-OFDM contig RB allocation in one CC: CA Class B and Class C

| DFT-S-OFDM: CA class B |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CBW1 | CBW2 | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | Inner/Outer | MPR (dB) |  |  |  |  |
|  |  |  |  |  |  |  |  | pi/2 BPSK | QPSK | 16QAM | 64QAM | 256QAM |
| 20 | 20 | 54 | 52 | 0 | 1 | 15 | inner | 0.0 | 0.0 | 0.0 | 0.3 | 2.0 |
| 20 | 20 | 52 | 52 | 0 | 1 | 15 | inner | 0.0 | 0.0 | 0.0 | 0.3 | 2.0 |
| 20 | 20 | 50 | 56 | 0 | 1 | 15 | inner | 0.0 | 0.0 | 0.0 | 0.3 | 2.0 |
| 50 | 50 | 136 | 134 | 0 | 1 | 15 | inner | 0.0 | 0.0 | 0.0 | 0.3 | 2.0 |
| 50 | 50 | 135 | 135 | 0 | 1 | 15 | inner | 0.0 | 0.0 | 0.0 | 0.3 | 2.0 |
| 50 | 50 | 144 | 126 | 0 | 1 | 15 | inner | 0.0 | 0.0 | 0.0 | 0.3 | 2.0 |
| Inner Max MPR |  |  |  |  |  |  |  | 0.0 | 0.0 | 0.0 | 0.3 | 2.0 |
| 20 | 20 | 106 | 1 | 0 | 1 | 15 | outer | 0.0 | 0.7 | 1.6 | 2.0 | 2.4 |
| 20 | 20 | 100 | 6 | 0 | 1 | 15 | outer | 0.0 | 0.7 | 1.6 | 1.6 | 2.0 |
| 20 | 20 | 90 | 16 | 0 | 1 | 15 | outer | 0.0 | 0.4 | 0.9 | 1.6 | 2.0 |
| 20 | 20 | 81 | 25 | 0 | 1 | 15 | outer | 0.0 | 0.4 | 0.9 | 0.9 | 2.0 |
| 20 | 20 | 80 | 26 | 0 | 1 | 15 | outer | 0.0 | 0.4 | 0.9 | 0.9 | 2.0 |
| 50 | 50 | 270 | 1 | 0 | 1 | 15 | outer | 0.0 | 0.4 | 0.9 | 0.9 | 2.0 |
| 50 | 50 | 216 | 54 | 0 | 1 | 15 | outer | 0.0 | 0.1 | 0.5 | 0.6 | 2.0 |
| 50 | 50 | 203 | 67 | 0 | 1 | 15 | outer | 0.0 | 0.0 | 0.3 | 0.6 | 2.0 |
| Outer Max MPR |  |  |  |  |  |  |  | 0.0 | 0.7 | 1.6 | 2.0 | 2.4 |
| DFT-S-OFDM: CA class C |  |  |  |  |  |  |  |  |  |  |  |  |
| CBW1 | CBW2 | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | Inner/Outer | MPR (dB) |  |  |  |  |
|  |  |  |  |  |  |  |  | pi/2 BPSK | QPSK | 16QAM | 64QAM | 256QAM |
| 60 | 100 | 55 | 107 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.3 | 1.6 |
| 60 | 100 | 60 | 102 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.3 | 1.6 |
| 60 | 100 | 50 | 112 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.3 | 2.0 |
| 100 | 100 | 136 | 137 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.3 | 2.0 |
| 100 | 100 | 144 | 129 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.3 | 2.0 |
| 100 | 100 | 135 | 138 | 0 | 1 | 30 | inner | 0.0 | 0.0 | 0.0 | 0.3 | 2.0 |
| Inner Max MPR |  |  |  |  |  |  |  | 0.0 | 0.0 | 0.0 | 0.3 | 2 |
| 60 | 100 | 162 | 1 | 0 | 1 | 30 | outer | 0.0 | 0.1 | 0.8 | 0.9 | 2.0 |
| 60 | 100 | 135 | 27 | 0 | 1 | 30 | outer | 0.0 | 0.1 | 0.5 | 0.6 | 2.0 |
| 60 | 100 | 125 | 37 | 0 | 1 | 30 | outer | 0.0 | 0.0 | 0.5 | 0.6 | 2.0 |
| 100 | 100 | 273 | 1 | 0 | 1 | 30 | outer | 0.0 | 0.1 | 0.9 | 0.9 | 2.0 |
| 100 | 100 | 270 | 3 | 0 | 1 | 30 | outer | 0.0 | 0.1 | 0.9 | 0.9 | 2.0 |
| 100 | 100 | 205 | 68 | 0 | 1 | 30 | outer | 0.0 | 0.0 | 0.0 | 0.3 | 1.6 |
| 100 | 100 | 216 | 57 | 0 | 1 | 30 | outer | 0.0 | 0.0 | 0.0 | 0.3 | 2.0 |
| Outer Max MPR |  |  |  |  |  |  |  | 0.0 | 0.1 | 0.9 | 0.9 | 2.0 |

2.2.2 Summary of MRP results for Contiguous RB allocation in one CC and two CCs

Table 7 has summarized the MPR results for contiguous RB allocations in two CCs case, and Table 8 has summarized the MPR results for contiguous RB allocations in one CC case.

Similar as non-contiguous RB allocation, 1dB is added to the above simulation results to consider the TxD reverse IMD impacts. And additional implementation margin is still needed, however, to facilitate alignment of results the implementation margin is not added here.

Table 7 MPR for contiguous RB allocation in two CCs

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 1.6 | 1.8 | 1.2 | 1.5 |
|  | QPSK | 1.6 | 2.4 | 1.5 | 2.1 |
|  | 16QAM | 1.9 | 2.9 | 1.7 | 2.6 |
|  | 64QAM | 2.0 | 2.9 | 1.8 | 2.8 |
|  | 256QAM | 1.9 | 2.9 | 1.8 | 2.8 |
| CP-OFDM | QPSK | 2.4 | 3.5 | 1.4 | 3.3 |
|  | 16QAM | 2.4 | 3.4 | 1.4 | 3.4 |
|  | 64QAM | 2.4 | 3.4 | 1.3 | 3.4 |
|  | 256QAM | 2.4 | 3.5 | 1.3 | 3.3 |

Table 8 MPR for contiguous RB allocation in one CC

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 1 | 1 | 1 | 1 |
|  | QPSK | 1 | 1.7 | 1 | 1.1 |
|  | 16QAM | 1 | 2.6 | 1 | 1.9 |
|  | 64QAM | 1.3 | 3 | 1.3 | 1.9 |
|  | 256QAM | 3 | 3.4 | 3 | 3 |
| CP-OFDM | QPSK | 1.4 | 4.1 | 1.4 | 3.6 |
|  | 16QAM | 1.6 | 4.1 | 1.6 | 3.6 |
|  | 64QAM | 2.8 | 4.6 | 2.8 | 3.6 |
|  | 256QAM | 3.7 | 4.6 | 3.6 | 4.6 |

The combined one CC and two CCs MPR results are given in Table 9 (additional implementation margin is needed in additional to these results).

Table 9 MPR for contiguous RB allocation in one CC and two CCs

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 1.6 | 1.8 | 1.2 | 1.5 |
|  | QPSK | 1.6 | 2.4 | 1.5 | 2.1 |
|  | 16QAM | 1.9 | 2.9 | 1.7 | 2.6 |
|  | 64QAM | 2 | 3 | 1.8 | 2.8 |
|  | 256QAM | 3 | 3.4 | 3 | 3 |
| CP-OFDM | QPSK | 2.4 | 4.1 | 1.4 | 3.6 |
|  | 16QAM | 2.4 | 4.1 | 1.6 | 3.6 |
|  | 64QAM | 2.8 | 4.6 | 2.8 | 3.6 |
|  | 256QAM | 3.7 | 4.6 | 3.6 | 4.6 |

Proposal 3:         Consider initial MPR results for contiguous RB allocations in table 9 (additional implementation margin is needed in additional to these results).

- Conclusion

This paper analyzed the MPR for PC1.5 intra-band contiguous CA, got below observations and proposals.

Observation 1:   For contiguous and NC CA, the EVM requirements only defined for the case of PRB configured in one of the CCs. And no EVM requirements apply when both CCs are configured with PRBs.

Observation 2:   For contiguous and NC CA, the IBE requirements only defined for the case of PRB configured in one of the CCs. And no IBE requirements apply when both CCs are configured with PRBs.

Observation 3:   It is unclear when defining MPR requirements for CA (especially non-contiguous RB allocations), the EVM/IBE are verified or not with current spec.

Proposal 1:         To align the intra-band CA MPR simulation/test configurations, clarify whether the EVM/IBE requirements are verified when both CCs are configured with PRBs?

Proposal 2:         Consider initial MPR results for non-contiguous RB allocations in two CCs in table 3 (additional implementation margin is needed in additional to these results, and Outer2 may need more check).

Table 3 MPR for non-contiguous RB allocation in two CCs (without implementation margin)

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | Outer2 | Inner | Outer1 | Outer2 |
| DFT-s-OFDM | Pi/2 BPSK | 3.3 | 4.5 | 6.4 | 3.1 | 4.4 | 5.9 |
|  | QPSK | 3.3 | 5.2 | 6.8 | 3.2 | 5.2 | 6 |
|  | 16QAM | 3.3 | 5.4 | 7.1 | 3.2 | 5.4 | 6.3 |
|  | 64QAM | 3.4 | 5.6 | 7.2 | 3.2 | 5.5 | 6.6 |
|  | 256QAM | 3.5 | 5.7 | 7.5 | 3.2 | 5.7 | 6.8 |
| CP-OFDM | QPSK | 3.3 | 6 | 7.8 | 2.9 | 6 | 7 |
|  | 16QAM | 3.2 | 5.8 | 7.6 | 2.6 | 5.8 | 6.8 |
|  | 64QAM | 3.2 | 6.4 | 8.3 | 2.7 | 6.4 | 7.1 |
|  | 256QAM | 3 | 6.2 | 7.8 | 2.5 | 6.2 | 6.9 |

Proposal 3:         Consider initial MPR results for contiguous RB allocations in table 9 (additional implementation margin is needed in additional to these results).

Table 9 MPR for contiguous RB allocation in one CC and two CCs (without implementation margin)

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 1.6 | 1.8 | 1.2 | 1.5 |
|  | QPSK | 1.6 | 2.4 | 1.5 | 2.1 |
|  | 16QAM | 1.9 | 2.9 | 1.7 | 2.6 |
|  | 64QAM | 2 | 3 | 1.8 | 2.8 |
|  | 256QAM | 3 | 3.4 | 3 | 3 |
| CP-OFDM | QPSK | 2.4 | 4.1 | 1.4 | 3.6 |
|  | 16QAM | 2.4 | 4.1 | 1.6 | 3.6 |
|  | 64QAM | 2.8 | 4.6 | 2.8 | 3.6 |
|  | 256QAM | 3.7 | 4.6 | 3.6 | 4.6 |

- Reference

[1] R4-2410750 WF on higher power UE, Samsung, RAN4#111 2024-05

[2] R4-2106304 PC2 Class C UL CA UE Architecture and MPR&A-MPR, Skyworks, RAN4#98b-e 2021-04

[3] R4-2103231 WF on RF architecture and requirements for PC2 intra-band UL contiguous CA, Huawei, RAN4#98e

[4] R4-2103232 Way forward on MPR and AMPR for intra-band UL contiguous CA, Skyworks Solutions Inc., Qualcomm, RAN4#98e