3GPP TSG-RAN WG4 Meeting #112	R4-2412350
Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	R19 MPR for PC1.5 NC CA

Source:	OPPO

Agenda item:	8.1.1.1.1

Document for:	Approval

- Introduction

This paper discusses the Rel-19 PC1.5 NC CA MPR per WF [1]. It clarifies the applicable emission requirements like EVM/IBE and would like to align the assumptions in the simulation/measurements, then it shares some MPR simulation results with two PC2 PAs and TxD capability.

- Discussion

2.1 Clarification of requirements

2.1.1 NCCA EVM

In 38.101-1, the EVM is defined as below figure 1 for UL NC CA, which says “Requirements only apply with PRB allocation in one of the component carriers”. If understood correctly, it means when PRBs are configured in both CCs then EVM requirements will not be verified.

Figure 1 EVM of UL NC CA

Observation 1:   For NC CA, the EVM requirements only defined for the case of PRB configured in one of the CCs. And no EVM requirements apply when both CCs are configured with PRBs.

2.1.2 NCCA IBE

In 38.101-1, the IBE is defined as below figure 2 for UL NCCA, which says “Requirements only apply with PRB allocation in one of the component carriers”.

If understand correctly, it means when PRBs are configured in both CCs the IBE requirements will not be verified.

Figure 2 NC CA IBE requirements

Observation 2:   For NC CA, the IBE requirements only defined for the case of PRB configured in one of the CCs. And no IBE requirements apply when both CCs are configured with PRBs.

2.1.3 NCCA SE

SE is measured in the more than FOOB region as shown in figure 3 [2].

Figure 3 Applicable SE requirements

Proposal 1:         To align the NCCA MPR simulation/test configurations, clarify EVM/IBE requirements are NOT verified when both CCs are configured with PRBs.

  - Non-Contig RB allocation MRP

2.2.1 Initial MRP results

With the understanding of EVM /IBE requirements are NOT verified when both CCs are configured with PRBs, below tables give the initial simulation results for alignment purpose.

The configurations are:

- CA power class PC1.5

- PC2 PA

- CP-OFDM /DFT-S-OFDM

- 20MHz+40MHz NCCA combinations with 20MHz gap in between

- scs is 15khz

- LO number include 2LO, and 1LO+TxD

- Modulations include QPSK or 64QAM in both CCs

And it should be noticed that the reverse IMD impacts are not accounted in the simulation, and some additional values need to be added to cope with it in the final MPR results.

The MPR results are in table 1 and 2 for DFT-S-OFDM and CP-OFDM respectively. It can be seen that MPR are similar for QPSK and 64QAM.

Table 1 DFT-S-OFDM NCCA MPR results

| DFT-S-OFDM |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | B | 2LO |  |  | 1LO + TxD |  |  |
|  |  |  |  |  | qpsk | 64qam | Max | qpsk | 64qam | Max |
| 100 | 1 | 216 | 1 | 56.88 | 2.0 | 3.1 | 3.1 | 4.8 | 5.5 | 5.5 |
| 100 | 1 | 150 | 66 | 45 | 2.0 | 2.6 | 2.6 | 4.8 | 5.5 | 5.5 |
| 100 | 1 | 100 | 116 | 36 | 1.6 | 2.2 | 2.2 | 4.8 | 5.5 | 5.5 |
| 72 | 1 | 50 | 166 | 21.96 | 1.6 | 2.6 | 2.6 | 5.5 | 6.3 | 6.3 |
| 60 | 1 | 60 | 156 | 21.6 | 1.6 | 2.2 | 2.2 | 6.2 | 6.3 | 6.3 |
| 32 | 1 | 60 | 156 | 16.56 | 2.0 | 3.1 | 3.1 | 10.3 | 9.1 | 10.3 |
| 32 | 1 | 32 | 184 | 11.52 | 2.0 | 3.1 | 3.1 | 10.3 | 10.3 | 10.3 |
| 24 | 1 | 32 | 184 | 10.08 | 2.5 | 3.1 | 3.1 | 11.7 | 11.7 | 11.7 |
| 18 | 1 | 18 | 198 | 6.48 | 2.5 | 3.6 | 3.6 | 13.3 | 13.3 | 13.3 |
| 15 | 1 | 15 | 201 | 5.4 | 3.0 | 3.6 | 3.6 | 13.3 | 13.3 | 13.3 |
| 12 | 1 | 18 | 198 | 5.4 | 3.0 | 3.6 | 3.6 | 13.3 | 13.3 | 13.3 |
| 12 | 1 | 16 | 200 | 5.04 | 3.0 | 3.6 | 3.6 | 13.3 | 13.3 | 13.3 |
| 12 | 1 | 12 | 204 | 4.32 | 3.0 | 3.6 | 3.6 | 13.3 | 13.3 | 13.3 |
| 9 | 1 | 9 | 207 | 3.24 | 3.0 | 3.6 | 3.6 | 13.3 | 13.3 | 13.3 |
| 8 | 1 | 8 | 208 | 2.88 | 3.5 | 4.2 | 4.2 | 15.3 | 15.3 | 15.3 |
| 6 | 1 | 12 | 204 | 3.24 | 3.5 | 4.1 | 4.1 | 15.3 | 15.3 | 15.3 |
| 6 | 1 | 6 | 210 | 2.16 | 3.5 | 4.1 | 4.1 | 15.3 | 15.3 | 15.3 |
| 4 | 1 | 8 | 208 | 2.16 | 3.5 | 4.2 | 4.2 | 15.3 | 15.4 | 15.4 |
| 4 | 1 | 4 | 212 | 1.44 | 4.1 | 4.8 | 4.8 | 15.3 | 15.4 | 15.4 |
| 3 | 1 | 3 | 213 | 1.08 | 4.1 | 4.8 | 4.8 | 17.9 | 17.9 | 17.9 |
| 2 | 1 | 2 | 214 | 0.72 | 4.1 | 4.8 | 4.8 | 15.3 | 15.3 | 15.3 |
| 1 | 1 | 2 | 214 | 0.54 | 4.1 | 4.7 | 4.7 | 15.3 | 17.8 | 17.8 |
| 1 | 1 | 1 | 215 | 0.36 | 4.1 | 5.4 | 5.4 | 17.9 | 17.8 | 17.9 |

Table 2 CP-OFDM NCCA MPR results

| CP-OFDM |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | B | 2LO |  |  | 1LO + TxD |  |  |
|  |  |  |  |  | qpsk | 64qam | Max | qpsk | 64qam | Max |
| 106 | 1 | 216 | 1 | 57.96 | 4.3 | 4.3 | 4.3 | 6.3 | 5.5 | 6.3 |
| 106 | 1 | 150 | 66 | 46.08 | 3.2 | 3.2 | 3.2 | 6.3 | 6.3 | 6.3 |
| 100 | 1 | 100 | 116 | 36 | 3.2 | 3.2 | 3.2 | 6.3 | 6.3 | 6.3 |
| 72 | 1 | 50 | 116 | 21.96 | 3.2 | 3.2 | 3.2 | 7.1 | 7.1 | 7.1 |
| 61 | 1 | 61 | 157 | 21.96 | 3.3 | 3.3 | 3.3 | 7.1 | 7.1 | 7.1 |
| 60 | 1 | 60 | 156 | 21.6 | 3.2 | 3.2 | 3.2 | 7.1 | 7.1 | 7.1 |
| 46 | 1 | 46 | 170 | 16.56 | 3.7 | 3.7 | 3.7 | 8.0 | 8.0 | 8.0 |
| 32 | 1 | 60 | 156 | 16.56 | 4.3 | 4.3 | 4.3 | 8.0 | 8.0 | 8.0 |
| 32 | 1 | 32 | 184 | 11.52 | 4.3 | 4.3 | 4.3 | 10.3 | 9.1 | 10.3 |
| 28 | 1 | 28 | 188 | 10.08 | 4.3 | 4.3 | 4.3 | 10.3 | 10.3 | 10.3 |
| 24 | 1 | 32 | 184 | 10.08 | 4.3 | 4.3 | 4.3 | 10.3 | 10.3 | 10.3 |
| 18 | 1 | 18 | 198 | 6.48 | 4.9 | 4.9 | 4.9 | 11.7 | 11.7 | 11.7 |
| 15 | 1 | 15 | 201 | 5.4 | 4.9 | 4.9 | 4.9 | 13.3 | 13.3 | 13.3 |
| 14 | 1 | 16 | 200 | 5.4 | 4.9 | 4.9 | 4.9 | 13.3 | 13.3 | 13.3 |
| 14 | 1 | 14 | 202 | 5.04 | 4.9 | 4.9 | 4.9 | 13.3 | 13.3 | 13.3 |
| 12 | 1 | 18 | 198 | 5.4 | 4.9 | 4.9 | 4.9 | 13.3 | 13.3 | 13.3 |
| 12 | 1 | 16 | 200 | 5.04 | 4.9 | 4.9 | 4.9 | 13.3 | 13.3 | 13.3 |
| 12 | 1 | 12 | 204 | 4.32 | 5.5 | 5.5 | 5.5 | 13.3 | 13.3 | 13.3 |
| 9 | 1 | 9 | 207 | 3.24 | 5.5 | 5.5 | 5.5 | 13.3 | 13.3 | 13.3 |
| 8 | 1 | 8 | 208 | 2.88 | 5.5 | 5.6 | 5.6 | 15.3 | 15.3 | 15.3 |
| 6 | 1 | 12 | 204 | 3.24 | 5.6 | 5.5 | 5.6 | 15.3 | 15.3 | 15.3 |
| 6 | 1 | 6 | 210 | 2.16 | 5.5 | 5.5 | 5.5 | 15.3 | 15.3 | 15.3 |
| 4 | 1 | 8 | 208 | 2.16 | 5.6 | 5.5 | 5.6 | 15.3 | 15.3 | 15.3 |
| 4 | 1 | 4 | 212 | 1.44 | 6.3 | 6.3 | 6.3 | 15.4 | 15.3 | 15.4 |
| 3 | 1 | 3 | 213 | 1.08 | 6.3 | 6.3 | 6.3 | 17.9 | 17.9 | 17.9 |
| 2 | 1 | 4 | 212 | 1.08 | 5.6 | 6.3 | 6.3 | 18.0 | 17.9 | 18.0 |
| 2 | 1 | 2 | 214 | 0.72 | 5.6 | 6.2 | 6.2 | 15.4 | 15.2 | 15.4 |
| 1 | 1 | 2 | 214 | 0.54 | 5.6 | 6.2 | 6.2 | 15.3 | 15.3 | 15.3 |
| 1 | 1 | 1 | 215 | 0.36 | 6.2 | 6.2 | 6.2 | 17.9 | 17.8 | 17.9 |

2.2.2 Summary of MRP results for NC CA

Above MPR results can be summarized as below figure 4 and 5.

It should be noticed that the reverse IMD impacts are not accounted in the simulation which is anticipated as 1dB, and some additional values (0.5~1dB) are also need to be added to cope with it in the final MPR results. Therefore, in the end 1.5~2dB need to be added to the curves.

These values are not added in below figures to facilitate alignment among companies.

Figure 4 DFT-S-OFDM NC CA MPR results

Figure 5 CP-OFDM NC CA MPR results

It can be seen from figure 4 and 5, the MPR values are similar for DFT-S-OFDM and CP-OFDM.

The proposed MPR values (with reverse IMD and margin considered) for PC1.5 NC CA are as below:

- If dualPA-Architecture is not indicated (1LO+TxD)

MPR =   9;		36 =< B

10;		21.6 =< B < 36

12;		11.52 =< B < 21.6

15;		3.24 =< B < 11.52

18;		1.44 =< B < 3.24

20;		0 < B < 1.44

- If dualPA-Architecture is indicated (2LO)

MPR =   6;		10.08 =< B

7;		5.04 =< B < 10.08

8;		2.16 =< B < 5.04

8.5;		0 < B < 2.16

Proposal 2:         Consider MPR results for NCCA as above.

- Conclusion

This paper analyzed the MPR for PC1.5 intra-band contiguous CA, got below observations and proposals.

Observation 1:   For NC CA, the EVM requirements only defined for the case of PRB configured in one of the CCs. And no EVM requirements apply when both CCs are configured with PRBs.

Observation 2:   For NC CA, the IBE requirements only defined for the case of PRB configured in one of the CCs. And no IBE requirements apply when both CCs are configured with PRBs.

Proposal 1:         To align the NCCA MPR simulation/test configurations, clarify EVM/IBE requirements are NOT verified when both CCs are configured with PRBs.

Proposal 2:         Consider MPR results for NCCA as below.

- If dualPA-Architecture is not indicated (1LO+TxD)

MPR =   9;		36 =< B

10;		21.6 =< B < 36

12;		11.52 =< B < 21.6

15;		3.24 =< B < 11.52

18;		1.44 =< B < 3.24

20;		0 < B < 1.44

- If dualPA-Architecture is indicated (2LO)

MPR =   6;		10.08 =< B

7;		5.04 =< B < 10.08

8;		2.16 =< B < 5.04

8.5;		0 < B < 2.16

- Reference

[1] R4-2410750 WF on higher power UE, Samsung, RAN4#111 2024-05

[2] R4-2010301 UE TX measurements and requirements for non-contiguous ULCA MPR and A-MPR, Skyworks, RAN4#96-e 2020-08

[3] R4-2002812 WF on Intra-band Non-contiguous ULCA TX Architectures and Issues, QC, RAN4#94e 2020-02