3GPP TSG-RAN WG4 Meeting #112	R4-2412351
Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	R19 MPR reduction for single carrier

Source:	OPPO

Agenda item:	8.1.1.2.1

Document for:	Approval

- Introduction

Power enhancements for single carrier has been discussed in last meeting, and some scenarios were brought out, WF [1] is as below. This paper studies the converting outer/edge RB allocation to inner allocation approach.

Mechanisms for enabling MPR reduction and/or power boosting Way forward: - Study the approach to convert full RB allocation in UE CBW to “inner RB allocation” with an extended UE BW, e.g. each side of the UE BW is equal to ½ of the UE BW, inside a larger BS CBW as starting point with consideration of the following aspects - The edge of the extended UE BW, i.e. the UE CBW plus the shifted frequency symmetrically at each side of the UE CBW, should be inside BS CBW or at least aligned with the BS CBW edge - ① FFS feasibility of case where extended UE CBW edge exceeds the BS CBW edge, i.e. gap between edges of UE CBW and BS CBW < 1/2 UE CBW - ② FFS whether IBE is used between edges of UE CBW and BS CBW or between edges of UE CBW and extended UE CBW - ③ FFS ACLR and SEM are applicable from the edge of extended UE CBW or edge of BS CBW, i.e. the start point of ΔfOOB - ④ FFS integral region and boundary of OOBE is based on extended UE CBW or UE CBW

- Discussion

2.1 MPR improvement by extended UE CBW

The basic idea of power boosting in Rel-19 for single CC is to exchange higher power with looser requirements. And WF has pointed out one way is to change the SEM/ACLR requirements for a UE CBW to IBE considering this UE CBW is extended to a larger CBW, as shown in figure 1.

Figure 1 power boosting by changing ACLR/SEM to IBE

To better understood how this will impact the MPR, below some simulations have done with UE 20MHz CBW, and extended CBWs.

    - UE CBW extend from 20MHz to 40MHz

Figure 2 gives the original 20MHz CBW (51RBs) MPR map with QPSK modulation.

And figure 3 gives the MPR when these 51RBs are within a 40MHz extended UE CBW and apply IBE between the 51RBs and 40MHz CBW boundary. It can be seen that the MPRs in this case are almost 0dB, i.e. the 51RBs are in the inner region of 40CBW.

And the MPR improvement with this extended UE CBW and requirement change can be found in figure 4.

Figure 2 MPR of 20MHz CBW QPSK (original)            Figure 3  MPR of 51RBs in 40MHz extended UE CBW

Figure 4 MPR improvement from 20MHz UE CBW to 40MHz extended UE CBW

It can be seen that with the extended UE CBW the MPR can be improved up to 2.5dB in the edge and outer region.

    - UE CBW extend from 20MHz to 30MHz

If we reduce the extended UE CBW from 40MHz to 30MHz, the MPR can be found in figure 5, and also the MPR improvement in figure 6.

Figure 5 MPR of 30MHz extended UE CBW           Figure 6 MPR improvement of 30MHz extended UE CBW

It can be seen that with the extended UE CBW of 30MHz the MPR can still be improved up to 2dB in the edge and outer region.

    - UE CBW extend from 20MHz to 25MHz

Then if the extended UE CBW is 25MHz which is similar as the original 20MHz UE CBW, then the MPR and its improvement can be found in figure 7 and figure 8.

Figure 7 MPR of 25MHz extended UE CBW           Figure 8 MPR improvement of 25MHz extended UE CBW

It can be seen that with the extended UE CBW of 25MHz the MPR improvement is not that attractive and only small part of regions can get improvement.

From above comparisons, it can be seen that MPR can be improved via extended UE CBW and converting ACLR/SEM requirements to IBE requirements. And larger extended UE CBW will have more MPR improvement.

Observation 1:   MPR can be improved via extended UE CBW and converting ACLR/SEM requirements to IBE requirements. And larger extended UE CBW will have more MPR improvement. When the extended UE CBW is large enough the MPR can be reduced to 0dB.

And it seems 2 times of the UE CBW is bit conservative to use this extended UE CBW approach to improve MPR. And 1.5 times of the UE CBW can be considered as the extended UE CBW starting point.

Proposal 1:         Consider extended UE CBW approach to improve MPR starting from 1.5*CBW.

2.2 System impacts by extended UE CBW

From above it can be seen that with this extended UE CBW approach, the MPR can be reduced to almost 0dB. However, one aspect that may need to be think more is how the system performance will be impacted by this kind of emission requirement relaxation and some system level simulations might be needed.

Proposal 2:         Study the system performance impacts by this extended UE CBW approach, i.e. impacts due to applying IBE instead of ACLR/SEM in the out of band emission regions.

- Conclusion

This paper analyzed the power enhancements by emission relaxation in the scenario of single operator in a band or adjacent operators in a band.

Observation 1:   MPR can be improved via extended UE CBW and converting ACLR/SEM requirements to IBE requirements. And larger extended UE CBW will have more MPR improvement. When the extended UE CBW is large enough the MPR can be reduced to 0dB.

Proposal 1:         Consider extended UE CBW approach to improve MPR starting from 1.5*CBW.

Proposal 2:         Study the system performance impacts by this extended UE CBW approach, i.e. impacts due to applying IBE instead of ACLR/SEM in the out of band emission regions.

- Reference

[1] R4-2410749 WF on power domain enhancement, Huawei, RAN4#111 2024-05